Copyright 2010 AIA
MasterSpec Professional
03/10

Copyright 2010 by The American Institute of Architects (AIA)

Exclusively published and distributed by Architectural Computer Services, Inc. (ARCOM) for the AIA

SECTION 224216.13 - COMMERCIAL LAVATORIES
TIPS:
To view non-printing Editor's Notes that provide guidance for editing, click on Masterworks/Single‑File Formatting/Toggle/Editor's Notes.
To read detailed research, technical information about products and materials, and coordination checklists, click on Masterworks/Supporting Information.

Revise this Section by deleting and inserting text to meet Project-specific requirements.

Verify that Section titles referenced in this Section are correct for this Project's Specifications; Section titles may have changed.

PART 1 -  GENERAL

1.1 SUMMARY

A. Section Includes:

1. Lavatories.

2. Faucets.

1.2 ACTION SUBMITTALS

A. Product Data: For each type of product.

B. LEED Submittals:

Retain "Product Data for Prerequisite WE 1 ( and Credit WE 3) (, Credit WE 2, and Credit WE 3)" Subparagraph below for LEED-NC, LEED-CS, and LEED for Schools for fixtures and equipment that consume water.

1. Product Data for Prerequisite WE 1[ and Credit WE 3] [, Credit WE 2, and Credit WE 3]: Documentation indicating flow and water consumption requirements.

Retain "Product Data for Prerequisite WE 1" Subparagraph below for LEED-CI for fixtures and equipment that consume water.

2. Product Data for Prerequisite WE 1: Documentation indicating flow and water consumption requirements.

Retain "Product Data for Prerequisite WE 1( and Credit WE 2)" Subparagraph below for LEED-EB for fixtures and equipment that consume water.

3. Product Data for Prerequisite WE 1[ and Credit WE 2]: Documentation indicating flow and water consumption requirements.

Retain "Shop Drawings" Paragraph below if automatic, electronic-sensor faucets are required for Project.

C. Shop Drawings: Include diagrams for power, signal, and control wiring of automatic faucets.

1.3 INFORMATIONAL SUBMITTALS

Retain "Coordination Drawings" Paragraph below if coordination is required for installation of products and materials by separate installers.

A. Coordination Drawings: Counter cutout templates for mounting of counter-mounted lavatories.

1.4 CLOSEOUT SUBMITTALS

A. Operation and Maintenance Data: For lavatories and faucets to include in operation and maintenance manuals.

Retain first subparagraph below if automatic, electronic-sensor faucets are required for Project.

1. In addition to items specified in Section 017823 "Operation and Maintenance Data," include the following:

a. Servicing and adjustments of automatic faucets.

PART 2 -  PRODUCTS

See Editing Instruction No. 1 in the Evaluations for cautions about named manufacturers and products. For an explanation of options and Contractor's product selection procedures, see Section 016000 "Product Requirements."

Do not use same lavatory designation as is used for another lavatory in Section 224100 "Residential Plumbing Fixtures," Section 224300 "Medical Plumbing Fixtures," and Section 224600 "Security Plumbing Fixtures."

2.1 VITREOUS-CHINA, COUNTER-MOUNTED LAVATORIES

Copy "Lavatory" Paragraph below and re-edit for each type of oval and round self-rimming, vitreous-china, counter-mounted lavatory required.

Insert number to complete drawing designation. Use these designations on Drawings to identify each lavatory.

A. Lavatory <Insert drawing designation>: [Oval] [Round], self rimming, vitreous china, counter mounted.

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

1. Manufacturers: Subject to compliance with requirements, [provide products by the following] [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to identify a specific product or a comparable product from manufacturers listed. Retain option and delete insert note if manufacturer's name and model number are indicated on Drawings.

2. Basis-of-Design Product: Subject to compliance with requirements, provide [product indicated on Drawings] <Insert manufacturer's name; product name or designation> or comparable product by one of the following:

a. American Standard America.

b. Briggs Plumbing Products, Inc.

c. Capizzi.

d. Crane Plumbing, L.L.C.

e. Ferguson Enterprises, Inc.; ProFlo Brand.

f. Gerber Plumbing Fixtures LLC.

g. Kohler Co.

h. Mansfield Plumbing Products LLC.

i. Peerless Pottery Sales, Inc.

j. TOTO USA, INC.

k. Zurn Industries, LLC; Commercial Brass and Fixtures.

l. <Insert manufacturer's name>.

3. Fixture:

a. Standard: ASME A112.19.2/CSA B45.1.

b. Type: Self-rimming for above-counter mounting.

Retain one of two "Nominal Size" subparagraphs below. Retain first for oval lavatories and second for round lavatories.

c. Nominal Size: Oval, [19 by 17 inches (483 by 432 mm)] [20 by 17 inches (508 by 432 mm)] <Insert dimensions>.

d. Nominal Size: Round, [19 inches (483 mm)] <Insert dimension>.

e. Faucet-Hole Punching: [One hole] [Three holes, 2-inch (51-mm) centers] [Three holes, 4-inch (102-mm) centers].

f. Faucet-Hole Location: Top.

g. Color: [White] <Insert color>.

h. Mounting Material: Sealant.

4. Faucet: <Insert lavatory faucet designation from "Solid-Brass, Manually Operated Faucets" Article>.

Copy "Lavatory" Paragraph below and re-edit for each type of oval, vitreous-china, undercounter-mounted lavatory required.

Insert number to complete drawing designation. Use these designations on Drawings to identify each lavatory.

B. Lavatory <Insert drawing designation>: Oval, vitreous china, undercounter mounted.

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

1. Manufacturers: Subject to compliance with requirements, [provide products by the following] [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to identify a specific product or a comparable product from manufacturers listed. Retain option and delete insert note if manufacturer's name and model number are indicated on Drawings.

2. Basis-of-Design Product: Subject to compliance with requirements, provide [product indicated on Drawings] <Insert manufacturer's name; product name or designation> or comparable product by one of the following:

a. American Standard America.

b. Capizzi.

c. Crane Plumbing, L.L.C.

d. Ferguson Enterprises, Inc.; ProFlo Brand.

e. Gerber Plumbing Fixtures LLC.

f. Kohler Co.

g. Mansfield Plumbing Products LLC.

h. Peerless Pottery Sales, Inc.

i. TOTO USA, INC.

j. Zurn Industries, LLC; Commercial Brass and Fixtures.

k. <Insert manufacturer's name>.

3. Fixture:

a. Standard: ASME A112.19.2/CSA B45.1.

b. Type: For undercounter mounting.

c. Nominal Size: Oval, [19 by 16 inches (483 by 406 mm)] [22 by 14 inches (559 by 356 mm)] [23 by 15 inches (584 by 381 mm)] <Insert dimensions>.

d. Faucet-Hole Punching: No holes.

e. Faucet-Hole Location: On countertop.

f. Color: [White] <Insert color>.

g. Mounting Material: Sealant and undercounter mounting kit.

4. Faucet: <Insert lavatory faucet designation from "Solid-Brass, Manually Operated Faucets" Article>.

2.2 VITREOUS-CHINA, WALL-MOUNTED LAVATORIES

Copy "Lavatory" Paragraph below and re-edit for each type of vitreous-china, wall-mounted lavatory with back required.

Insert number to complete drawing designation. Use these designations on Drawings to identify each lavatory.

A. Lavatory <Insert drawing designation>: Vitreous china, wall mounted, with back.

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

1. Manufacturers: Subject to compliance with requirements, [provide products by the following] [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to identify a specific product or a comparable product from manufacturers listed. Retain option and delete insert note if manufacturer's name and model number are indicated on Drawings.

2. Basis-of-Design Product: Subject to compliance with requirements, provide [product indicated on Drawings] <Insert manufacturer's name; product name or designation> or comparable product by one of the following:

a. American Standard America.

b. Briggs Plumbing Products, Inc.

c. Crane Plumbing, L.L.C.

d. Ferguson Enterprises, Inc.; ProFlo Brand.

e. Gerber Plumbing Fixtures LLC.

f. Kohler Co.

g. Mansfield Plumbing Products LLC.

h. Peerless Pottery Sales, Inc.

i. Zurn Industries, LLC; Commercial Brass and Fixtures.

j. <Insert manufacturer's name>.

3. Fixture:

a. Standard: ASME A112.19.2/CSA B45.1.

b. Type: For wall hanging.

c. Nominal Size: Oval, [19 by 16 inches (483 by 406 mm)] [22 by 14 inches (559 by 356 mm)] [23 by 15 inches (584 by 381 mm)] <Insert dimensions>.

d. Faucet-Hole Punching: [One hole] [Three holes, 2-inch (51-mm) centers] [Three holes, 4-inch (102-mm) centers].

e. Faucet-Hole Location: Top.

f. Color: [White] <Insert color>.

g. Mounting Material: Chair carrier.

4. Faucet: <Insert lavatory faucet designation from "Solid-Brass, Manually Operated Faucets" Article>.

5. Support: ASME A112.6.1M, [Type I, exposed-arm lavatory carrier] [Type II, concealed-arm lavatory carrier] [Type II, concealed-arm lavatory carrier with escutcheons].[ Include rectangular, steel uprights.]

Copy "Lavatory" Paragraph below and re-edit for each type of ledge-back, vitreous-china, wall-mounted lavatory required.

Insert number to complete drawing designation. Use these designations on Drawings to identify each lavatory.

B. Lavatory <Insert drawing designation>: Ledge back, vitreous china, wall mounted.

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

1. Manufacturers: Subject to compliance with requirements, [provide products by the following] [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to identify a specific product or a comparable product from manufacturers listed. Retain option and delete insert note if manufacturer's name and model number are indicated on Drawings.

2. Basis-of-Design Product: Subject to compliance with requirements, provide [product indicated on Drawings] <Insert manufacturer's name; product name or designation> or comparable product by one of the following:

a. American Standard America.

b. Briggs Plumbing Products, Inc.

c. Crane Plumbing, L.L.C.

d. Ferguson Enterprises, Inc.; ProFlo Brand.

e. Gerber Plumbing Fixtures LLC.

f. Kohler Co.

g. Mansfield Plumbing Products LLC.

h. Peerless Pottery Sales, Inc.

i. <Insert manufacturer's name>.

3. Fixture:

a. Standard: ASME A112.19.2/CSA B45.1.

b. Type: For wall hanging.

c. Nominal Size: Oval, [19 by 16 inches (483 by 406 mm)] [22 by 14 inches (559 by 356 mm)] [23 by 15 inches (584 by 381 mm)] <Insert dimensions>.

d. Faucet-Hole Punching: [One hole] [Three holes, 2-inch (51-mm) centers] [Three holes, 4-inch (102-mm) centers].

e. Faucet-Hole Location: Top.

f. Color: [White] <Insert color>.

g. Mounting Material: Chair carrier.

4. Faucet: <Insert lavatory faucet designation from "Solid-Brass, Manually Operated Faucets" Article>.

5. Support: ASME A112.6.1M, [Type I, exposed-arm lavatory carrier] [Type II, concealed-arm lavatory carrier] [Type II, concealed-arm lavatory carrier with escutcheons].[ Include rectangular, steel uprights.]

Copy "Lavatory" Paragraph below and re-edit for each type of wheelchair, vitreous-china, wall-mounted lavatory required.

Insert number to complete drawing designation. Use these designations on Drawings to identify each lavatory.

C. Lavatory <Insert drawing designation>: Wheelchair, vitreous china, wall mounted.

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

1. Manufacturers: Subject to compliance with requirements, [provide products by the following] [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to identify a specific product or a comparable product from manufacturers listed. Retain option and delete insert note if manufacturer's name and model number are indicated on Drawings.

2. Basis-of-Design Product: Subject to compliance with requirements, provide [product indicated on Drawings] <Insert manufacturer's name; product name or designation> or comparable product by one of the following:

a. American Standard America.

b. Crane Plumbing, L.L.C.

c. Ferguson Enterprises, Inc.; ProFlo Brand.

d. Gerber Plumbing Fixtures LLC.

e. Kohler Co.

f. Mansfield Plumbing Products LLC.

g. Peerless Pottery Sales, Inc.

h. <Insert manufacturer's name>.

3. Fixture:

a. Standard: ASME A112.19.2/CSA B45.1.

b. Type: Slab or wheelchair.

c. Nominal Size: Rectangular, 27 by 20 inches (686 by 508 mm).

d. Faucet-Hole Punching: Three holes, [2-inch (51-mm)] <Insert dimension> centers.

e. Faucet-Hole Location: Top.

f. Color: [White] <Insert color>.

g. Mounting: For concealed-arm carrier.

4. Faucet: <Insert lavatory faucet designation from "Solid-Brass, Manually Operated Faucets" Article>.

5. Support: ASME A112.6.1M, Type II, concealed-arm lavatory carrier with rectangular, steel uprights.

2.3 SOLID-BRASS, MANUALLY OPERATED FAUCETS

A. NSF Standard: Comply with NSF/ANSI 61, "Drinking Water System Components - Health Effects," for faucet materials that will be in contact with potable water.

Copy "Lavatory Faucets" Paragraph below and re-edit for each type of manual-type solid-brass valve required.

Insert number to complete drawing designation. Use these designations in lavatory description to identify required faucet for that lavatory.

B. Lavatory Faucets <Insert designation>: Manual-type, [single-control mixing] [single-control nonmixing] [two-handle mixing], [commercial] [general-duty], solid-brass valve.

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

1. Manufacturers: Subject to compliance with requirements, [provide products by the following] [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to identify a specific product or a comparable product from manufacturers listed. Retain option and delete insert note if manufacturer's name and model number are indicated on Drawings.

2. Basis-of-Design Product: Subject to compliance with requirements, provide [product indicated on Drawings] <Insert manufacturer's name; product name or designation> or comparable product by one of the following:

a. American Standard America.

b. Bradley Corporation.

c. Chicago Faucets.

d. Delta Faucet Company.

e. Elkay Manufacturing Co.

f. Grohe America, Inc.

g. Just Manufacturing.

h. Kohler Co.

i. Moen Incorporated.

j. Speakman Company.

k. T & S Brass and Bronze Works, Inc.

l. Zurn Industries, LLC; Commercial Brass and Fixtures.

m. <Insert manufacturer's name>.

3. Standard: ASME A112.18.1/CSA B125.1.

4. General: Include hot- and cold-water indicators; coordinate faucet inlets with supplies and fixture hole punchings; coordinate outlet with spout and fixture receptor.

Body type in "Body Type" Subparagraph below must match fixture hole punching.

5. Body Type: [Centerset] [Widespread] [Single hole] <Insert type>.

6. Body Material: Commercial, solid brass.

7. Finish: [Polished chrome plate] <Insert finish>.

Retain "Maximum Flow Rate" or "Maximum Flow" Subparagraph below.

8. Maximum Flow Rate: [0.5 gpm (1.5 L/min.)] <Insert value>.

9. Maximum Flow: [0.25 gal. (0.95 L)] <Insert value> per metering cycle.

10. Mounting Type: [Deck, exposed] [Deck, concealed] [Back/wall, exposed] [Back/wall, concealed].

11. Valve Handle(s): [Single lever] [Knob] [Knob, nonmetallic] [Cross, three arm] [Cross, four arm] [Wrist blade, 4 inches (102 mm)] [Elbow, 6 inches (152 mm)] [Push button] <Insert type>.

12. Spout: [Rigid] [Swing] [Rigid, gooseneck] [Swivel, gooseneck] type.

13. Spout Outlet: [Aerator] [Laminar flow] [Spray] <Insert type>.

14. Operation: [Compression, manual] [Noncompression, manual].

15. Drain: [Not part of faucet] <Insert type>.

2.4 SUPPLY FITTINGS

A. NSF Standard: Comply with NSF/ANSI 61, "Drinking Water System Components - Health Effects," for supply-fitting materials that will be in contact with potable water.

B. Standard: ASME A112.18.1/CSA B125.1.

C. Supply Piping: Chrome-plated-brass pipe or chrome-plated copper tube matching water-supply piping size. Include chrome-plated-brass or stainless-steel wall flange.

D. Supply Stops: Chrome-plated-brass, one-quarter-turn, ball-type or compression valve with inlet connection matching supply piping.

Retain "Wheel handle" option in "Operation" Paragraph below for one-quarter-turn, ball-type stops.

E. Operation: [Loose key] [Wheel handle] <Insert type>.

F. Risers:

1. [NPS 3/8 (DN 10)] [NPS 1/2 (DN 15)].

2. [Chrome-plated, soft-copper flexible tube] [ASME A112.18.6, braided- or corrugated-stainless-steel, flexible hose] riser.

2.5 WASTE FITTINGS

A. Standard: ASME A112.18.2/CSA B125.2.

B. Drain: Grid type with NPS 1-1/4 (DN 32) offset and straight tailpiece.

C. Trap:

1. Size: [NPS 1-1/2 by NPS 1-1/4 (DN 40 by DN 32)] [NPS 1-1/4 (DN 32)].

Retain one of two "Material" subparagraphs below.

2. Material: Chrome-plated, [two-piece, cast-brass trap and swivel elbow with 0.032-inch- (0.83-mm-) thick brass tube to wall] [two-piece, cast-brass trap and ground-joint swivel elbow with 0.032-inch- (0.83-mm-) thick brass tube to wall] [one-piece, cast-brass trap with swivel 0.029-inch- (73-mm-) thick tubular brass wall bend] <Insert trap type>; and chrome-plated, brass or steel wall flange.

3. Material: Stainless-steel, two-piece trap and swivel elbow with 0.012-inch- (0.30-mm-) thick stainless-steel tube to wall; and stainless-steel wall flange.

PART 3 -  EXECUTION

3.1 EXAMINATION

A. Examine roughing-in of water supply and sanitary drainage and vent piping systems to verify actual locations of piping connections before lavatory installation.

B. Examine counters and walls for suitable conditions where lavatories will be installed.

C. Proceed with installation only after unsatisfactory conditions have been corrected.

3.2 INSTALLATION

A. Install lavatories level and plumb according to roughing-in drawings.

B. Install supports, affixed to building substrate, for wall-mounted lavatories.

Indicate on Drawing, those lavatories that are required to be accessible.

C. Install accessible wall-mounted lavatories at handicapped/elderly mounting height for people with disabilities or the elderly, according to ICC/ANSI A117.1.

D. Install wall flanges or escutcheons at piping wall penetrations in exposed, finished locations. Use deep-pattern escutcheons if required to conceal protruding fittings. Comply with escutcheon requirements specified in Section 220518 "Escutcheons for Plumbing Piping."

E. Seal joints between lavatories and counters and walls using sanitary-type, one-part, mildew-resistant silicone sealant. Match sealant color to fixture color. Comply with sealant requirements specified in Section 079200 "Joint Sealants."

Indicate on Drawings those lavatories that are required to be accessible.

F. Install protective shielding pipe covers and enclosures on exposed supplies and waste piping of accessible lavatories. Comply with requirements in Section 220719 "Plumbing Piping Insulation."

3.3 CONNECTIONS

Coordinate piping installations and specialty arrangements with schematics on Drawings and with requirements specified in piping systems. If Drawings are explicit enough, these requirements may be reduced or omitted.

A. Connect fixtures with water supplies, stops, and risers, and with traps, soil, waste, and vent piping. Use size fittings required to match fixtures.

B. Comply with water piping requirements specified in Section 221116 "Domestic Water Piping."

C. Comply with soil and waste piping requirements specified in Section 221316 "Sanitary Waste and Vent Piping."

3.4 ADJUSTING

A. Operate and adjust lavatories and controls. Replace damaged and malfunctioning lavatories, fittings, and controls.

B. Adjust water pressure at faucets to produce proper flow.

C. Install fresh batteries in battery-powered, electronic-sensor mechanisms.

3.5 CLEANING AND PROTECTION

A. After completing installation of lavatories, inspect and repair damaged finishes.

B. Clean lavatories, faucets, and other fittings with manufacturers' recommended cleaning methods and materials.

C. Provide protective covering for installed lavatories and fittings.

D. Do not allow use of lavatories for temporary facilities unless approved in writing by Owner.

END OF SECTION 224216.13

	COMMERCIAL LAVATORIES
	224216.13 - 1



