Copyright 2011 AIA
MasterSpec Professional
03/11

Copyright 2011 by The American Institute of Architects (AIA)

Exclusively published and distributed by Architectural Computer Services, Inc. (ARCOM) for the AIA

SECTION 224713 - DRINKING FOUNTAINS
TIPS:
To view non-printing Editor's Notes that provide guidance for editing, click on Masterworks/Single‑File Formatting/Toggle/Editor's Notes.
To read detailed research, technical information about products and materials, and coordination checklists, click on Masterworks/Supporting Information.

Revise this Section by deleting and inserting text to meet Project-specific requirements.

Verify that Section titles referenced in this Section are correct for this Project's Specifications; Section titles may have changed.

PART 1 -  GENERAL

1.1 SUMMARY

A. Section includes drinking fountains and related components.

1.2 ACTION SUBMITTALS

A. Product Data: For each type of drinking fountains.

B. LEED Submittals:

Retain "Product Data for Prerequisite WE 1( and Credit WE 3) (, Credit WE 2, and Credit WE 3)" Subparagraph below for LEED-NC, LEED-CS, and LEED for Schools for fixtures and equipment that consume water.

1. Product Data for Prerequisite WE 1[ and Credit WE 3] [, Credit WE 2, and Credit WE 3]: Documentation indicating flow and water consumption requirements.

1.3 CLOSEOUT SUBMITTALS

A. Maintenance Data: For drinking fountains to include in maintenance manuals.

PART 2 -  PRODUCTS

See Editing Instruction No. 1 in the Evaluations for cautions about named manufacturers and products. For an explanation of options and Contractor's product selection procedures, see SECTION 016000 "Product Requirements."

2.1 DRINKING FOUNTAINS

Copy "Drinking Fountains" Paragraph below and re-edit for each type of drinking fountain required.

Insert number to complete drawing designation. Use these designations on Drawings to identify each drinking fountain.

A. Drinking Fountains <Insert drawing designation>: [Concrete] [Painted cast iron or steel], [pedestal] [wheelchair accessible].

1. Concrete Drinking Fountains:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

a. Manufacturers: Subject to compliance with requirements, [provide products by the following] [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to identify a specific product or a comparable product from manufacturers listed. Retain option and delete insert note if manufacturer's name and model number are indicated on Drawings.

b. Basis-of-Design Product: Subject to compliance with requirements, provide [product indicated on Drawings] <Insert manufacturer's name; product name or designation> or comparable product by one of the following:

1) Belson Outdoors, Inc.

2) Haws Corporation.

3) Petersen Manufacturing Co., Inc.

4) Sanderson Concrete Inc.

5) Stern-Williams Co., Inc.

6) <Insert manufacturer's name>.

2. Cast-Iron or Steel Drinking Fountains:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

a. Manufacturers: Subject to compliance with requirements, [provide products by the following] [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to identify a specific product or a comparable product from manufacturers listed. Retain option and delete insert note if manufacturer's name and model number are indicated on Drawings.

b. Basis-of-Design Product: Subject to compliance with requirements, provide [product indicated on Drawings] <Insert manufacturer's name; product name or designation> or comparable product by one of the following:

1) Belson Outdoors, Inc.

2) Halsey Taylor.

3) Haws Corporation.

4) Most Dependable Fountains, Inc.

5) Murdock-Super Secur; a division of Acorn Engineering Company.

6) Stern-Williams Co., Inc.

7) Tri Palm International, LLC.

8) <Insert manufacturer's name>.

Retain "Standards" Subparagraph below for wheelchair-accessible drinking fountains.

3. Standards: Comply with ICC A117.1 and NSF 61.

Retain "with offset to receptor" option in "Pedestal" Subparagraph below for wheelchair-accessible drinking fountains.

4. Pedestal: [Rectangular] [Round] <Insert shape>, [with offset to receptor] [with side receptor(s)].

5. Receptor(s):

a. Number: [One] [Two] [Three].

b. Material: [Bronze] [Chrome-plated brass or stainless steel] <Insert material>.

c. Shape: [Rectangular] [Round] [Rounded front] <Insert shape>.

d. Bubbler: One for each receptor, with adjustable stream regulator.

e. Drain: Grid type with NPS 1-1/4 (DN 32) tailpiece.

6. Controls: [Foot pedal] [Push bar] [Push button] <Insert control>.

7. Access to Internal Components: Panel in pedestal.

8. Supply Piping: [NPS 3/8 (DN 10)] [NPS 1/2 (DN 15)] with shutoff valve.

9. Drain Piping: [NPS 1-1/4 (DN 32)] [NPS 1-1/2 (DN 40)] minimum trap and waste.

Copy "Drinking Fountains" Paragraph below and re-edit for each type of wall-mounted drinking fountain required.

Insert number to complete drawing designation. Use these designations on Drawings to identify each drinking fountain.

B. Drinking Fountains <Insert drawing designation>: [Bronze] [Stainless steel], wall mounted.

1. Bronze Drinking Fountains:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

a. Manufacturers: Subject to compliance with requirements, [provide products by the following] [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to identify a specific product or a comparable product from manufacturers listed. Retain option and delete insert note if manufacturer's name and model number are indicated on Drawings.

b. Basis-of-Design Product: Subject to compliance with requirements, provide [product indicated on Drawings] <Insert manufacturer's name; product name or designation> or comparable product by one of the following:

1) Elkay Manufacturing Co.

2) Filtrine Manufacturing Company.

3) Halsey Taylor.

4) Haws Corporation.

5) <Insert manufacturer's name>.

2. Stainless-Steel Drinking Fountains:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

a. Manufacturers: Subject to compliance with requirements, [provide products by the following] [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to identify a specific product or a comparable product from manufacturers listed. Retain option and delete insert note if manufacturer's name and model number are indicated on Drawings.

b. Basis-of-Design Product: Subject to compliance with requirements, provide [product indicated on Drawings] <Insert manufacturer's name; product name or designation> or comparable product by one of the following:

1) Elkay Manufacturing Co.

2) Filtrine Manufacturing Company.

3) Halsey Taylor.

4) Haws Corporation.

5) Murdock-Super Secur; a division of Acorn Engineering Company.

6) Stern-Williams Co., Inc.

7) Tri Palm International, LLC.

8) <Insert manufacturer's name>.

3. Standards:

Retain first subparagraph below for stainless-steel drinking fountains.

a. Comply with ASME A112.19.3/CSA B45.4.

b. Comply with NSF 61.

4. Type Receptor: [Slab] [With back] [On horizontal support].

5. Receptor Shape: [Rectangular] [Round] <Insert shape>.

Retain "Back Panel" Subparagraph below only for slab-type and on-horizontal-support-type, brass or stainless-steel drinking fountains and only if required. Include size if necessary.

6. Back Panel: [Stainless-steel] <Insert material> wall plate behind drinking fountain.

7. Bubblers: [One] [Two] [Three], with adjustable stream regulator, located on deck.

8. Control: [Push button] [Push bar] <Insert control>.

9. Drain: Grid type with NPS 1-1/4 (DN 32) tailpiece.

10. Supply Piping: NPS 3/8 (DN 10) with shutoff valve.

11. Drain Piping: ASME A112.18.2/CSA B125.2, NPS 1-1/4 (DN 32) chrome-plated brass P-trap and waste.

12. Support: ASME A112.6.1M, Type III lavatory carrier.

PART 3 -  EXECUTION

3.1 EXAMINATION

A. Examine roughing-in for water-supply and sanitary drainage and vent piping systems to verify actual locations of piping connections before fixture installation.

B. Examine walls and floors for suitable conditions where fixtures will be installed.

C. Proceed with installation only after unsatisfactory conditions have been corrected.

3.2 INSTALLATION

A. Install fixtures level and plumb according to roughing-in drawings. For fixtures indicated for children, install at height required by authorities having jurisdiction.

B. Set pedestal drinking fountains on floor.

C. Install off-the-floor carrier supports, affixed to building substrate, for wall-mounted fixtures.

D. Install water-supply piping with shutoff valve on supply to each fixture to be connected to domestic-water distribution piping. Use ball, gate, or globe valve. Install valves in locations where they can be easily reached for operation. Valves are specified in Section 220523 "General-Duty Valves for Plumbing Piping."

E. Install trap and waste piping on drain outlet of each fixture to be connected to sanitary drainage system.

F. Install wall flanges or escutcheons at piping wall penetrations in exposed, finished locations. Use deep-pattern escutcheons where required to conceal protruding fittings. Comply with escutcheon requirements specified in Section 220518 "Escutcheons for Plumbing Piping."

G. Seal joints between fixtures and walls using sanitary-type, one-part, mildew-resistant, silicone sealant. Match sealant color to fixture color. Comply with sealant requirements specified in Section 079200 "Joint Sealants."

H. Adjust fixture flow regulators for proper flow and stream height.

3.3 CONNECTIONS

Coordinate piping installations and specialty arrangements with Drawings and with requirements specified in piping systems. If Drawings are explicit enough, these requirements may be reduced or omitted.

A. Connect fixtures with water supplies, stops, and risers, and with traps, soil, waste, and vent piping. Use size fittings required to match fixtures.

B. Comply with water piping requirements specified in Section 221116 "Domestic Water Piping."

C. Install ball, gate, or globe shutoff valve on water supply to each fixture. Comply with valve requirements specified in Section 220523 "General-Duty Valves for Plumbing Piping."

D. Comply with soil and waste piping requirements specified in Section 221316 "Sanitary Waste and Vent Piping."

3.4 CLEANING

A. After installation, inspect unit. Remove paint splatters and other spots, dirt, and debris. Repair damaged finish to match original finish.

B. Clean fixtures, on completion of installation, according to manufacturer's written instructions.

C. Provide protective covering for installed fixtures.

D. Do not allow use of fixtures for temporary facilities unless approved in writing by Owner.

END OF SECTION 224713

	DRINKING FOUNTAINS
	224713 - 1



