Louisiana Army National Guard

Preliminary Review Documents

141st Readiness Center (RC)

Jackson Barracks, New Orleans, Louisiana

Technical Specifications


Copyright 2003 and 2005 by The American Institute of Architects (AIA)

Exclusively published and distributed by Architectural Computer Services, Inc. (ARCOM) for the AIA

SECTION 09681 – CARPET TILE
This Section uses the term "Architect."  Change this term to match that used to identify the design professional as defined in the General and Supplementary Conditions.

Verify that Section titles referenced in this Section are correct for this Project's Specifications; Section titles may have changed.

PART 1 -  GENERAL

1.1 SUMMARY

A. This Section includes modular, tufted carpet tile.

1.2 SUBMITTALS

A. Product Data:  For each product indicated.

B. Shop Drawings:  Show the following:

1. Carpet tile type, color, and dye lot.

2. Pattern of installation.

3. Insets and borders.

4. Edge, transition, and other accessory strips.

5. Transition details to other flooring materials.

C. Samples:  For each color and texture required.

1. Carpet Tile:  Full-size Sample.

2. Exposed Edge, Transition, and other Accessory Stripping:  12-inch- (300-mm-) long Samples.

D. SPiRiT Submittals:

Retain subparagraph below if low-emitting materials are required for LEED Credit EQ 4.3; coordinate with requirements selected in Part 2 for carpet tile and adhesives.

1. Credit IEQ 5.C4:  Manufacturers' product data for  carpet tile and installation adhesive, including printed statement of VOC content.

E. Product Schedule:  For carpet tile.  

F. Maintenance data.

1.3 QUALITY ASSURANCE

A. Installer Qualifications:  An experienced installer who is certified by the Floor Covering Installation Board or who can demonstrate compliance with its certification program requirements.

B. Mockups:  Before installing carpet tile, build mockups to verify selections made under sample submittals and to demonstrate aesthetic effects and set quality standards for materials and execution.

Retain subparagraph below if mockups are erected as part of building rather than separately and the intention is to make an exception to the default requirement in Division 01 Section "Quality Requirements" for demolishing and removing mockups when directed, unless otherwise indicated.

1. Approved mockups may become part of the completed Work if undamaged at time of Substantial Completion.

1.4 DELIVERY, STORAGE, AND HANDLING

A. Comply with CRI 104, Section 5, "Storage and Handling."

1.5 PROJECT CONDITIONS

A. Comply with CRI 104, Section 7.2, "Site Conditions; Temperature and Humidity" and Section 7.12, "Ventilation."

B. Environmental Limitations:  Do not install carpet tiles until wet work in spaces is complete and dry, and ambient temperature and humidity conditions are maintained at the levels indicated for Project when occupied for its intended use.

Delete first paragraph below if products are not adhesively applied to concrete substrates.  Revise to suit Project.

C. Do not install carpet tiles over concrete slabs until slabs have cured and are sufficiently dry to bond with adhesive and concrete slabs have pH range recommended by carpet tile manufacturer.

D. Where demountable partitions or other items are indicated for installation on top of carpet tiles, install carpet tiles before installing these items.

1.6 WARRANTY

A. Special Warranty for Carpet Tiles:  Manufacturer's standard form in which manufacturer agrees to repair or replace components of carpet tile installation that fail in materials or workmanship within specified warranty period.  Failures include, but are not limited to, more than 10 percent loss of face fiber, edge raveling, snags, runs, loss of tuft bind strength, dimensional stability, excess static discharge, and delamination.

Verify available warranties and insert number in subparagraph below.

1. Warranty Period:  15 years from date of Substantial Completion.

1.7 EXTRA MATERIALS

Extra materials may not be allowed for publicly funded projects.

A. Furnish extra materials described below, before installation begins, that match products installed and that are packaged with protective covering for storage and identified with labels describing contents.

1. Carpet Tile:  Full-size units equal to 5 percent of amount installed for each type indicated, but not less than 10 sq. yd. (8.3 sq. m).

PART 2 -  PRODUCTS

Copy Article below and re-edit it for each product required.

2.1 CARPET TILE

Retain one of two paragraphs and list of manufacturers and products below.  See Division 01 Section "Product Requirements."

A. Basis-of-Design Product:  Subject to compliance with requirements, provide Mannington Commercial, Inc. – “Means” modular carpet tile or a comparable product.
Delete remainder of this Article if products are limited to those listed above.

Retain first paragraph below and insert proprietary fiber type if required.

1. Fiber Type:  DSDN® solution dyed nylon.
2. Pile Characteristic:  Patterned loop pile.

3. Average Density:  7,200 oz./cu. yd. (0.267 g/cu. cm.).
4. Weight Density:  144,000 oz./cu. yd. (5.34 g/cu. cm.).
5. Pile Thickness:  0.100 inches (2.54 mm) for finished carpet tile.

6. Stitches:  9.6 stitches per inch (0.38 stiches per mm).
7. Gage:  12 ends per inch (0.47 ends per mm).
8. Surface Pile Weight:  20 oz./sq. yd. (678 g/sq. m).
9. Primary Backing:  100% woven synthetic.
10. Primary Precoat:  100% vinyl non-aqueous closed cell polymer.

11. Secondary Backing: .Infinity™ modular reinforced vinyl composite closed cell polymer with recycled content.
12. Size:  18 by 18 inches (457 by 457 mm).

B. Applied Soil-Resistance Treatment:  Manufacturer's standard material.

C. Antimicrobial Treatment:  Manufacturer's standard material.

D. Performance Characteristics:  As follows:

1. Methenamine Pill Test (ASTM-D-2859):  Passes

2. Flooring Radiant Panel Test (ASTM-E-648):  Class 1 (Direct Glue)

3. N.B.S. Smoke Chamber Test (ASTM-E-662):
Less than 450 (Flamming Mode)

4. Dimensional Stability:  0.2 percent or less per ISO 2551 (Aachen Test).

5. Electrostatic Propensity:  Less than 3.0 kV per AATCC 134.

Retain subparagraph and associated subparagraphs below if low-emitting materials are required for LEED Credit EQ 4.3.  VOC limits below are based on CRI's Green Label Indoor Air Quality Testing Program.

6. VOC Limits:  Provide carpet tile that complies with the following limits for VOC content when tested according to ASTM D 5116:

a. Total VOCs:  0.5 mg/sq. m x h.

b. 4-PC (4-Phenylcyclohexene):  0.05 mg/sq. m x h.

c. Formaldehyde:  0.05 mg/sq. m x h.

d. Styrene:  0.4 mg/sq. m x h.

2.2 INSTALLATION ACCESSORIES

A. Trowelable Leveling and Patching Compounds:  Latex-modified, hydraulic-cement-based formulation provided or recommended by carpet tile manufacturer.

B. Adhesives:  Water-resistant, mildew-resistant, nonstaining, pressure-sensitive type to suit products and subfloor conditions indicated, that complies with flammability requirements for installed carpet tile and is recommended by carpet tile manufacturer for releasable installation.

Retain subparagraph and associated subparagraphs below if low-emitting materials are required for LEED Credit EQ 4.3.  VOC limits below are based on CRI's Green Label Indoor Air Quality Testing Program.

1. VOC Limits:  Provide adhesives that comply with the following limits for VOC content when tested according to ASTM D 5116:

a. Total VOCs:  10.00 mg/sq. m x h.

b. Formaldehyde:  0.05 mg/sq. m x h.

c. 2-Ethyl-1-Hexanol:  3.00 mg/sq. m x h.

PART 3 -  EXECUTION

3.1 INSTALLATION

A. General:  Comply with CRI 104, Section 14, "Carpet Modules," and with carpet tile manufacturer's written installation instructions.

Revise first paragraph below if mill-applied peel-and-stick adhesives or standard adhesives are used.  Revise second option if spot-glued installation is recommended by manufacturer.

B. Installation Method:  As recommended in writing by carpet tile manufacturer.

C. Extend carpet tile into toe spaces, door reveals, closets, open-bottomed obstructions, removable flanges, alcoves, and similar openings.

D. Install pattern parallel to walls and borders.

END OF SECTION 09681


CARPET TILE

09681 - 4

