Copyright 2003 AIA
MASTERSPEC Full Length
11/03 (R 05/06)

Copyright 2003 by The American Institute of Architects (AIA)

Exclusively published and distributed by Architectural Computer Services, Inc. (ARCOM) for the AIA

SECTION 09680 - CARPET
This Section uses the term "Architect."  Change this term to match that used to identify the design professional as defined in the General and Supplementary Conditions.

Verify that Section titles referenced in this Section are correct for this Project's Specifications; Section titles may have changed.

PART 1 -  GENERAL

1.1 RELATED DOCUMENTS

A. Drawings and general provisions of the Contract, including General and Supplementary Conditions and Division 1 Specification Sections, apply to this Section.

1.2 SUMMARY

A. This Section includes the following:

Adjust list below to suit Project.  Insert fusion-bonded, needle-punched, knitted, or another carpet construction if required.

1. Tufted carpet.

2. Woven carpet.

3. <Insert carpet construction.>

Delete subparagraph below if not applicable; e.g., direct-glue-down installations.

4. Carpet cushion.

B. Related Sections include the following:

List below only products that the reader might expect to find in this Section but are specified elsewhere.

1. Division 1 Section "Selective Demolition" for removing existing floor coverings.

2. Division 9 Section "[Resilient Floor Tile] [Resilient Wall Base and Accessories]" for resilient wall base and accessories installed with carpet.

3. Division 9 Section "Carpet Tile."

1.3 SUBMITTALS

A. Product Data:  For the following, including installation recommendations for each type of substrate:

1. Carpet:  For each type indicated.  Include manufacturer's written data on physical characteristics, durability, and fade resistance.

2. Carpet Cushion:  For each type indicated.  Include manufacturer's written data on physical characteristics and durability.

B. Shop Drawings:  Show the following:

Retain, delete, or revise subparagraphs below to suit Project.

1. Columns, doorways, enclosing walls or partitions, built-in cabinets, and locations where cutouts are required in carpet.

2. Existing flooring materials to be removed.

3. Existing flooring materials to remain.

4. Carpet type, color, and dye lot.

Retain first subparagraph below only for large installations where multiple dye lots are unavoidable.

5. Locations where dye lot changes occur.

6. Seam locations, types, and methods.

7. Type of subfloor.

8. Type of installation.

9. Pattern type, repeat size, location, direction, and starting point.

10. Pile direction.

11. Type, color, and location of insets and borders.

12. Type, color, and location of edge, transition, and other accessory strips.

13. Transition details to other flooring materials.

Delete subparagraph below if not applicable.

14. Type of carpet cushion.

C. Samples:  For each of the following products and for each color and texture required.  Label each Sample with manufacturer's name, material description, color, pattern, and designation indicated on Drawings and in schedules.

Delete subparagraphs below if not applicable.

1. Carpet:  12-inch- (300-mm-) square Sample.

2. Exposed Edge, Transition, and other Accessory Stripping:  12-inch- (300-mm-) long Samples.

3. Carpet Cushion:  6-inch- (150-mm-) square Sample.

4. Carpet Seam:  6-inch (150-mm) Sample.

5. Mitered Carpet Border Seam:  12-inch- (300-mm-) square Sample.  Show carpet pattern alignment.

D. LEED Submittals:

Retain subparagraph below if low-emitting materials are required for LEED-NC or LEED-CI Credit EQ 4.3; coordinate with requirements selected in Part 2 for carpet, carpet cushion, and adhesives.

1. Product Data for Credit EQ 4.3:

a. For carpet, documentation indicating compliance with testing and product requirements of Carpet and Rug Institute's "Green Label Plus" program.

b. For carpet cushion, documentation indicating compliance with testing and product requirements of Carpet and Rug Institute's "Green Label" program.

c. For installation adhesive, including printed statement of VOC content.

E. Product Schedule:  For carpet[ and carpet cushion.] Use same designations indicated on Drawings.

Coordinate first paragraph below with qualification requirements in Division 1 Section "Quality Requirements" and as supplemented in "Quality Assurance" Article.

F. Qualification Data:  For Installer.

G. Product Test Reports:  Based on evaluation of comprehensive tests performed by a qualified testing agency.

H. Maintenance Data:  For carpet to include in maintenance manuals.  Include the following:

1. Methods for maintaining carpet, including cleaning and stain-removal products and procedures and manufacturer's recommended maintenance schedule.

2. Precautions for cleaning materials and methods that could be detrimental to carpet[ and carpet cushion].

I. Warranties:  Special warranties specified in this Section.

1.4 QUALITY ASSURANCE

Delete paragraph below if Installer certification is not required; other Installer qualifications are included in Division 1 Section "Quality Requirements."

A. Installer Qualifications:  An experienced installer who is certified by the Floor Covering Installation Board or who can demonstrate compliance with its certification program requirements.

Delete paragraph below if none of the products listed in Part 2 are required to have a critical radiant flux classification.  Revise if authorities having jurisdiction require other fire tests.

B. Fire-Test-Response Characteristics:  Provide products with the critical radiant flux classification indicated in Part 2, as determined by testing identical products per ASTM E 648 by an independent testing and inspecting agency acceptable to authorities having jurisdiction.

Delete paragraph and subparagraph below if not required.  If retaining, indicate location, size, and other details of mockups on Drawings or by inserts.  Revise wording if only one mockup is required.

C. Mockups:  Before installing carpet, build mockups to verify selections made under sample submittals and to demonstrate aesthetic effects and set quality standards for materials and execution.

Retain subparagraph below if mockups are erected as part of building rather than separately and the intention is to make an exception to the default requirement in Division 1 Section "Quality Requirements" for demolishing and removing mockups when directed, unless otherwise indicated.

1. Approved mockups may become part of the completed Work if undisturbed at time of Substantial Completion.

Delete paragraph below if Work of this Section is not extensive or complex enough to justify a preinstallation conference.  If retaining, coordinate with Division 1.

D. Preinstallation Conference:  Conduct conference at Project site to comply with requirements in Division 1 Section "Project Management and Coordination."

Retain paragraph above if requirements in Division 1 are adequate for Project.  Retain paragraph below if additional requirements are necessary; include information about conference.  Identify specific participants not mentioned in Division 1.

E. Preinstallation Conference:  Conduct conference at Project site to comply with requirements in Division 1 Section "Project Management and Coordination." Review methods and procedures related to carpet installation including, but not limited to, the following:

Delete subparagraphs below if not required.  If retaining, revise to include product-specific requirements.  Insert additional requirements to suit Project.  Retain first subparagraph for occupied buildings if delivery must be controlled and coordinated, e.g., for tenant spaces in occupied office buildings with multiple tenants.

1. Review delivery, storage, and handling procedures.

2. Review ambient conditions and ventilation procedures.

1.5 DELIVERY, STORAGE, AND HANDLING

A. Comply with CRI 104, Section 5, "Storage and Handling."

1.6 PROJECT CONDITIONS

A. Comply with CRI 104, Section 7.2, "Site Conditions; Temperature and Humidity" and Section 7.12, "Ventilation."

B. Environmental Limitations:  Do not install carpet[ and carpet cushion] until wet work in spaces is complete and dry, and ambient temperature and humidity conditions are maintained at the levels indicated for Project when occupied for its intended use.

Delete first paragraph below if products are not adhesively applied to concrete substrates.  Revise to suit Project.

C. Do not install carpet[ and carpet cushion] over concrete slabs until slabs have cured, are sufficiently dry to bond with adhesive, and have pH range recommended by carpet manufacturer.

D. Where demountable partitions or other items are indicated for installation on top of carpet, install carpet before installing these items.

1.7 WARRANTY

When warranties are required, verify with Owner's counsel that special warranties stated in this Article are not less than remedies available to Owner under prevailing local laws.  Coordinate with Division 1 Section "Product Requirements."

Revise first paragraph and subparagraphs below if manufacturers have separate warranties for various characteristics.

A. Special Warranty for Carpet:  Manufacturer's standard form in which manufacturer agrees to repair or replace components of carpet installation that fail in materials or workmanship within specified warranty period.

1. Warranty does not include deterioration or failure of carpet due to unusual traffic, failure of substrate, vandalism, or abuse.

2. Failures include, but are not limited to, more than 10 percent loss of face fiber, edge raveling, snags, runs, [loss of tuft bind strength,] [excess static discharge,] <Insert failure characteristic> and delamination.

Verify available warranties and insert number in subparagraph below.

3. Warranty Period:  [10] <Insert number> years from date of Substantial Completion.

B. Special Warranty for Carpet Cushion:  Manufacturer's standard form in which manufacturer agrees to repair or replace components of carpet cushion installation that fail in materials or workmanship within specified warranty period.

1. Warranty includes consequent removal and replacement of carpet and accessories.

2. Warranty does not include deterioration or failure of carpet cushion due to unusual traffic, failure of substrate, vandalism, or abuse.

3. Failure includes, but is not limited to, permanent indentation or compression.

Verify available warranties and insert number in subparagraph below.

4. Warranty Period:  [10] <Insert number> years from date of Substantial Completion.

If Installer warranty is available and required to suit Project, add here.

1.8 EXTRA MATERIALS

Extra materials may not be allowed for publicly funded projects.

A. Furnish extra materials described below, before installation begins, that match products installed and that are packaged with protective covering for storage and identified with labels describing contents.

Revise subparagraph below to suit Project.  Replace percentage with a specific number of square yards (square meters).

1. Carpet:  Full-width rolls equal to [5] <Insert number> percent of amount installed for each type indicated, but not less than 10 sq. yd. (8.3 sq. m).

PART 2 -  PRODUCTS

Insert designations in three article titles below to coordinate Specifications with Drawings.  Replace insert note with the designation used on Drawings or in schedules to indicate locations where a specific type of carpet is required.  Copy applicable articles and re-edit for each product required.  If a project includes only one type of carpet or carpet cushion, consider deleting the designation in each article title.

2.1 TUFTED CARPET <Insert designation>

Revise article title above if fusion-bonded, needle-punched, knitted, or another carpet construction is required to suit Project.

Retain one of two paragraphs and list of manufacturers and products below.  See Division 1 Section "Product Requirements."

A. Available Products:  Subject to compliance with requirements, products that may be incorporated into the Work include, but are not limited to, the following:

B. Products:  Subject to compliance with requirements, provide one of the following:

For competitive bidding, copy subparagraph and associated subparagraphs below and repeat requirements for each additional product.

1. <Insert manufacturer's name; product name or designation.>

a. Color:  [Match Architect's samples] [As selected by Architect from manufacturer's full range] <Insert color>.

Generally delete color above and pattern below if they are described by product designation.

b. Pattern:  [Match Architect's samples] <Insert pattern>.

For competitive bidding from several sources, delete subparagraph below.

c. Source:  <Insert carpet supplier's name and contact information.>

Delete remainder of this Article if products are limited to those listed above.

C. Fiber Content:  [100 percent nylon 6, 6] [100 percent nylon 6] [100 percent polypropylene] <Insert fiber and content by percentage.>

Retain first paragraph below and insert proprietary fiber type such as Antron, Ultron, or Zeftron if required.

D. Fiber Type:  <Insert proprietary fiber type.>

E. Pile Characteristic:  [Level-loop] [Cut] [Cut-and-loop] [Multilevel-loop] [Level tip shear] [Random shear] [Frieze] [Sculptured] <Insert characteristic> pile.

Revise paragraphs and subparagraphs below to suit product and add additional requirements if any.  There are many criteria for specifying carpet construction and many test methods for specifying carpet performance, but there is little consistency among manufacturers in test methods used or in readily providing comprehensive technical information for carpet.  See the Evaluations for guidelines.

F. Yarn Twist:  <Insert twist in TPI (TPCM).>

G. Yarn Count:  <Insert yarn count.>

H. Density:  <Insert oz./cu. yd. (g/cu. cm).>

Test method in first paragraph below is applicable only to finished level-loop and cut-pile constructions with tuft heights less than 0.25 inch (6 mm).  See the Evaluations for an explanation of pile thickness, pile height, and tuft height.

I. Pile Thickness:  <Insert inches (mm)> for finished carpet[ per ASTM D 6859].

J. Stitches:  <Insert stitches per inch (mm).>

K. Gage:  <Insert gage in ends per inch (mm).>

Do not include weight of backings in first paragraph below.

L. Face Weight:  <Insert oz./sq. yd. (g/sq. m).>

M. Total Weight:  <Insert oz./sq. yd. (g/sq. m)> for finished carpet.

N. Primary Backing:  [Manufacturer's standard material] [Woven polypropylene] [Nonwoven, polypropylene or polyester] <Insert specific primary backing material>.

O. Secondary Backing:  [Manufacturer's standard material] [Woven polypropylene] [Nonwoven, polypropylene or polyester] [Woven jute] [Fiberglass] <Insert specific secondary backing material>.

P. Backcoating:  [Manufacturer's standard material] [SBR latex] [PVC] [Thermoplastic copolymer] <Insert backcoating; consult manufacturers>.

Retain first, last, or all three "backing" paragraphs above or first paragraph below to describe backing system.  Retain below and delete three paragraphs above if proprietary backing system, such as "ActionBac," is required.

Q. Backing System:  <Insert proprietary name.>

R. Width:  [12 feet (3.7 m)] [6 feet (1.8 m)] [13.5 feet (4.1 m)] [15 feet (4.6 m)] <Insert width>.

Delete paragraph below if soil resistance is integral to carpet fiber or if soil resistance is not needed for Project; consult manufacturer.

S. Applied Soil-Resistance Treatment:  [Manufacturer's standard material] <Insert treatment>.

Delete first paragraph below if antimicrobial treatment is not needed for Project; consult manufacturer.

T. Antimicrobial Treatment:  [Manufacturer's standard material] <Insert treatment>.

U. Performance Characteristics:  As follows:

Delete first subparagraph below if not required.  Verify requirements of authorities having jurisdiction.

1. Critical Radiant Flux Classification:  Not less than [0.45 W/sq. cm] [0.22 W/sq. cm].

2. Dry Breaking Strength:  Not less than 100 lbf (445 N) per ASTM D 2646.

First and second options in subparagraph below are HUD-recommended minimums for cut pile and loop pile, respectively, per its Use of Materials Bulletin No. 44d.  Third option is minimum average recommended by CRI for school carpet with backing other than unitary backing.  DuPont recommends 5 lbf (22 N) for cut pile and 10 lbf (45 N) for loop pile.  More than 5 lbf (22 N) for cut pile and 10 lbf (45 N) for loop pile may be required for high-frequency traffic locations.

3. Tuft Bind:  Not less than [3 lbf (13 N)] [5 lbf (22 N)] [6.2 lbf (28 N)] [8 lbf (36 N)] [10 lbf (45 N)] <Insert tuft bind> per ASTM D 1335.

Delete subparagraph below if carpet has no secondary backing.  First option is HUD-recommended minimum for carpet per its Use of Materials Bulletin No. 44d.  Second option is recommended by CRI for carpet with primary and secondary backing installed in schools.  Third option is required by GSA per its "Technical Requirements Booklet for Carpet, Carpet Tiles and Carpet Cushion Fsc Group 72, Part I, Section A."

4. Delamination:  Not less than [2.5 lbf/in. (12 N/mm)] [3.5 lbf/in. (15 N/mm)] [4 lbf/in. (18 N/mm)] <Insert force> per ASTM D 3936.

Retain subparagraph below for wool or wool blends only.

5. Resistance to Insects:  Comply with AATCC 24.

Delete first subparagraph below if not required.  Revise if Impact Insulation Class is available and is required to suit Project.

6. Noise Reduction Coefficient (NRC):  <Insert NRC> per ASTM C 423.

7. Colorfastness to Crocking:  Not less than 4, wet and dry, per AATCC 165.

First option in subparagraph below is minimum recommended by CRI for carpet for general use; second option is for carpet installed in schools.

8. Colorfastness to Light:  Not less than 4 after [40] [60] <Insert number> AFU (AATCC fading units) per AATCC 16, Option E.

Retain subparagraph below if retaining antimicrobial treatment.

9. Antimicrobial Activity:  Not less than 2-mm halo of inhibition for gram-positive bacteria; not less than 1-mm halo of inhibition for gram-negative bacteria; no fungal growth; per AATCC 174.

First option in subparagraph below is typical for commercial carpet for general use; second option may be available for greater static control.

10. Electrostatic Propensity:  Less than [3.5] [2] <Insert number> kV per AATCC 134.

Retain subparagraph below if low-emitting materials are required for LEED-NC or LEED-CI Credit EQ 4.3.

11. Environmental Requirements:  Provide carpet that complies with testing and product requirements of Carpet and Rug Institute's "Green Label Plus" program.

Insert other performance characteristics to suit Project, such as smoke density, texture retention, additional colorfastness characteristics, or quantification of soil-resistance treatment.

2.2 WOVEN CARPET <Insert designation>

Retain one of two paragraphs and list of manufacturers and products below.  See Division 1 Section "Product Requirements."

A. Available Products:  Subject to compliance with requirements, products that may be incorporated into the Work include, but are not limited to, the following:

B. Products:  Subject to compliance with requirements, provide one of the following:

For competitive bidding, copy subparagraph and associated subparagraphs below and repeat requirements for each additional product.

1. <Insert manufacturer's name; product name or designation.>

a. Color:  [Match Architect's samples] [As selected by Architect from manufacturer's full range] <Insert color>.

Generally delete color above and pattern below if they are described by product designation.

b. Pattern:  [Match Architect's samples] <Insert pattern>.

For competitive bidding from several sources, delete subparagraph below.

c. Source:  <Insert carpet supplier's name and contact information.>

Delete remainder of this Article if products are limited to those listed above.

C. Fiber Content:  [100 percent wool] [80 percent wool; 20 percent nylon 6, 6] [80 percent wool; 20 percent nylon 6] <Insert fiber and content by percentage.>

D. Face Construction:  [Axminster] [Wilton] [Velvet] <Insert construction>.

E. Pile Characteristic:  [Level-loop] [Cut] [Cut-and-loop] pile.

Revise paragraphs and subparagraphs below to suit product and add additional requirements if any.  There are many criteria for specifying carpet construction and many test methods for specifying carpet performance, but there is little consistency among manufacturers in test methods used or in readily providing comprehensive technical information for carpet.  See the Evaluations for guidelines.

F. Yarn Twist:  <Insert twist in TPI (TPCM).>

G. Yarn Count:  <Insert yarn count.>

H. Density:  <Insert oz./cu. yd. (g/cu. cm).>

Test method in first paragraph below is applicable only to finished level-loop and cut-pile constructions with tuft heights less than 0.25 inch (6 mm).  See the Evaluations for an explanation of pile thickness, pile height, and tuft height.

I. Pile Thickness:  <Insert inches (mm)> for finished carpet[ per ASTM D 6859].

J. Rows:  <Insert number of lengthwise tufts per inch (mm).>

K. Pitch:  <Insert number of rows in 27 inches (686 mm).>

Do not include weight of backings in first paragraph below.

L. Face Weight:  <Insert oz./sq. yd. (g/sq. m).>

M. Total Weight:  <Insert oz./sq. yd. (g/sq. m)> for finished carpet.

N. Backing:  [Manufacturers standard.] [As follows:]

Retain backing details in four subparagraphs below if retaining second option in paragraph above; consult manufacturer.

1. Chain Warp:  <Insert material.>

2. Stuffer Warp:  <Insert material.>

3. Shot or Fill Weft:  <Insert material.>

4. Backcoating:  <Insert backcoating.>

Delete first paragraph below if soil resistance is integral to carpet fiber or if soil resistance is not needed for Project; consult manufacturer.

O. Applied Soil-Resistance Treatment:  [Manufacturer's standard material] <Insert treatment>.

P. Performance Characteristics:  As follows:

Delete first subparagraph below if not required.  Verify requirements of authorities having jurisdiction.

1. Critical Radiant Flux Classification:  Not less than [0.45 W/sq. cm] [0.22 W/sq. cm].

2. Dry Breaking Strength:  Not less than 100 lbf (445 N) per ASTM D 2646.

3. Resistance to Insects:  Comply with AATCC 24.

Delete first subparagraph below if not required.  Revise if Impact Insulation Class is available and is required to suit Project.

4. Noise Reduction Coefficient (NRC):  <Insert NRC> per ASTM C 423.

5. Colorfastness to Crocking:  Not less than 4, wet and dry, per AATCC 165.

First option in subparagraph below is minimum recommended by CRI for carpet for general use; second option is for carpet installed in schools.

6. Colorfastness to Light:  Not less than 4 after [40] [60] <Insert number> AFU (AATCC fading units) per AATCC 16, Option E.

First option in subparagraph below is typical for commercial carpet for general use; second option may be available for greater static control.

7. Electrostatic Propensity:  Less than [3.5] [2] <Insert number> kV per AATCC 134.

Retain subparagraph below if low-emitting materials are required for LEED-NC or LEED-CI Credit EQ 4.3.

8. Environmental Requirements:  Provide carpet that complies with testing and product requirements of Carpet and Rug Institute's "Green Label Plus" program.

Insert other performance characteristics to suit Project, such as smoke density, texture retention, additional colorfastness characteristics, antimicrobial treatment, or quantification of soil-resistance treatment.

2.3 CARPET CUSHION <Insert designation>

Retain one of first two paragraphs and list of manufacturers and products below.  See Division 1 Section "Product Requirements."

For a descriptive specification that does not name carpet cushion products or manufacturers, delete first two paragraphs and list.

A. Available Products:  Subject to compliance with requirements, products that may be incorporated into the Work include, but are not limited to, the following:

B. Products:  Subject to compliance with requirements, provide one of the following:

1. <Insert manufacturer's name; product name or designation.>

C. Traffic Classification:  CCC Class [I, moderate] [II, heavy] [III, extra-heavy] traffic.

Retain paragraph and subparagraphs below for fiber carpet cushion.

D. Fiber Cushion:  [Rubberized hair, mothproofed and sterilized] [Rubberized jute, mothproofed and sterilized] [Synthetic] [Resinated, recycled textile].

1. Weight:  <Insert oz./sq. yd. (g/sq. m).>

2. Thickness:  <Insert inches (mm)> plus 5 percent maximum.

3. Density:  <Insert lb/cu. ft. (kg/cu. m).>

Retain paragraph and subparagraphs below for rubber carpet cushion.

E. Rubber Cushion:  [Flat] [Rippled waffle] [Textured flat] [Reinforced].

1. Weight:  <Insert oz./sq. yd. (g/sq. m).>

2. Thickness:  <Insert inches (mm)> plus 5 percent maximum.

3. Compression Resistance:  <Insert lb/sq. in. (kg/sq. mm)> at [25] [65] percent per ASTM D 3676.

4. Density:  <Insert lb/cu. ft. (kg/cu. m).>

Retain first paragraph and subparagraphs below for polyurethane-foam carpet cushion.

F. Polyurethane-Foam Cushion:  [Grafted prime] [Densified] [Bonded] [Mechanically frothed].

1. Compression Force Deflection at 65 Percent:  <Insert lb/sq. in. (kg/sq. mm) of polymer density> per ASTM D 3574.

2. Thickness:  <Insert inches (mm).>

3. Density:  <Insert lb/cu. ft. (kg/cu. m).>

G. Performance Characteristics:  As follows:

Delete subparagraph below if not required.  Verify requirements of authorities having jurisdiction.

1. Critical Radiant Flux Classification:  Not less than [0.45 W/sq. cm] [0.22 W/sq. cm].

Delete subparagraph below if not required.  Revise if Impact Insulation Class is available and is required to suit Project.

2. Noise Reduction Coefficient (NRC):  <Insert NRC> per ASTM C 423.

Retain subparagraph below if low-emitting materials are required for LEED-NC or LEED-CI Credit EQ 4.3.

3. Environmental Requirements:  Provide carpet cushion that complies with testing and product requirements of Carpet and Rug Institute's "Green Label" program.

2.4 INSTALLATION ACCESSORIES

Revise paragraph below to suit Project.  Retain option for double-glue-down installations.

A. Trowelable Leveling and Patching Compounds:  Latex-modified, hydraulic-cement-based formulation provided or recommended by carpet[ cushion] manufacturer.

Revise paragraph below to suit product(s) and Project.  Retain second option for double-glue-down installations.  Pressure-sensitive adhesives are commonly used for carpet tiles but can be used in carpet applications.

B. Adhesives:  Water-resistant, mildew-resistant, nonstaining type to suit products and subfloor conditions indicated, that complies with flammability requirements for installed carpet and is recommended or provided by [carpet manufacturer] [carpet and carpet cushion manufacturers].

Retain subparagraph below if low-emitting materials are required for LEED-NC or LEED-CI Credit EQ 4.3.

1. VOC Limits:  Provide adhesives with VOC content not more than 50g/L when calculated according to 40 CFR 59, Subpart D (EPA method 24).

Retain first paragraph below for stretch-in installations.

C. Tackless Carpet Stripping:  Water-resistant plywood, in strips as required to match cushion thickness and that comply with CRI 104, Section 12.2.

D. Seam Adhesive:  Hot-melt adhesive tape or similar product recommended by carpet manufacturer for sealing and taping seams and butting cut edges at backing to form secure seams and to prevent pile loss at seams.

Delete paragraph below if resilient edge strips are specified in another Division 9 Section.  If retaining, revise finish or description to suit Project and show width requirements on Drawings.

E. Metal Edge Strips:  Extruded aluminum with mill finish of width shown, of height required to protect exposed edge of carpet, and of maximum lengths to minimize running joints.

PART 3 -  EXECUTION

3.1 EXAMINATION

A. Examine substrates, areas, and conditions, with Installer present, for compliance with requirements for maximum moisture content, alkalinity range, installation tolerances, and other conditions affecting carpet performance.  Examine carpet for type, color, pattern, and potential defects.

Retain paragraph and subparagraphs below if products are adhesively attached to concrete subfloors.

B. Concrete Subfloors:  Verify that concrete slabs comply with ASTM F 710 and the following:

Revise first subparagraph below to suit Project.  Retain option for double-glue-down installations.

1. Slab substrates are dry and free of curing compounds, sealers, hardeners, and other materials that may interfere with adhesive bond.  Determine adhesion and dryness characteristics by performing bond and moisture tests recommended by carpet[ cushion] manufacturer.

2. Subfloor finishes comply with requirements specified in Division 3 Section "Cast-in-Place Concrete" for slabs receiving carpet.

3. Subfloors are free of cracks, ridges, depressions, scale, and foreign deposits.

Delete first paragraph and subparagraphs below if no wood subfloors.  Carpet manufacturers recommend underlayment over wood subfloors; below assumes underlayment is specified.  Coordinate with Division 6 Section "Rough Carpentry" to ensure that underlayment material and installation method specified are acceptable to carpet or carpet cushion manufacturers selected.

C. For wood subfloors, verify the following:

1. Underlayment over subfloor complies with requirements specified in Division 6 Section "Rough Carpentry."

2. Underlayment surface is free of irregularities and substances that may interfere with adhesive bond or show through surface.

D. Proceed with installation only after unsatisfactory conditions have been corrected.

3.2 PREPARATION

A. General:  Comply with CRI 104, Section 7.3, "Site Conditions; Floor Preparation," and with carpet manufacturer's written installation instructions for preparing substrates.

Revise paragraph below to suit product and Project.  Dimensions below are recommended by CRI.

B. Use trowelable leveling and patching compounds, according to manufacturer's written instructions, to fill cracks, holes, depressions, and protrusions in substrates.  Fill or level cracks, holes and depressions 1/8 inch (3 mm) wide or wider, and protrusions more than 1/32 inch (0.8 mm), unless more stringent requirements are required by manufacturer's written instructions.

Retain first paragraph below if carpet is adhesively attached to concrete substrates.  Retain option for double-glue-down installations.

C. Remove coatings, including curing compounds, and other substances that are incompatible with adhesives and that contain soap, wax, oil, or silicone, without using solvents.  Use mechanical methods recommended in writing by carpet[ cushion] manufacturer.

D. Broom and vacuum clean substrates to be covered immediately before installing carpet.

3.3 INSTALLATION

A. Comply with CRI 104 and [carpet manufacturer's] [carpet and carpet cushion manufacturers'] written installation instructions for the following:

1. Direct-Glue-Down Installation:  Comply with CRI 104, Section 9, "Direct Glue-Down Installation."

2. Double-Glue-Down Installation:  Comply with CRI 104, Section 10, "Double Glue-Down Installation."

3. Carpet with Attached-Cushion Installation:  Comply with CRI 104, Section 11, "Attached-Cushion Installations."

4. Preapplied Adhesive Installation:  Comply with CRI 104, Section 11.4, "Pre-Applied Adhesive Systems (Peel and Stick)."

5. Hook-and-Loop Installation:  Comply with CRI 104, Section 11.5, "Hook and Loop Technology."

6. Stretch-in Installation:  Comply with CRI 104, Section 12, "Stretch-in Installation."

7. Stair Installation:  Comply with CRI 104, Section 13, "Carpet on Stairs" for [stretch-in] [glue-down] installation.

B. Comply with carpet manufacturer's written recommendations and Shop Drawings for seam locations and direction of carpet; maintain uniformity of carpet direction and lay of pile.  At doorways, center seams under the door in closed position.

Retain one of two options in subparagraph below for seamed border installation.

1. [Bevel adjoining border edges at seams with hand shears] [Level adjoining border edges].

C. Do not bridge building expansion joints with carpet.

D. Cut and fit carpet to butt tightly to vertical surfaces, permanent fixtures, and built-in furniture including cabinets, pipes, outlets, edgings, thresholds, and nosings.  Bind or seal cut edges as recommended by carpet manufacturer.

E. Extend carpet into toe spaces, door reveals, closets, open-bottomed obstructions, removable flanges, alcoves, and similar openings.

F. Maintain reference markers, holes, and openings that are in place or marked for future cutting by repeating on finish flooring as marked on subfloor.  Use nonpermanent, nonstaining marking device.

Delete paragraph below if carpet has no pattern.

G. Install pattern parallel to walls and borders to comply with CRI 104, Section 15, "Patterned Carpet Installations" and with carpet manufacturer's written recommendations.

Retain option in paragraph below for stretch-in installations.

H. Comply with carpet cushion manufacturer's written recommendations.[  Install carpet cushion seams at 90-degree angle with carpet seams.]

3.4 CLEANING AND PROTECTING

A. Perform the following operations immediately after installing carpet:

1. Remove excess adhesive, seam sealer, and other surface blemishes using cleaner recommended by carpet manufacturer.

2. Remove yarns that protrude from carpet surface.

3. Vacuum carpet using commercial machine with face-beater element.

B. Protect installed carpet to comply with CRI 104, Section 16, "Protection of Indoor Installations."

Retain first option in paragraph below if carpet cushion is required for stretch-in applications.  Retain second for glue-down applications, third for double-glue-down applications.

C. Protect carpet against damage from construction operations and placement of equipment and fixtures during the remainder of construction period.  Use protection methods indicated or recommended in writing by carpet manufacturer [and carpet cushion manufacturer] [and carpet adhesive manufacturer] [and carpet cushion and adhesive manufacturers].

END OF SECTION 09680END OF SECTION 09680

CARPET
09680 - 1

