Copyright 2003 AIA
MASTERSPEC
11/03

Copyright 2003 by The American Institute of Architects (AIA)

Exclusively published and distributed by Architectural Computer Services, Inc. (ARCOM) for the AIA

SECTION 12491 - HORIZONTAL LOUVER BLINDS
This Section uses the term "Architect."  Change this term to match that used to identify the design professional as defined in the General and Supplementary Conditions.

Verify that Section titles referenced in this Section are correct for this Project's Specifications; Section titles may have changed.

PART 1 -  GENERAL

1.1 RELATED DOCUMENTS

A. Drawings and general provisions of the Contract, including General and Supplementary Conditions and Division 1 Specification Sections, apply to this Section.

1.2 SUMMARY

A. This Section includes the following:

Adjust list below to suit Project.

1. Horizontal louver blinds with [aluminum] [wood] [polymer] slats.

2. Motorized blind operators.

B. Related Sections include the following:

List below only products and construction that the reader might expect to find in this Section but are specified elsewhere.

1. Division 6 Section "Miscellaneous Carpentry" for wood blocking and grounds for mounting horizontal louver blinds and accessories.

2. Division 16 Sections for electrical service and connections for motor operators, controls, limit switches, and other powered devices and for system disconnect switches for motorized blind operation.

If alternates, allowances, or unit prices apply to Work of this Section, insert brief paragraphs here to alert Contractor and reference appropriate Division 1 Sections for specific details.

1.3 SUBMITTALS

A. Product Data:  For each type of product indicated.

B. Shop Drawings:  Show fabrication and installation details for horizontal louver blinds[ and motorized blind operators].

Retain subparagraph below if equipment includes wiring.

1. Wiring Diagrams:  Power, system, and control wiring.

C. Samples for Initial Selection:  For each type and color of horizontal louver blind indicated.

1. Include similar Samples of accessories involving color selection.

Delete paragraph and subparagraph above if colors and other characteristics are preselected and specified or scheduled.  Retain paragraph and subparagraphs below with or without above.

D. Samples for Verification:  For each type and color of horizontal louver blind indicated.

1. Slat:  Not less than 12 inches (300 mm) long.

Retain or revise applicable subparagraphs below to suit Project.  Typically, colored or decorative tapes are available only for 1-inch- (25-mm-) wide or wider slats.

2. Tapes:  Full width, not less than 6 inches (150 mm) long.

3. Horizontal Louver Blind:  Full-size unit, not less than 16 inches (400 mm) wide by 24 inches (600 mm) long.

4. Valance:  Full-size unit, not less than 12 inches (300 mm) wide.

5. Cornice:  Full-size unit, not less than 12 inches (300 mm) wide.

Retain first paragraph below if a window treatment schedule that includes horizontal louver blinds is required.

E. Window Treatment Schedule:  For horizontal louver blinds.  Use same designations indicated on Drawings.

F. Product Certificates:  For each type of horizontal louver blind, signed by product manufacturer.

Retain paragraph above if manufacturer certification is required or first paragraph below if test reports are required.

G. Product Test Reports:  Based on evaluation of comprehensive tests performed by [manufacturer and witnessed by a qualified testing agency] [a qualified testing agency], for each type of horizontal louver blind.

H. Maintenance Data:  For horizontal louver blinds to include in maintenance manuals.

1.4 QUALITY ASSURANCE

Delete or revise paragraph below if horizontal louver blinds from more than one manufacturer are required for Project.

A. Source Limitations:  Obtain horizontal louver blinds through one source from a single manufacturer.

Retain paragraph and subparagraphs below if needed to comply with requirements of authorities having jurisdiction.  If retaining, verify with manufacturers that products comply with requirements.

B. Fire-Test-Response Characteristics:  Provide horizontal louver blinds with the fire-test-response characteristics indicated, as determined by testing identical products per test method indicated below by UL or another testing and inspecting agency acceptable to authorities having jurisdiction.  Identify materials with appropriate markings of applicable testing and inspecting agency.

Retain applicable subparagraphs below.

1. Flame-Resistance Ratings:  Passes NFPA 701.

2. <Insert fire-test-response characteristic>:  <Insert fire-test-response requirements and test method>.

Delete first paragraph below if no motorized blind operator.

C. Electrical Components, Devices, and Accessories:  Listed and labeled as defined in NFPA 70, Article 100, by a testing agency acceptable to authorities having jurisdiction, and marked for intended use.

D. Product Standard:  Provide horizontal louver blinds complying with WCSC A 100.1.

Delete paragraph and subparagraph below if not required.  If retaining, indicate location, size, and other details of mockups on Drawings or by inserts.  Revise wording if only one mockup is required.

E. Mockups:  Build mockups to verify selections made under sample submittals and to demonstrate aesthetic effects and set quality standards for materials and execution.

Retain subparagraph below if mockups are erected as part of building rather than separately and the intention is to make an exception to the default requirement in Division 1 Section "Quality Requirements" for demolishing and removing mockups when directed, unless otherwise indicated.

1. Approved mockups may become part of the completed Work if undisturbed at time of Substantial Completion.

1.5 DELIVERY, STORAGE, AND HANDLING

A. Deliver horizontal louver blinds in factory packages, marked with manufacturer and product name, [fire-test-response characteristics,] [lead-free designation,] and location of installation using same designations indicated on Drawings and in a window treatment schedule.

1.6 PROJECT CONDITIONS

A. Environmental Limitations:  Do not install horizontal louver blinds until construction and wet and dirty finish work in spaces, including painting, is complete and dry and ambient temperature and humidity conditions are maintained at the levels indicated for Project when occupied for its intended use.

B. Field Measurements:  Where horizontal louver blinds are indicated to fit to other construction, verify dimensions of other construction by field measurements before fabrication and indicate measurements on Shop Drawings.  Allow clearances for operable glazed units' operation hardware throughout the entire operating range.  Notify Architect of discrepancies.  Coordinate fabrication schedule with construction progress to avoid delaying the Work.

1.7 EXTRA MATERIALS

Extra materials may not be allowed for publicly funded projects.

A. Furnish extra materials described below that match products installed and that are packaged with protective covering for storage and identified with labels describing contents.

1. Horizontal Louver Blinds:  Before installation begins, for each size, color, texture, pattern, and gloss indicated, full-size units equal to 5 percent of amount installed[, but no fewer than <Insert number> units].

PART 2 -  PRODUCTS

2.1 HORIZONTAL LOUVER BLINDS, ALUMINUM SLATS

See Editing Instruction No. 1 in the Evaluations for cautions about naming manufacturers and products.  Retain one of first three paragraphs below.  Retain list of manufacturers with either of first two paragraphs; insert list of manufacturers if retaining third paragraph.  See Division 1 Section "Product Requirements."

A. Available Products:  Subject to compliance with requirements, products that may be incorporated into the Work include, but are not limited to, the following:

B. Products:  Subject to compliance with requirements, provide one of the following:

1. Hunter Douglas; <Insert product name or designation>.

2. Levolor, a Newell Rubbermaid Company; <Insert product name or designation>.

3. Springs Window Fashions Division, Inc.; <Insert product name or designation>.

4. <Insert manufacturer's name; product name or designation.>

C. Basis-of-Design Product:  Subject to compliance with requirements, provide [the product indicated on Drawings] <Insert manufacturer's name; product name or designation> or a comparable product by one of the following:

1. <Insert, in separate subparagraphs, manufacturer's name.>

D. Slats:  Aluminum; alloy and temper recommended by producer for type of use and finish indicated; with crowned profile[ and radiused corners].

Retain one of three nominal slat widths and spacing options in first two subparagraphs and associated subparagraphs below.  Verify availability with manufacturers.

1. Width:  2 inches (51 mm).

a. Spacing:  [Manufacturer's standard] [1.74 inch (44 mm)] <Insert dimension>.

2. Width:  [1 inch (25 mm)] [1/2 to 5/8 inch (13 to 16 mm)].

a. Spacing:  [Not less than every] [Every] [0.71 inch (18 mm)] [0.77 inch (19.5 mm)] [0.79 inch (20 mm)] [0.85 inch (21.5 mm)] [0.87 inch (22 mm)].

3. Thickness:  [Manufacturer's standard] [Not less than 0.006 inch (0.15 mm)] [Not less than 0.008 inch (0.20 mm)] <Insert dimension>.

4. Finish:  [One color] [One color each side] [As indicated].

a. Ionized Coating:  Antistatic, dust-repellent, baked polyester finish.

Retain last subparagraph above or subparagraph below.  Availability of reflective coating below is limited.

b. Reflective Coating:  Manufacturer's special coating enhancing the reflection of solar energy on the outside-facing slat surface.

Retain subparagraph below for perforated slats.  Color selection of perforated-aluminum slats is limited, and darker colors have less affect on the visible image through slats.

5. Perforated Slats:  Openness factor of [6 to 7] <Insert number> percent.

Delete paragraph and subparagraphs below if headrail is specified with motorized operator.

E. Headrail:  Formed steel or extruded aluminum; long edges returned or rolled; fully enclosing operating mechanisms on three sides[ and end plugs] and the following:

1. Capacity:  [One blind per headrail] [Two blinds per headrail] [As indicated].

2. Integrated Headrail/Valance:  [Curved face] <Insert description>.

3. Light-blocking lower back lip.

4. Tilt limiter with preselected degree settings.

Select one option in first paragraph below or insert another bottom rail type to suit Project.

F. Bottom Rail:  Formed-steel or extruded-aluminum tube, with plastic or metal capped ends [top contoured to match crowned shape of slat] [bottom contoured for minimizing light gaps] <Insert type>; with enclosed ladders and tapes to prevent contact with sill.

Retain first paragraph and subparagraph below for premium-quality blinds with superior light blockage.  Coordinate with tape spacing specified previously.

G. Maximum Light-Blocking Blinds:  Designed for eliminating all visible light gaps if slats are tilted closed; with tight tape spacing indicated and slats with minimal-sized rout holes for ladders hidden and placed near back edge for maximum slat overlap; with headrail and bottom rail extended and formed for light-tight joints between rail and adjacent slats or construction.

1. Finish:  [Match color, texture, pattern, and gloss of slats] [Color, texture, pattern, and gloss differing from slats as indicated by manufacturer's designations] [Color, texture, pattern, and gloss differing from slats, matching Architect's samples] [Color texture, pattern, and gloss differing from slats as selected by Architect from manufacturer's full range].

H. Ladders:  Evenly spaced to prevent long-term slat sag.

1. For Blinds with Nominal Slat Width 1 Inch (25 mm) or Less:  Braided string.

Retain subparagraph above or below.

2. For Blinds with Nominal Slat Width [1 Inch (25 mm)] [2 Inches (51 mm)] or More:  [Braided string] [Manufacturer's standard-width reinforced vinyl tapes] [Manufacturer's standard-width cloth tapes] <Insert description>.

Delete subparagraph below unless option for tapes is retained in subparagraph above.

a. Tape Color, Texture, and Pattern:  [Color, texture, and pattern as indicated by manufacturer's designations] [Color, texture, and pattern matching Architect's samples] [Color, texture, and pattern as selected by Architect from manufacturer's full range].

I. Lift-and-Tilt Control:  Motorized operator.

If retaining paragraph above, coordinate with "Motorized Horizontal Louver Blind Operators" Article and delete first four paragraphs and associated subparagraphs below.

J. Lift Cords:  Manufacturer's standard.

K. Tilt Control:  Enclosed worm-gear mechanism[, slip clutch or detachable wand preventing overrotation,] and linkage rod, and the following:

1. Tilt Operation:  Manual with [clear plastic wand] [cord-operated tilter].

Retain subparagraph below or delete if one-third to one-half length of blind specified in WCSC standard is acceptable.

2. Length of Tilt Control:  [Manufacturer's standard] [Full length of blind] [Length required to make operation convenient from floor level] [As indicated on Drawings].

Retain one of two subparagraphs below.  Limited tilt is generally required where heat buildup between glass and blind is a potential problem.  Tilt limited to 60 degrees from zero-degree horizontal is recommended by GANA in its "Glazing Manual."  Tilt limited to 70 degrees from zero-degree horizontal is an example for coordinating appearance and performance of 2-color slat blinds.

3. Tilt:  Full.

4. Tilt:  [One] [Two]-direction, positive stop or lock-out limited at an angle of [40] [60] [70] [75] <Insert number> degrees from zero-degree horizontal[, both directions].

L. Lift Operation:  Manual, cord lock; locks pull cord to stop blind at any position in ascending or descending travel.

Retain paragraph above or below for manual operation.  If top-locking, cord-lock blinds are improperly operated, ring pull prevents blind from dropping and damaging itself or sill.  Revise positions or length of ring pull below to suit Project.

M. Lift Operation:  Manual, top-locking cord lock; locks pull cord to stop blind in either fully opened or fully closed position only and is equipped with a ring pull not more than 4 inches (100 mm) long.

Retain paragraph below, or delete if default left-side position for tilt control and right-side position for cord lock specified in WCSC standard is acceptable.  Verify availability of positions with manufacturers for each product.  Industry standard positions are referenced to operator hands as they face blind.  Center tilt-control position option is generally required for small-width blinds.

N. Tilt-Control and Cord-Lock Position:  [Right and left side of headrail, respectively] [Right side of headrail] [Left side of headrail] [Center blind and left side of headrail, respectively] [Center blind and right side of headrail, respectively], unless otherwise indicated.

Delete first paragraph and subparagraph below if retaining "Headrail" Paragraph in this Article.

O. Valance:  [Two slats] [PVC strip] [Manufacturer's standard] <Insert description>.

1. Finish Color Characteristics:  [Match color, texture, pattern, and gloss of slats] [Color, texture, pattern, and gloss differing from slats as indicated by manufacturer's designations] [Color, texture, pattern, and gloss differing from slats, matching Architect's samples] [Color texture, pattern, and gloss differing from slats as selected by Architect from manufacturer's full range].

P. Mounting:  [Wall mounting] [Ceiling mounting] [End mounting] [Wall extension brackets] [As indicated], permitting easy removal and replacement without damaging blind or adjacent surfaces and finishes; with spacers and shims required for blind placement and alignment indicated.

1. Provide intermediate support brackets if end support spacing exceeds spacing recommended by manufacturer for weight and size of blind.

Q. Hold-Down Brackets and Hooks or Pins:  Manufacturer's standard.

Retain, revise, or delete paragraph above and below as required for Project and product.  Hold-down brackets and pins prevent swaying caused by air movement and are used on doors so blind remains in place when door is used.  Side channels reduce light leakage at blind edge.  If retaining above or below, or both, but not all blinds need hold-down brackets and pins or side channels, indicate by designation in article title and copy and edit article content; or differentiate between blinds on Drawings or in a window treatment schedule.

R. Side Channels and Perimeter Light Gap Seals:  Manufacturer's standard.

Delete paragraph below if color, texture, pattern, and gloss are specified with manufacturer's name and product in this Article.

S. Colors, Textures, Patterns, and Gloss:  [Match Architect's samples] [Match Architect's samples for custom color and other characteristics] [As selected by Architect from manufacturer's full range] [As indicated in a window treatment schedule].

2.2 HORIZONTAL LOUVER BLINDS, WOOD SLATS

See Editing Instruction No. 1 in the Evaluations for cautions about naming manufacturers and products.  Retain one of the first three paragraphs below.  Retain list of manufacturers with either of the first two paragraphs; insert list of manufacturers if retaining third paragraph.  See Division 1 Section "Product Requirements."

A. Available Products:  Subject to compliance with requirements, products that may be incorporated into the Work include, but are not limited to, the following:

B. Products:  Subject to compliance with requirements, provide one of the following:

1. Hunter Douglas; <Insert product name or designation>.

2. Levolor, a Newell Rubbermaid Company; <Insert product name or designation>.

3. Mark Window Products; <Insert product name or designation>.

4. Springs Window Fashions Division, Inc.; <Insert product name or designation>.

5. <Insert manufacturer's name; product name or designation.>

C. Basis-of-Design Product:  Subject to compliance with requirements, provide [the product indicated on Drawings] <Insert manufacturer's name; product name or designation> or a comparable product by one of the following:

1. <Insert, in separate subparagraphs, manufacturer's name.>

Verify slat characteristics with manufacturers.  For example, only Springs Window Fashions offers wood finished with a flame-retardant treatment.

D. Slats:  Hardwood, [North American] [basswood] [poplar] [ramin] <Insert species> species[, flame-retardant treated]; with flat profile [and radiused corners] [and beaded edges] [and double beaded edges].

Retain one of four nominal slat widths and spacings in first two subparagraphs and associated subparagraphs below.  Revise slat spacing to suit Project and product.  Verify availability of slat widths with manufacturers.

1. Width:  [1-3/8 inch (35 mm)] [2 inches (51 mm)] [2-3/8-inch (60-mm)]<Insert dimension>.

a. Spacing:  Manufacturer's standard.

2. Width:  1 inch (25 mm).

a. Spacing:  [Manufacturer's standard] [Every] [0.79 inch (20 mm)] <Insert dimension>.

3. Finish:  [Manufacturer's standard] [<Insert number> colors as indicated, for striped blind with pattern as indicated on Drawings].

Delete first paragraph below if motorized operation requires substituting headrail by motorized blind operator manufacturer.

E. Headrail:  Formed steel or extruded aluminum; long edges returned or rolled; fully enclosing operating mechanisms on three sides and ends; capacity for [one] [two] <Insert number> blind(s) per headrail, unless otherwise indicated [on Drawings] [in a window treatment schedule].

F. Bottom Rail:  Hardwood matching slats[ and trapezoid-shaped bottom angled for minimizing light gaps]; with enclosed ladders and tapes to prevent contact with sill.

1. Finish Color Characteristics:  [Match color, texture, pattern, and gloss of slats] [Match color, texture, pattern, and gloss of valance] [Color, texture, pattern, and gloss differing from slats as indicated by manufacturer's designations] [Color, texture, pattern, and gloss differing from slats, matching Architect's samples] [Color texture, pattern, and gloss differing from slats as selected by Architect from manufacturer's full range].

Retain first paragraph below if premium-quality blinds with superior light blockage suit Project.  Correlate with tape spacing specified previously.

G. Maximum Light-Blocking Blinds:  Designed for eliminating all visible light gaps if slats are tilted closed; with tight tape spacing indicated and slats with minimal-sized rout holes for ladders hidden and placed near back edge for maximum slat overlap; with headrail and bottom rail extended and formed for light-tight joints between rail and adjacent slats or construction.

H. Ladders:  [Braided string] [Manufacturer's standard-width cloth tapes] <Insert description>.  Evenly spaced to prevent long-term louver sag.

Delete subparagraph below unless option for tapes is retained in paragraph above.

1. Tape Color, Texture, and Pattern:  [Color, texture, and pattern as indicated by manufacturer's designations] [Color, texture, and pattern matching Architect's samples] [Color, texture, and pattern as selected by Architect from manufacturer's full range].

I. Tilt Control:  Enclosed worm gear mechanism,[ slip clutch or detachable wand preventing overrotation,] and linkage rod, for the following operation:

1. Tilt Operation:  Manual with [clear plastic wand] [hardwood wand finished to match slats] [cord-operated tilter].

Retain subparagraph above or below.  If retaining below, coordinate with "Motorized Horizontal Louver Blind Operators" Article.

2. Tilt Operation:  Motorized operator.

Retain first subparagraph below or delete if one-third to one-half length of blind specified in WCSC standard is acceptable.

3. Length of Tilt Control:  [Full length of blind] [Length required to make operation convenient from floor level] [As indicated on Drawings].

4. Tilt:  [Full] <Insert limited tilt description>.

J. Lift Operation:  Manual, cord lock; locks pull cord to stop blind at any position in ascending or descending travel.

Retain paragraph above or below for manual operation.  If top-locking cord-lock blinds are improperly operated, ring pull prevents blind from dropping and damaging itself or sill.  Revise length of ring pull below if required for Project.

K. Lift Operation:  Manual, top-locking cord lock; locks pull cord to stop blind in either fully opened or fully closed position only and is equipped with a ring pull not more than 4 inches (100 mm) long.

If retaining first paragraph below, coordinate with "Motorized Horizontal Louver Blind Operators" Article.

L. Lift Operation:  Motorized operator.

M. Tilt-Control and Cord-Lock Position:  As indicated [on Drawings] [in a window treatment schedule].

Retain paragraph above or first paragraph below, or delete both if default left-side position for tilt control and right-side position for cord lock specified in WCSC standard is acceptable.  Verify availability of positions with manufacturers for each product.  Industry standard positions are referenced to operator hands as they face blind.  Center tilt-control position option is generally required for small-width blinds.

N. Tilt-Control and Cord-Lock Position:  [Right side of headrail] [Left side of headrail] [Center blind and left side of headrail, respectively] [Center blind and right side of headrail, respectively], unless otherwise indicated.

Delete paragraph and subparagraph below if not applicable.

O. Valance:  [Manufacturer's standard] <Insert description>.

1. Finish Color Characteristics:  [Match color, texture, pattern, and gloss of slats] [Color, texture, pattern, and gloss differing from slats as indicated by manufacturer's designations] [Color, texture, pattern, and gloss differing from slats, matching Architect's samples] [Color texture, pattern, and gloss differing from slats as selected by Architect from manufacturer's full range].

Delete paragraph and subparagraph below if not applicable.

P. Cornice:  <Insert description.>

1. Finish Color Characteristics:  [Match color, texture, pattern, and gloss of slats] [Color, texture, pattern, and gloss differing from slats as indicated by manufacturer's designations] [Color, texture, pattern, and gloss differing from slats, matching Architect's samples] [Color texture, pattern, and gloss differing from slats as selected by Architect from manufacturer's full range].

Delete first paragraph below if not applicable.  Verify availability of wood tassels with manufacturers.

Q. Tassels:  Hardwood, [manufacturer's standard] <Insert style>[ finished to match slats].

R. Mounting:  [Wall mounting] [Ceiling mounting] [End mounting] [Wall extension brackets] [As indicated], permitting easy removal and replacement without damaging blind or adjacent surfaces and finishes; with spacers and shims required for blind placement and alignment indicated.

1. Provide intermediate support brackets if end support spacing exceeds spacing recommended by manufacturer for weight and size of blind.

Retain, revise, or delete paragraph below as required for Project and product.  Hold-down brackets and pins prevent swaying caused by air movement and are used on doors so blind remains in place when door is used.  If retaining below but not all blinds need hold-down brackets and pins, indicate by designation in article title and copy and edit article content; or differentiate between blinds on Drawings or in a window treatment schedule.

S. Hold-Down Brackets and Hooks or Pins:  Manufacturer's standard, as indicated.

Delete paragraph below if color, texture, pattern, and gloss are specified with manufacturer's name and product in this Article.

T. Colors, Textures, Patterns, and Gloss:  [Match Architect's samples] [Match Architect's samples for custom color and other characteristics] [As selected by Architect from manufacturer's full range] [As indicated in a window treatment schedule].

2.3 HORIZONTAL LOUVER BLINDS, POLYMER SLATS

See Editing Instruction No. 1 in the Evaluations for cautions about naming manufacturers and products.  Retain one of first three paragraphs below.  Retain list of manufacturers with either of first two paragraphs; insert list of manufacturers if retaining third paragraph.  See Division 1 Section "Product Requirements."

A. Available Products:  Subject to compliance with requirements, products that may be incorporated into the Work include, but are not limited to, the following:

B. Products:  Subject to compliance with requirements, provide one of the following:

1. Comfortex Window Fashions; <Insert product name or designation>.

2. Hunter Douglas; <Insert product name or designation>.

3. Levolor, a Newell Rubbermaid Company; <Insert product name or designation>.

4. Mark Window Products; <Insert product name or designation>.

5. Springs Window Fashions Division, Inc.; <Insert product name or designation>.

6. <Insert manufacturer's name; product name or designation.>

C. Basis-of-Design Product:  Subject to compliance with requirements, provide [the product indicated on Drawings] <Insert manufacturer's name; product name or designation> or a comparable product by one of the following:

1. <Insert, in separate subparagraphs, manufacturer's name.>

D. Slats:  Lead-free, UV-stabilized, integrally colored, opaque, [permanently flexible, extruded PVC] [polymer/wood alloy] that will not crack or yellow; antistatic, dust-repellent treated; with [crowned] [manufacturer's standard] profile.

1. Width:  [2 inches (51 mm)] [2-1/2 inches (64 mm)].

a. Spacing:  Manufacturer's standard.

Retain one of two subparagraphs below.  If different finishes on topside and underside of slat are required to suit Project and product, select second subparagraph and one option; otherwise, delete.

2. Finish:  [Wood-tone] [Painted] color as indicated.

3. Finish:  Two [colors] [textures] [patterns] as indicated, one per side of slat.

Delete paragraph below if motorized operation requires substituting headrail by motorized blind operator manufacturer.

E. Headrail:  Formed steel or extruded aluminum; long edges returned or rolled; fully enclosing operating mechanisms on three sides and ends; capacity for [one] [two] <Insert number> blind(s) per headrail, unless otherwise indicated [on Drawings] [in a window treatment schedule].

Delete first paragraph below if bar or open-channel-type bottom rail suits Project.

F. Bottom Rail:  [Manufacturers standard] [Formed-steel or extruded-aluminum tube, with plastic or metal capped ends] [Hardwood matching slats and trapezoid-shaped bottom angled for minimizing light gaps]; with enclosed and protected ladders and tapes to prevent contact with sill.

G. Ladders:  [Braided string] [Manufacturer's standard-width cloth tapes] <Insert description>.  Evenly spaced to prevent long-term slat sag.

Delete subparagraph below unless option for tapes is retained in paragraph above.

1. Tape Color, Texture, and Pattern:  [Color, texture, and pattern as indicated by manufacturer's designations] [Color, texture, and pattern matching Architect's samples] [Color, texture, and pattern as selected by Architect from manufacturer's full range].

H. Tilt Control:  Enclosed worm-gear mechanism and linkage rod, for the following operation:

1. Tilt Operation:  Manual with [clear plastic wand] [wood-tone wand to match slats] [cord-operated tilter].

Retain subparagraph above or below.  If retaining below, coordinate with "Motorized Horizontal Louver Blind Operators" Article.

2. Tilt Operation:  Motorized operator.

Retain first subparagraph below or delete if one-third to one-half length of blind specified in WCSC standard is acceptable.

3. Length of Tilt Control:  [Full length of blind] [Length required to make operation convenient from floor level] [As indicated on Drawings].

4. Tilt:  [Full] <Insert limited tilt description>.

I. Lift Operation:  Manual, cord lock; locks pull cord to stop blind at any position in ascending or descending travel.

Retain paragraph above or below for manual operation.  If top-locking, cord-lock blinds are improperly operated, ring pull prevents blind from dropping and damaging itself or sill.  Revise positions or length below to suit Project.

J. Lift Operation:  Manual, top-locking cord lock; locks pull cord to stop blind in either fully opened or fully closed position only and is equipped with a ring pull not more than 4 inches (100 mm) long.

If retaining first paragraph below, coordinate with "Motorized Horizontal Louver Blind Operators" Article.

K. Lift Operation:  Motorized operator.

L. Tilt-Control and Cord-Lock Position:  As indicated [on Drawings] [in a window treatment schedule].

Retain paragraph above or below, or delete both if default left-side position for tilt control and right-side position for cord lock specified in WCSC standard is acceptable.  Verify availability of positions with manufacturers for each product.  Industry standard positions are referenced to operator hands as they face blind.

M. Tilt-Control and Cord-Lock Position:  [Right and left side of headrail, respectively] [Right side of headrail] [Left side of headrail], unless otherwise indicated.

Delete first paragraph and subparagraph below if not applicable.

N. Valance:  [Manufacturer's standard] <Insert description>.

1. Finish Color Characteristics:  [Match color, texture, pattern, and gloss of slats] [Color, texture, pattern, and gloss differing from slats as indicated by manufacturer's designations] [Color, texture, pattern, and gloss differing from slats, matching Architect's samples] [Color texture, pattern, and gloss differing from slats as selected by Architect from manufacturer's full range].

O. Mounting:  [Wall mounting] [Ceiling mounting] [End mounting] [Wall extension brackets] [As indicated], permitting easy removal and replacement without damaging blind or adjacent surfaces and finishes; with spacers and shims required for blind placement and alignment indicated.

1. Provide intermediate support brackets if end support spacing exceeds spacing recommended by manufacturer for weight and size of blind.

Retain, revise, or delete paragraph below as required for Project and product.  Hold-down brackets and pins prevent swaying caused by air movement and are used on doors so blind remains in place when door is used.  If retaining below but not all blinds need hold-down brackets and pins, indicate by designation in article title and copy and edit article content; or differentiate between blinds on Drawings or in a window treatment schedule.

P. Hold-Down Brackets and Hooks or Pins:  Manufacturer's standard, as indicated.

Delete paragraph below if color, texture, pattern, and gloss are specified with manufacturer's name and product in this Article.

Q. Colors, Textures, Patterns, and Gloss:  [Match Architect's samples] [As selected by Architect from manufacturer's full range] [As indicated in a window treatment schedule].

2.4 HORIZONTAL LOUVER BLIND FABRICATION

A. Concealed Components:  Noncorrodible or corrosion-resistant-coated materials.

1. Lift-and-Tilt Mechanisms:  With permanently lubricated moving parts.

B. Unit Sizes:  Obtain units fabricated in sizes to fill window and other openings as follows, measured at 74 deg F (23 deg C):

1. Blind Units Installed between (inside) Jambs:  Width equal to 1/4 inch (6 mm) per side or 1/2 inch (13 mm) total, plus or minus 1/8 inch (3.1 mm), less than jamb-to-jamb dimension of opening in which each blind is installed.  Length equal to 1/4 inch (6 mm), plus or minus 1/8 inch (3.1 mm), less than head-to-sill dimension of opening in which each blind is installed.

Delete subparagraph above or below if not applicable.  Revise above if heat-strengthened glass is not specified and if compliance with GANA recommendations in its "Glazing Manual" for clearances between top, bottom, and sides of opening and blind suits Project.  Coordinate requirements for perimeter clearances with distance between blinds and glass, tilt limits, glass type, and placement of heating/cooling air supplies to avoid heat buildup and possible damage to glass.

2. Blind Units Installed outside Jambs:  Width and length as indicated, with terminations between blinds of end-to-end installations at centerlines of mullion or other defined vertical separations between openings.

C. Installation Brackets:  Designed for easy removal and reinstallation of blind, for supporting headrail[, valance,] and operating hardware, and for hardware position and blind mounting method indicated.

D. Installation Fasteners:  No fewer than two fasteners per bracket, fabricated from metal noncorrosive to blind hardware and adjoining construction; type designed for securing to supporting substrate; and supporting blinds and accessories under conditions of normal use.

E. Color-Coated Finish:

1. Metal:  For components exposed to view, apply manufacturer's standard baked finish complying with manufacturer's written instructions for surface preparation including pretreatment, application, baking, and minimum dry film thickness.

Delete subparagraph below if no wood components.

2. Wood:  Apply manufacturer's standard [opaque] [transparent] factory-applied finish complying with manufacturer's written instructions for surface preparation, application, and minimum dry film thickness.

Revise paragraph below if color-coordinated components, hardware, and cords are not required for Project.

F. Component Color:  Provide rails, cords, ladders, and exposed-to-view metal[, wood,] and plastic matching or coordinating with slat color, unless otherwise indicated.

2.5 MOTORIZED HORIZONTAL LOUVER BLIND OPERATORS

Delete this Article if no motorized horizontal louver blind operators.  If more than one type of operator is required, insert designation in title above and indicate blind(s) that each operator type is required to operate by designation on Drawings or in a window treatment schedule.

See Editing Instruction No. 1 in the Evaluations for cautions about naming manufacturers and products.  Retain one of first three paragraphs and list of manufacturers below.  See Division 1 Section "Product Requirements."

A. Available Products:  Subject to compliance with requirements, products that may be incorporated into the Work include, but are not limited to, the following:

B. Products:  Subject to compliance with requirements, provide one of the following:

1. Am-Source International.

2. BTX Window Automation Inc.

3. Silent Gliss USA, Inc.

4. SM Automatic, Inc.

5. <Insert manufacturer's name.>

C. Basis-of-Design Product:  Subject to compliance with requirements, provide [the product indicated on Drawings] <Insert manufacturer's name; product name or designation> or a comparable product by one of the following:

D. General:  Provide factory-assembled blind operation systems designed for blind type, size, weight, construction, use, and operation frequency indicated, with [lift] [tilt] [lift-and-tilt] functions.  Provide operation systems of size and capacity and with features, characteristics, and accessories suitable for Project conditions and recommended by blind manufacturer, complete with electric motors and factory-prewired motor controls, remote-control stations, remote-control devices, power disconnect switches, enclosures protecting controls and all operating parts, headrail, and accessories required for reliable operation without malfunction.  Include wiring from motor controls to motors.  Coordinate operator wiring requirements and electrical characteristics with building electrical system.

E. Comply with NFPA 70.

Usually retain option in first paragraph below for systems with low-voltage components (low-voltage motors and switches) or digital control (group, individual/group, sensor, infrared, timer, and microprocessor controllers).

F. Control Equipment:  Comply with NEMA ICS 1, NEMA ICS 2, and NEMA ICS 6[ with NFPA 70, Class 2 control circuit, maximum 24-V ac or dc].

G. Electric Motors:  UL-approved or -recognized, totally enclosed, insulated motor, complying with NEMA MG 1, with thermal-overload protection and internal limit switches; sized by blind manufacturer to start and operate size and weight of blind considering service factor or Project's service conditions without exceeding nameplate ratings.

1. Service Factor:  According to NEMA MG 1, unless otherwise indicated.

2. Motor Characteristics:  Single phase, [24] [110] [220] <Insert number> V, 60 Hz.

3. Motor Mounting:  Within manufacturer's standard headrail enclosure.

H. Remote Controls:  Electric controls with NEMA ICS 6, Type 1 enclosure for [surface] [recessed or flush] [within headrail] <Insert type> mounting.  Provide the following devices for remote-control activation of blinds:

Retain at least 1 of 10 subparagraphs and associated subparagraph below or insert another.  More than one subparagraph may be retained, for example, if sensor system has switch or remote override or if infrared remote-control systems also include switch control.

1. Control Stations:  Keyed, [maintained] [momentary]-contact, three-position, switch-operated control station with open, close, and off functions.  Provide two keys per station.

Retain subparagraph below if lift or tilt operation controlled by a switch is required for Project.

2. Control Stations:  [Maintained] [Momentary]-contact, three-position, [toggle] [rocker]-style, wall-switch-operated control station with open, close, and center off functions.

Retain first subparagraph below if lift-and-tilt operation controlled by a switch is required for Project.

3. Control Stations:  Combination maintained-momentary-contact, five-position, rocker-style, wall-switch-operated control station with open, close, tilt forward, tilt backward, and center off functions.

4. Group Control Stations:  [Maintained] [Momentary]-contact, three-position, rocker-style, wall-switch-operated control station with open, close, and center off functions for single-switch group control.

5. Individual/Group Control Stations:  [Maintained] [Momentary]-contact, three-position, rocker-style, wall-switch-operated control station with open, close, and center off functions for individual and group control.

6. Sun Sensor Controls:  Adjustable system consisting of LEDs detecting sun intensity and responding by automatically adjusting blinds.

7. Infrared Controls:  System consisting of concealed receiver complete with external eye and connecting modular cable, and [two] <Insert number> portable, multiple-channel transmitters with separate buttons to open and close [up to 12] <Insert number> individual blinds or groups of blinds, to open and close all blinds simultaneously, [to tilt forward,] [to tilt backward,] and to stop.

8. Timer Controls:  Clock timer, [24-hour] [seven-day] <Insert time period> programmable for regular events.

9. Microprocessor Controls:  Electronic programmable means for setting, changing, and adjusting control features; isolated from voltage spikes and surges.

Retain one option in subparagraph below.

a. Color:  [Ivory] [White] [As indicated].

I. Limit Switches:  Adjustable switches, interlocked with motor controls and set to automatically stop blind at fully raised and fully lowered positions.

J. Operating Features:  Include the following:

1. Group switching with integrated switch control; single faceplate for multiple switch cut-outs.

2. Capable of interface with [audiovisual] [multiroom] <Insert description> control system.

3. Capable of accepting input from building automation control system.

Retain subparagraph below only for sensor-, infrared-, or timer-controlled systems.

4. Override switch.

K. Accessories:

1. Solar Power Unit:  For use with control system indicated.

2. <Insert accessory.>

PART 3 -  EXECUTION

3.1 EXAMINATION

A. Examine substrates, areas, and conditions, with Installer present, for compliance with requirements for installation tolerances, operational clearances, [accurate locations of connections to building electrical system, ]and other conditions affecting performance.

1. Proceed with installation only after unsatisfactory conditions have been corrected.

3.2 INSTALLATION

Coordinate requirements for distance between blinds and glass with clearances between blind perimeter and surrounding construction, tilt limits, glass type, and placement of heating/cooling air supplies to avoid heat buildup and possible damage to glass.  Retain 1-inch (25-mm) clearance only if tilt is limited; verify extent of tilt limit required with manufacturers.  Retain 2-inch (51-mm) clearance to comply with GANA requirements in its "Glazing Manual" for slats that can be tilted fully closed.

A. Install horizontal louver blinds level and plumb and aligned with adjacent units according to manufacturer's written instructions, and located so exterior slat edges in any position are not closer than [1 inch (25 mm)] [2 inches (51 mm)] <Insert dimension> to interior face of glass.  Install intermediate support as required to prevent deflection in headrail.  Allow clearances between adjacent blinds and for operating glazed opening's operation hardware if any.

Retain or revise one of four paragraphs below, or delete all if mounting is shown on Drawings.

B. Flush Mounted:  Install horizontal louver blinds with slat edges flush with finish face of opening if slats are tilted open.

C. Jamb Mounted:  Install headrail flush with face of opening jamb and head.

D. Head Mounted:  Install headrail on face of opening head.

E. Recessed:  Install headrail concealed within blind pocket.

Delete paragraph below if horizontal louver blinds are manually operated only.  See Evaluations for cautions about improper wiring.

F. Connections:  Connect motorized operators to building electrical system.

3.3 ADJUSTING

A. Adjust horizontal louver blinds to operate smoothly, easily, safely, and free of binding or malfunction throughout entire operational range.

3.4 CLEANING AND PROTECTION

A. Clean horizontal louver blind surfaces after installation, according to manufacturer's written instructions.

B. Provide final protection and maintain conditions, in a manner acceptable to manufacturer and Installer, that ensure that horizontal louver blinds are without damage or deterioration at time of Substantial Completion.

C. Replace damaged horizontal louver blinds that cannot be repaired, in a manner approved by Architect, before time of Substantial Completion.

3.5 DEMONSTRATION

Delete this Article if horizontal louver blinds are manually operated.

A. Engage a factory-authorized service representative to train Owner's maintenance personnel to adjust, operate, and maintain systems.  Refer to Division 1 Section "Demonstration and Training."

END OF SECTION 12491

HORIZONTAL LOUVER BLINDS
12491 - 1

