Copyright 2000 AIA
MASTERSPEC
11/00

NIH Version dated January 2002

This section uses the term Contracting Officer, Contracting Officer's Technical Representative and/or Project Officer in specific locations within this section based on specific authorities granted each individual under the terms of the contract.  Any changes in these terms at specific locations within these specifications must be coordinated with the Contracting Officer.

This section establishes the minimum safety requirements for a NIH construction project and shall be included in all NIH construction project specifications.

The construction contractor is responsible for the safety of the construction project and subject to the current laws and regulations that are in effect.  Including a safety requirement beyond that currently covered by this section and the applicable laws and regulations may subject the NIH to additional liability.  Prior to adding safety requirements to this section, design A&Es are to consult with the Project Officer.

This section includes detailed safety and health requirements and is closely related to Division 1 Section "Use, Handling, Storage, Transporting, Accumulation and Disposal of NIH Controlled Materials" (always included in Specifications) and Division 13 Section "Removal of Asbestos Materials" (may be included in Specifications).  These sections need to be closely coordinated to avoid including conflicting detailed requirements or conflicting references.  Detailed requirements should only be included in one of the sections with this section being the primary source for safety and health requirements.  Division 1 Section "Use, Handling, Storage, Transporting, Accumulation and Disposal of NIH Controlled Materials" is the primary source for hazardous material requirements.  Division 13 Section "Removal of Asbestos Materials" is the primary section for asbestos related information.

If the project does not require the removal of asbestos and if Division 13 Section "Removal of Asbestos Materials" is not included in the specifications, remove references to that division from various locations in this section.

SECTION 01595 - SAFETY AND HEALTH
PART 1 -  GENERAL

1.1 RELATED DOCUMENTS

A. Drawings and general provisions of the Contract, including General and Supplementary Conditions and other Division 1 Specification Sections, apply to this Section.

1.2 SUMMARY

A. References:  In addition to publications referenced in the Construction Contract Clauses, the following Code of Federal Regulations (CFR) publications designate and define hazardous materials and conditions, and establish procedures for handling these materials and conditions.  Omission of any publication in this section does not remove any obligation or legal requirement on the part of the contractor to comply with all legal requirements for the location of the work.

1. 29 CFR, Part 1910:  Occupational Safety and Health Administration (OSHA) General Industry and Health Standards.

2. 29 CFR, Part 1926:  OSHA Construction Industry Standards.

3. 40 CFR, Part 61:  National Emission Standards for Hazardous Air Pollutants.

4. 40 CFR, Part 261:  Environmental Protection Agency (EPA) Characteristics of Hazardous Waste.

5. 40 CFR, Part 761, EPA Polychlorinated Biphenyls (PCBs), Manufacturing, Processing, Distribution in Commerce and Use Prohibitions.

6. 40 CFR, Part 763:  EPA Asbestos.

7. Federal Standard 313A: Material Safety Data Sheets, Preparation and the Submission of.

8. NIH DES Instruction 1340-7: Precautions and Procedures for Entering Manholes or Other Below Grade Confined Spaces.

9. NIH DCAB publication "Standards for Temporary Construction," March 1988.

B. Related Sections:  This specification section is related to any and all specification sections with explicit or implicit reference to cutting and patching.  Specific submittal requirements of these related specification sections are not included in this section.  Related sections include but are not limited to the following specification sections:

Edit the following list to include pertinent specification sections included in the project.

1. Division 1 Section "Summary"

2. Division 1 Section "Work Restrictions"

3. Division 1 Section "Project Management and Coordination"

4. Division 1 Section "Construction Progress Documentation"

5. Division 1 Section "Photographic Documentation"

6. Division 1 Section "Submittal Procedures"

7. Division 1 Section "Quality Requirements"

8. Division 1 Section "References"

9. Division 1 Section "Construction Quality Control"

10. Division 1 Section "Temporary Facilities and Controls"

11. Division 1 Section "Use, Handling, Storage, Transporting, Accumulation and Disposal of NIH Controlled Material"

12. Division 1 Section "Temporary Traffic Controls"

13. Division 1 Section "Safety and Health"

14. Division 1 Section "Product Requirements"

15. Division 1 Section "Execution Requirements"

16. Division 1 Section "Cutting and Patching"

17. Division 1 Section "Selective Demolition"

18. Division 1 Section "Fire Precautions for Hot Work"

19. Division 1 Section "Closeout Procedures"

20. Division 1 Section "Operation and Maintenance Documentation"

21. Division 1 Section "Project Record Documents"

22. Division 1 Section "Demonstration and Training"

23. Division 13 Section "Removal of Asbestos Materials"

C. Hazardous Materials:  Some hazardous and toxic materials and substances are included in 29 CFR Part 1910, subparts H and Z, and in 29 CFR Part 1926 and others additionally defined in Federal Standard 313A.  Commonly encountered hazardous materials include but are not limited to asbestos, PCBs, explosives and radioactive material.

Edit the following paragraph to retain the second sentence with the reference to Division 13 Section "removal of Asbestos Materials " if section is used.  Otherwise delete the sentence with the reference but retain first sentence as an indicator that these materials might be encountered.

1. Asbestos may be found in spray-on fireproofing, insulation, boiler lagging, pipe coverings and other materials.  See Division 13 Section "Removal of Asbestos Materials" for removal requirements.

2. PCBs may be contained in transformers, capacitors, voltage regulators, oil switches, mechanical insulation and other materials.

D. Acquisition of Publications:  Referenced CFR publications may be purchased from the Superintendent of Documents, U.S. Government Printing Office, Washington, D.C. 20402.

1.3 SUBMITTALS

A. Contractor's Safety and Health Program:  The contractor shall submit a Site Specific Safety and Health Program which includes a Site Specific Safety and Health Plan and a copy of the Company Safety and Health Plan to the Contracting Officer within 14 calendar days of the Notice to Proceed or before work commences on the project site, whichever is earlier.

1. NIH will review the submitted Safety and Health Plans for general conformance to the requirements of the Contract Documents.

2. NIH will not make any judgment as to the adequacy of the submitted plans with respect to federal, state, and local safety laws and safety regulations.  If the plans are found to meet the requirements of the Contract Documents, NIH will acknowledge that the Contractor has provided copies of the plans and accept them as meeting the contract requirement for the Contractor to provide copies of his safety program.

3. NIH will not approve any contractor Safety and Health Plan.  Safety is the Contractor's responsibility under the current laws and governing regulation.

4. Submission of the Safety and Health Plans to NIH does not relieve the contractor of any safety responsibility.

5. Specific content requirements for the Contractor's Safety and Health Program are included in this section.

B. Hazardous Materials/Waste Management Plan:  The contractor will coordinate the Safety and Health Program with the Hazardous Material/Waste Management Plan required by Division 1 Section "Use, Handling, Storage, Transporting, Accumulation and Disposal of NIH Controlled Materials."  The Hazardous Material/Waste Management Plan is included in this section for reference only.

C. HAZMAT disposal Certificates: The contractor will provide copies of all permits and certificates for any hazardous materials disposed of off site as required by Division 1 Section "Use, Handling, Storage, Transporting, Accumulation and Disposal of NIH Controlled Materials."

D. Accident Reports.  The Contractor must submit to the Contracting Officer and Project Officer a written report within three calendar days of any accident, fire, emergency, theft or incident in which any personal or property damage took place, regardless of any other notifications performed.  Include a copy of each accident report that is submitted by the Contractor or Subcontractors to their insurance carriers, within seven calendar days after the date of the accident.

E. Gas Tests:  Copies of the Daily Log of gas tests shall be provided to the Contracting Officer.

Coordinate the following paragraph with similar language contained in Division 1 Section "Project Management and Controls."

F. Severe Storm Plan.  The Contractor shall develop and submit to the Contracting Officer within 14 calendar days of receipt of the notice to proceed a plan describing contractor procedures for responding to threats of Severe Weather.

1.4 PRECONSTRUCTION SAFETY MEETING

A. Prior to commencing construction, representatives of the Contractor, including the general superintendent and one or more safety representatives, shall meet with the Contracting Officer for the purpose of reviewing Contract safety and health requirements.

1. The Contractor's Safety and Health Program shall be reviewed, and implementation of safety and health provisions pertinent to the Work shall be discussed.

2. The Contractor shall be prepared to discuss, in detail, the Contractor's site specific Safety and Health Plan including measures intended to control any unsafe or unhealthy conditions associated with the work to be performed under the contract.

3. This meeting may be held in conjunction with the preconstruction conference, if so directed by the Contracting Officer.  The conduct of this meeting is not contingent upon a general preconstruction meeting.

4. The level of detail for the safety meeting is dependent upon the nature of the work and the potential inherent hazards.

5. The Contractor shall advise the Contracting Officer of any special safety restrictions he has established so that NIH personnel can be notified of these restrictions.

6. No later than 3 calendar days after the Preconstruction Safety Meeting, the Contractor shall distribute minutes of the meeting to each party present and to other concerned parties, including the Contracting Officer.

1.5 COMPLIANCE WITH REGULATIONS

Coordinate this article with requirements contained in Division 1 Section "Use, Handling, Storage, Transporting, Accumulation and Disposal of NIH Controlled Materials" and verify the same references and regulations are cited.  Avoid including detailed requirements in two Division 1 sections.

A. The work, including contact with or handling of hazardous materials, disturbance or dismantling of structures containing hazardous materials, and disposal of hazardous materials, shall comply with the applicable requirements of 29 CFR Parts 1910 and 1926, and 40 CFR Parts 61, 261, 761 and 763 and any other requirements contained in Division 1 Section "Use, Handling, Storage, Transporting, Accumulation and Disposal of NIH Controlled Materials."

Edit the following paragraph if the project does not contain asbestos removal and remove the reference to Division 13 Section "Removal of Asbestos Materials."

B. Work involving disturbance or dismantling of asbestos or asbestos containing materials, demolition of structures containing asbestos and removal of asbestos, shall comply with 40 CFR Part 61, Subparts A and M, and 40 CFR Part 763, and any other requirements contained in Division 1 Section "Removal of Asbestos Materials."

C. Work shall additionally comply with all applicable state and local safety and health regulations.

D. In case of a conflict between applicable regulations, the more stringent requirements shall apply.

E. Contractor Responsibility: The Contractor shall obtain all required permits for work to be performed.  The Contractor shall assume full responsibility and liability for compliance with all applicable codes, standards and regulations pertaining to the health and safety of personnel during execution of the Work, and shall hold the Government harmless for any action on the Contractor's part, or that of the Contractor's employees or subcontractors, that results in illness, injury or death.

1. The Contractor shall have written safety and health programs in compliance with 29 CFR Parts 1910 and 1926.

2. Permits, Inspections, Tests, and Reports:  The required permits inspections, tests, and reports made by the Contractor, subcontractors, specially trained technicians, equipment manufacturers, and others as required, shall be at the Contractor's expense.

1.6 USE OF EXPLOSIVES

A. Use of explosives shall be prohibited.  Explosive actuated tools (ex: nailguns, etc) shall not be used or brought to the project site without prior written approval from the Project Officer.  Such approval shall not relieve the Contractor of responsibility for injury to persons or for damage to property due to the use of such explosives.

1.7 WORK UNDERGROUND OR IN CONFINED SPACES

A. Work underground or in confined spaces shall comply with the NIH Division of Engineering Services Instruction Manual 1340-7, "Precautions and Procedures for Entering Manholes or Other Below Grade Confined Spaces" (DES INST 1340-7).  A copy of the instruction may be obtained from the Project Officer on request.

B. Work shall also comply with appropriate MOSHA and OSHA regulations including but not limited to 29CFR 1910.146 and COMAR 09.12.32B

C. The Contractor shall remove water and debris and properly vent manholes before commencement and during execution of work in manholes.

1.8 ELECTRICAL

A. The Contractor shall appoint an individual responsible for the electrical safety of each work team to restrict entry to dangerous locations to those authorized by him jointly with NIH.

B. Electrical arc welding equipment shall not be connected to the building power supply.

1.9 GAS PROTECTION

A. The Contractor shall have one or more employees properly trained in operation of gas testing equipment and formally qualified as gas inspectors who shall be on duty during times workmen are in confined spaces.  Their primary functions shall be to test for gas and operate testing equipment.  Unless equipment of constant supervisory type with automatic alarm is employed, gas tests shall be made at least every 2 hours or more often when character of ground or experience indicates gas may be encountered.  A gas test shall be made before workmen are permitted to enter the excavation after an idle period exceeding one-half hour.

B. Readings shall be permanently recorded daily, indicating the concentration of gas, number and location of drilled piers, point of test, date, and time of test.

C. Special requirements, coordination, and precautions will apply to areas that contain a hazardous atmosphere or, by virtue of their use or physical character, may be oxygen deficient.  A check by NIH is required prior to entering areas that contain hazardous or oxygen-deficient atmospheres.  Surveillance and monitoring shall be required in these types of work spaces by both Contractor and Government personnel.

1.10 SEVERE STORM PLAN

A. The Contractor shall develop and maintain a Severe Storm Plan and submit a copy to the Contracting Officer for information.  The Contracting Officer will review the plan for general content and for the adequacy of the contractor's provisions to protect NIH property entrusted to the contractor as part of the project.  The Contracting Officer reserves the right to require improvements to the plan for the protection of NIH personnel and property.  The Contracting Officer assumes no liability for damage to Contractor Equipment or work not turned over to NIH which might be damaged by severe weather, regardless of the effectiveness of the contractor's Severe Storm Plan.

B. In the event that the National Weather Service or the Project Officer issues a severe storm warning for the Bethesda, MD area, the Contractor shall:

1. Secure outside equipment and materials and place materials susceptible to damage in protected locations.

2. Check surrounding area, including roof, for loose material, equipment, debris, and other objects that could be blown away or against existing facilities.

3. Ensure that temporary erosion controls are adequate.

4. Provide the Project Officer with emergency contact information and provide for rapid response of contractor forces to correct project related damage during and immediately after a storm warning period.

C. The Contractor is responsible for completing routine severe storm preparations prior to vacating the project site at the close of each business day regardless of the weather forecast or active severe storm warnings.

1.11 MATERIAL DELIVERIES

A. Whenever practicable, deliveries shall be made during regular NIH working hours and only when the Contractor's representative is available to receive them.  Deliver material in approved containers and with properly licensed vehicles and operators.  Open delivery vehicles are not permitted.  Deliver materials in fully closed vehicles or tarp covered vehicles.  All dump trucks shall be fully covered while in transport to and from the unloading site.  All loads shall be securely fastened until unloading.  Engines shall not be left running while vehicles are loading, unloading, waiting or parked.  Do not block roads, walks, building entrances/exits, fire hydrants and standpipes, exterior tanks or building gas connections.

Coordinate the following article with the requirements contained in Division 1 Section "Use, Handling, Storage, Transporting, Accumulation and Disposal of NIH Controlled Materials" and verify the same references and regulations are cited.  Avoid including detailed requirements in two Division 1 sections.

1.12 HAZARDOUS MATERIALS

A. The Contractor shall bring to the attention of the Contracting Officer, or the Contracting Officer's authorized representative, any material encountered during execution of the Work that the Contractor suspects is hazardous.  The Contracting Officer shall determine whether the Contractor shall perform tests to determine if the material is hazardous.  If the suspected material is leaking or spilling from its location the contractor is to contact the NIH Fire Department immediately.

B. If the Contracting Officer directs the Contractor to perform tests on suspected hazardous materials and the material is found to be hazardous, or if the material is found to be hazardous without Contractor testing, and/or if additional protective measures are required, a change to the Contract price may be provided, subject to the applicable provisions of the Contract.

1.13 ADDITIONAL NIH SAFETY REQUIREMENTS

In order to include the following paragraph, the designer should have a basic understanding of the applicable regulations and codes.  Coordinate with the Project Officer for copies of pertinent documents and points of contact at NIH offices which may provide amplifying information.  The designer should keep records of conversations with NIH offices regarding NIH Safety.

Consider including sub paragraphs to the following paragraph listing the appropriate codes and regulations not listed elsewhere in this section.

A. The Contractor shall comply with all established NIH Standards, Codes and Regulations and obtain appropriate approvals from the NIH Divisions of Safety, Health, Security and Fire Protection.

B. No work shall be performed in any area occupied by the public or NIH employees unless approved by the Project Officer.

C. In the event of an emergency in a construction site that has been secured with a chain, the NIH Fire Department will cut the chain to gain entry. The Contractor shall be responsible for a new chain.

D. Accident Treatment and Records: The Contractor shall post emergency first aid information and NIH emergency Telephone Numbers at the work site.

E. NIH Emergency Telephone Numbers: Emergency response for medical and other emergencies will be provided by the Government.  The Division of Public Safety has implemented 9-1-1 service on the NIH Campus.  Calls requesting the assistance of the NIH Police or Fire Department (fires, ambulance, rescue or hazardous materials incidents) are received at a state of the art Emergency Communications Center where trained and certified Emergency Services Dispatchers ensure that the appropriate NIH Police and/or Fire Department personnel and equipment are promptly dispatched.  Contractors shall instruct their employees to immediately report any fire even if the fire has been extinguished as well as any hazardous material spill, ambulance or rescue emergency.

Confirm with the Project Officer that these are still the correct emergency numbers.

	ON-CAMPUS Emergency (Police/Fire/Rescue)
	Dial 9-1-1

	TTY Telephone Line
	496-0063

	NIH Police Non-Emergency
	496-5685

	NIH Fire Department Non-Emergency
	496-2372

	NIH Fire Prevention Section
	496-0487

	NIH Occupational Safety and Health Branch
	496-2960

	OFF-CAMPUS Emergency POLICE & FIRE/RESCUE
	Dial 9-9-1-1

	(For off-campus locations when using a telephone with an NIH exchange, e.g. 496, 402,435, 480 or 594)
	


F. Safety Clearance Procedures (Lockout/Tagout Tag System).  The following Lockout/Tagout procedures will be followed unless more stringent procedures are required by current laws and regulations.  The following procedures do not relieve the contractor from any more stringent requirements of the applicable codes and regulations.  The safety of contractor and NIH personnel in areas impacted by the project remains the contractor's responsibility during construction.

1. Lockout/Tagout procedures shall be in accordance with OSHA 29 CFR 1910 and OSHA 29 CFR 1926.  Lockout/Tagout procedures shall be included in the Contractor's Safety and Health Plan.

2. Contractor shall ensure that each employee is familiar with and complies with these procedures.

3. The Project Officer will, at the Contractor's request, apply lockouttagout tags and take other actions that, because of experience and knowledge, are known to be necessary to make the particular NIH equipment safe to work on.

4. No person, regardless of position or authority, shall operate any switch, valve, or equipment that has an official lockout/tagout tag attached to it, nor shall such tag be removed except as provided in this section.

5. No person shall work on any equipment that requires a lockout/tagout tag unless he, his immediate supervisor, project leader, or a subordinate has in his possession the keys to the required lockout/tagout tags.

6. When work is to be performed on electrical circuits, the work shall be performed only by qualified personnel following the required safety procedures.

7. A Contractor's supervisor who is required to enter an area protected by a lockout/tagout tag will be considered a member of the protected group provided he notifies the holder of the tag stub each time he enters and departs from the protected area.

8. Identification markings on building light and power distribution circuit breakers shall not be relied on for establishing safe work conditions.

9. Before clearance will be given on any equipment other than electrical (generally referred to as mechanical apparatus), the apparatus, valves, or systems shall be secured in a passive condition with the appropriate vents, pins, and locks.

10. Pressurized or vacuum systems shall be vented to relieve differential pressure completely.

11. Vent valves shall be lockout/tagout tagged open during the course of the work.

12. Where dangerous gas or fluid systems are involved, or in areas where the environment may be oxygen deficient, system or areas shall be purged, ventilated, or otherwise made safe prior to entry.  See paragraph "Gas Protection."

13. Tag Placement

a. Lockout/tagout tags shall be completed in accordance with the regulations printed on the back thereof and attached to any device which, if operated, could cause an unsafe condition to exist.

b. If more than one group is to work on any circuit or equipment, the employee in charge of each group shall have a separate set of lockout/tagout tags completed and properly attached and locked.

c. When it is required that certain NIH owned and operated equipment be tagged, the NIH will review the characteristics of the various systems involved that affect the safety of the operations and the work to be done; take the necessary actions, including voltage and pressure checks, grounding, and venting, to make the system and equipment safe to work on; and apply such lockout/tagout tags to those switches, valves, vents, or other mechanical devices needed to preserve the safety provided.  This operation is referred to as "Providing Safety Clearance."

14. Tag Removal

a. Lockout/tagout tags shall be removed only by those persons who initiated the lockout/tagout tag and who retain possession of the keys.  Otherwise, lockout/tagout tags may be removed only with the authorization of the Project Officer.

Add additional subparagraphs for any operations with atypical safety considerations that are part of this project or additional specific safety requirements of NIH.  Consult with NIH Safety Office for any unique requirements.  Examples might be radiation hazards if working in a radiation zone, high heat, biohazard etc.  Additions should not attempt to define the safety requirements if the requirements are addressed in the OSHA safety regulations.  If OSHA does not cover the know hazards then consideration should be given to requiring the contractor to provide a safety consultant.  Coordinate any requirement for a Safety Consultant with Division 1 Section "Project Management and Coordination."

1.14 PERSONNEL PROTECTIVE EQUIPMENT

A. Special facilities, devices, equipment and similar items used by the Contractor in execution of the work shall comply with 29 CFR, Part 1910, Subpart I and other applicable regulations.

PART 2 -  PRODUCTS

2.1 Safety and Health Programs:  The Contractor shall submit copies of the written site specific project safety and health plan and emergency action procedures, as applicable to the work scope, as required as a result of the safety meeting, or as required by OSHA 29 CFR, Part 1926 including but not necessarily limited to the procedures and programs that support the requirements of the following:

A. Designation of Safety Competent Person

B. Occupational Noise Exposure.

C. Fall Protection.

D. Personnel Protective Equipment.

E. Control of Hazardous Energy.

F. Hazardous Materials Waste Management Plan (draft if final plan has not been accepted)

G. Electrical Safety Related Work Practices.

H. Lead.

I. Asbestos.

J. Respirator Protection.

K. Confined spaces.

L. Emergency evacuation and reporting

2.2 Contractor's Safety and Health Plan:  In addition to specific safety and health programs applicable to the project, Contractor shall submit to the Project Officer a copy of the firm's general Safety and Health Plan listing emergency procedures and contact persons with home addresses and telephone numbers.

Coordinate language in the following paragraph with similar language contained in Division 1 Section "Use, Handling, Storage, Transporting, Accumulation and Disposal of NIH Controlled Materials."

2.3 Permits:  If hazardous materials are disposed of off-site, submit copies of shipping manifests and permits from applicable federal, state or local authorities and disposal facilities, and submit certificates that the material has been disposed of in accordance with regulations as required in Division 1 Section "Use, Handling, Storage, Transporting, Accumulation and Disposal of NIH Controlled Material".

2.4 Accident Reporting:  The Contractor must submit to the Contracting Officer a written report within three calendar days of any accident, fire, emergency, theft or incident in which any personal or property damage took place, regardless of any other notifications performed.  Include a copy of each accident report that is submitted by the Contractor or Subcontractors to their insurance carriers, within seven calendar days after the date of the accident.

A. Gas Test Reports:  Submit copies of daily log of gas tests

PART 3 -  EXECUTION

3.1 EMERGENCY SUSPENSION OF WORK

A. When the Contractor is notified by the Contracting Officer, or the Project Officer, of non-compliance with the safety or health provisions of the Contract, the Contractor shall immediately, unless otherwise instructed, correct the unsafe or unhealthy condition.

1. If the Contractor fails to comply promptly, all or part of the work will be stopped by notice from the Contracting Officer.

2. When, in the opinion of and by notice given by the Contracting Officer, satisfactory corrective action has been taken by the Contractor, work shall resume.

3. The Contractor shall not be allowed any extension of time or compensation for damages in connection with a work stoppage for an unsafe or unhealthy condition.

3.2 PROTECTION OF PERSONNEL

A. The Contractor shall take all necessary precautions to prevent injury to the public, occupants, or damage to property of others.  The public and occupants includes all persons not employed by the Contractor or a subcontractor.

B. Wherever practical, the work area shall be fenced, barricaded or otherwise blocked off from the public or occupants to prevent unauthorized entry into the work area.

1. Provide traffic barricades and traffic control signage where construction activities occur in vehicular areas.

2. Corridors, aisles, stairways, doors and exit ways shall not be obstructed or used in a manner to encroach upon routes of ingress or egress utilized by the public or occupants, or to present an unsafe or unhealthy condition to the public or occupants.

3. Store, position and use equipment, tools, materials, scraps and trash in a manner that does not present a hazard to the public or occupants by accidental shifting, ignition or other hazardous activity.

4. Store and transport refuse and debris in a manner to prevent unsafe and unhealthy conditions for the public and occupants.  Cover refuse containers, and remove refuse on a frequent regular basis acceptable to the Contracting Officer.  Use tarpaulins or other means to prevent loose transported materials from dropping from trucks.

C. Alternate Precautions: When the nature of the work prevents isolation of the work area and the public or building occupants may be in or pass through, under or over the work area, alternate precautions such as the posting of signs, the use of signal persons, the erection of barricades or similar protection around particularly hazardous operations shall be used as appropriate.

D. Public Thoroughfare: When work is to be performed over a public thoroughfare such as a sidewalk, roadway or other site access way, the thoroughfare shall be closed, if possi-ble, or other precautions taken such as the installa-tion of screens or barricades. When the exposure to heavy falling objects exists, as during the erection of building walls or during demolition, special protection of the type detailed in 29 CFR, Parts 1910 and 1926 shall be provided.

3.3 ENVIRONMENTAL PROTECTION

Coordinate language in the following paragraph with similar language contained in Division 1 Section "Use, Handling, Storage, Transporting, Accumulation and Disposal of NIH Controlled Materials."

A. Dispose of solid, liquid and gaseous contaminants in accordance with local codes, laws, ordinances and regulations.

B. Comply with applicable federal, state and local noise control laws, ordinances and regulations, including but not limited to 29 CFR, Part 1910.95 and 29 CFR, Part 1926.52.

Coordinate the following article with the requirements contained in Division 1 Section "Use, Handling, Storage, Transporting, Accumulation and Disposal of NIH Controlled Materials" and verify the same references and regulations are cited.  Avoid including detailed requirements in two Division 1 sections.

3.4 HAZARDOUS MATERIALS

A. Requirements for hazardous Materials are contained in Division 1 Section "Use, Handling, Storage, Transporting, Accumulation and Disposal of NIH Controlled Materials."

END OF SECTION 01595

SAFETY AND HEALTH
01595 - 12

