Draw a 1-A3 standard web joint connection using 3/4" diameter, 2 L's 4 x 3 1/2 x 5 / 16 x 8 1/2. 

Top of beams are flush with +56'-8" elevation. 

Cope cut is to be made on the W14 x 30 beam. 

The minimum cope on the W14 x 30 beam should be equal to the "k" distance of the W24 x 76 beam. 

Measuring the cut from the backs of the connecting angles, the minimum cut is equal to 1/2" plus the "a" distance of the W24 x 76 beam. 

If the fractional part of the cut dimension is not in fourths of an inch, it is customary to raise the fractional part of the cut dimension to the next highest 1/4". 

Show all detailing dimensions

Layout the drawing to an appropriate scale on a size A protrait sheet with a titile block that includes the title, sheet number 1, date, draftsmen name, and scale.

File Name: Lastname_Steel

