Louisiana Army National Guard

Preliminary Review Documents

141st Readiness Center (RC)

Jackson Barracks, New Orleans, Louisiana

Technical Specifications

SECTION 11458 – DISAPPEARING STAIRWAYS 
PART 1 -  GENERAL

1.1 SUMMARY
A. This Section includes the following:
1. One (1) fire-rated disappearing stairway.
1.2 SUBMITTALS

A. Product Data:  For each type of product specified.

B. Shop Drawings:  Show the following:

1. Showing fabrication and installation, including fully dimensioned plans, elevations, sections, details of components and attachments to other units of Work.

2. Show layout, anchorage details, rough-in requirements.
PART 2 -  PRODUCTS
2.1 MANUFACTURERS
A. Available Manufacturers/Products:  Subject to compliance with requirements, manufacturers/ products that may be incorporated in the Work include, but are not limited to, the following:

1. Precision Stair Corp.; Super Simplex Disappearing Stairway.
2.2 DISAPPEARING STAIRWAY

A. General:  Fabricate unit to withstand and support 500 lbs.  Construct unit with a minimum 1/8” thick (formed channel) steel box; 6” deep with a one-hour rated fire door.

B. Risers:  Maximum 9-1/2-inches (240 mm) height.

C. Treads:  Skid serrated (rubber on aluminum).

D. Hardware:  Cadmium plated.

E. Size:  30-inches (760 mm) x 54-inches (1370 mm).
F. Fire Door:  1-3/4-inches (32 mm) ‘B’ label recessed into frame flush with bottom lip.

G. Door Hinge:  Steel piano.

H. Side Runner:  5-inches (127 mm) aluminum.

I. Operation:  Pole operated.
PART 3 -  EXECUTION
3.1 INSTALLATION
A. General:  Comply with manufacturer’s instructions and recommendations.  

B. Anchor units securely to supporting structural substrates, adequate to withstand specified loading.

3.2 CLEANING AND PROTECTION

A. Clean exposed metal and other surfaces according to manufacturer’s instructions.  Touch-up damaged metal coatings.
END OF SECTION 11458

DISAPPEARING STAIRWAYS

11458 - 2

