Copyright 2006 AIA
MASTERSPEC Full Length
05/06

Copyright 2006 by The American Institute of Architects (AIA)

Exclusively published and distributed by Architectural Computer Services, Inc. (ARCOM) for the AIA

SECTION 066400 - PLASTIC PANELING
Revise this Section by deleting and inserting text to meet Project-specific requirements.

This Section uses the term "Architect."  Change this term to match that used to identify the design professional as defined in the General and Supplementary Conditions.

Verify that Section titles referenced in this Section are correct for this Project's Specifications; Section titles may have changed.

PART 1 -  GENERAL

1.1 RELATED DOCUMENTS

Retain or delete this article in all Sections of Project Manual.

A. Drawings and general provisions of the Contract, including General and Supplementary Conditions and Division 01 Specification Sections, apply to this Section.

1.2 SUMMARY

A. Section includes glass-fiber reinforced plastic (FRP) wall paneling[ and trim accessories].

B. Related Sections:

Retain Sections in subparagraphs below that contain requirements Contractor might expect to find in this Section but are specified in other Sections.

Retain first subparagraph below if plastic paneling is installed directly on wood furring.

1. Division 06 Section "Rough Carpentry" for wood furring for installing plastic paneling.

2. Division 10 Section "Wall and Door Protection" for corner guards installed over plastic paneling.

1.3 SUBMITTALS

Paragraphs below are defined in Division 01 Section "Submittal Procedures" as "Action Submittals."

A. Product Data:  For each type of product indicated.

B. LEED Submittals:

Retain first subparagraph below if low-emitting materials are required for LEED-NC or LEED-CI Credit EQ 4.1; coordinate with requirements selected in Part 2 for adhesives and sealants.

1. Product Data for Credit EQ 4.1:  For [adhesives] [and] [sealants], including printed statement of VOC content and chemical components.

Retain subparagraph below if factory-laminated panels are used and low-emitting materials are required for LEED-NC or LEED-CI Credit EQ 4.4.

2. Product Data for Credit EQ 4.4:  For laminating adhesive [and composite wood products] used in factory-laminated plastic panels, indicating that product contains no urea formaldehyde.

C. Samples for Initial Selection:  For plastic paneling[ and trim accessories].

Delete "Samples for Initial Selection" Paragraph above if colors and other characteristics are preselected and specified or scheduled.  Retain paragraph below with or without above.

D. Samples for Verification:  For plastic paneling[ and trim accessories], in manufacturer's standard sizes.

1.4 QUALITY ASSURANCE

A. Source Limitations:  Obtain plastic paneling[ and trim accessories] from single manufacturer.

Products with flame-spread index of 25 or less are readily available and are usually designated Class A.  Most other products have flame-spread index of 200 or less and are designated Class C.

B. Surface-Burning Characteristics:  As determined by testing identical products according to ASTM E 84 by a qualified testing agency.  Identify products with appropriate markings of applicable testing agency.

1. Flame-Spread Index:  [25] [200] <Insert value> or less.

2. Smoke-Developed Index:  [450] <Insert value> or less.

Retain second option in subparagraph below if FMG approval is required for insurance purposes.

3. Testing Agency:  [Acceptable to authorities having jurisdiction] [FM Approvals] [UL].

1.5 PROJECT CONDITIONS

A. Environmental Limitations:  Do not deliver or install plastic paneling until spaces are enclosed and weathertight and temporary HVAC system is operating and maintaining ambient temperature and humidity conditions at occupancy levels during the remainder of the construction period.

PART 2 -  PRODUCTS

2.1 PLASTIC SHEET PANELING

A. General:  Gelcoat-finished, glass-fiber reinforced plastic panels complying with ASTM D 5319.

See Editing Instruction No. 1 in the Evaluations for cautions about naming manufacturers.  Retain one of first two subparagraphs and list of manufacturers below.  See Division 01 Section "Product Requirements."

1. Manufacturers:  Subject to compliance with requirements, [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

2. Basis-of-Design Product:  Subject to compliance with requirements, provide [product indicated on Drawings] <Insert manufacturer's name; product name or designation> or comparable product by one of the following:

a. Kemlite Company Inc.

b. Marlite.

c. Nudo Products, Inc.

d. <Insert manufacturer's name>.

Verify availability with manufacturers selected before retaining a thickness in first subparagraph below.  3/32 inch is slightly greater than 0.09 inch (2.3 mm).

3. Nominal Thickness:  Not less than [0.075 inch (1.9 mm)] [0.09 inch (2.3 mm)] [0.12 inch (3.0 mm)].

4. Surface Finish:  [Smooth] [Molded pebble texture] [Smooth surface with filled grooves at 4 inches (102 mm) o.c. to resemble tile] [As indicated by manufacturer's designations] [Match Architect's sample] [As selected by Architect from manufacturer's full range].

5. Color:  [White] [As indicated by manufacturer's designations] [Match Architect's sample] [As selected by Architect from manufacturer's full range].

2.2 FACTORY-LAMINATED PLASTIC PANELS

A. General:  Gelcoat-finished, glass-fiber reinforced plastic panels complying with ASTM D 5319, laminated to [plywood] [oriented strand board] [fire-retardant particleboard] [gypsum board] [high-impact gypsum board] [moisture- and mold-resistant gypsum board].

See Editing Instruction No. 1 in the Evaluations for cautions about naming manufacturers.  Retain one of first two subparagraphs and list of manufacturers below.  See Division 01 Section "Product Requirements."

1. Manufacturers:  Subject to compliance with requirements, [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

2. Basis-of-Design Product:  Subject to compliance with requirements, provide [product indicated on Drawings] <Insert manufacturer's name; product name or designation> or comparable product by one of the following:

a. Kemlite Company Inc.

b. Nudo Products, Inc.

c. <Insert manufacturer's name>.

Verify availability with manufacturers selected before retaining a thickness in first subparagraph below.  3/32 inch is slightly greater than 0.09 inch (2.3 mm).

3. Glass-Fiber Reinforced Plastic Panel Nominal Thickness:  Not less than [0.03 inch (0.76 mm)] [0.05 inch (1.3 mm)] [0.075 inch (1.9 mm)] [0.09 inch (2.3 mm)].

4. Surface Finish:  [Smooth] [Molded pebble texture] [Smooth surface with filled grooves at 4 inches (102 mm) o.c. to resemble tile] [As indicated by manufacturer's designations] [Match Architect's sample] [As selected by Architect from manufacturer's full range].

5. Color:  [White] [As indicated by manufacturer's designations] [Match Architect's sample] [As selected by Architect from manufacturer's full range].

6. Plywood:  DOC PS 1, Exterior B-C, [1/4 inch (6.4 mm)] [3/8 inch (9.5 mm)] [1/2 inch (12.7 mm)] [5/8 inch (15.9 mm)] [3/4 inch (19.1 mm)] thick.

7. Oriented Strand Board:  DOC PS 2, [1/4 inch (6.4 mm)] [3/8 inch (9.5 mm)] [1/2 inch (12.7 mm)] [3/4 inch (19.1 mm)] thick.

Particleboard described in first subparagraph below contains urea formaldehyde.  Delete if low-emitting materials are required for LEED-NC or LEED-CI Credit EQ 4.4.  It complies with requirements for Class I (Class A) finish but not requirements of model codes for use where noncombustible materials are required.

8. Fire-Retardant Particleboard:  Product complying with ANSI A208.1, Grade M-S, except for modulus of rupture; with flame-spread index of 25 or less per ASTM E 84; and [3/8 inch (9.5 mm)] [1/2 inch (12.7 mm)] thick.

9. Gypsum Board:  ASTM C 1396/C 1396M, [Regular, 1/2 inch (12.7 mm)] [Type X, 5/8 inch (15.9 mm)].

First subparagraph below specifies National Gypsum's "High-Impact Brand Wallboard" gypsum board.

10. High-Impact Gypsum Board:  ASTM C 1396/C 1396M, 5/8 inch (15.9 mm), with Type X core, and [0.010-inch (0.254-mm)] [0.020-inch (0.508-mm)] [0.030-inch (0.762-mm)] [0.081-inch (2.057-mm)] plastic film laminated to back side for greater resistance to through penetration (impact resistance).

Examples of moisture- and mold-resistant panels include USG's "SHEETROCK Brand HUMITEK" panels and National Gypsum's "XP Wallboard," which are both paper faced; and Georgia-Pacific's "DensArmor Interior Guard" panels, which have coated glass-mat facings.

11. Moisture- and Mold-Resistant Gypsum Board:  ASTM C 1396/C 1396M or ASTM C 1178/C 1178M, 5/8 inch (15.9 mm), Type X, with moisture- and mold-resistant core and surfaces.

Retain subparagraph below if low-emitting materials are required for LEED-NC or LEED-CI Credit EQ 4.4.

12. Laminating Adhesive:  Manufacturers standard adhesive that does not contain urea formaldehyde.

2.3 ACCESSORIES

Retain one of first two paragraphs below if using trim.  Delete "outside corners" option in either paragraph if using corner guards specified in Division 10 Section "Wall and Door Protection" instead of using outside corner trim.  Product in first paragraph is available only for plastic sheet paneling and factory-laminated panels with 3/8-inch- (9.5-mm-) thick backing.

A. Trim Accessories:  Manufacturer's standard one-piece vinyl extrusions designed to retain and cover edges of panels.  Provide division bars, inside corners,[ outside corners,] and caps as needed to conceal edges.

1. Color:  [White] [Match panels] [As indicated by manufacturer's designations] [Match Architect's sample] [As selected by Architect from manufacturer's full range].

B. Trim Accessories:  Manufacturer's standard two-piece, snap-on vinyl extrusions designed to cover edges of panels.  Provide division bars, inside corners,[ outside corners,] and caps as needed to conceal edges.

1. Color:  [White] [Match panels] [As indicated by manufacturer's designations] [Match Architect's sample] [As selected by Architect from manufacturer's full range].

C. Exposed Fasteners:  Nylon drive rivets recommended by panel manufacturer.

D. Concealed Mounting Splines:  Continuous, H-shaped aluminum extrusions designed to fit into grooves routed in edges of factory-laminated panels and to be fastened to substrate.

E. Adhesive:  As recommended by plastic paneling manufacturer.

Retain subparagraph below if low-emitting materials are required for LEED-NC or LEED-CI Credit EQ 4.1.  VOC content limit is that for gypsum board and panel adhesive.

1. VOC Content:  [50] <Insert limit> g/L or less when calculated according to 40 CFR 59, Subpart D (EPA Method 24).

Manufacturers recommend the use of sealant with trim accessories in high-moisture or wet areas.  Sealant can also be used with factory-laminated panels without trim in areas where moisture and wetting are not a concern.

F. Sealant:  [Single-component, mildew-resistant, neutral-curing silicone] [Single-component, mildew-resistant, acid-curing silicone] [Latex] sealant recommended by plastic paneling manufacturer and complying with requirements in Division 07 Section "Joint Sealants."

Retain subparagraph below if low-emitting materials are required for LEED-NC or LEED-CI Credit EQ 4.1.

1. VOC Content:  [250] <Insert limit> g/L or less when calculated according to 40 CFR 59, Subpart D (EPA Method 24).

PART 3 -  EXECUTION

3.1 EXAMINATION

A. Examine substrates and conditions, with Installer present, for compliance with requirements for installation tolerances and other conditions affecting performance of the Work.

B. Proceed with installation only after unsatisfactory conditions have been corrected.

3.2 PREPARATION

Retain any or all of first three paragraphs below for adhesive application; first two are primarily for renovation work.

A. Remove wallpaper, vinyl wall covering, loose or soluble paint, and other materials that might interfere with adhesive bond.

B. Prepare substrate by sanding high spots and filling low spots as needed to provide flat, even surface for panel installation.

C. Clean substrates of substances that could impair bond of adhesive, including oil, grease, dirt, and dust.

Manufacturers' recommendations for conditioning vary.  Marlite recommends 48 hours; Kemlite and Nudo recommend 24 hours.

D. Condition panels by unpacking and placing in installation space before installation according to manufacturer's written recommendations.

E. Lay out paneling before installing.  Locate panel joints [where indicated] [to provide equal panels at ends of walls not less than half the width of full panels] [so that trimmed panels at corners are not less than 12 inches (300 mm) wide].

1. Mark plumb lines on substrate at [trim accessory] [panel joint] locations for accurate installation.

2. Locate [trim accessories] [panel joints] to allow clearance at panel edges according to manufacturer's written instructions.

3.3 INSTALLATION

A. Install plastic paneling according to manufacturer's written instructions.

Retain one of first two paragraphs below to suit installation method required for substrate.

B. Install panels in a full spread of adhesive.

C. Install panels with fasteners.  Layout fastener locations and mark on face of panels so that fasteners are accurately aligned.

1. Drill oversized fastener holes in panels and center fasteners in holes.

2. Apply sealant to fastener holes before installing fasteners.

Retain first paragraph below in addition to one of last two paragraphs above if required for factory-laminated panels.

D. Install factory-laminated panels using concealed mounting splines in panel joints.

E. Install trim accessories with [adhesive] [and] [nails] [or] [staples].[  Do not fasten through panels.]

Retain first paragraph below if using sealant with trim accessories.  Manufacturers recommend the use of sealant in high-moisture or wet areas.

F. Fill grooves in trim accessories with sealant before installing panels and bed inside corner trim in a bead of sealant.

Usually retain first paragraph below whether trim accessories are used or not.  Delete if not applicable.

G. Maintain uniform space between panels and wall fixtures.  Fill space with sealant.

Retain first paragraph below if installing panels without trim accessories.

H. Maintain uniform space between adjacent panels and between panels and floors, ceilings, and fixtures.  Fill space with sealant.

I. Remove excess sealant and smears as paneling is installed.  Clean with solvent recommended by sealant manufacturer and then wipe with clean dry cloths until no residue remains.

END OF SECTION 066400

	PLASTIC PANELING
	066400 - 1



