4.13
WIRING DEVICES
4.13.1
Light Switches

Wall switches for lighting circuits shall have the following features and ratings:


SYMBOL 183 \f "Symbol" \s 10 \h
20 A


SYMBOL 183 \f "Symbol" \s 10 \h
120/277 V


SYMBOL 183 \f "Symbol" \s 10 \h
Toggle type


SYMBOL 183 \f "Symbol" \s 10 \h
Brown handle


Light switches shall be Hubbell #1221.  Switch plates for factory areas shall be stainless steel; plastic switch plates are acceptable in office areas only.

4.13.2
Receptacles
4.13.2.1
Office Convenience Receptacles


Convenience receptacles for office areas shall have the following features and ratings:


SYMBOL 183 \f "Symbol" \s 10 \h
15 A


SYMBOL 183 \f "Symbol" \s 10 \h
125 V


SYMBOL 183 \f "Symbol" \s 10 \h
2 pole


SYMBOL 183 \f "Symbol" \s 10 \h
3 wire


SYMBOL 183 \f "Symbol" \s 10 \h
Straight blade


SYMBOL 183 \f "Symbol" \s 10 \h
Grounding type


SYMBOL 183 \f "Symbol" \s 10 \h
Duplex


SYMBOL 183 \f "Symbol" \s 10 \h
NEMA 5-15R configuration


Office convenience receptacles shall be Hubbell #5262.  Plastic cover plates are acceptable for office areas only.

4.13.2.2
Factory Convenience Receptacles


Convenience receptacles for factory areas shall have the following features and ratings:


SYMBOL 183 \f "Symbol" \s 10 \h
20 A


SYMBOL 183 \f "Symbol" \s 10 \h
125 V


SYMBOL 183 \f "Symbol" \s 10 \h
2 pole


SYMBOL 183 \f "Symbol" \s 10 \h
3 wire


SYMBOL 183 \f "Symbol" \s 10 \h
Grounding type


SYMBOL 183 \f "Symbol" \s 10 \h
Single


SYMBOL 183 \f "Symbol" \s 10 \h
Locking type


SYMBOL 183 \f "Symbol" \s 10 \h
NEMA L5-20R configuration


Factory convenience receptacles shall be Hubbell #2310A.  Cover plates for factory areas shall be stainless steel.

STD SPEC
4.13 - 1

