Copyright 2002 and 2005 AIA
MASTERSPEC Full Length
11/02 (R 05/06)

Copyright 2002 and 2005, The American Institute of Architects (AIA)

Exclusively published and distributed by Architectural Computer Services, Inc. (ARCOM) for the AIA

SECTION 015000 - TEMPORARY FACILITIES AND CONTROLS
This Section uses the term "Architect."  Change this term to match that used to identify the design professional as defined in the General and Supplementary Conditions.

Verify that Section titles referenced in this Section are correct for this Project's Specifications; Section titles may have changed.

PART 1 -  GENERAL

1.1 RELATED DOCUMENTS

A. Drawings and general provisions of the Contract, including General and Supplementary Conditions and other Division 01 Specification Sections, apply to this Section.

1.2 SUMMARY

A. This Section includes requirements for temporary utilities, support facilities, and security and protection facilities.

B. Related Sections include the following:

Subparagraphs below include requirements related to temporary facilities and controls; revise to suit Project.

1. Division 01 Section "Summary" for limitations on utility interruptions and other work restrictions.

2. Division 01 Section "Multiple Contract Summary" for division of responsibilities for temporary facilities and controls.

3. Division 01 Section "Submittal Procedures" for procedures for submitting copies of implementation and termination schedule and utility reports.

4. Division 01 Section "Execution" for progress cleaning requirements.

5. Divisions 02 through 49 Sections for temporary heat, ventilation, and humidity requirements for products in those Sections.

6. Division 31 Section "Dewatering" for disposal of ground water at Project site.

7. Division 31 Section "Termite Control" for pest control.

8. Division 31 Section "Asphalt Paving" for construction and maintenance of asphalt paving for temporary roads and paved areas.

Retain subparagraph above or below.  Coordinate with materials specified in Part 2 "Materials" Article.

9. Division 32 Section "Concrete Paving" for construction and maintenance of cement concrete pavement for temporary roads and paved areas.

1.3 DEFINITIONS

A. Permanent Enclosure:  As determined by Architect, permanent or temporary roofing is complete, insulated, and weathertight; exterior walls are insulated and weathertight; and all openings are closed with permanent construction or substantial temporary closures.

1.4 USE CHARGES

Retain first paragraph below to prevent extra cost charges to Owner.

A. General:  Cost or use charges for temporary facilities shall be included in the Contract Sum.  Allow other entities to use temporary services and facilities without cost, including, but not limited to,[ Owner's construction forces,] Architect,[ occupants of Project,] testing agencies, and authorities having jurisdiction.

B. Sewer Service:  Pay sewer service use charges for sewer usage by all entities for construction operations.

C. Water Service:  Pay water service use charges for water used by all entities for construction operations.

D. Electric Power Service:  Pay electric power service use charges for electricity used by all entities for construction operations.

Delete paragraph below if multiple contracts are not required.

E. Sewer, Water, and Electric Power Service:  Use charges are specified in Division 01 Section "Multiple Contract Summary."

Retain two paragraphs below only if utilities are available on-site and Owner will pay for their use for construction operations.

F. Water Service:  Water from Owner's existing water system is available for use without metering and without payment of use charges.  Provide connections and extensions of services as required for construction operations.

G. Electric Power Service:  Electric power from Owner's existing system is available for use without metering and without payment of use charges.  Provide connections and extensions of services as required for construction operations.

If needed, insert use-charge requirements for other utilities needed for construction operations.

1.5 SUBMITTALS

A. Site Plan:  Show temporary facilities, utility hookups, staging areas, and parking areas for construction personnel.

1.6 QUALITY ASSURANCE

A. Electric Service:  Comply with NECA, NEMA, and UL standards and regulations for temporary electric service.  Install service to comply with NFPA 70.

B. Tests and Inspections:  Arrange for authorities having jurisdiction to test and inspect each temporary utility before use.  Obtain required certifications and permits.

1.7 PROJECT CONDITIONS

Retain this Article to impose responsibility for maintenance and protection of permanent services and facilities used to provide temporary service and facility.

A. Temporary Use of Permanent Facilities:  Installer of each permanent service shall assume responsibility for operation, maintenance, and protection of each permanent service during its use as a construction facility before Owner's acceptance, regardless of previously assigned responsibilities.

PART 2 -  PRODUCTS

2.1 MATERIALS

A. Pavement:  Comply with Division 32 [Section "Asphalt Paving."] [Section "Concrete Paving."] [pavement Sections.]

Delete first two paragraphs below if wood fence in third paragraph is used or covered walkway is used.  For more security, increase fence height to 8 feet (2.4 m) and revise size of posts.  If appearance is important for highly visible projects, revise paragraphs to require new materials; otherwise, Contractor is likely to provide reused material.  Delete all three paragraphs if fence material is Contractor's option.

B. Chain-Link Fencing:  Minimum 2-inch (50-mm), 0.148-inch- (3.76-mm-) thick, galvanized steel, chain-link fabric fencing; minimum 6 feet (1.8 m) high with galvanized steel pipe posts; minimum 2-3/8-inch- (60-mm-) OD line posts and 2-7/8-inch- (73-mm-) OD corner and pull posts[, with 1-5/8-inch- (42-mm-) OD top rails] [, with galvanized barbed-wire top strand].

C. Portable Chain-Link Fencing:  Minimum 2-inch (50-mm), 9-gage, galvanized steel, chain-link fabric fencing; minimum 6 feet (1.8 m) high with galvanized steel pipe posts; minimum 2-3/8-inch- (60-mm-) OD line posts and 2-7/8-inch- (73-mm-) OD corner and pull posts, with 1-5/8-inch- (42-mm-) OD top and bottom rails.  Provide [concrete] [galvanized steel] bases for supporting posts.

D. Wood Enclosure Fence:  Plywood, [6 feet (1.8 m)] [8 feet (2.4 m)] high, framed with four 2-by-4-inch (50-by-100-mm) rails, with preservative-treated wood posts spaced not more than 8 feet (2.4 m) apart.

If Sections in paragraph below are not included in Project, add requirements here.

E. Lumber and Plywood:  Comply with requirements in Division 06 Section "[Rough Carpentry] [Miscellaneous Rough Carpentry]."

Retain paragraph below if gypsum board temporary partitions for enclosure or dust control are specified.

F. Gypsum Board:  Minimum 1/2 inch (12.7 mm) thick by 48 inches (1219 mm) wide by maximum available lengths; regular-type panels with tapered edges.  Comply with ASTM C 36/C 36M.

Retain first paragraph below for insulated temporary enclosures and partitions.

G. Insulation:  Unfaced mineral-fiber blanket, manufactured from glass, slag wool, or rock wool; with maximum flame-spread and smoke-developed indexes of 25 and 50, respectively.

H. Paint:  Comply with requirements in Division 09 painting Sections.

2.2 TEMPORARY FACILITIES

A. Field Offices, General:  Prefabricated or mobile units with serviceable finishes, temperature controls, and foundations adequate for normal loading.

B. Common-Use Field Office:  Of sufficient size to accommodate needs of construction personnel.  Keep office clean and orderly.  Furnish and equip offices as follows:

1. Furniture required for Project-site documents including file cabinets, plan tables, plan racks, and bookcases.

2. Conference room of sufficient size to accommodate meetings of [10] <Insert number> individuals.  Provide electrical power service and 120-V ac duplex receptacles, with not less than 1 receptacle on each wall.  Furnish room with conference table, chairs, and 4-foot- (1.2-m-) square tack board.

3. Drinking water and private toilet.

4. Coffee machine and supplies.

5. Heating and cooling equipment necessary to maintain a uniform indoor temperature of 68 to 72 deg F (20 to 22 deg C).

6. Lighting fixtures capable of maintaining average illumination of 20 fc (215 lx) at desk height.

Coordinate paragraph and subparagraph below with Owner for use of existing building for storage and protection of materials to be incorporated into Project.

C. Storage and Fabrication Sheds:  Provide sheds sized, furnished, and equipped to accommodate materials and equipment for construction operations.

1. Store combustible materials apart from building.

2.3 EQUIPMENT

A. Fire Extinguishers:  Portable, UL rated; with class and extinguishing agent as required by locations and classes of fire exposures.

Revise paragraph and subparagraphs below to suit Project.  Liquid-propane-gas or fuel-oil heaters are commonly used.  Steam or hot-water heaters, gas-fired space heaters, or electric unit heaters are also used.

B. HVAC Equipment:  Unless Owner authorizes use of permanent HVAC system, provide vented, self-contained, liquid-propane-gas or fuel-oil heaters with individual space thermostatic control.

Usually retain first subparagraph below.  Gasoline-burning and salamander-type heating units are usually prohibited.

1. Use of gasoline-burning space heaters, open-flame heaters, or salamander-type heating units is prohibited.

2. Heating Units:  Listed and labeled for type of fuel being consumed, by a testing agency acceptable to authorities having jurisdiction, and marked for intended use.

Retain option in subparagraph below for LEED-NC or LEED-CI Credit EQ 3.1 or insert another number.  This requirement is only part of an Indoor Air Quality (IAQ) Management Plan required by this Credit for the construction and pre-occupancy phases of the building.

3. Permanent HVAC System:  If Owner authorizes use of permanent HVAC system for temporary use during construction, provide filter with MERV of [8] <Insert number> at each return air grille in system and remove at end of construction.

PART 3 -  EXECUTION

3.1 INSTALLATION, GENERAL

A. Locate facilities where they will serve Project adequately and result in minimum interference with performance of the Work.  Relocate and modify facilities as required by progress of the Work.

Retain subparagraph below for greenfield sites if reduced site disturbance is required for LEED-NC Credit SS 5.1.

1. Locate facilities to limit site disturbance as specified in Division 01 Section "Summary."

B. Provide each facility ready for use when needed to avoid delay.  Do not remove until facilities are no longer needed or are replaced by authorized use of completed permanent facilities.

3.2 TEMPORARY UTILITY INSTALLATION

A. General:  Install temporary service or connect to existing service.

1. Arrange with utility company, Owner, and existing users for time when service can be interrupted, if necessary, to make connections for temporary services.

B. Sewers and Drainage:  Provide temporary utilities to remove effluent lawfully.

1. Connect temporary sewers to [municipal system] [private system indicated] as directed by authorities having jurisdiction.

C. Water Service:  Install water service and distribution piping in sizes and pressures adequate for construction.

Retain paragraph above or first paragraph and subparagraph below.  Delete below if facilities are not available or use is not permitted even if available.

D. Water Service:  Use of Owner's existing water service facilities will be permitted, as long as facilities are cleaned and maintained in a condition acceptable to Owner.  At Substantial Completion, restore these facilities to condition existing before initial use.

Generally retain subparagraph below only for work in existing buildings.

1. Where installations below an outlet might be damaged by spillage or leakage, provide a drip pan of suitable size to minimize water damage.  Drain accumulated water promptly from pans.

E. Sanitary Facilities:  Provide temporary toilets, wash facilities, and drinking water for use of construction personnel.  Comply with authorities having jurisdiction for type, number, location, operation, and maintenance of fixtures and facilities.

Delete subparagraph below if facilities are not available or their use is not permitted even if available.  Indicate location of Owner's existing toilets in Division 01 Section "Summary" or show on Drawings.

1. Toilets:  Use of Owner's existing toilet facilities will be permitted, as long as facilities are cleaned and maintained in a condition acceptable to Owner.  At Substantial Completion, restore these facilities to condition existing before initial use.

F. Heating[ and Cooling]:  Provide temporary heating[ and cooling] required by construction activities for curing or drying of completed installations or for protecting installed construction from adverse effects of low temperatures or high humidity.  Select equipment that will not have a harmful effect on completed installations or elements being installed.

G. Ventilation and Humidity Control:  Provide temporary ventilation required by construction activities for curing or drying of completed installations or for protecting installed construction from adverse effects of high humidity.  Select equipment that will not have a harmful effect on completed installations or elements being installed.  Coordinate ventilation requirements to produce ambient condition required and minimize energy consumption.

Insert gas or other utility services if required for Project.

H. Electric Power Service:  Use of Owner's existing electric power service will be permitted, as long as equipment is maintained in a condition acceptable to Owner.

Retain paragraph above or first paragraph and subparagraphs below.  Delete above if facilities are not available or their use is not permitted even if available.

I. Electric Power Service:  Provide electric power service and distribution system of sufficient size, capacity, and power characteristics required for construction operations.

1. Install electric power service [overhead] [underground], unless otherwise indicated.

Retain subparagraph below if Owner is unwilling to permit use of building's existing electric service.

2. Connect temporary service to Owner's existing power source, as directed by Owner.

J. Lighting:  Provide temporary lighting with local switching that provides adequate illumination for construction operations, observations, inspections, and traffic conditions.

1. Install and operate temporary lighting that fulfills security and protection requirements without operating entire system.

2. Install lighting for Project identification sign.

K. Telephone Service:  Provide temporary telephone service in common-use facilities for use by all construction personnel.  Install [one] <Insert number> telephone line(s) for each field office.

1. Provide additional telephone lines for the following:

a. Provide a dedicated telephone line for each facsimile machine and computer in each field office.

Delete first subparagraph below if Owner maintains a separate on-site field office.

b. Provide [one] <Insert number> telephone line(s) for Owner's use.

2. At each telephone, post a list of important telephone numbers.

Revise list below to suit Project.  See Evaluations.

a. Police and fire departments.

b. Ambulance service.

c. Contractor's home office.

d. Architect's office.

e. Engineers' offices.

f. Owner's office.

g. Principal subcontractors' field and home offices.

3. Provide superintendent with cellular telephone or portable two-way radio for use when away from field office.

L. Electronic Communication Service:  Provide temporary electronic communication service, including electronic mail, in common-use facilities.

1. Provide [DSL] [T-1 line] in primary field office.

3.3 SUPPORT FACILITIES INSTALLATION

A. General:  Comply with the following:

Review first subparagraph below with Owner's insurance carrier.  Revise to suit Project.

1. Provide incombustible construction for offices, shops, and sheds located within construction area or within 30 feet (9 m) of building lines.  Comply with NFPA 241.

2. Maintain support facilities until near Substantial Completion.  Remove before Substantial Completion.  Personnel remaining after Substantial Completion will be permitted to use permanent facilities, under conditions acceptable to Owner.

B. Temporary Roads and Paved Areas:  Construct and maintain temporary roads and paved areas adequate for construction operations.  Locate temporary roads and paved areas [as indicated] [within construction limits indicated] on Drawings.

1. Provide dust-control treatment that is nonpolluting and nontracking.  Reapply treatment as required to minimize dust.

Retain paragraph and subparagraph above or first paragraph and subparagraphs below.  Retain above if temporary roads and paved areas are not in same location as permanent roads or paved areas.  See Evaluations.

C. Temporary Roads and Paved Areas:  Construct and maintain temporary roads and paved areas adequate for construction operations.  Locate temporary roads and paved areas in same location as permanent roads and paved areas.  Extend temporary roads and paved areas, within construction limits indicated, as necessary for construction operations.

1. Coordinate elevations of temporary roads and paved areas with permanent roads and paved areas.

2. Prepare subgrade and install subbase and base for temporary roads and paved areas according to Division 31 Section "Earth Moving."

Retain one of two subparagraphs below.  First allows subbase and base of permanent roads and paved areas to be used for temporary use.  Second allows base course of permanent hot-mix asphalt pavement to be used for temporary use.  First is for use with either concrete or hot-mix asphalt pavement; second is for use only with hot-mix asphalt pavement.

3. Recondition base after temporary use, including removing contaminated material, regrading, proofrolling, compacting, and testing.

4. Delay installation of final course of permanent hot-mix asphalt pavement until immediately before Substantial Completion.  Repair hot-mix asphalt base-course pavement before installation of final course according to Division 32 Section "Asphalt Paving."

D. Traffic Controls:  Comply with requirements of authorities having jurisdiction.

1. Protect existing site improvements to remain including curbs, pavement, and utilities.

2. Maintain access for fire-fighting equipment and access to fire hydrants.

E. Parking:  [Provide temporary] [Use designated areas of Owner's existing] parking areas for construction personnel.

Revise first paragraph and subparagraphs below to satisfy soil-conservation district requirements.  See Evaluations.  Consult soil engineer about expected ground water.

F. Dewatering Facilities and Drains:  Comply with requirements of authorities having jurisdiction.  Maintain Project site, excavations, and construction free of water.

1. Dispose of rainwater in a lawful manner that will not result in flooding Project or adjoining properties nor endanger permanent Work or temporary facilities.

2. Remove snow and ice as required to minimize accumulations.

G. Project Identification and Temporary Signs:  Provide Project identification and other signs[ as indicated on Drawings].  Install signs where indicated to inform public and individuals seeking entrance to Project.  Unauthorized signs are not permitted.

If required, insert a list of necessary signs and add Project-specific provisions such as special graphics and special lighting.  See Evaluations.

1. Provide temporary, directional signs for construction personnel and visitors.

2. Maintain and touchup signs so they are legible at all times.

H. Waste Disposal Facilities:  Comply with requirements specified in Division 01 Section "Construction Waste Management and Disposal."

Retain paragraph above or below.  See Evaluations.

I. Waste Disposal Facilities:  Provide waste-collection containers in sizes adequate to handle waste from construction operations.  Comply with requirements of authorities having jurisdiction.  Comply with Division 01 Section "Execution" for progress cleaning requirements.

Delete paragraph and subparagraph below for low-rise construction of less than three stories.  Subparagraph could be important for cost-plus contracts.

J. Lifts and Hoists:  Provide facilities necessary for hoisting materials and personnel.

1. Truck cranes and similar devices used for hoisting materials are considered "tools and equipment" and not temporary facilities.

Paragraph below is simplest method of specifying temporary elevator use.

K. Temporary Elevator Use:  Refer to Division 14 Sections for temporary use of new elevators.

Retain paragraph above or first paragraph and subparagraphs below.  Delete below if elevators are not available or their use is not permitted even if available.  Indicate location of Owner's existing elevators in Division 01 Section "Summary" or show on Drawings.

L. Existing Elevator Use:  Use of Owner's existing elevators will be permitted, as long as elevators are cleaned and maintained in a condition acceptable to Owner.  At Substantial Completion, restore elevators to condition existing before initial use, including replacing worn cables, guide shoes, and similar items of limited life.

1. Do not load elevators beyond their rated weight capacity.

2. Provide protective coverings, barriers, devices, signs, or other procedures to protect elevator car and entrance doors and frame.  If, despite such protection, elevators become damaged, engage elevator Installer to restore damaged work so no evidence remains of correction work.  Return items that cannot be refinished in field to the shop, make required repairs and refinish entire unit, or provide new units as required.

M. Temporary Stairs:  Until permanent stairs are available, provide temporary stairs where ladders are not adequate.

Retain paragraph above or first paragraph and subparagraph below.  Delete below if stairs are not available or their use is not permitted even if available.  Indicate location of Owner's existing stairs in Division 01 Section "Summary" or show on Drawings.

N. Existing Stair Usage:  Use of Owner's existing stairs will be permitted, as long as stairs are cleaned and maintained in a condition acceptable to Owner.  At Substantial Completion, restore stairs to condition existing before initial use.

1. Provide protective coverings, barriers, devices, signs, or other procedures to protect stairs and to maintain means of egress.  If, despite such protection, stairs become damaged, restore damaged areas so no evidence remains of correction work.

O. Temporary Use of Permanent Stairs:  Cover finished, permanent stairs with protective covering of plywood or similar material so finishes will be undamaged at time of acceptance.

Individual Project circumstances may require use of other construction aids and miscellaneous facilities, such as scaffolds, platforms, swing stages, ramps and bridges, incidental sheeting and shoring, and demolition waste chutes.  Add requirements to suit Project.  See Evaluations.

3.4 SECURITY AND PROTECTION FACILITIES INSTALLATION

A. Environmental Protection:  Provide protection, operate temporary facilities, and conduct construction in ways and by methods that comply with environmental regulations and that minimize possible air, waterway, and subsoil contamination or pollution or other undesirable effects.

1. Comply with work restrictions specified in Division 01 Section "Summary."

B. Temporary Erosion and Sedimentation Control:  Comply with requirements specified in Division 31 Section "Site Clearing."

Retain paragraph above or paragraph and subparagraph below.

C. Temporary Erosion and Sedimentation Control:  Provide measures to prevent soil erosion and discharge of soil-bearing water runoff and airborne dust to adjacent properties and walkways, according to requirements of authorities having jurisdiction.

1. Inspect, repair, and maintain erosion- and sedimentation-control measures during construction until permanent vegetation has been established.

Insert local regulations for environmental protection.  Restrictions on noisemaking activities should be specified in Division 01 Section "Summary."

D. Stormwater Control:  Comply with authorities having jurisdiction.  Provide barriers in and around excavations and subgrade construction to prevent flooding by runoff of stormwater from heavy rains.

E. Tree and Plant Protection:  Comply with requirements specified in Division 01 Section "Temporary Tree and Plant Protection."

Retain paragraph above or below.

F. Tree and Plant Protection:  Install temporary fencing located as indicated or outside the drip line of trees to protect vegetation from damage from construction operations.  Protect tree root systems from damage, flooding, and erosion.

Delete paragraph below only if cost considerations are paramount.  Revise if specific pests, such as termites or pigeons, are known to be a problem.

G. Pest Control:  Engage pest-control service to recommend practices to minimize attraction and harboring of rodents, roaches, and other pests and to perform extermination and control procedures at regular intervals so Project will be free of pests and their residues at Substantial Completion.  Obtain extended warranty for Owner.  Perform control operations lawfully, using environmentally safe materials.

Delete paragraph and subparagraphs below if not required.

H. Site Enclosure Fence:  [Before construction operations begin] [When excavation begins], furnish and install site enclosure fence in a manner that will prevent people and animals from easily entering site except by entrance gates.

1. Extent of Fence:  [As required to enclose entire Project site or portion determined sufficient to accommodate construction operations] [As indicated on Drawings].

Option in subparagraph below is only for projects connected to existing construction.

2. Maintain security by limiting number of keys and restricting distribution to authorized personnel.[  Provide Owner with one set of keys.]

Retain first paragraph below because enclosure fence does not always provide adequate security against theft and vandalism.

I. Security Enclosure and Lockup:  Install substantial temporary enclosure around partially completed areas of construction.  Provide lockable entrances to prevent unauthorized entrance, vandalism, theft, and similar violations of security.

J. Barricades, Warning Signs, and Lights:  Comply with requirements of authorities having jurisdiction for erecting structurally adequate barricades, including warning signs and lighting.

Delete first paragraph and subparagraphs below if covered walkways are not required and area is sufficient to conduct operations, including delivery of materials.  Include installation details on Drawings.

K. Covered Walkway:  Erect structurally adequate, protective, covered walkway for passage of individuals along adjacent public street(s).  Coordinate with entrance gates, other facilities, and obstructions.  Comply with regulations of authorities having jurisdiction[ and requirements indicated on Drawings].

1. Construct covered walkways using scaffold or shoring framing.

2. Provide wood-plank overhead decking, protective plywood enclosure walls, handrails, barricades, warning signs, lights, safe and well-drained walkways, and similar provisions for protection and safe passage.

3. Extend back wall beyond the structure to complete enclosure fence.

4. Paint and maintain in a manner approved by Owner and Architect.

L. Temporary Enclosures:  Provide temporary enclosures for protection of construction, in progress and completed, from exposure, foul weather, other construction operations, and similar activities.  Provide temporary weathertight enclosure for building exterior.

1. Where heating or cooling is needed and permanent enclosure is not complete, insulate temporary enclosures.

M. Temporary Partitions:  Provide floor-to-ceiling dustproof partitions to limit dust and dirt migration and to separate areas occupied by [Owner] [and] [tenants] from fumes and noise.

Delete first two subparagraphs and associated subparagraph if not required.  Revise construction to a fire-rated partition if necessary.

1. Construct dustproof partitions with gypsum wallboard with joints taped on occupied side, and fire-retardant plywood on construction operations side.

Retain subparagraph and associated subparagraph below if containment of airborne particles and dust generated by construction activities is critical to occupants of other spaces in building, e.g., occupied healthcare facilities.

2. Construct dustproof partitions with 2 layers of 3-mil (0.07-mm) polyethylene sheet on each side.  Cover floor with 2 layers of 3-mil (0.07-mm) polyethylene sheet, extending sheets 18 inches (460 mm) up the sidewalls.  Overlap and tape full length of joints.  Cover floor with fire-retardant plywood.

a. Construct vestibule and airlock at each entrance through temporary partition with not less than 48 inches (1219 mm) between doors.  Maintain water-dampened foot mats in vestibule.

3. Insulate partitions to provide noise protection to occupied areas.

4. Seal joints and perimeter.  Equip partitions with dustproof doors and security locks.

5. Protect air-handling equipment.

6. Weather strip openings.

7. Provide walk-off mats at each entrance through temporary partition.

Paragraph and subparagraphs below specify currently accepted requirements; revise to suit local conditions.  See Evaluations.

N. Temporary Fire Protection:  Install and maintain temporary fire-protection facilities of types needed to protect against reasonably predictable and controllable fire losses.  Comply with NFPA 241.

1. Prohibit smoking in [hazardous fire-exposure] [construction] areas.

2. Supervise welding operations, combustion-type temporary heating units, and similar sources of fire ignition according to requirements of authorities having jurisdiction.

3. Develop and supervise an overall fire-prevention and -protection program for personnel at Project site.  Review needs with local fire department and establish procedures to be followed.  Instruct personnel in methods and procedures.  Post warnings and information.

4. Provide temporary standpipes and hoses for fire protection.  Hang hoses with a warning sign stating that hoses are for fire-protection purposes only and are not to be removed.  Match hose size with outlet size and equip with suitable nozzles.

If fire-suppression sprinkler systems or other permanent fire-protection systems are used, insert specific requirements.

3.5 OPERATION, TERMINATION, AND REMOVAL

First paragraph below is important if allowances for metered use of temporary facilities have been established.

A. Supervision:  Enforce strict discipline in use of temporary facilities.  To minimize waste and abuse, limit availability of temporary facilities to essential and intended uses.

B. Maintenance:  Maintain facilities in good operating condition until removal.

1. Maintain operation of temporary enclosures, heating, cooling, humidity control, ventilation, and similar facilities on a 24-hour basis where required to achieve indicated results and to avoid possibility of damage.

Delete first paragraph below if not required.

C. Operate Project-identification-sign lighting daily from dusk until 12:00 midnight.

D. Temporary Facility Changeover:  Do not change over from using temporary security and protection facilities to permanent facilities until Substantial Completion.

E. Termination and Removal:  Remove each temporary facility when need for its service has ended, when it has been replaced by authorized use of a permanent facility, or no later than Substantial Completion.  Complete or, if necessary, restore permanent construction that may have been delayed because of interference with temporary facility.  Repair damaged Work, clean exposed surfaces, and replace construction that cannot be satisfactorily repaired.

1. Materials and facilities that constitute temporary facilities are property of Contractor.  Owner reserves right to take possession of Project identification signs.

Consider inserting specific removal requirements, as illustrated in first subparagraph below.

2. Remove temporary paving not intended for or acceptable for integration into permanent paving.  Where area is intended for landscape development, remove soil and aggregate fill that do not comply with requirements for fill or subsoil.  Remove materials contaminated with road oil, asphalt and other petrochemical compounds, and other substances that might impair growth of plant materials or lawns.  Repair or replace street paving, curbs, and sidewalks at temporary entrances, as required by authorities having jurisdiction.

3. At Substantial Completion, clean and renovate permanent facilities used during construction period.  Comply with final cleaning requirements specified in Division 01 Section "Closeout Procedures."

Review final draft of this Section with Owner.  Advise Owner that any exceptions to its provisions might translate into costs borne by Owner.

END OF SECTION 015000

TEMPORARY FACILITIES AND CONTROLS
015000 - 1

