Copyright 2011 AIA
MasterSpec Professional
02/11

Copyright 2011 by The American Institute of Architects (AIA)

Exclusively published and distributed by Architectural Computer Services, Inc. (ARCOM) for the AIA

SECTION 017419 - CONSTRUCTION WASTE MANAGEMENT AND DISPOSAL
TIPS:
To view non-printing Editor's Notes that provide guidance for editing, click on Masterworks/Single‑File Formatting/Toggle/Editor's Notes.
To read detailed research, technical information about products and materials, and coordination checklists, click on Masterworks/Supporting Information.

Revise this Section by deleting and inserting text to meet Project-specific requirements.

This Section uses the term "Architect." Change this term to match that used to identify the design professional as defined in the General and Supplementary Conditions.

Verify that Section titles referenced in this Section are correct for this Project's Specifications; Section titles may have changed.

PART 1 -  GENERAL

1.1 SUMMARY

A. Section includes administrative and procedural requirements for the following:

1. Salvaging nonhazardous [demolition] [and] [construction] waste.

2. Recycling nonhazardous [demolition] [and] [construction] waste.

3. Disposing of nonhazardous [demolition] [and] [construction] waste.

B. Related Requirements:

Retain subparagraphs below to cross-reference requirements Contractor might expect to find in this Section but are specified in other Sections.

1. Section 024116 "Structure Demolition" for disposition of waste resulting from demolition of buildings, structures, and site improvements[, and for disposition of hazardous waste].

2. Section 024119 "Selective Demolition" for disposition of waste resulting from partial demolition of buildings, structures, and site improvements[, and for disposition of hazardous waste].

3. Section 042000 "Unit Masonry" for disposal requirements for masonry waste.

4. Section 044313.13 "Anchored Stone Masonry Veneer" for disposal requirements for excess stone and stone waste.

5. Section 044313.16 "Adhered Stone Masonry Veneer" for disposal requirements for excess stone and stone waste.

6. Section 311000 "Site Clearing" for disposition of waste resulting from site clearing and removal of above- and below-grade improvements.

1.2 DEFINITIONS

Retain terms that remain after this Section has been edited for a project.

A. Construction Waste: Building and site improvement materials and other solid waste resulting from construction, remodeling, renovation, or repair operations. Construction waste includes packaging.

B. Demolition Waste: Building and site improvement materials resulting from demolition or selective demolition operations.

C. Disposal: Removal off-site of demolition and construction waste and subsequent sale, recycling, reuse, or deposit in landfill or incinerator acceptable to authorities having jurisdiction.

D. Recycle: Recovery of demolition or construction waste for subsequent processing in preparation for reuse.

E. Salvage: Recovery of demolition or construction waste and subsequent sale or reuse in another facility.

F. Salvage and Reuse: Recovery of demolition or construction waste and subsequent incorporation into the Work.

1.3 PERFORMANCE REQUIREMENTS

First option in "General" Paragraph below is requirement for LEED Credit MR 2.1; second option is for Credit MR 2.2.

A. General: Achieve end-of-Project rates for salvage/recycling of [50] [75] <Insert number> percent by weight of total non-hazardous solid waste generated by the Work. Facilitate recycling and salvage of materials[.][, including the following:]

Retain subparagraph below with last option in "General" Paragraph above to suit Project. Verify capabilities of local recycling facilities.

1. <Insert, in separate subparagraphs, materials to be recycled or salvaged>.

1.4 ACTION SUBMITTALS

A. Waste Management Plan: Submit plan within [7] [30] <Insert number> days of date established for [commencement of the Work] [the Notice to Proceed] [the Notice of Award].

1.5 INFORMATIONAL SUBMITTALS

See Evaluations for example of Progress Reports.

A. Waste Reduction Progress Reports: Concurrent with each Application for Payment, submit report. Include the following information:

1. Material category.

2. Generation point of waste.

3. Total quantity of waste in tons (tonnes).

4. Quantity of waste salvaged, both estimated and actual in tons (tonnes).

5. Quantity of waste recycled, both estimated and actual in tons (tonnes).

6. Total quantity of waste recovered (salvaged plus recycled) in tons (tonnes).

7. Total quantity of waste recovered (salvaged plus recycled) as a percentage of total waste.

Submittals in "Waste Reduction Calculations," "Records of Donations," "Records of Sales," and "Recycling and Processing Facility Records" paragraphs below document compliance with requirements for LEED Credit MR 2.1 and Credit MR 2.2. Submittals are not required to be submitted to the USGBC as part of LEED certification process but will be needed if Architect or Owner signs letter for LEED credits.

B. Waste Reduction Calculations: Before request for Substantial Completion, submit calculated end-of-Project rates for salvage, recycling, and disposal as a percentage of total waste generated by the Work.

C. Records of Donations: Indicate receipt and acceptance of salvageable waste donated to individuals and organizations. Indicate whether organization is tax exempt.

D. Records of Sales: Indicate receipt and acceptance of salvageable waste sold to individuals and organizations. Indicate whether organization is tax exempt.

E. Recycling and Processing Facility Records: Indicate receipt and acceptance of recyclable waste by recycling and processing facilities licensed to accept them. Include manifests, weight tickets, receipts, and invoices.

F. Landfill and Incinerator Disposal Records: Indicate receipt and acceptance of waste by landfills and incinerator facilities licensed to accept them. Include manifests, weight tickets, receipts, and invoices.

Retain "LEED Submittal" Paragraph below if Contractor rather than Architect or Owner is required to sign letter for LEED credits.

G. LEED Submittal: LEED letter template for Credit MR 2.1[ and Credit MR 2.2], signed by Contractor, tabulating total waste material, quantities diverted and means by which it is diverted, and statement that requirements for the credit have been met.

Coordinate "Qualification Data" Paragraph below with qualification requirements in Section 014000 "Quality Requirements" and as may be supplemented in "Quality Assurance" Article.

H. Qualification Data: For waste management coordinator.

1.6 QUALITY ASSURANCE

Some Contractors may use waste management coordination services of an outside waste management firm; retain option in "Waste Management Coordinator Qualifications" Paragraph below if acceptable. LEED accreditation is desirable; consider revising where LEED certification is not a Project goal.

A. Waste Management Coordinator Qualifications: LEED-Accredited Professional, certified by USGBC.[ Waste management coordinator may also serve as LEED coordinator.]

B. Waste Management Conference: Conduct conference at Project site to comply with requirements in Section 013100 "Project Management and Coordination."

1.7 WASTE MANAGEMENT PLAN

Retain option in "General" Paragraph below if Project requires selective demolition or building demolition.

A. General: Develop a waste management plan according to ASTM E 1609 and requirements in this Section. Plan shall consist of waste identification, waste reduction work plan, and cost/revenue analysis.[ Distinguish between demolition and construction waste.] Indicate quantities by weight or volume, but use same units of measure throughout waste management plan.

B. Waste Identification: Indicate anticipated types and quantities of [demolition] [site-clearing] [and] [construction] waste generated by the Work. Include estimated quantities and assumptions for estimates.

C. Waste Reduction Work Plan: List each type of waste and whether it will be salvaged, recycled, or disposed of in landfill or incinerator. Include points of waste generation, total quantity of each type of waste, quantity for each means of recovery, and handling and transportation procedures.

1. Salvaged Materials for Reuse: For materials that will be salvaged and reused in this Project, describe methods for preparing salvaged materials before incorporation into the Work.

2. Salvaged Materials for Sale: For materials that will be sold to individuals and organizations, include list of their names, addresses, and telephone numbers.

If applicable, list local charitable organizations (such as the Habitat for Humanity) in "Salvaged Materials for Donation" Subparagraph below.

3. Salvaged Materials for Donation: For materials that will be donated to individuals and organizations, include list of their names, addresses, and telephone numbers.

4. Recycled Materials: Include list of local receivers and processors and type of recycled materials each will accept. Include names, addresses, and telephone numbers.

5. Disposed Materials: Indicate how and where materials will be disposed of. Include name, address, and telephone number of each landfill and incinerator facility.

6. Handling and Transportation Procedures: Include method that will be used for separating recyclable waste including sizes of containers, container labeling, and designated location where materials separation will be performed.

PART 2 -  PRODUCTS (Not Used)

PART 3 -  EXECUTION

3.1 PLAN IMPLEMENTATION

A. General: Implement approved waste management plan. Provide handling, containers, storage, signage, transportation, and other items as required to implement waste management plan during the entire duration of the Contract.

Retain option in "Waste Management Coordinator" Paragraph below when Project size and complexity of waste management justifies full-time on-site waste management coordinator.

B. Waste Management Coordinator: Engage a waste management coordinator to be responsible for implementing, monitoring, and reporting status of waste management work plan.[ Coordinator shall be present at Project site full time for duration of Project.]

C. Training: Train workers, subcontractors, and suppliers on proper waste management procedures, as appropriate for the Work occurring at Project site.

1. Distribute waste management plan to everyone concerned within [three] <Insert number> days of submittal return.

2. Distribute waste management plan to entities when they first begin work on-site. Review plan procedures and locations established for salvage, recycling, and disposal.

D. Site Access and Temporary Controls: Conduct waste management operations to ensure minimum interference with roads, streets, walks, walkways, and other adjacent occupied and used facilities.

1. Designate and label specific areas on Project site necessary for separating materials that are to be salvaged, recycled, reused, donated, and sold.

2. Comply with Section 015000 "Temporary Facilities and Controls" for controlling dust and dirt, environmental protection, and noise control.

3.2 SALVAGING DEMOLITION WASTE

A. Salvaged Items for Reuse in the Work:

1. Clean salvaged items.

2. Pack or crate items after cleaning. Identify contents of containers.

3. Store items in a secure area until installation.

4. Protect items from damage during transport and storage.

5. Install salvaged items to comply with installation requirements for new materials and equipment. Provide connections, supports, and miscellaneous materials necessary to make items functional for use indicated.

B. Salvaged Items for [Sale] [and] [Donation]: [Permitted] [Not permitted] on Project site.

C. Salvaged Items for Owner's Use:

1. Clean salvaged items.

2. Pack or crate items after cleaning. Identify contents of containers.

3. Store items in a secure area until delivery to Owner.

4. Transport items to Owner's storage area [on-site] [off-site] [designated by Owner].

5. Protect items from damage during transport and storage.

3.3 RECYCLING [DEMOLITION] [AND] [CONSTRUCTION] WASTE, GENERAL

A. General: Recycle paper and beverage containers used by on-site workers.

"Recycling Receivers and Processors" Paragraph below may be helpful for bidders with little or no previous experience with recycling. List of recycling and processing facilities is available from telephone directories and many local and state authorities.

B. Recycling Receivers and Processors: List below is provided for information only; available recycling receivers and processors include, but are not limited to, the following:

1. <Insert names and telephone numbers of local recycling receivers and processors of recyclable materials>.

Allowing Contractor to accrue some portion of the incentives in "Recycling Incentives" Paragraph below could result in better recovery rates than if Owner accrues all of the incentives.

C. Recycling Incentives: Revenues, savings, rebates, tax credits, and other incentives received for recycling waste materials shall [accrue to Owner] [accrue to Contractor] [be shared equally by Owner and Contractor].

Procedures in "Procedures" Paragraph below describe the "source-separated" method for handling recyclable waste. If space at Project site is limited, consider revising below to allow "co-mingled" method, which takes less space because it permits all recyclable waste to be placed in a single container that is separated later at the recycling facility.

D. Procedures: Separate recyclable waste from other waste materials, trash, and debris. Separate recyclable waste by type at Project site to the maximum extent practical according to approved construction waste management plan.

1. Provide appropriately marked containers or bins for controlling recyclable waste until they are removed from Project site. Include list of acceptable and unacceptable materials at each container and bin.

a. Inspect containers and bins for contamination and remove contaminated materials if found.

2. Stockpile processed materials on-site without intermixing with other materials. Place, grade, and shape stockpiles to drain surface water. Cover to prevent windblown dust.

3. Stockpile materials away from construction area. Do not store within drip line of remaining trees.

4. Store components off the ground and protect from the weather.

5. Remove recyclable waste from Owner's property and transport to recycling receiver or processor.

3.4 RECYCLING DEMOLITION WASTE

Paragraphs in this article are examples only; retain or add other specific disposal, cleanup, or removal requirements to suit Project or recycling facilities. Ceramic tile is also often recycled.

Retain one of two "Asphaltic Concrete Paving" paragraphs below; first paragraph can be used for general fill.

A. Asphalt Paving: Grind asphalt to maximum [1-1/2-inch (38-mm)] [4-inch (100-mm)] size.

B. Asphalt Paving: Break up and transport paving to asphalt-recycling facility.

C. Concrete: Remove reinforcement and other metals from concrete and sort with other metals.

Option for larger size in subparagraph below can be used for general fill or riprap; option for smaller size can be used as satisfactory soil for fill or subbase.

1. Pulverize concrete to maximum [1-1/2-inch (38-mm)] [4-inch (100-mm)] size.

D. Masonry: Remove metal reinforcement, anchors, and ties from masonry and sort with other metals.

Retain one of two subparagraphs below. Option for largest size in first subparagraph can be used for general fill; brick masonry in smaller sizes can be used for landscape mulch; concrete masonry in smaller sizes can be used as satisfactory soil for fill or subbase.

1. Pulverize masonry to maximum [3/4-inch (19-mm)] [1-inch (25-mm)] [1-1/2-inch (38-mm)] [4-inch (100-mm)] size.

2. Clean and stack undamaged, whole masonry units on wood pallets.

E. Wood Materials: Sort and stack members according to size, type, and length. Separate lumber, engineered wood products, panel products, and treated wood materials.

F. Metals: Separate metals by type.

1. Structural Steel: Stack members according to size, type of member, and length.

2. Remove and dispose of bolts, nuts, washers, and other rough hardware.

G. Asphalt Shingle Roofing: Separate organic and glass-fiber asphalt shingles and felts. Remove and dispose of nails, staples, and accessories.

H. Gypsum Board: Stack large clean pieces on wood pallets or in container and store in a dry location. Remove edge trim and sort with other metals. Remove and dispose of fasteners.

I. Acoustical Ceiling Panels and Tile: Stack large clean pieces on wood pallets and store in a dry location.

J. Metal Suspension System: Separate metal members including trim, and other metals from acoustical panels and tile and sort with other metals.

K. Carpet[ and Pad]: Roll large pieces tightly after removing debris, trash, adhesive, and tack strips.

1. Store clean, dry carpet[ and pad] in a closed container or trailer provided by Carpet Reclamation Agency or carpet recycler.

L. Carpet Tile: Remove debris, trash, and adhesive.

1. Stack tile on pallet and store clean, dry carpet in a closed container or trailer provided by Carpet Reclamation Agency or carpet recycler.

M. Piping: Reduce piping to straight lengths and store by type and size. Separate supports, hangers, valves, sprinklers, and other components by type and size.

N. Conduit: Reduce conduit to straight lengths and store by type and size.

3.5 RECYCLING CONSTRUCTION WASTE

Paragraphs in this article are examples of items that are common to normal construction operations; retain or insert other specific disposal, cleanup, or removal requirements to suit Project or recycling facilities.

A. Packaging:

1. Cardboard and Boxes: Break down packaging into flat sheets. Bundle and store in a dry location.

2. Polystyrene Packaging: Separate and bag materials.

3. Pallets: As much as possible, require deliveries using pallets to remove pallets from Project site. For pallets that remain on-site, break down pallets into component wood pieces and comply with requirements for recycling wood.

4. Crates: Break down crates into component wood pieces and comply with requirements for recycling wood.

B. Wood Materials:

1. Clean Cut-Offs of Lumber: Grind or chip into small pieces.

2. Clean Sawdust: Bag sawdust that does not contain painted or treated wood.

C. Gypsum Board: Stack large clean pieces on wood pallets or in container and store in a dry location.

Retain "Clean Gypsum Board" Subparagraph below if gypsum board will be chipped on-site; delete if gypsum board to be processed off-site.

1. Clean Gypsum Board: Grind scraps of clean gypsum board using small mobile chipper or hammer mill. Screen out paper after grinding.

3.6 DISPOSAL OF WASTE

Insert other specific disposal, cleanup, or removal requirements to suit Project.

A. General: Except for items or materials to be salvaged, recycled, or otherwise reused, remove waste materials from Project site and legally dispose of them in a landfill or incinerator acceptable to authorities having jurisdiction.

1. Except as otherwise specified, do not allow waste materials that are to be disposed of accumulate on-site.

2. Remove and transport debris in a manner that will prevent spillage on adjacent surfaces and areas.

Retain one of two "Burning" paragraphs below; burning is usually not permitted.

B. Burning: Do not burn waste materials.

C. Burning: Burning of waste materials is permitted only at designated areas on Owner's property, provided required permits are obtained. Provide full-time monitoring for burning materials until fires are extinguished.

Retain "Disposal" Paragraph below if disposal is permitted on Owner's property; revise, if applicable, to indicate limits on type of materials that may be disposed of on-site.

Delete first paragraph and retain second paragraph when disposal off Owner's property is required. Add specific requirements for off-site disposal to suit Project.

D. Disposal: Remove waste materials and dispose of at designated spoil areas on Owner's property.

E. Disposal: Remove waste materials from Owner's property and legally dispose of them.

3.7 SAMPLE FORMS

Attach samples of forms that will be issued with this Section here or after end of Section according to office preference.

END OF SECTION 017419

	CONSTRUCTION WASTE MANAGEMENT AND DISPOSAL
	017419 - 1



