Copyright 2012 AIA
MasterSpec Professional
05/12

Copyright 2012 by The American Institute of Architects (AIA)

Exclusively published and distributed by Architectural Computer Services, Inc. (ARCOM) for the AIA

SECTION 074113.13 - FORMED METAL ROOF PANELS
TIPS:
To view non-printing Editor's Notes that provide guidance for editing, click on Masterworks/Single‑File Formatting/Toggle/Editor's Notes.
To read detailed research, technical information about products and materials, and coordination checklists, click on Masterworks/Supporting Information.

Revise this Section by deleting and inserting text to meet Project-specific requirements.

This Section uses the term "Architect." Change this term to match that used to identify the design professional as defined in the General and Supplementary Conditions.

Verify that Section titles referenced in this Section are correct for this Project's Specifications; Section titles may have changed.

PART 1 -  GENERAL

1.1 SUMMARY

A. Section includes exposed-fastener, lap-seam, metal roof panels.

1.2 PREINSTALLATION MEETINGS

A. Preinstallation Conference: Conduct conference at [Project site] <Insert location>.

1.3 ACTION SUBMITTALS

A. Product Data: For each type of product.

B. LEED Submittals:

1. Product Data for Credit MR 4: For products having recycled content, documentation indicating percentages by weight of postconsumer and preconsumer recycled content. Include statement indicating cost for each product having recycled content.

"Product Test Reports for Credit SS 7.2" Subparagraph below applies to LEED-NC, LEED-CS, and LEED for Schools; coordinate with requirements for roof coatings.

2. Product Test Reports for Credit SS 7.2: For roofing materials, documentation indicating that roofing materials comply with Solar Reflectance Index requirement.

C. Shop Drawings: Include fabrication and installation layouts of metal panels; details of edge conditions, joints, panel profiles, corners, anchorages, attachment system, trim, flashings, closures, and accessories; and special details.

D. Samples: For each type of metal panel indicated.

1.4 INFORMATIONAL SUBMITTALS

A. Product test reports.

B. Warranties: Sample of special warranties.

1.5 CLOSEOUT SUBMITTALS

A. Maintenance data.

1.6 QUALITY ASSURANCE

A. Installer Qualifications: An entity that employs installers and supervisors who are trained and approved by manufacturer.

Retain "UL-Certified, Portable Roll-Forming Equipment" Paragraph below if portable roll-forming equipment is allowed for on-site roll forming.

B. UL-Certified, Portable Roll-Forming Equipment: UL-certified, portable roll-forming equipment capable of producing metal panels warranted by manufacturer to be the same as factory-formed products. Maintain UL certification of portable roll-forming equipment for duration of work.

1.7 WARRANTY

When warranties are required, verify with Owner's counsel that special warranties stated in this article are not less than remedies available to Owner under prevailing local laws.

A. Special Warranty: Manufacturer's standard form in which manufacturer agrees to repair or replace components of metal panel systems that fail in materials or workmanship within specified warranty period.

Verify available warranties and warranty periods for metal panels.

1. Warranty Period: [Two] <Insert number> years from date of Substantial Completion.

B. Special Warranty on Panel Finishes: Manufacturer's standard form in which manufacturer agrees to repair finish or replace metal panels that show evidence of deterioration of factory-applied finishes within specified warranty period.

Verify available warranties for metal panel finishes and insert number in "Finish Warranty Period" Subparagraph below. A 20-year period is available for fluoropolymer finish and is the maximum included with manufacturers' published data; a 10-year period is usually available for siliconized polyester. Longer periods for premium finishes may be available.

1. Finish Warranty Period: [20] [10] <Insert number> years from date of Substantial Completion.

Insert requirements for special weathertightness warranty if needed; panel manufacturers do not typically offer such warranties.

PART 2 -  PRODUCTS

See Editing Instruction No. 1 in the Evaluations for cautions about named manufacturers and products. For an explanation of options and Contractor's product selection procedures, see Section 016000 "Product Requirements."

2.1 PERFORMANCE REQUIREMENTS

Retain "Recycled Content of Steel Products" Paragraph below to specify recycled content if applying for LEED Credit MR 4. USGBC allows a default value of 25 percent to be used for steel, without documentation; higher percentages can be claimed if they are supported by appropriate documentation. The Steel Recycling Institute indicates that steel sheet typically has 23 percent postconsumer recycled content and 1.5 percent preconsumer recycled content.

A. Recycled Content of Steel Products: Postconsumer recycled content plus one-half of preconsumer recycled content not less than [25] <Insert number> percent.

Retain "Solar Reflectance Index" Paragraph below if required for LEED-NC, LEED-CS, or LEED for Schools Credit SS 7.2. First option is the minimum value for roofs with slopes of 2:12 or less; second option is the minimum value for roofs with slopes steeper than 2:12.

B. Solar Reflectance Index: Not less than [78] [29] according to ASTM E 1980.

Retain first "Energy Performance" Paragraph below for roofs that must comply with the EPA/DOE's ENERGY STAR requirements. The EPA/DOE's ENERGY STAR "Roof Product List" is available in PDF at www.energystar.gov.

C. Energy Performance: Provide roof panels that are listed on the EPA/DOE's ENERGY STAR "Roof Product List" for [low] [steep]-slope roof products.

Retain "Energy Performance" Paragraph below for roofs that must comply with local "cool-roof" energy legislation; verify requirements with authorities having jurisdiction. Example and options below are for low-slope roofs that must comply with prescriptive approach of CCR Title 24 (California Building Standards Code).

D. Energy Performance: Provide roof panels according to one of the following when tested according to CRRC-1:

1. Three-year, aged solar reflectance of not less than [0.55] <Insert value> and emissivity of not less than [0.75] <Insert value>.

2. Three-year, aged Solar Reflectance Index of not less than [64] <Insert value> when calculated according to ASTM E 1980.

E. Structural Performance: Provide metal panel systems capable of withstanding the effects of the following loads, based on testing according to ASTM E 1592:

1. Wind Loads: As indicated on Drawings.

2. Other Design Loads: [As indicated on Drawings] <Insert loads>.

3. Deflection Limits: For wind loads, no greater than [1/180] [1/240] <Insert deflection> of the span.

4. <Insert serviceability requirements>.

Retain "Air Infiltration" and "Water Penetration under Static Pressure" paragraphs below for single-skin metal panels that span openings between supports; usually delete for panels mounted on solid substrates.

ASTM E 1680 in "Air Filtration" Paragraph below has replaced ASTM E 283 for testing metal roof panels; retain option to allow products to be tested according to ASTM E 283.

F. Air Infiltration: Air leakage of not more than 0.06 cfm/sq. ft. (0.3 L/s per sq. m) when tested according to ASTM E 1680[ or ASTM E 283] at the following test-pressure difference:

Value in first option in "Test-Pressure Difference" Subparagraph below is equivalent to a 25-mph (40-km/h) wind and is ASTM E 1680 default. Products tested to value in second option below, equivalent to a 50-mph (80-km/h) wind, are widely available. Revise to suit Project.

1. Test-Pressure Difference: [1.57 lbf/sq. ft. (75 Pa)] [6.24 lbf/sq. ft. (300 Pa)].

ASTM E 1646 in "Water Penetration under Static Pressure" Paragraph below has replaced ASTM E 331 for testing metal roof panels; retain option to allow products to be tested according to ASTM E 331.

G. Water Penetration under Static Pressure: No water penetration when tested according to ASTM E 1646[ or ASTM E 331] at the following test-pressure difference:

Value in first option in "Test-Pressure Difference" Subparagraph below is equivalent to a 34-mph (55-km/h) wind and is ASTM E 1646 default. Products tested to value in second option below, equivalent to a 50-mph wind (80-km/h), are widely available. Revise to suit Project.

1. Test-Pressure Difference: [2.86 lbf/sq. ft. (137 Pa)] [6.24 lbf/sq. ft. (300 Pa)].

Test in "Hydrostatic-Head Resistance" Paragraph below is for resistance to hydrostatic pressure from standing water. Retain only for panels capable of watertight (hydrostatic) installation, not for panels used in water-shedding (hydrokinetic) installations that rely on underlayment for watertightness.

H. Hydrostatic-Head Resistance: No water penetration when tested according to ASTM E 2140.

Retain "Wind-Uplift Resistance" Paragraph below if UL-class roof is required. Verify that product is listed in UL's "Roofing Materials & Systems Directory." UL listings include requirements for the entire assembly and not solely the metal roof panels.

I. Wind-Uplift Resistance: Provide metal roof panel assemblies that comply with UL 580 for wind-uplift-resistance class indicated.

The higher the value in the options in "Uplift Rating" Subparagraph below, the greater the uplift resistance.

1. Uplift Rating: [UL 30] [UL 60] [UL 90].

Retain "FM Global Listing" Paragraph below if Project is FM Global insured or if FM Global requirements set a minimum quality standard. Coordinate requirements of FM Global classification with other requirements in this Section. Verify availability of metal roof panel systems that meet these classifications.

J. FM Global Listing: Provide metal roof panels and component materials that comply with requirements in FM Global 4471 as part of a panel roofing system and that are listed in FM Global's "Approval Guide" for Class 1 or noncombustible construction, as applicable. Identify materials with FM Global markings.

Retain one option in "Fire/Windstorm Classification" Subparagraph below based on windstorm classification of Project; the higher the value in the option, the greater the uplift resistance. FM Global Loss Prevention Data Sheet 1-28 multiplies the actual field-of-roof uplift pressure by a factor of 2 to obtain the factored pressure, the number that establishes the minimum FM Global approval rating. Verify availability of roofing systems that meet these classifications. Other options for classifications increase in increments of 15 (e.g., Class 1A-135, Class 1A-150, Class 1A-165, and higher). "Class 1A" signifies complying with ASTM E 108, Class A fire performance for FM Global-approved, Class 1 panel roofs.

1. Fire/Windstorm Classification: Class 1A-[60] [75] [90] [105] [120] <Insert number>.

Retain one option in "Hail Resistance" Subparagraph below. For areas that experience three or more hailstorms annually, FM Global recommends roofing systems rated SH (severe hail) instead of MH (moderate hail).

2. Hail Resistance: [MH] [SH].

K. Thermal Movements: Allow for thermal movements from ambient and surface temperature changes by preventing buckling, opening of joints, overstressing of components, failure of joint sealants, failure of connections, and other detrimental effects. Base calculations on surface temperatures of materials due to both solar heat gain and nighttime-sky heat loss.

Differential values (for aluminum in particular) in "Temperature Change (Range)" Subparagraph below are suitable for most of the U.S.; revise to suit Project.

1. Temperature Change (Range): [120 deg F (67 deg C), ambient; 180 deg F (100 deg C), material surfaces] <Insert temperature range>.

2.2 EXPOSED-FASTENER, LAP-SEAM, METAL ROOF PANELS

A. General: Provide factory-formed metal roof panels designed to be installed by lapping side edges of adjacent panels and mechanically attaching panels to supports using exposed fasteners in side laps. Include accessories required for weathertight installation.

Copy paragraphs below and re-edit for each product.

Insert drawing designation for each product required. Use these designations on Drawings to identify each product.

B. Corrugated-Profile, Exposed-Fastener Metal Roof Panels <Insert drawing designation>: Formed with alternating curved ribs spaced at [2.67 inches (68 mm)] <Insert dimension> o.c. across width of panel.

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

1. Manufacturers: Subject to compliance with requirements, [provide products by the following] [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to identify a specific product or a comparable product from manufacturers listed. Retain option and delete insert note if manufacturer's name and model number are indicated on Drawings.

2. Basis-of-Design Product: Subject to compliance with requirements, provide [product indicated on Drawings] <Insert manufacturer's name; product name or designation> or comparable product by one of the following:

a. AEP Span; a BlueScope Steel company.

b. Alcoa Inc.

c. CENTRIA Architectural Systems.

d. Fabral.

e. Firestone Metal Products, LLC.

f. Flexospan Steel Buildings, Inc.

g. MBCI; a division of NCI Building Systems, L.P.
h. McElroy Metal, Inc.
i. Metal Sales Manufacturing Corporation.

j. Morin; a Kingspan Group company.

k. Union Corrugating Company.

l. VICWEST.

m. <Insert manufacturer's name>.

3. Metallic-Coated Steel Sheet: Zinc-coated (galvanized) steel sheet complying with ASTM A 653/A 653M, G90 (Z275) coating designation, or aluminum-zinc alloy-coated steel sheet complying with ASTM A 792/A 792M, Class AZ50 (Class AZM150) coating designation; structural quality. Prepainted by the coil-coating process to comply with ASTM A 755/A 755M.

a. Nominal Thickness: [0.022 inch (0.56 mm)] [0.028 inch (0.71 mm)] [0.034 inch (0.86 mm)] [0.040 inch (1.02 mm)] [0.052 inch (1.32 mm)].

b. Exterior Finish: [Two-coat fluoropolymer] [Three-coat fluoropolymer] [Siliconized polyester] <Insert finish>.

c. Color: [As indicated by manufacturer's designations] [Match Architect's samples] [As selected by Architect from manufacturer's full range] <Insert color>.

4. Aluminum Sheet: Coil-coated sheet, ASTM B 209 (ASTM B 209M), alloy as standard with manufacturer, with temper as required to suit forming operations and structural performance required.

a. Thickness: [0.032 inch (0.81 mm)] [0.040 inch (1.02 mm)].

b. Surface: [Smooth, flat] [Embossed] finish.

c. Exterior Finish: [Two-coat fluoropolymer] [Three-coat fluoropolymer] [Siliconized polyester] <Insert finish>.

d. Color: [As indicated by manufacturer's designations] [Match Architect's samples] [As selected by Architect from manufacturer's full range] <Insert color>.

5. Panel Coverage: [21.3 inches (541 mm)] [29.3 inches (744 mm)] [34.67 inches (881 mm)] [37.3 inches (947 mm)] [42.67 inches (1084 mm)] [45.3 inches (1151 mm)] <Insert dimension>.

6. Panel Height: [0.5 inch (13 mm)] [0.875 inch (22 mm)] <Insert dimension>.

C. Tapered-Rib-Profile, Exposed-Fastener Metal Roof Panels <Insert drawing designation>: Formed with raised, trapezoidal major ribs and [intermediate stiffening ribs symmetrically spaced] [flat pan] between major ribs.

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

1. Manufacturers: Subject to compliance with requirements, [provide products by the following] [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to identify a specific product or a comparable product from manufacturers listed. Retain option and delete insert note if manufacturer's name and model number are indicated on Drawings.

2. Basis-of-Design Product: Subject to compliance with requirements, provide [product indicated on Drawings] <Insert manufacturer's name; product name or designation> or comparable product by one of the following:

a. AEP Span; a BlueScope Steel company.

b. Architectural Metal Systems; a Nucor company.

c. Berridge Manufacturing Company.

d. Butler Manufacturing; a BlueScope Steel company.

e. CENTRIA Architectural Systems.

f. Fabral.

g. Firestone Metal Products, LLC.

h. Flexospan Steel Buildings, Inc.

i. MBCI; a division of NCI Building Systems, L.P.
j. McElroy Metal, Inc.

k. Metal Sales Manufacturing Corporation.

l. Morin; a Kingspan Group company.

m. Petersen Aluminum Corporation.

n. Union Corrugating Company.

o. VICWEST.

p. <Insert manufacturer's name>.

3. Metallic-Coated Steel Sheet: Zinc-coated (galvanized) steel sheet complying with ASTM A 653/A 653M, G90 (Z275) coating designation, or aluminum-zinc alloy-coated steel sheet complying with ASTM A 792/A 792M, Class AZ50 (Class AZM150) coating designation; structural quality. Prepainted by the coil-coating process to comply with ASTM A 755/A 755M.

a. Nominal Thickness: [0.022 inch (0.56 mm)] [0.028 inch (0.71 mm)] [0.034 inch (0.86 mm)] [0.040 inch (1.02 mm)] [0.052 inch (1.32 mm)].

b. Exterior Finish: [Two-coat fluoropolymer] [Three-coat fluoropolymer] [Siliconized polyester] <Insert finish>.

c. Color: [As indicated by manufacturer's designations] [Match Architect's samples] [As selected by Architect from manufacturer's full range] <Insert color>.

4. Aluminum Sheet: Coil-coated sheet, ASTM B 209 (ASTM B 209M), alloy as standard with manufacturer, with temper as required to suit forming operations and structural performance required.

a. Thickness: [0.032 inch (0.81 mm)] [0.040 inch (1.02 mm)].

b. Surface: [Smooth, flat] [Embossed] finish.

c. Exterior Finish: [Two-coat fluoropolymer] [Three-coat fluoropolymer] [Siliconized polyester] <Insert finish>.

d. Color: [As indicated by manufacturer's designations] [Match Architect's samples] [As selected by Architect from manufacturer's full range] <Insert color>.

5. Major-Rib Spacing: [6 inches (152 mm)] [8 inches (203 mm)] [9 inches (229 mm)] [12 inches (305 mm)] <Insert dimension> o.c.

6. Panel Coverage: [24 inches (610 mm)] [36 inches (914 mm)] <Insert dimension>.

7. Panel Height: [0.625 inch (16 mm)] [0.75 inch (19 mm)] [1.0 inch (25 mm)] [1.25 inches (32 mm)] [1.5 inches (38 mm)] <Insert dimension>.

D. Vee-Rib-Profile, Exposed-Fastener Metal Roof Panels <Insert drawing designation>: Formed with raised, V-shaped ribs and recesses that are approximately same size, evenly spaced across panel width, and with rib/recess sides angled at approximately 45 degrees.

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

1. Manufacturers: Subject to compliance with requirements, [provide products by the following] [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to identify a specific product or a comparable product from manufacturers listed. Retain option and delete insert note if manufacturer's name and model number are indicated on Drawings.

2. Basis-of-Design Product: Subject to compliance with requirements, provide [product indicated on Drawings] <Insert manufacturer's name; product name or designation> or comparable product by one of the following:

a. AEP Span; a BlueScope Steel company.

b. Alcoa Inc.

c. Berridge Manufacturing Company.

d. CENTRIA Architectural Systems.

e. Fabral.

f. Flexospan Steel Buildings, Inc.

g. MBCI; a division of NCI Building Systems, L.P.
h. Metal Sales Manufacturing Corporation.

i. Morin; a Kingspan Group company.

j. Petersen Aluminum Corporation.

k. VICWEST.

l. <Insert manufacturer's name>.

3. Metallic-Coated Steel Sheet: Zinc-coated (galvanized) steel sheet complying with ASTM A 653/A 653M, G90 (Z275) coating designation, or aluminum-zinc alloy-coated steel sheet complying with ASTM A 792/A 792M, Class AZ50 (Class AZM150) coating designation; structural quality. Prepainted by the coil-coating process to comply with ASTM A 755/A 755M.

a. Nominal Thickness: [0.022 inch (0.56 mm)] [0.028 inch (0.71 mm)] [0.034 inch (0.86 mm)] [0.040 inch (1.02 mm)] [0.052 inch (1.32 mm)].

b. Exterior Finish: [Two-coat fluoropolymer] [Three-coat fluoropolymer] [Siliconized polyester] <Insert finish>.

c. Color: [As indicated by manufacturer's designations] [Match Architect's samples] [As selected by Architect from manufacturer's full range] <Insert color>.

4. Aluminum Sheet: Coil-coated sheet, ASTM B 209 (ASTM B 209M), alloy as standard with manufacturer, with temper as required to suit forming operations and structural performance required.

a. Thickness: [0.032 inch (0.81 mm)] [0.040 inch (1.02 mm)].

b. Surface: [Smooth, flat] [Embossed] finish.

c. Exterior Finish: [Two-coat fluoropolymer] [Three-coat fluoropolymer] [Siliconized polyester] <Insert finish>.

d. Color: [As indicated by manufacturer's designations] [Match Architect's samples] [As selected by Architect from manufacturer's full range] <Insert color>.

5. Rib Spacing: [5.3 inches (135 mm)] [7.2 inches (183 mm)] [12 inches (305 mm)] <Insert dimension> o.c.

6. Panel Coverage: [30 inches (762 mm)] [32 inches (813 mm)] [36 inches (914 mm)] [40 inches (1016 mm)] <Insert dimension>.

7. Panel Height: [1.375 inches (35 mm)] [1.5 inches (38 mm)] [1.75 inches (44 mm)] [2.0 inches (51 mm)] [3.0 inches (76 mm)] <Insert dimension>.

E. Box-Rib-Profile, Exposed-Fastener Metal Roof Panels <Insert drawing designation>: Formed with raised, box-shaped ribs that are wider than recesses, evenly spaced across panel width, and with rib/recess sides angled 60 degrees or more.

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

1. Manufacturers: Subject to compliance with requirements, [provide products by the following] [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to identify a specific product or a comparable product from manufacturers listed. Retain option and delete insert note if manufacturer's name and model number are indicated on Drawings.

2. Basis-of-Design Product: Subject to compliance with requirements, provide [product indicated on Drawings] <Insert manufacturer's name; product name or designation> or comparable product by one of the following:

a. Alcoa Inc.

b. Fabral.

c. MBCI; a division of NCI Building Systems, L.P.
d. Metal Sales Manufacturing Corporation.

e. <Insert manufacturer's name>.

3. Metallic-Coated Steel Sheet: Zinc-coated (galvanized) steel sheet complying with ASTM A 653/A 653M, G90 (Z275) coating designation, or aluminum-zinc alloy-coated steel sheet complying with ASTM A 792/A 792M, Class AZ50 (Class AZM150) coating designation; structural quality. Prepainted by the coil-coating process to comply with ASTM A 755/A 755M.

a. Nominal Thickness: [0.022 inch (0.56 mm)] [0.028 inch (0.71 mm)] [0.034 inch (0.86 mm)] [0.040 inch (1.02 mm)] [0.052 inch (1.32 mm)].

b. Exterior Finish: [Two-coat fluoropolymer] [Three-coat fluoropolymer] [Siliconized polyester] <Insert finish>.

c. Color: [As indicated by manufacturer's designations] [Match Architect's samples] [As selected by Architect from manufacturer's full range] <Insert color>.

4. Aluminum Sheet: Coil-coated sheet, ASTM B 209 (ASTM B 209M), alloy as standard with manufacturer, with temper as required to suit forming operations and structural performance required.

a. Thickness: [0.032 inch (0.81 mm)] [0.040 inch (1.02 mm)].

b. Surface: [Smooth, flat] [Embossed] finish.

c. Exterior Finish: [Two-coat fluoropolymer] [Three-coat fluoropolymer] [Siliconized polyester] <Insert finish>.

d. Color: [As indicated by manufacturer's designations] [Match Architect's samples] [As selected by Architect from manufacturer's full range] <Insert color>.

5. Rib Spacing: [2.67 inches (68 mm)] [4.0 inches (102 mm)] [5.3 inches (135 mm)] [6.0 inches (152 mm)] <Insert dimension> o.c.

6. Panel Coverage: [24 inches (610 mm)] [28 inches (711 mm)] [30 inches (762 mm)] [32 inches (813 mm)] [36 inches (914 mm)] <Insert dimension>.

7. Panel Height: [0.625 inch (16 mm)] [1.0 inch (25 mm)] [1.5 inches (38 mm)] [2.0 inches (51 mm)] <Insert dimension>.

F. Deep-Box-Rib-Profile, Exposed-Fastener Metal Roof Panels <Insert drawing designation>: Formed with raised, box-shaped ribs that are wider than recesses, evenly spaced across panel width, and with rib/recess sides angled more than 60 degrees.

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

1. Manufacturers: Subject to compliance with requirements, [provide products by the following] [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to identify a specific product or a comparable product from manufacturers listed. Retain option and delete insert note if manufacturer's name and model number are indicated on Drawings.

2. Basis-of-Design Product: Subject to compliance with requirements, provide [product indicated on Drawings] <Insert manufacturer's name; product name or designation> or comparable product by one of the following:

a. CENTRIA Architectural Systems.

b. Fabral.

c. Metal Sales Manufacturing Corporation.

d. Morin; a Kingspan Group company.

e. <Insert manufacturer's name>.

3. Metallic-Coated Steel Sheet: Zinc-coated (galvanized) steel sheet complying with ASTM A 653/A 653M, G90 (Z275) coating designation, or aluminum-zinc alloy-coated steel sheet complying with ASTM A 792/A 792M, Class AZ50 (Class AZM150) coating designation; structural quality. Prepainted by the coil-coating process to comply with ASTM A 755/A 755M.

a. Nominal Thickness: [0.022 inch (0.56 mm)] [0.028 inch (0.71 mm)] [0.034 inch (0.86 mm)] [0.040 inch (1.02 mm)] [0.052 inch (1.32 mm)].

b. Exterior Finish: [Two-coat fluoropolymer] [Three-coat fluoropolymer] [Siliconized polyester] <Insert finish>.

c. Color: [As indicated by manufacturer's designations] [Match Architect's samples] [As selected by Architect from manufacturer's full range] <Insert color>.

4. Aluminum Sheet: Coil-coated sheet, ASTM B 209 (ASTM B 209M), alloy as standard with manufacturer, with temper as required to suit forming operations and structural performance required.

a. Thickness: [0.032 inch (0.81 mm)] [0.040 inch (1.02 mm)].

b. Surface: [Smooth, flat] [Embossed] finish.

c. Exterior Finish: [Two-coat fluoropolymer] [Three-coat fluoropolymer] [Siliconized polyester] <Insert finish>.

d. Color: [As indicated by manufacturer's designations] [Match Architect's samples] [As selected by Architect from manufacturer's full range] <Insert color>.

5. Rib Spacing: [12 inches (305 mm)] <Insert dimension> o.c.

6. Panel Coverage: [24 inches (610 mm)] <Insert dimension>.

7. Panel Height: [3 inches (76 mm)] [4 inches (102 mm)] <Insert dimension>.

2.3 UNDERLAYMENT MATERIALS

Retain appropriate paragraphs in this article for metal roof panels applied over sheathing. Self-adhering, high-temperature underlayment may be applied at eaves, valleys, and other areas vulnerable to leakage, with felt applied over the remainder of roof; or underlayment may be applied over the entire roof area.

Underlayments listed in "Self-Adhering, High-Temperature Underlayment" Paragraph below are suitable for higher temperatures associated with metal roofing. Verify, with underlayment manufacturer, acceptability of use on slopes less than 2:12.

A. Self-Adhering, High-Temperature Underlayment: Provide self-adhering, cold-applied, sheet underlayment, a minimum of 30 mils (0.76 mm) thick, specifically designed to withstand high metal temperatures beneath metal roofing. Provide primer when recommended by underlayment manufacturer.

1. Thermal Stability: Stable after testing at 220 deg F (111 deg C); ASTM D 1970.

2. Low-Temperature Flexibility: Passes after testing at minus 20 deg F (29 deg C); ASTM D 1970.

Retain "Products" Subparagraph and list of manufacturers and products below to require specific products or a comparable product from other manufacturers.

3. Products: Subject to compliance with requirements, [provide the following] [provide one of the following] [available products that may be incorporated into the Work include, but are not limited to, the following]:

a. Carlisle Residential, a division of Carlisle Construction Materials; WIP 300HT.

b. Grace Construction Products, a unit of W. R. Grace & Co.; [Grace Ice and Water Shield HT] [Ultra].

c. Henry Company; Blueskin PE200 HT.

d. Kirsch Building Products, LLC; Sharkskin Ultra SA.

e. Metal-Fab Manufacturing, LLC; MetShield.

f. Owens Corning; WeatherLock Specialty Tile and Metal Underlayment.

g. <Insert manufacturer's name; product name or designation>.

Retain "Felt Underlayment" Paragraph below if not using self-adhering, high-temperature underlayment over entire roof area.

B. Felt Underlayment: ASTM D 226/D 226M, Type II (No. 30), asphalt-saturated organic felts.

Sheet in "Slip Sheet" Paragraph below is used to separate metal roof panels from underlayment. Retain if required by manufacturer.

C. Slip Sheet: Manufacturer's recommended slip sheet, of type required for application.

2.4 MISCELLANEOUS MATERIALS

A. Miscellaneous Metal Subframing and Furring: ASTM C 645; cold-formed, metallic-coated steel sheet, ASTM A 653/A 653M, G90 (Z275 hot-dip galvanized) coating designation or ASTM A 792/A 792M, Class AZ50 (Class AZM150) aluminum-zinc-alloy coating designation unless otherwise indicated. Provide manufacturer's standard sections as required for support and alignment of metal panel system.

Retain panel accessories, flashing, and trim as required and coordinate with those specified in Section 076200 "Sheet Metal Flashing and Trim."

B. Panel Accessories: Provide components required for a complete, weathertight panel system including trim, copings, fasciae, mullions, sills, corner units, clips, flashings, sealants, gaskets, fillers, closure strips, and similar items. Match material and finish of metal panels unless otherwise indicated.

1. Closures: Provide closures at eaves and ridges, fabricated of same metal as metal panels.

2. Backing Plates: Provide metal backing plates at panel end splices, fabricated from material recommended by manufacturer.

3. Closure Strips: Closed-cell, expanded, cellular, rubber or crosslinked, polyolefin-foam or closed-cell laminated polyethylene; minimum 1-inch- (25-mm-) thick, flexible closure strips; cut or premolded to match metal panel profile. Provide closure strips where indicated or necessary to ensure weathertight construction.

C. Flashing and Trim: Provide flashing and trim formed from same material as metal panelsas required to seal against weather and to provide finished appearance. Locations include, but are not limited to, eaves, rakes, corners, bases, framed openings, ridges, fasciae, and fillers. Finish flashing and trim with same finish system as adjacent metal panels.

If required, insert special requirements for ridge closures, corner units, copings, fasciae, and fillers.

D. Gutters and Downspouts: Formed from same material as roof panels according to SMACNA's "Architectural Sheet Metal Manual." Finish to match [metal roof panels] [roof fascia and rake trim].

E. Roof Curbs: Fabricated from same material as roof panels, [0.048-inch- (1.2-mm-)] <Insert dimension> nominal thickness; with bottom of skirt profiled to match roof panel profiles and with welded top box and integral full-length cricket. Fabricate curb subframing of 0.060-inch- (1.52-mm-) nominal thickness, angle-, C-, or Z-shaped steel sheet. Fabricate curb and subframing to withstand indicated loads of size and height indicated. Finish roof curbs to match metal roof panels.

F. Panel Fasteners: Self-tapping screws designed to withstand design loads. Provide exposed fasteners with heads matching color of metal panels by means of plastic caps or factory-applied coating. Provide EPDM or PVC sealing washers for exposed fasteners.

G. Panel Sealants: Provide sealant types recommended by manufacturer that are compatible with panel materials, are nonstaining, and do not damage panel finish.

1. Sealant Tape: Pressure-sensitive, 100 percent solids, gray polyisobutylene compound sealant tape with release-paper backing; 1/2 inch (13 mm) wide and 1/8 inch (3 mm) thick.

2. Joint Sealant: ASTM C 920; as recommended in writing by metal panel manufacturer.

3. Butyl-Rubber-Based, Solvent-Release Sealant: ASTM C 1311.

2.5 FABRICATION

A. General: Fabricate and finish metal panels and accessories at the factory, by manufacturer's standard procedures and processes, as necessary to fulfill indicated performance requirements demonstrated by laboratory testing. Comply with indicated profiles and with dimensional and structural requirements.

Retain "On-Site Fabrication" Paragraph below to permit use of on-site, portable roll-forming equipment.

B. On-Site Fabrication: Subject to compliance with requirements of this Section, metal panels may be fabricated on-site using UL-certified, portable roll-forming equipment if panels are of same profile and warranted by manufacturer to be equal to factory-formed panels. Fabricate according to equipment manufacturer's written instructions and to comply with details shown.

C. Provide panel profile, including major ribs and intermediate stiffening ribs, if any, for full length of panel.

Retain first paragraph below if gaskets or sealants are factory installed.

D. Fabricate metal panel joints with factory-installed captive gaskets or separator strips that provide a weathertight seal and prevent metal-to-metal contact, and that minimize noise from movements.

E. Sheet Metal Flashing and Trim: Fabricate flashing and trim to comply with manufacturer's recommendations and recommendations in SMACNA's "Architectural Sheet Metal Manual" that apply to design, dimensions, metal, and other characteristics of item indicated.

2.6 FINISHES

A. Panels and Accessories:

Retain one exposed fluoropolymer or siliconized-polyester finish from subparagraphs below. Verify availability of finishes for products specified. If retaining more than one, indicate location of each on Drawings or by inserts. To obtain a proprietary finish system, insert names of coating manufacturers and products. Retain AAMA 621 for steel and AAMA 620 for aluminum.

1. Two-Coat Fluoropolymer: [AAMA 621] [AAMA 620]. Fluoropolymer finish containing not less than 70 percent PVDF resin by weight in color coat.

2. Three-Coat Fluoropolymer: [AAMA 621] [AAMA 620]. Fluoropolymer finish containing not less than 70 percent PVDF resin by weight in both color coat and clear topcoat.

3. Siliconized Polyester: Epoxy primer and silicone-modified, polyester-enamel topcoat; with a dry film thickness of not less than 0.2 mil (0.005 mm) for primer and 0.8 mil (0.02 mm) for topcoat.

Finish in "Concealed Finish" Subparagraph below is frequently used as a factory finish for interior surfaces.

4. Concealed Finish: White or light-colored acrylic or polyester backer finish.

PART 3 -  EXECUTION

3.1 PREPARATION

A. Miscellaneous Supports: Install subframing, furring, and other miscellaneous panel support members and anchorages according to ASTM C 754 and metal panel manufacturer's written recommendations.

3.2 UNDERLAYMENT INSTALLATION

Retain this article for metal roof panels applied over solid roof sheathing.

Retain "Self-Adhering Sheet Underlayment" or "Felt Underlayment" Paragraph below or retain both if required.

A. Self-Adhering Sheet Underlayment: Apply primer if required by manufacturer. Comply with temperature restrictions of underlayment manufacturer for installation. Apply at locations indicated [below] [on Drawings], wrinkle free, in shingle fashion to shed water, and with end laps of not less than 6 inches (152 mm) staggered 24 inches (610 mm) between courses. Overlap side edges not less than 3-1/2 inches (90 mm).[ Extend underlayment into gutter trough.] Roll laps with roller. Cover underlayment within 14 days.

Retain one of two subparagraphs below or delete if indicated on Drawings.

1. Apply over the entire roof surface.

2. Apply over the roof area indicated below:

Revise subparagraphs below to suit Project or delete if indicated on Drawings. If inserting dimensions, note that many self-adhering sheet underlayments are manufactured in 36-inch- (914-mm-) wide rolls.

a. Roof perimeter for a distance up from eaves of [24 inches (610 mm)] [36 inches (914 mm)] <Insert dimension> beyond interior wall line.

b. Valleys, from lowest point to highest point, for a distance on each side of [18 inches (460 mm)] <Insert dimension>. Overlap ends of sheets not less than 6 inches (152 mm).

c. Rake edges for a distance of [18 inches (460 mm)] <Insert dimension>.

d. Hips and ridges for a distance on each side of [12 inches (305 mm)] <Insert dimension>.

e. Roof-to-wall intersections for a distance from wall of [18 inches (460 mm)] <Insert dimension>.

f. Around dormers, chimneys, skylights, and other penetrating elements for a distance from element of [18 inches (460 mm)] <Insert dimension>.

B. Felt Underlayment: Apply at locations indicated [below] [on Drawings], in shingle fashion to shed water, and with lapped joints of not less than 2 inches (50 mm).

Retain one of two subparagraphs below or delete both if indicated on Drawings.

1. Apply over the entire roof surface.

2. Apply on roof not covered by self-adhering sheet underlayment. Lap over edges of self-adhering sheet underlayment not less than 3 inches (75 mm), in shingle fashion to shed water.

Retain "Slip Sheet" Paragraph below if required.

C. Slip Sheet: Apply slip sheet over underlayment before installing metal roof panels.

D. Flashings: Install flashings to cover underlayment to comply with requirements specified in Section 076200 "Sheet Metal Flashing and Trim."

3.3 METAL PANEL INSTALLATION

A. Lap-Seam Metal Panels: Fasten metal panels to supports with fasteners at each lapped joint at location and spacing recommended by manufacturer.

1. Lap ribbed or fluted sheets one full rib. Apply panels and associated items true to line for neat and weathertight enclosure.

2. Provide metal-backed washers under heads of exposed fasteners bearing on weather side of metal panels.

3. Locate and space exposed fasteners in uniform vertical and horizontal alignment. Use proper tools to obtain controlled uniform compression for positive seal without rupture of washer.

4. Install screw fasteners with power tools having controlled torque adjusted to compress washer tightly without damage to washer, screw threads, or panels. Install screws in predrilled holes.

5. Flash and seal panels with weather closures at perimeter of all openings.

Retain "Watertight Installation" Subparagraph below for hydrostatic (watertight) panel installation; delete if not required.

6. Watertight Installation:

a. Apply a continuous ribbon of sealant or tape to seal lapped joints of metal panels, using sealant or tape as recommend by manufacturer on side laps of nesting-type panels and elsewhere as needed to make panels watertight.

b. Provide sealant or tape between panels and protruding equipment, vents, and accessories.

c. At panel splices, nest panels with minimum 6-inch (152-mm) end lap, sealed with sealant and fastened together by interlocking clamping plates.

B. Accessory Installation: Install accessories with positive anchorage to building and weathertight mounting, and provide for thermal expansion. Coordinate installation with flashings and other components.

C. Flashing and Trim: Comply with performance requirements, manufacturer's written installation instructions, and SMACNA's "Architectural Sheet Metal Manual." Provide concealed fasteners where possible, and set units true to line and level. Install work with laps, joints, and seams that are permanently watertight.

3.4 CLEANING

A. Remove temporary protective coverings and strippable films, if any, as metal panels are installed, unless otherwise indicated in manufacturer's written installation instructions. On completion of metal panel installation, clean finished surfaces as recommended by metal panel manufacturer. Maintain in a clean condition during construction.

END OF SECTION 074113.13

	FORMED METAL ROOF PANELS
	074113.13 - 1



