Copyright 2012 AIA
MasterSpec Professional
05/12

Copyright 2012 by The American Institute of Architects (AIA)

Exclusively published and distributed by Architectural Computer Services, Inc. (ARCOM) for the AIA

SECTION 074616 - ALUMINUM SIDING
TIPS:
To view non-printing Editor's Notes that provide guidance for editing, click on Masterworks/Single‑File Formatting/Toggle/Editor's Notes.
To read detailed research, technical information about products and materials, and coordination checklists, click on Masterworks/Supporting Information.

Revise this Section by inserting or deleting text to meet Project-specific requirements.

This Section uses the term "Architect." Change this term to match that used to identify the design professional as defined in the General and Supplementary Conditions.

Verify that Section titles referenced in this Section are correct for this Project's Specifications; Section titles may have changed.

PART 1 -  GENERAL

1.1 SUMMARY

A. Section includes aluminum [siding] [and] [soffit].

1.2 ACTION SUBMITTALS

A. Product Data: For each type of product.

B. Samples: For aluminum [siding] [and] [soffit] including related accessories.

1.3 INFORMATIONAL SUBMITTALS

Retain "Product Certificates" Paragraph below to require submittal of product certificates from manufacturers.

A. Product certificates.

B. Research/evaluation reports.

C. Sample warranty.

1.4 CLOSEOUT SUBMITTALS

A. Maintenance data.

1.5 QUALITY ASSURANCE

A. Mockups: Build mockups to verify selections made under Sample submittals and to demonstrate aesthetic effects and to set quality standards for fabrication and installation.

Indicate portion of wall represented by mockup on Drawings or draw mockup as separate element.

1. Build mockup of typical wall area as shown on Drawings.

Retain subparagraph below if the intention is to make an exception to the default requirement in Section 014000 "Quality Requirements" for demolishing and removing mockups.

2. Subject to compliance with requirements, approved mockups may become part of the completed Work if undisturbed at time of Substantial Completion.

1.6 WARRANTY

When warranties are required, verify with Owner's counsel that warranties stated in this article are not less than remedies available to Owner under prevailing local laws.

A. Special Warranty: Manufacturer agrees to repair or replace products that fail in materials or workmanship within specified warranty period.

Verify available warranties and warranty periods for aluminum siding and soffit.

1. Warranty Period: [10] [25] [50] <Insert number> years from date of Substantial Completion.

PART 2 -  PRODUCTS

See Editing Instruction No. 1 in the Evaluations for cautions about named manufacturers and products. For an explanation of options and Contractor's product selection procedures, see Section 016000 "Product Requirements."

2.1 ALUMINUM SIDING

A. Aluminum Siding: Formed and coated product complying with AAMA 1402.

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

1. Manufacturers: Subject to compliance with requirements, [provide products by the following] [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to identify a specific product or a comparable product from manufacturers listed. Retain option and delete insert note if manufacturer's name and model number are indicated on Drawings.

2. Basis-of-Design Product: Subject to compliance with requirements, provide [product indicated on Drawings] <Insert manufacturer's name; product name or designation> or comparable product by one of the following:

a. Gentek Building Products, Inc.

b. Kaycan Ltd.

c. Mastic Home Exteriors.

d. Norandex; Building Materials Distribution, Inc.

e. Rollex Corporation.

f. <Insert manufacturer's name>.

Retain one or more of first four paragraphs below or revise to suit Project.

B. Horizontal Pattern: 8-inch (203-mm) exposure in [plain, single-board] [beaded-edge, single-board] [plain, double, 4-inch (102-mm) board] <Insert requirement> style.

C. Horizontal Pattern: 10-inch (254-mm) exposure in [plain,] [Dutch-lap,] <Insert requirement,> double, 5-inch (127-mm) board style.

D. Vertical Pattern: 12-inch (300-mm) exposure in board-and-batten, single-board style.

E. Vertical Pattern: 16-inch (400-mm) exposure in V-grooved, triple, 5-1/3-inch (135-mm) board style.

F. Texture: [Smooth] [Wood grain] <Insert requirement>.

G. Nominal Thickness: [0.019 inch (0.5 mm)] [0.024 inch (0.6 mm)] <Insert dimension>.

Retain "Insulation" Paragraph below if required.

H. Insulation: Manufacturer's standard integral insulation panels.

I. Finish: Manufacturer's standard [three-coat PVDF] [primer and baked-on acrylic] [primer and baked-on polyester] <Insert requirement>.

Retain one of three options in "Colors" Subparagraph below. If retaining first option, indicate colors in a separate schedule.

1. Colors: [As indicated by manufacturer's designations] [Match Architect's samples] [As selected by Architect from manufacturer's full range of colors].

2.2 ALUMINUM SOFFIT

A. Aluminum Soffit: Formed and coated product complying with AAMA 1402.

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

1. Manufacturers: Subject to compliance with requirements, [provide products by the following] [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to identify a specific product or a comparable product from manufacturers listed. Retain option and delete insert note if manufacturer's name and model number are indicated on Drawings.

2. Basis-of-Design Product: Subject to compliance with requirements, provide [product indicated on Drawings] <Insert manufacturer's name; product name or designation> or comparable product by one of the following:

a. Gentek Building Products, Inc.

b. Kaycan Ltd.

c. Mastic Home Exteriors.

d. Norandex; Building Materials Distribution, Inc.

e. Rollex Corporation.

f. <Insert manufacturer's name>.

Retain one of four "Pattern" paragraphs below or revise to suit Project.

B. Pattern: 6-inch (152-mm) exposure in V-grooved, single-board style.

C. Pattern: 10-inch (254-mm) exposure in V-grooved, double, 5-inch (127-mm) board style.

D. Pattern: 12-inch (300-mm) exposure in V-grooved, double, 6-inch (152-mm) board style.

E. Pattern: 16-inch (400-mm) exposure in V-grooved, [triple, 5-1/3-inch (135-mm)] [quadruple, 4-inch (102-mm)] <Insert requirement> board style.

F. Texture: [Smooth] [Wood grain] <Insert requirement>.

G. Ventilation: Provide [perforated] [unperforated] soffit[ unless otherwise indicated].

H. Nominal Thickness: [0.019 inch (0.5 mm)] [0.024 inch (0.6 mm)] <Insert dimension>.

I. Finish: Manufacturer's standard [three-coat PVDF] [primer and baked-on acrylic] [primer and baked-on polyester] <Insert requirement>.

Retain one of four options in "Colors" Subparagraph below. If retaining first option, indicate colors in a separate schedule.

1. Colors: [As indicated by manufacturer's designations] [Match Architect's samples] [As selected by Architect from manufacturer's full range of colors] [Match adjacent siding].

2.3 ACCESSORIES

Retain "Siding Accessories, General" Paragraph below if using aluminum or vinyl siding.

A. Siding Accessories, General: Provide starter strips, edge trim, outside and inside corner caps, and other items as recommended by siding manufacturer for building configuration.

1. Provide accessories [made from same material as] [matching color and texture of] adjacent siding unless otherwise indicated.

B. Aluminum Accessories: Where aluminum accessories are indicated, provide accessories complying with AAMA 1402.

1. Texture: [Smooth] [Wood grain] <Insert requirement>.

2. Nominal Thickness: [0.019 inch (0.5 mm)] [0.024 inch (0.6 mm)] <Insert dimension>.

3. Finish: Manufacturer's standard [three-coat PVDF] [primer and baked-on acrylic] [primer and baked-on polyester] <Insert requirement>.

Retain one of four options in "Colors for Decorative Accessories" Paragraph below. If retaining first option, indicate colors in a separate schedule.

C. Colors for Decorative Accessories: [As indicated by manufacturer's designations] [Match Architect's samples] [As selected by Architect from manufacturer's full range of colors] [Match adjacent siding].

D. Flashing: Provide aluminum flashing complying with Section 076200 "Sheet Metal Flashing and Trim" at window and door heads and where indicated.

1. Finish for Aluminum Flashing: [Same as aluminum siding] [Siliconized polyester coating, same color as siding] [High-performance organic finish, same color as siding] [Factory-prime coating] <Insert finish>.

E. Fasteners:

1. For fastening to wood, use [siding nails] [ribbed bugle-head screws] of sufficient length to penetrate a minimum of 1 inch (25 mm) into substrate.

2. For fastening to metal, use ribbed bugle-head screws of sufficient length to penetrate a minimum of 1/4 inch (6 mm), or three screw-threads, into substrate.

3. For fastening aluminum, use aluminum fasteners. Where fasteners are exposed to view, use prefinished aluminum fasteners in color to match item being fastened.

PART 3 -  EXECUTION

3.1 INSTALLATION

A. General: Comply with manufacturer's written installation instructions applicable to products and applications indicated unless more stringent requirements apply.

1. Center nails in elongated nailing slots without binding siding to allow for thermal movement.

B. Install aluminum [siding] [and] [soffit] and related accessories according to AAMA 1402.

1. Install fasteners no more than [24 inches (600 mm)] <Insert dimension> o.c.

C. Install joint sealants as specified in Section 079200 "Joint Sealants" and to produce a weathertight installation.

D. Where aluminum siding contacts dissimilar metals, protect against galvanic action by painting contact surfaces with primer or by applying sealant or tape or installing nonconductive spacers as recommended by manufacturer for this purpose.

3.2 ADJUSTING AND CLEANING

A. Remove damaged, improperly installed, or otherwise defective materials and replace with new materials complying with specified requirements.

B. Clean finished surfaces according to manufacturer's written instructions and maintain in a clean condition during construction.

END OF SECTION 074616

	ALUMINUM SIDING
	074616 - 1



