Copyright 2010 AIA
MasterSpec Professional
02/10

Copyright 2010 by The American Institute of Architects (AIA)

Exclusively published and distributed by Architectural Computer Services, Inc. (ARCOM) for the AIA

SECTION 101419 - DIMENSIONAL LETTER SIGNAGE
TIPS:
To view non-printing Editor's Notes that provide guidance for editing, click on Masterworks/Single‑File Formatting/Toggle/Editor's Notes.
To read detailed research, technical information about products and materials, and coordination checklists, click on Masterworks/Supporting Information.

Revise this Section by deleting and inserting text to meet Project-specific requirements.

This Section uses the term "Architect." Change this term to match that used to identify the design professional as defined in the General and Supplementary Conditions.

Verify that Section titles referenced in this Section are correct for this Project's Specifications; Section titles may have changed.

PART 1 -  GENERAL

1.1 SUMMARY

A. Section Includes:

1. Cast dimensional characters.

2. Cutout dimensional characters.

3. Fabricated channel dimensional characters.

4. Illuminated, fabricated channel dimensional characters.

5. Molded-plastic dimensional characters.

6. Illuminated, molded-plastic dimensional characters.

1.2 ACTION SUBMITTALS

A. Product Data: For each type of product.

B. LEED Submittals:

"Product Certificates for Credit MR 5" Subparagraph below applies to LEED-CI.

1. Product Certificates for Credit MR 5: For products and materials required to comply with requirements for regionally manufactured materials. Include statement indicating cost for each regionally manufactured material.

First subparagraph below applies to LEED-CI Credit MR 5, Option 1.

a. Include statement indicating location of manufacturer and distance to Project for each regionally manufactured material.

"Product Data for Credit IEQ 4.1" Subparagraph below applies to LEED-NC, LEED-CI, and LEED-CS; coordinate with requirements for adhesives.

2. Product Data for Credit IEQ 4.1: For adhesives, documentation including printed statement of VOC content.

"Laboratory Test Reports for Credit IEQ 4.1" Subparagraph below applies to LEED for Schools.

3. Laboratory Test Reports for Credit IEQ 4.1: For adhesives, documentation indicating that products comply with the testing and product requirements of the California Department of Health Services' "Standard Practice for the Testing of Volatile Organic Emissions from Various Sources Using Small-Scale Environmental Chambers."

C. Shop Drawings: For dimensional letter signs.

1. Include fabrication and installation details and attachments to other work.

2. Show sign mounting heights, locations of supplementary supports to be provided by others, and accessories.

3. Show message list, typestyles, graphic elements, and layout for each sign at least [half size] <Insert scale>.

Retain two subparagraphs below if signs are electrically powered.

4. Show locations of electrical service connections.

5. Include diagrams for power, signal, and control wiring.

D. Samples: For each exposed product and for each color and texture specified.

E. Sign Schedule: Use same designations specified or indicated on Drawings or in a sign schedule.

Retain "Delegated-Design Submittal" Paragraph below if design services have been delegated to Contractor.

F. Delegated-Design Submittal: For [signs indicated in "Performance Requirements" Article] <Insert sign designations>.

1. Include structural analysis calculations for signs indicated to comply with design loads; signed and sealed by the qualified professional engineer responsible for their preparation.

1.3 INFORMATIONAL SUBMITTALS

A. Sample warranty.

1.4 CLOSEOUT SUBMITTALS

A. Maintenance data.

1.5 WARRANTY

A. Special Warranty: Manufacturer agrees to repair or replace components of signs that fail in materials or workmanship within specified warranty period.

Verify available warranties and warranty periods for units and components.

1. Warranty Period: [Five] <Insert number> years from date of Substantial Completion.

PART 2 -  PRODUCTS

See Editing Instruction No. 1 in the Evaluations for cautions about named manufacturers and products. For an explanation of options and Contractor's product selection procedures, see Section 016000 "Product Requirements."

2.1 DIMENSIONAL LETTER SIGNS, GENERAL

Retain "Regional Materials" Paragraph below for LEED-CI Credit MR 5, Option 1; before retaining, verify availability of signs that comply.

A. Regional Materials: Dimensional letter signs shall be manufactured within 500 miles (800 km) of Project site.

2.2 PERFORMANCE REQUIREMENTS

Retain "Delegated Design" Paragraph below if Contractor is required to assume responsibility for design.

A. Delegated Design: Engage a qualified professional engineer, as defined in Section 014000 "Quality Requirements," to design sign structure and anchorage of [rooftop] [dimensional character] <Insert description> sign type(s) <Insert drawing designation of sign(s)> to withstand design loads [as indicated on Drawings] <Insert loads>.

B. Thermal Movements: For exterior [fabricated channel dimensional characters] <Insert item>, allow for thermal movements from ambient and surface temperature changes.

Differential values in "Temperature Change" Subparagraph below (for aluminum in particular) are suitable for most of the U.S.

1. Temperature Change: [120 deg F (67 deg C), ambient; 180 deg F (100 deg C), material surfaces] <Insert temperature change>.

C. Electrical Components, Devices, and Accessories: Listed and labeled as defined in NFPA 70, by a qualified testing agency, and marked for intended location and application.

2.3 DIMENSIONAL CHARACTERS

Retain this article for signs consisting of multiple, individually mounted characters.

Copy "Cast Characters," "Cutout Characters," "Fabricated Channel Characters," and "Molded-Plastic Characters" paragraphs below and re-edit for each product.

Insert number to complete drawing designation for each product required. Use these designations on Drawings to identify each product.

A. Cast Characters <Insert drawing designation>: Characters with uniform faces, sharp corners, and precisely formed lines and profiles, and as follows:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

1. Manufacturers: Subject to compliance with requirements, [provide products by the following] [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to identify a specific product or a comparable product from manufacturers listed. Retain option and delete insert note if manufacturer's name and model number are indicated on Drawings.

2. Basis-of-Design Product: Subject to compliance with requirements, provide [product indicated on Drawings] <Insert manufacturer's name; product name or designation> or comparable product by one of the following:

a. ACE Sign Systems, Inc.

b. Allen Markings International.

c. APCO Graphics, Inc.

d. A. R. K. Ramos Signage Systems.

e. ASI Sign Systems, Inc.

f. Diskey Sign Company.

g. Gemini Incorporated.

h. Matthews International Corporation; Bronze Division.

i. Metal Arts; Division of L & H Mfg. Co.

j. Metallic Arts.

k. Seton Identification Products.

l. Southwell Company (The).

m. <Insert manufacturer's name>.

3. Character Material: Cast [aluminum] [brass] [bronze] [zinc] <Insert material>.

4. Character Height: [As indicated] <Insert dimension>.

5. Finishes:

Retain "Integral Metal Finish," "Integral Aluminum Finish," "Baked-Enamel or Powder-Coat Finish," and "Overcoat" subparagraphs below as required.

Retain "Integral Metal Finish" Subparagraph for integral finishes on metals other than aluminum.

a. Integral Metal Finish: [Mill] [Antique oxidized] [As indicated by manufacturer's designation] [Match Architect's sample] [As selected by Architect from full range of industry finishes] <Insert finish>.

Retain "Integral Aluminum Finish" Subparagraph below for integral finishes on aluminum. Some manufacturers do not recommend anodized finishes on sand-cast aluminum.

b. Integral Aluminum Finish: [Clear anodized] [Light bronze anodized] [Medium bronze anodized] [Match Architect's sample] [Anodized color as selected by Architect from full range of industry colors and color densities] <Insert finish>.

Retain "Baked-Enamel or Powder-Coat Finish" Subparagraph below for applied finish on metals; revise to suit Project.

c. Baked-Enamel or Powder-Coat Finish: Manufacturer's standard, in color [as indicated by manufacturer's designation] [matching Architect's sample] [as selected by Architect from manufacturer's full range] <Insert color>.

Retain "Overcoat" Subparagraph below if required.

d. Overcoat: [Manufacturer's standard baked-on clear coating] <Insert requirement>.

Retain one option in "Mounting" Subparagraph below; revise to suit Project.

6. Mounting: [As indicated] [Concealed studs] [Projecting studs] [Rosette-head through fasteners] [Countersunk flathead through fasteners] <Insert requirement>.

B. Cutout Characters <Insert drawing designation>: Characters with uniform faces; square-cut, smooth[, eased] edges; precisely formed lines and profiles; and as follows:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

1. Manufacturers: Subject to compliance with requirements, [provide products by the following] [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to identify a specific product or a comparable product from manufacturers listed. Retain option and delete insert note if manufacturer's name and model number are indicated on Drawings.

2. Basis-of-Design Product: Subject to compliance with requirements, provide [product indicated on Drawings] <Insert manufacturer's name; product name or designation> or comparable product by one of the following:

a. ACE Sign Systems, Inc.

b. APCO Graphics, Inc.

c. A. R. K. Ramos Signage Systems.

d. ASI Sign Systems, Inc.

e. Charleston Industries, Inc.

f. Diskey Sign Company.

g. Gemini Incorporated.

h. InPro Corporation.

i. Matthews International Corporation; Bronze Division.

j. Metal Arts; Division of L & H Mfg. Co.

k. Metallic Arts.

l. Nelson-Harkins Industries.

m. Southwell Company (The).

n. Steel Art Company.

o. <Insert manufacturer's name>.

3. Character Material: Sheet or plate [aluminum] [brass] [bronze] [copper] [stainless steel] [zinc] [acrylic] <Insert material>.

4. Character Height: [As indicated] <Insert dimension>.

5. Thickness: [As indicated] [Manufacturer's standard for size of character] [0.125 inch (3.18 mm)] [0.25 inch (6.35 mm)] <Insert dimension>.

6. Finishes:

Retain "Integral Metal Finish," "Integral Aluminum Finish," "Integral Stainless-Steel Finish, "Integral Acrylic Color," "Baked-Enamel or Powder-Coat Finish," and "Overcoat" subparagraphs below as required.

Retain "Integral Metal Finish" Subparagraph for integral finishes on metals other than aluminum and stainless steel.

a. Integral Metal Finish: [Mill] [Antique oxidized] [As indicated by manufacturer's designation] [Match Architect's sample] [As selected by Architect from full range of industry finishes] <Insert finish>.

Retain "Integral Aluminum Finish" Subparagraph below for integral finishes on aluminum.

b. Integral Aluminum Finish: [Clear anodized] [Light bronze anodized] [Medium bronze anodized] [Match Architect's sample] [Anodized color as selected by Architect from full range of industry colors and color densities] <Insert finish>.

Retain "Integral Stainless-Steel Finish" Subparagraph below for integral finishes on stainless steel.

c. Integral Stainless-Steel Finish: [No. 4] [No. 8] [Match Architect's sample] [As selected by Architect from full range of industry finishes] <Insert description>.

d. Integral Acrylic Color: [As indicated by manufacturer's designation] [Match Architect's sample] [As selected by Architect from full range of industry colors] <Insert color>.

Retain "Baked-Enamel or Powder-Coat Finish" Subparagraph below for applied finish on metals; revise to suit Project.

e. Baked-Enamel or Powder-Coat Finish: Manufacturer's standard, in color [as indicated by manufacturer's designation] [matching Architect's sample] [as selected by Architect from manufacturer's full range] <Insert color>.

Retain "Overcoat" Subparagraph below if required.

f. Overcoat: [Manufacturer's standard baked-on clear coating] <Insert requirement>.

Retain one option in "Mounting" Subparagraph below; revise to suit Project.

7. Mounting: [As indicated] [Concealed studs] [Projecting studs] [Rosette-head through fasteners] [Countersunk flathead through fasteners] [Concealed, painted aluminum back bar or bracket assembly] [Concealed, stainless-steel back bar or bracket assembly] [Adhesive] <Insert requirement>.

C. Fabricated Channel Characters <Insert drawing designation>: [Metal face and side returns] [Open face with metal side returns] [Translucent face with metal side returns], formed free from warp and distortion; with uniform faces, sharp corners, and precisely formed lines and profiles; internally braced for stability and for securing fasteners; and as follows.

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

1. Manufacturers: Subject to compliance with requirements, [provide products by the following] [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to identify a specific product or a comparable product from manufacturers listed. Retain option and delete insert note if manufacturer's name and model number are indicated on Drawings.

2. Basis-of-Design Product: Subject to compliance with requirements, provide [product indicated on Drawings] <Insert manufacturer's name; product name or designation> or comparable product by one of the following:

a. ACE Sign Systems, Inc.

b. Allen Industries, Inc.; Architectural Division.

c. APCO Graphics, Inc.

d. A. R. K. Ramos Signage Systems.

e. ASI Sign Systems, Inc.

f. Diskey Sign Company.

g. Gemini Incorporated.

h. Metallic Arts.

i. Nelson-Harkins Industries.

j. Poblocki Sign Company, LLC.

k. Steel Art Company.

l. <Insert manufacturer's name>.

Retain "Illuminated Characters" Subparagraph below if required.

3. Illuminated Characters: [Backlighted] [Frontlighted] character construction with [fluorescent tube] [fiber-optic] [LED] [neon tube] <Insert requirement> lighting including transformers, insulators, and other accessories for operability, with provision for servicing and concealing connections to building electrical system. Use tight or sealed joint construction to prevent unintentional light leakage. Space lamps apart from each other and away from character surfaces as needed to illuminate evenly.

a. Power: [As indicated on electrical Drawings] [120 V, 60 Hz, 1 phase, 15 A] <Insert requirement>.

4. Character Material: Sheet or plate [aluminum] <Insert material>.

Retain "Translucent Face Sheet" Subparagraph below for translucent-face characters.

5. Translucent Face Sheet: Acrylic sheet with integral color [matching Architect's sample] [as selected by Architect from manufacturer's full range] <Insert color>.

6. Character Height: [As indicated] <Insert dimension>.

7. Character Depth: [As indicated] <Insert dimension>.

8. Finishes:

Retain "Integral Aluminum Finish" and "Baked-Enamel or Powder-Coat Finish" subparagraphs below as required.

Retain "Integral Aluminum Finish" Subparagraph for integral finishes on aluminum.

a. Integral Aluminum Finish: [Clear anodized] [Light bronze anodized] [Medium bronze anodized] [Match Architect's sample] [Anodized color as selected by Architect from full range of industry colors and color densities] <Insert finish>.

Retain "Baked-Enamel or Powder-Coat Finish" Subparagraph below for applied finish on metals; revise to suit Project.

b. Baked-Enamel or Powder-Coat Finish: Manufacturer's standard, in color [as indicated by manufacturer's designation] [matching Architect's sample] [as selected by Architect from manufacturer's full range] <Insert color>.

Retain one option in "Mounting" Subparagraph below; revise to suit Project.

9. Mounting: [As indicated] [Manufacturer's standard for size and design of character] [Projecting studs] [Concealed, painted aluminum back bar or bracket assembly] [Concealed, stainless-steel back bar or bracket assembly] <Insert requirement>.

Retain subparagraph below for characters projecting from wall surface.

a. Hold characters at [2-inch (51-mm) distance] [manufacturer's recommended distance] [distance as selected by Architect] <Insert dimension> from wall surface.

D. Molded-Plastic Characters <Insert drawing designation>: Characters having uniform faces and profiles, and as follows:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

1. Manufacturers: Subject to compliance with requirements, [provide products by the following] [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to identify a specific product or a comparable product from manufacturers listed. Retain option and delete insert note if manufacturer's name and model number are indicated on Drawings.

2. Basis-of-Design Product: Subject to compliance with requirements, provide [product indicated on Drawings] <Insert manufacturer's name; product name or designation> or comparable product by one of the following:

a. ACE Sign Systems, Inc.

b. ASI Sign Systems, Inc.

c. Diskey Sign Company.

d. Gemini Incorporated.

e. Metallic Arts.

f. <Insert manufacturer's name>.

Retain "Illuminated Characters" Subparagraph below if required.

3. Illuminated Characters: Characters with concealed [fluorescent tube] [fiber-optic] [LED] [neon tube] lighting including transformers, insulators, and other accessories; with provision for servicing and concealing connections to building electrical system. Space lamps apart from each other and away from character surfaces as needed to illuminate evenly.

a. Power: [As indicated on electrical Drawings] [120 V, 60 Hz, 1 phase, 15 A] <Insert requirement>.

4. Color: Manufacturer's standard [integral color] [painted finish] process, in color [as indicated by manufacturer's designation] [matching Architect's sample] [as selected by Architect from manufacturer's full range] <Insert color>.

2.4 DIMENSIONAL CHARACTER MATERIALS

Alloy UNS No. C85200 in "Brass Castings" Paragraph below has 3 percent lead content. Verify availability of lead-free alloy with sign manufacturer before retaining first option.

A. Brass Castings: [Lead-free alloy] [Alloy UNS No. C85200 (high-copper yellow brass)] <Insert requirement>.

Alloy UNS No. C26000 (yellow brass) in "Brass Sheet (Yellow Brass)" Paragraph below has maximum 0.07 percent lead content. Verify availability of lead-free alloy with sign manufacturer before retaining first option.

B. Brass Sheet (Yellow Brass): [Lead-free alloy] [Alloy UNS No. C26000 (yellow brass)] <Insert requirement>.

Alloy UNS No. C86500 in "Bronze Castings" Paragraph below has no listed lead content. Verify availability of lead-free alloy with sign manufacturer before retaining first option.

C. Bronze Castings: [Lead-free alloy] [Alloy UNS No. C86500 (No. 1 manganese bronze)] <Insert requirement>.

Alloy UNS No. C22000 in "Bronze Plate" Paragraph below has a maximum 0.05 percent lead content. Verify availability of lead-free alloy with sign manufacturer before retaining first option.

D. Bronze Plate: [Lead-free alloy] [Alloy UNS No. C22000 (commercial bronze)] <Insert requirement>.

Generally retain one option in "Stainless-Steel Sheet" Paragraph below. Type 316 is more corrosion resistant and more expensive than Type 304.

E. Stainless-Steel Sheet: [Type 304] [Type 316].

F. Acrylic Sheet: ASTM D 4802, Type UVF (UV filtering).

2.5 ACCESSORIES

A. Fasteners and Anchors: Manufacturer's standard as required for secure anchorage of signage, noncorrosive and compatible with each material joined, and complying with the following:

1. Use concealed fasteners and anchors unless indicated to be exposed.

2. For exterior exposure, furnish [nonferrous-metal] [stainless-steel] [or] [hot-dip galvanized] <Insert requirement> devices unless otherwise indicated.

3. Exposed Metal-Fastener Components, General:

a. Fabricated from same basic metal and finish of fastened metal unless otherwise indicated.

4. Sign Mounting Fasteners:

a. Concealed Studs: Concealed (blind), threaded studs welded or brazed to back of sign material, screwed into back of sign assembly, or screwed into tapped lugs cast integrally into back of cast sign material, unless otherwise indicated.

b. Projecting Studs: Threaded studs with sleeve spacer, welded or brazed to back of sign material, screwed into back of sign assembly, or screwed into tapped lugs cast integrally into back of cast sign material, unless otherwise indicated.

c. Through Fasteners: Exposed metal fasteners matching sign finish, with type of head indicated, installed in predrilled holes.

Retain first "Adhesives" Paragraph below if required for LEED-NC, LEED-CI, or LEED-CS Credit IEQ 4.1. VOC content limit is that for multipurpose construction adhesives. Revise VOC content limit based on application; see Section 018113.13 "Sustainable Design Requirements - LEED for New Construction and Major Renovations," Section 018113.16 "Sustainable Design Requirements - LEED for Commercial Interiors," or Section 018113.19 "Sustainable Design Requirements - LEED for Core and Shell Development" for a list of adhesive types and corresponding VOC content limits.

B. Adhesives: As recommended by sign manufacturer and with a VOC content of [70] <Insert value> g/L or less for adhesives used inside the weatherproofing system and applied on-site when calculated according to 40 CFR 59, Subpart D (EPA Method 24).

Retain "Adhesives" Paragraph below if required for LEED for Schools Credit IEQ 4.1.

C. Adhesives: As recommended by sign manufacturer and that comply with the testing and product requirements of the California Department of Health Services' "Standard Practice for the Testing of Volatile Organic Emissions from Various Sources Using Small-Scale Environmental Chambers."

D. Bituminous Paint: Cold-applied asphalt emulsion complying with ASTM D 1187.

2.6 FABRICATION

A. General: Provide manufacturer's standard sign assemblies according to requirements indicated.

1. Mill joints to a tight, hairline fit. Form assemblies and joints exposed to weather to resist water penetration and retention.

2. Provide welds and brazes behind finished surfaces without distorting or discoloring exposed side. Clean exposed welded and brazed connections of flux, and dress exposed and contact surfaces.

3. Conceal connections if possible; otherwise, locate connections where they are inconspicuous.

4. Internally brace signs for stability and for securing fasteners.

5. Provide rebates, lugs, and brackets necessary to assemble components and to attach to existing work. Drill and tap for required fasteners. Use concealed fasteners where possible; use exposed fasteners that match sign finish.

6. Castings: Fabricate castings free of warp, cracks, blowholes, pits, scale, sand holes, and other defects that impair appearance or strength. Grind, wire brush, sandblast, and buff castings to remove seams, gate marks, casting flash, and other casting marks before finishing.

B. Brackets: Fabricate brackets, fittings, and hardware for bracket-mounted signs to suit sign construction and mounting conditions indicated. Modify manufacturer's standard brackets as required.

1. Aluminum Brackets: Factory finish brackets with baked-enamel or powder-coat finish [to match sign-background color] [to match Architect's sample] <Insert requirement> color unless otherwise indicated.

2. Stainless-Steel Brackets: Factory finish brackets [to match sign background] [to match Architect's sample] [with No. 4] <Insert finish> finish unless otherwise indicated.

PART 3 -  EXECUTION

3.1 INSTALLATION

Retain mounting methods in this article that coordinate with mounting requirements in sign-description paragraphs retained in Part 2.

A. General: Install signs using mounting methods indicated and according to manufacturer's written instructions.

1. Install signs level, plumb, true to line, and at locations and heights indicated, with sign surfaces free of distortion and other defects in appearance.

2. Before installation, verify that sign surfaces are clean and free of materials or debris that would impair installation.

3. Corrosion Protection: Coat concealed surfaces of exterior aluminum in contact with grout, concrete, masonry, wood, or dissimilar metals, with a heavy coat of bituminous paint.

B. Mounting Methods:

1. Concealed Studs: Using a template, drill holes in substrate aligning with studs on back of sign. Remove loose debris from hole and substrate surface.

a. Masonry Substrates: Fill holes with adhesive. Leave recess space in hole for displaced adhesive. Place sign in position and push until flush to surface, embedding studs in holes. Temporarily support sign in position until adhesive fully sets.

b. Thin or Hollow Surfaces: Place sign in position and flush to surface, install washers and nuts on studs projecting through opposite side of surface, and tighten.

2. Projecting Studs: Using a template, drill holes in substrate aligning with studs on back of sign. Remove loose debris from hole and substrate surface.

a. Masonry Substrates: Fill holes with adhesive. Leave recess space in hole for displaced adhesive. Place spacers on studs, place sign in position, and push until spacers are pinched between sign and substrate, embedding the stud ends in holes. Temporarily support sign in position until adhesive fully sets.

b. Thin or Hollow Surfaces: Place spacers on studs, place sign in position with spacers pinched between sign and substrate, and install washers and nuts on stud ends projecting through opposite side of surface, and tighten.

3. Through Fasteners: Drill holes in substrate using predrilled holes in sign as template. Countersink holes in sign if required. Place sign in position and flush to surface. Install through fasteners and tighten.

4. Back Bar and Brackets: Remove loose debris from substrate surface and install backbar or bracket supports in position so that signage is correctly located and aligned.

5. Adhesive: Clean bond-breaking materials from substrate surface and remove loose debris. Apply linear beads or spots of adhesive symmetrically to back of sign and of suitable quantity to support weight of sign after cure without slippage. Keep adhesive away from edges to prevent adhesive extrusion as sign is applied and to prevent visibility of cured adhesive at sign edges. Place sign in position, and push to engage adhesive. Temporarily support sign in position until adhesive fully sets.

C. Remove temporary protective coverings and strippable films as signs are installed.

END OF SECTION 101419

	DIMENSIONAL LETTER SIGNAGE
	101419 - 1



