Copyright 2011 AIA
MasterSpec Professional
11/11

Copyright 2011 by The American Institute of Architects (AIA)

Exclusively published and distributed by Architectural Computer Services, Inc. (ARCOM) for the AIA

SECTION 102239.13 - FOLDING GLASS-PANEL PARTITIONS
TIPS:
To view non-printing Editor's Notes that provide guidance for editing, click on Masterworks/Single‑File Formatting/Toggle/Editor's Notes.
To read detailed research, technical information about products and materials, and coordination checklists, click on Masterworks/Supporting Information.

Revise this Section by deleting and inserting text to meet Project-specific requirements.

This Section uses the term "Architect." Change this term to match that used to identify the design professional as defined in the General and Supplementary Conditions.

Verify that Section titles referenced in this Section are correct for this Project's Specifications; Section titles may have changed.

PART 1 -  GENERAL

1.1 SUMMARY

A. Section includes manually operated, glass-panel partitions.

1.2 PREINSTALLATION MEETINGS

A. Preinstallation Conference: Conduct conference at [Project site] <Insert location>.

1.3 ACTION SUBMITTALS

A. Product Data: For each type of product.

B. LEED Submittals:

1. Product Data for Credit MR 4: For products having recycled content, documentation indicating percentages by weight of postconsumer and preconsumer recycled content. Include statement indicating cost for each product having recycled content.

"Certificates for Credit MR 7" Subparagraph below applies to LEED-NC, LEED-CI, and LEED for Schools.

2. Certificates for Credit MR 7: Chain-of-custody certificates certifying that operable glass-panel partitions comply with forest certification and chain-of-custody requirements. Include statement indicating cost for each certified wood product.

C. Shop Drawings: For operable glass-panel partitions.

1. Include plans, elevations, sections, details, and attachments to other work.

2. Indicate stacking and operating clearances. Indicate location and installation requirements for hardware and track, blocking, and direction of travel.

D. Samples: For each exposed product and for each color and texture specified.

Retain "Delegated-Design Submittal" Paragraph below if design services have been delegated to Contractor.

E. Delegated-Design Submittal: For operable glass-panel partitions.

1. Include design calculations for seismic restraints.

1.4 INFORMATIONAL SUBMITTALS

Retain "Coordination Drawings" Paragraph below for situations where limited space necessitates maximum utilization for efficient installation of different components or if coordination is required for installation of products and materials by separate installers.

A. Coordination Drawings: Reflected ceiling plans, drawn to scale, and coordinated with each other, using input from installers of the items involved.

Revise "Setting Drawings" Paragraph below to suit Project.

B. Setting Drawings: For embedded items and cutouts required in other work[, including support-beam, mounting-hole template].

Retain "Seismic Qualification Certificates" Paragraph below if required by seismic criteria applicable to Project. See ASCE/SEI 7 for certification requirements for equipment and components.

C. Seismic Qualification Certificates: For operable glass-panel partitions, tracks, accessories, and components, from manufacturer.

D. Product certificates.

E. Product test reports.

F. Sample warranty.

1.5 CLOSEOUT SUBMITTALS

A. Operation and maintenance data.

1.6 QUALITY ASSURANCE

Retain "Manufacturer Qualifications" and "Vendor Qualifications" paragraphs below if required for LEED for wood products. Before retaining, verify that manufacturers comply.

A. Manufacturer Qualifications: A qualified manufacturer that is certified for chain of custody by an FSC-accredited certification body.

B. Vendor Qualifications: A vendor that is certified for chain of custody by an FSC-accredited certification body.

C. Installer Qualifications: An entity that employs installers and supervisors who are trained and approved by manufacturer.

1.7 WARRANTY

A. Special Warranty: Manufacturer agrees to repair or replace components of operable glass-panel partitions that fail in materials or workmanship within specified warranty period.

Verify available warranties and warranty periods.

1. Warranty Period: [Two] <Insert number> years from date of Substantial Completion.

PART 2 -  PRODUCTS

See Editing Instruction No. 1 in the Evaluations for cautions about named manufacturers and products. For an explanation of options and Contractor's product selection procedures, see Section 016000 "Product Requirements."

2.1 PERFORMANCE REQUIREMENTS

Retain "Delegated Design" Paragraph below if Contractor is required to assume responsibility for design.

A. Delegated Design: Engage a qualified professional engineer, as defined in Section 014000 "Quality Requirements," to design seismic bracing of tracks to structure above.

Retain "Seismic Performance" Paragraph below with "Seismic Qualification Certificates" Paragraph in "Informational Submittals" Article for projects requiring seismic design. Delete paragraph if performance requirements are indicated on Drawings. Model building codes and ASCE/SEI 7 establish criteria for buildings subject to earthquake motions. Coordinate requirements with structural engineer.

B. Seismic Performance: Operable glass-panel partitions shall withstand the effects of earthquake motions determined according to [ASCE/SEI 7] <Insert requirement>.

Retain subparagraph below to define the term "withstand" as it applies to this Project. Definition varies with type of building and occupancy and is critical to valid certification.

1. The term "withstand" means "the partition panels will remain in place without separation of any parts from the system when subjected to the seismic forces specified."

Retain "Acoustical Performance" Paragraph below only if required for Project. This may significantly limit available products. See the Operable Glass-Panel Partition Table in the Evaluations.

C. Acoustical Performance: Provide operable glass-panel partitions tested by a qualified testing agency for the following acoustical properties according to test methods indicated:

If retaining "Sound-Transmission Requirements" Subparagraph below, indicate ratings in Part 2 or on Drawings.

1. Sound-Transmission Requirements: Operable glass-panel partition assembly tested for laboratory sound-transmission loss performance according to ASTM E 90, determined by ASTM E 413, and rated for not less than the STC indicated.

Retain "Fire-Test-Response Characteristics" Paragraph below for wood-framed panels if the amount of exposed wood on the wall surface is significant (usually greater than 10 percent of the wall area). Revise as required by governing code for partition location, occupancy classification, and whether facility is sprinklered or not. Verify and insert requirements of authorities having jurisdiction.

D. Fire-Test-Response Characteristics: Provide wood-framed panels complying with one of the following as determined by testing identical products by UL or another testing and inspecting agency acceptable to authorities having jurisdiction:

1. Surface-Burning Characteristics: Comply with ASTM E 84 or UL 723; testing by a qualified testing agency.

a. Flame-Spread Index: [25] [75] <Insert value> or less.

b. Smoke-Developed Index: [450] <Insert value> or less.

2.2 OPERABLE GLASS PANELS

Retain fourth option in "Operable Glass Panels" Paragraph below for glass partitions with acoustical properties.

A. Operable Glass Panels: [Frameless aluminum] [Aluminum-framed] [Wood-framed] glass-panel partition system, including panels, [seals, ]suspension system, operators, and accessories.

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

1. Manufacturers: Subject to compliance with requirements, [provide products by the following] [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to identify a specific product or a comparable product from manufacturers listed. Retain option and delete insert note if manufacturer's name and model number are indicated on Drawings.

2. Basis-of-Design Product: Subject to compliance with requirements, provide [product indicated on Drawings] <Insert manufacturer's name; product name or designation> or comparable product by one of the following:

a. Hufcor, Inc.
b. KWIK-WALL Company.
c. Moderco Inc.
d. Modernfold, Inc.; a DORMA Group company.
e. NanaWall Systems, Inc.
f. <Insert manufacturer's name>.

B. Panel Operation: Manually operated, [individual] [paired] [continuously hinged] panels.

C. Panel Construction: As required to support panel from suspension components and with reinforcement for hardware attachment. Fabricate panels with tight hairline joints and concealed fasteners. Fabricate panels so finished in-place partition is rigid; level; plumb; aligned, with tight joints and uniform appearance; and free of bow, warp, twist, deformation, and surface and finish irregularities.

1. Factory-Glazed Fabrication: Glaze operable glass panels in the factory where practical and possible for applications indicated. Comply with manufacturer's written instructions and with requirements in Section 088000 "Glazing."

Retain one of two "Glass and Glazing" paragraphs below; revise to suit Project.

D. Glass and Glazing: Glass type <Insert description> as specified in Section 088000 "Glazing."

E. Glass and Glazing: As follows:

1. Glass: [Manufacturer's standard] [Custom] safety glass and glass assemblies as indicated and complying with [the following] [requirements in Section 088000 "Glazing" and as follows]:

Retain one or more of "Tempered Glass," "Tempered Patterned Glass," "Laminated Glass," and "Insulating-Glass Units" subparagraphs below; revise to suit Project. Verify availability of required options with manufacturer.

a. Tempered Glass: ASTM C 1048, Kind FT (fully tempered), Type I (transparent flat glass), [Class 1 (clear)] [Class 2 (tinted)], Quality-Q3.

b. Tempered Patterned Glass: ASTM C 1048, Kind FT (fully tempered), Type II (patterned flat glass), Class 1 (clear), Form 3 (patterned); and of quality, finish, and pattern specified.

c. Laminated Glass: ASTM C 1172.

1) Annealed Float Glass: ASTM C 1036, Type I (transparent flat glass), [Class 1 (clear)] [Class 2 (tinted)], Quality-Q3.

2) Patterned Glass: ASTM C 1036, Type II (patterned and wired flat glass), Class 1 (clear), Form 3 (patterned); and of quality, finish, and pattern specified.

Retain "Insulating-Glass Units" Subparagraph below if required for acoustical reasons.

d. Insulating-Glass Units: Factory-assembled units consisting of sealed lites of glass as indicated, separated by a dehydrated interspace, qualified according to ASTM E 2190, and complying with other requirements specified.

F. Dimensions: Fabricate operable glass-panel partitions to form an assembled system of dimensions indicated and verified by field measurements.

Retain "STC" Paragraph below for glass partitions with acoustical properties.

G. STC: Not less than [36] [44] <Insert number>.

Consider deleting "Panel Frame Materials" Paragraph below if products are adequately specified by performance requirements.

H. Panel Frame Materials:

Retain "Certified Wood" Subparagraph below if wood components of operable glass-panel partitions are required to be certified for LEED. An alternative method of dealing with this credit requirement is to retain requirement in Section 018113.13 "Sustainable Design Requirements - LEED for New Construction and Major Renovations," Section 018113.16 "Sustainable Design Requirements - LEED for Commercial Interiors," or Section 018113.23 "Sustainable Design Requirements - LEED for Schools" that gives Contractor the option and responsibility to determine how the credit requirement will be met. Manufacturers certifying products as "FSC Mixed Credit" do not have to use 100 percent certified wood in the products, but in their total production must use an amount equal to or greater than the percentage of their production that is labeled "FSC Mixed Credit."

1. Certified Wood: Wood for operable glass-panel partitions shall be certified as "FSC Pure"[ or "FSC Mixed Credit"] according to FSC STD-01-001, "FSC Principles and Criteria for Forest Stewardship," and to FSC STD-40-004, "FSC Standard for Chain of Custody Certification."

Retain "Recycled Content of Operable Glass-Panel Partitions" Subparagraph below to specify recycled content if applying for LEED Credit MR 4. An alternative method of dealing with these credit requirements is to retain requirement in Section 018113.13 "Sustainable Design Requirements - LEED for New Construction and Major Renovations," Section 018113.16 "Sustainable Design Requirements - LEED for Commercial Interiors," or Section 018113.23 "Sustainable Design Requirements - LEED for Schools" that gives Contractor the option and responsibility to determine how these credit requirements will be met.

2. Recycled Content of Operable Glass-Panel Partitions:

Retain "Recycled Content of Aluminum" and "Recycled Content of Glass" subparagraphs below to suit Project; verify available recycled content with manufacturer.

a. Recycled Content of Aluminum: Postconsumer recycled content plus one-half of preconsumer recycled content not less than <Insert number> percent by weight.

b. Recycled Content of Glass: Postconsumer recycled content plus one-half of preconsumer recycled content not less than <Insert number> percent by weight.

3. Aluminum: Alloy and temper recommended by aluminum producer and finisher for type of use, corrosion resistance, and finish indicated; manufacturer's standard strengths and thicknesses for type of use.

Retain "Wood Frame" Subparagraph below if required. Insert requirements describing fire-retardant-treated wood, if necessary, to suit Project and to comply with requirements of authorities having jurisdiction. Verify availability with manufacturer before retaining specific species.

4. Wood Frame: Clear, vertical-grain, straight, kiln-dried[, fire-retardant-treated] wood as follows:

a. Species: [Match Architect's sample] [As selected by Architect from manufacturer's full range] [Cherry] [Hemlock] [Maple] [Meranti] [Poplar] [Red oak] <Insert species>.

I. Panel Closure: [Manufacturer's standard unless otherwise indicated] <Insert requirement>.

J. Hardware: Manufacturer's standard as required to operate operable glass-panel partition and accessories; with decorative, protective finish.

Revise "Panel Frame Finishes" Paragraph below for each finish required. Verify, with manufacturer, the availability of each finish.

K. Panel Frame Finishes:

1. Exposed Metal: [Match Architect's sample] [As selected by Architect from manufacturer's full range] as follows:

a. Aluminum: [Clear anodized] [Light bronze anodized] [Medium bronze anodized] [Dark bronze anodized] [Black anodized] [Baked powder coating, black color] <Insert finish and color>.

b. Metal-Clad Aluminum: [Satin stainless steel] [Polished stainless steel] [Satin brass] [Polished brass] [Satin bronze] [Polished bronze] <Insert finish>.

Retain one of two "Wood Finish" subparagraphs below; revise to suit Project.

2. Wood Finish: [Match Architect's sample] [As selected by Architect from manufacturer's full range], as follows:

a. Type: [Transparent finish] [Transparent finish over stain] <Insert finish> over wood variety indicated.

3. Wood Finish: [As specified in Section 099300 "Staining and Transparent Finishing."] <Insert description.>

2.3 SEALS

A. General: Provide seals that produce operable glass-panel partitions complying with performance requirements and the following:

1. Seals made from materials and in profiles that minimize sound leakage.

2. Seals fitting tight at contact surfaces and sealing continuously between adjacent panels and between operable glass-panel partition perimeter and adjacent surfaces, when operable glass-panel partition is extended and closed.

2.4 SUSPENSION SYSTEMS

A. Tracks: Steel or aluminum [mounted directly to overhead structural support,] [with adjustable steel hanger rods for overhead support,] designed for operation, size, and weight of operable glass-panel partition indicated. Size track to support partition operation and storage without damage to suspension system, operable glass-panel partitions, or adjacent construction. Limit track deflection to no more than 0.10 inch (2.54 mm) between bracket supports. Provide a continuous system of track sections and accessories to accommodate configuration and layout indicated for partition operation and storage.

B. Carriers: Trolley system as required for configuration type, size, and weight of partition and for easy operation; with ball-bearing wheels.

C. Track Intersections, Switches, and Accessories: As required for operation, storage, track configuration, and layout indicated for operable glass-panel partitions, and compatible with partition assembly specified. Fabricate track intersections and switches from steel or aluminum.

2.5 ACCESSORIES

A. Pass Doors: Swinging door built into and matching panel [materials,] [construction,] [acoustical qualities,] finish and thickness, complete with frames and operating hardware. Hinges finished to match other exposed hardware.

Retain "Accessibility Standard" Subparagraph below to suit Project. Doors operated only by security personnel are exempt from cited standards.

1. Accessibility Standard: Fabricate doors to comply with applicable provisions in [ICC A117.1] [and] [the U.S. Architectural & Transportation Barriers Compliance Board's ADA-ABA Accessibility Guidelines for Buildings and Facilities] <Insert requirements of authorities having jurisdiction>.

2. Single Pass Door: [36 by 80 inches (914 by 2032 mm)] [36 by 84 inches (914 by 2134 mm)] <Insert dimensions>.

3. Double Pass Door: [72 by 80 inches (1829 by 2032 mm)] [72 by 84 inches (1829 by 2134 mm)] <Insert dimensions>.

4. Pass-Door Hardware: Equip pass door with the following:

Features in six subparagraphs below can be used with either a single or double pass door. Retain those required and revise to suit Project.

a. Door Seals: [Mechanically operated floor seal on panels containing pass doors] [Sweep floor seals].

b. Panic hardware.

c. Concealed door closer.

Retain "Latchset" Subparagraph or one of two "Lock" subparagraphs below.

d. Latchset: Passage set.

e. Lock: Key-operated lock with cylinder[, keyed to master key system,] operable from both sides of door. Include two keys per lock.

f. Lock: Deadlock to receive cylinder, operable from both sides of door. See [Section 087100 "Door Hardware"] [Section 087111 "Door Hardware (Descriptive Specification)"] for lock cylinder and keying requirements.

B. Storage Pocket Door: Full height at end of partition runs to conceal stacked partition; of same frame material, finish, thickness, and acoustical qualities as panels; complete with operating hardware[ and acoustical seals at soffit, floor, and jambs]. Hinges in finish to match other exposed hardware.

Retain one of three subparagraphs below.

1. Manufacturer's standard method to secure storage pocket door in closed position.

2. Rim Lock: Key-operated lock cylinder[, keyed to master key system,] to secure storage pocket door in closed position. Include two keys per lock.

Lock in "Rim Lock" Subparagraph below may be required for certain situations such as schools.

3. Rim Lock: Deadlock to receive cylinder, to secure storage pocket door in closed position. See [Section 087100 "Door Hardware"] [Section 087111 "Door Hardware (Descriptive Specification)"] for lock cylinder and keying requirements.

PART 3 -  EXECUTION

3.1 INSTALLATION

A. General: Comply with ASTM E 557 except as otherwise required by operable glass-panel partition manufacturer's written installation instructions.

B. Install operable glass-panel partitions and accessories after other finishing operations, including painting, have been completed in area of partition installation.

C. Broken, cracked, chipped, deformed, or unmatched panels are not acceptable.

D. Broken, cracked, deformed, or unmatched gasketing or gasketing with gaps at butted ends is not acceptable.

E. Adjust partitions for alignment and full closure of vertical joints and full closure along top and bottom seals.

F. Adjust [pass doors] [and] [storage pocket doors] to operate smoothly and easily, without binding or warping.

3.2 DEMONSTRATION

A. Engage a factory-authorized service representative to train Owner's maintenance personnel to adjust, operate, and maintain operable glass-panel partitions.

END OF SECTION 102239.13

	FOLDING GLASS-PANEL PARTITIONS
	102239.13 - 1



