Copyright 2011 AIA
MasterSpec Professional
02/11

Copyright 2011 by The American Institute of Architects (AIA)

Exclusively published and distributed by Architectural Computer Services, Inc. (ARCOM) for the AIA

SECTION 313116 - TERMITE CONTROL
TIPS:
To view non-printing Editor's Notes that provide guidance for editing, click on Masterworks/Single‑File Formatting/Toggle/Editor's Notes.
To read detailed research, technical information about products and materials, and coordination checklists, click on Masterworks/Supporting Information.

Revise this Section by deleting and inserting text to meet Project-specific requirements.

Verify that Section titles referenced in this Section are correct for this Project's Specifications; Section titles may have changed.

PART 1 -  GENERAL

1.1 SUMMARY

A. Section Includes:

Adjust list below to suit Project.

1. Soil treatment with termiticide.

2. Bait-station system.

1.2 ACTION SUBMITTALS

A. Product Data: For each type of product indicated. Include the EPA-Registered Label for termiticide products.

1.3 INFORMATIONAL SUBMITTALS

A. Product certificates.

B. Soil Treatment Application Report: Include the following:

1. Date and time of application.

2. Moisture content of soil before application.

3. Termiticide brand name and manufacturer.

4. Quantity of undiluted termiticide used.

5. Dilutions, methods, volumes used, and rates of application.

6. Areas of application.

7. Water source for application.

C. Bait-Station System Application Report: Include the following:

1. Location of areas and sites conducive to termite feeding and activity.

2. Plan drawing showing number and locations of bait stations.

3. Dated report for each monitoring and inspection occurrence indicating level of termite activity, procedure, and treatment applied before time of Substantial Completion.

4. Termiticide brand name and manufacturer.

5. Quantities of [termiticide] [and] [nontoxic termite bait] used.

6. Schedule of inspections for one year from date of Substantial Completion.

D. Warranties: Sample of special warranties.

1.4 QUALITY ASSURANCE

A. Installer Qualifications: A specialist who is licensed according to regulations of authorities having jurisdiction to apply termite control treatment and products in jurisdiction where Project is located and who employs workers trained and approved by manufacturer to install manufacturer's products.

B. Regulatory Requirements: Formulate and apply termiticides and termiticide devices according to the EPA-Registered Label.

C. Preinstallation Conference: Conduct conference at [Project site] <Insert location>.

1.5 PROJECT CONDITIONS

Retain first two paragraphs below if soil treatment is required.

A. Environmental Limitations: To ensure penetration, do not treat soil that is water saturated or frozen. Do not treat soil while precipitation is occurring. Comply with requirements of the EPA-Registered Label and requirements of authorities having jurisdiction.

B. Coordinate soil treatment application with excavating, filling, grading, and concreting operations. Treat soil under footings, grade beams, and ground-supported slabs before construction.

Retain paragraph below if bait-station system concurrent with construction is required to suit Project.

C. Install bait-station system [during construction to determine areas of termite activity] [and] [after construction, including landscaping, is completed].

1.6 WARRANTY

Warranty in paragraph below is for protection of construction from infestation and not specifically for methods used. Insert bait-station system and warranty terms if applicable. If required, contact HUD for its current "Subterranean Termite Soil Treatment Builder's Guarantee."

A. Soil Treatment Special Warranty: Manufacturer's standard form, signed by Applicator and Contractor, certifying that termite control work, consisting of applied soil termiticide treatment, will prevent infestation of subterranean termites. If subterranean termite activity or damage is discovered during warranty period, re-treat soil and repair or replace damage caused by termite infestation.

Verify available warranties and warranty periods with manufacturers listed in Part 2 articles.

1. Warranty Period: [Three] [Five] <Insert number> years from date of Substantial Completion.

1.7 MAINTENANCE SERVICE

Retain or delete this article to suit products and the needs of Owner. Coordinate with warranty requirements if any. Example below is for soil treatment. Bait-station systems require product-specific and continuing postconstruction monitoring and treatment.

A. Continuing Service: Beginning at Substantial Completion, provide [12 months'] <Insert number> continuing service including monitoring, inspection, and re-treatment for occurrences of termite activity. Provide a standard continuing service agreement. State services, obligations, conditions, terms for agreement period, and terms for future renewal options.

PART 2 -  PRODUCTS

2.1 SOIL TREATMENT

A. Termiticide: Provide an EPA-Registered termiticide, complying with requirements of authorities having jurisdiction, in an aqueous solution formulated to prevent termite infestation. Provide quantity required for application at the label volume and rate for the maximum termiticide concentration allowed for each specific use, according to product's EPA-Registered Label.

See Editing Instruction No. 1 in the Evaluations for cautions about naming manufacturers and products. See SECTION 016000 "Product Requirements."

1. Products: Subject to compliance with requirements, [provide the following] [provide one of the following] [available products that may be incorporated into the Work include, but are not limited to, the following]:

a. BASF Corporation, Agricultural Products; Termidor.

b. Bayer Environmental Science; Premise 75.

c. FMC Corporation, Agricultural Products Group; [Dragnet FT] [Talstar] [Prevail].

d. Syngenta; [Demon TC] [Prelude] [Probuild TC].

e. <Insert manufacturer's name; product name or designation>.

2. Service Life of Treatment: Soil treatment termiticide that is effective for not less than [three] [five] <Insert number> years against infestation of subterranean termites.

2.2 BAIT-STATION SYSTEM

A. Provide bait stations based on the dimensions of building perimeter indicated on Drawings, according to manufacturer's EPA-Registered Label for product, manufacturer's written instructions, and the following:

See Editing Instruction No. 1 in the Evaluations for cautions about naming manufacturers and products. See Section 016000 "Product Requirements."

1. Products: Subject to compliance with requirements, [provide the following] [provide one of the following] [available products that may be incorporated into the Work include, but are not limited to, the following]:

a. BASF Corporation, Agricultural Products; Subterfuge.

b. Dow AgroSciences LLC; Sentricon System.

c. Ensystex, Inc.; Exterra System.

d. FMC Corporation, Agricultural Products Group; First Line.

e. Whitmire Micro-Gen Research Laboratories, Inc.; Advance TBS.

f. <Insert manufacturer's name; product name or designation>.

Retain one of first two subparagraphs below based on manufacturer's recommended minimum spacing and number of bait stations in a cluster.

2. No fewer than one bait station per [8 linear feet (2.4 linear meters)] [20 linear feet (6.1 linear meters)] <Insert value>.

3. No fewer than one cluster of bait stations per 20 linear feet (6.1 linear meters), consisting of no fewer than three bait stations per cluster.

4. <Insert requirements>.

PART 3 -  EXECUTION

3.1 APPLICATION, GENERAL

A. General: Comply with the most stringent requirements of authorities having jurisdiction and with manufacturer's EPA-Registered Label for products.

3.2 APPLYING SOIL TREATMENT

A. Examine substrates, areas, and conditions, with Applicator present, for compliance with requirements for[ moisture content of soil per termiticide label requirements,] interfaces with earthwork, slab and foundation work, landscaping, utility installation, and other conditions affecting performance of termite control.

B. Proceed with application only after unsatisfactory conditions have been corrected.

C. Soil Treatment Preparation: Remove foreign matter and impermeable soil materials that could decrease treatment effectiveness on areas to be treated. Loosen, rake, and level soil to be treated except previously compacted areas under slabs and footings. Termiticides may be applied before placing compacted fill under slabs if recommended in writing by termiticide manufacturer.

1. Fit filling hose connected to water source at the site with a backflow preventer, complying with requirements of authorities having jurisdiction.

D. Application: Mix soil treatment termiticide solution to a uniform consistency. Provide quantity required for application at the label volume and rate for the maximum specified concentration of termiticide, according to manufacturer's EPA-Registered Label, to the following so that a continuous horizontal and vertical termiticidal barrier or treated zone is established around and under building construction. Distribute treatment evenly.

Revise five subparagraphs below to suit Project.

1. Slabs-on-Grade and Basement Slabs: Under ground-supported slab construction, including footings, building slabs, and attached slabs as an overall treatment. Treat soil materials before concrete footings and slabs are placed.

2. Foundations: Adjacent soil, including soil along the entire inside perimeter of foundation walls; along both sides of interior partition walls; around plumbing pipes and electric conduit penetrating the slab; around interior column footers, piers, and chimney bases; and along the entire outside perimeter, from grade to bottom of footing. Avoid soil washout around footings.

3. Crawlspaces: Soil under and adjacent to foundations as previously indicated. Treat adjacent areas including around entrance platform, porches, and equipment bases. Apply overall treatment only where attached concrete platform and porches are on fill or ground.

4. Masonry: Treat voids.

5. Penetrations: At expansion joints, control joints, and areas where slabs will be penetrated.

E. Avoid disturbance of treated soil after application. Keep off treated areas until completely dry.

F. Protect termiticide solution, dispersed in treated soils and fills, from being diluted until ground-supported slabs are installed. Use waterproof barrier according to EPA-Registered Label instructions.

G. Post warning signs in areas of application.

H. Reapply soil treatment solution to areas disturbed by subsequent excavation, grading, landscaping, or other construction activities following application.

3.3 INSTALLING BAIT-STATION SYSTEM

A. Place bait stations according to the EPA-Registered Label for the product and manufacturer's written instructions, in the following areas that are conducive to termite feeding and activity:

Revise examples in subparagraphs below and insert others to suit Project; if applicable, show precise locations on Drawings.

1. Conducive sites and locations indicated on Drawings.

2. In and around infested trees and stumps.

3. In mulch beds.

4. Where wood directly contacts soil.

5. Areas of high soil moisture.

6. Near irrigation sprinkler heads.

7. Each area where roof drainage system, including downspouts and scuppers, drains to soil.

8. Along driplines of roof overhangs without gutters.

9. Where condensate lines from mechanical equipment drip or drain to soil.

10. At plumbing penetrations through ground-supported slabs.

11. Other sites and locations as determined by licensed Installer.

Retain paragraph below if retaining "Maintenance Service" Article and if Owner agrees to continuing service.

B. Inspect and service bait stations from time of their application until Substantial Completion[ unless extended by continuing service agreement], according to the EPA-Registered Label for product and manufacturer's written instructions for termite management system and bait products.

If retaining subparagraph below, revise to suit Project.

1. Service Frequency: Inspect bait stations not less than once [every] [every three] <Insert number> month(s).

END OF SECTION 313116

	TERMITE CONTROL
	313116 - 1



