Copyright 2011 AIA
MasterSpec Professional
11/11

Copyright 2011 by The American Institute of Architects (AIA)

Exclusively published and distributed by Architectural Computer Services, Inc. (ARCOM) for the AIA

SECTION 329300 - PLANTS
TIPS:
To view non-printing Editor's Notes that provide guidance for editing, click on Masterworks/Single‑File Formatting/Toggle/Editor's Notes.
To read detailed research, technical information about products and materials, and coordination checklists, click on Masterworks/Supporting Information.

Revise this Section by deleting and inserting text to meet Project-specific requirements.

This Section uses the term "Architect." Change this term to match that used to identify the design professional as defined in the General and Supplementary Conditions.

PART 1 -  GENERAL

1.1 SUMMARY

A. Section Includes:

1. Plants.

2. Tree-watering devices.

3. Landscape edgings.

B. Related Requirements:

Retain subparagraph below to cross-reference requirements Contractor might expect to find in this Section but are specified in other Sections.

1. Section 329600 "Transplanting" for transplanting non-nursery-grown trees.

1.2 DEFINITIONS

Retain terms that remain after this Section has been edited for a project.

A. Backfill: The earth used to replace or the act of replacing earth in an excavation.

B. Pesticide: A substance or mixture intended for preventing, destroying, repelling, or mitigating a pest. Pesticides include insecticides, miticides, herbicides, fungicides, rodenticides, and molluscicides. They also include substances or mixtures intended for use as a plant regulator, defoliant, or desiccant. Some sources classify herbicides separately from pesticides.

C. Planting Soil: Existing, on-site soil; imported soil; or manufactured soil that has been modified with soil amendments and perhaps fertilizers to produce a soil mixture best for plant growth. See [Section 329113 "Soil Preparation"] [Section 329115 "Soil Preparation (Performance Specification)"] for drawing designations for planting soils.

D. Root Flare: Also called "trunk flare." The area at the base of the plant's stem or trunk where the stem or trunk broadens to form roots; the area of transition between the root system and the stem or trunk.

1.3 PREINSTALLATION MEETINGS

A. Preinstallation Conference: Conduct conference at [Project site] <Insert location>.

1.4 ACTION SUBMITTALS

A. Product Data: For each type of product.

Retain paragraph below if required. Revise to suit Project.

B. Samples of each type of mulch.

1.5 INFORMATIONAL SUBMITTALS

A. Product certificates.

B. Sample warranty.

1.6 CLOSEOUT SUBMITTALS

A. Maintenance Data: Recommended procedures to be established by Owner for maintenance of plants during a calendar year.

1.7 QUALITY ASSURANCE

A. Installer's Field Supervision: Require Installer to maintain an experienced full-time supervisor on Project site when work is in progress.

1. Pesticide Applicator: State licensed, commercial.

B. Provide quality, size, genus, species, and variety of plants indicated, complying with applicable requirements in ANSI Z60.1.

1.8 DELIVERY, STORAGE, AND HANDLING

Retain one or more paragraphs in this article to suit Project. Limit inserts to only those unusual requirements not included in Section 016000 "Product Requirements."

A. Deliver bare-root stock plants within [24 hours] [36 hours] <Insert time> of digging. Immediately after digging up bare-root stock, pack root system in wet straw, hay, or other suitable material to keep root system moist until planting. Transport in covered, temperature-controlled vehicles, and keep plants cool and protected from sun and wind at all times.

B. Do not prune trees and shrubs before delivery. Protect bark, branches, and root systems from sun scald, drying, wind burn, sweating, whipping, and other handling and tying damage. Do not bend or bind-tie trees or shrubs in such a manner as to destroy their natural shape. Provide protective covering of plants during shipping and delivery. Do not drop plants during delivery and handling.

C. Handle planting stock by root ball.

D. Store bulbs, corms, and tubers in a dry place at 60 to 65 deg F (16 to 18 deg C) until planting.

E. Deliver plants after preparations for planting have been completed, and install immediately. If planting is delayed more than six hours after delivery, set plants and trees in their appropriate aspect (sun, filtered sun, or shade), protect from weather and mechanical damage, and keep roots moist.

1.9 WARRANTY

A. Special Warranty: Installer agrees to repair or replace plantings and accessories that fail in materials, workmanship, or growth within specified warranty period.

1. Failures include, but are not limited to, the following:

a. Death and unsatisfactory growth, except for defects resulting from abuse, lack of adequate maintenance, or neglect by Owner.

b. Structural failures including plantings falling or blowing over.

Revise start date in "Warranty Periods" Subparagraph below according to local practice and the types of planting required and coordinate with the maintenance period.

2. Warranty Periods: From date of [planting completion] [Substantial Completion] <Insert starting time>.

Warranty periods in "Trees, Shrubs, Vines, and Ornamental Grasses," "Ground Covers, Biennials, Perennials, and Other Plants," and "Annuals" subparagraphs below are examples only for some categories of plants; revise or insert other plant categories to suit Project.

a. Trees, Shrubs, Vines, and Ornamental Grasses: [12] <Insert number> months.

b. Ground Covers, Biennials, Perennials, and Other Plants: [12] [Nine] [Six] [Three] <Insert number> months.

c. Annuals: [Three] [Two] <Insert number> months.

PART 2 -  PRODUCTS

See Editing Instruction No. 1 in the Evaluations for cautions about named manufacturers and products. For an explanation of options and Contractor's product selection procedures, see Section 016000 "Product Requirements."

2.1 PLANT MATERIAL

A. General: Furnish nursery-grown plants true to genus, species, variety, cultivar, stem form, shearing, and other features indicated in Plant List, Plant Schedule, or Plant Legend indicated on Drawings and complying with ANSI Z60.1; and with healthy root systems developed by transplanting or root pruning. Provide well-shaped, fully branched, healthy, vigorous stock, densely foliated when in leaf and free of disease, pests, eggs, larvae, and defects such as knots, sun scald, injuries, abrasions, and disfigurement.

B. Root-Ball Depth: Furnish trees and shrubs with root balls measured from top of root ball, which begins at root flare according to ANSI Z60.1. Root flare shall be visible before planting.

Retain "(Annuals) (and) (Biennials)" Paragraph below for annuals or biennials. ANSI Z60.1 does not include them.

C. [Annuals] [and] [Biennials]: Provide healthy, disease-free plants of species and variety shown or listed, with well-established root systems reaching to sides of the container to maintain a firm ball, but not with excessive root growth encircling the container. Provide only plants that are acclimated to outdoor conditions before delivery[ and that are in bud but not yet in bloom].

2.2 FERTILIZERS

A. Planting Tablets: Tightly compressed chip-type, long-lasting, slow-release, commercial-grade planting fertilizer in tablet form. Tablets shall break down with soil bacteria, converting nutrients into a form that can be absorbed by plant roots.

1. Size: [5-gram] [10-gram] [21-gram] <Insert size> tablets.

2. Nutrient Composition: 20 percent nitrogen, 10 percent phosphorous, and 5 percent potassium, by weight plus micronutrients.

2.3 MULCHES

A. Organic Mulch: [Shredded hardwood] [Ground or shredded bark] [Wood and bark chips] [Pine straw] [Salt hay or threshed straw] [Pine needles] [Peanut, pecan, and cocoa-bean shells] <Insert mulch type>.

B. Mineral Mulch: [Rounded riverbed gravel or smooth-faced stone] [Crushed stone or gravel] [Marble chips] [Granite chips] <Insert stone type>.

1. Size Range: [1-1/2 inches (38 mm) maximum, 3/4 inch (19 mm) minimum] [3/4 inch (19 mm) maximum, 1/4 inch (6.4 mm) minimum] <Insert dimensions>.

2. Color: [Uniform tan-beige color range acceptable to Architect] [Readily available natural gravel color range] <Insert color>.

2.4 WEED-CONTROL BARRIERS

Retain "Nonwoven Geotextile Filter Fabric" or "Composite Fabric" Paragraph below, or insert another type if required. If retaining more than one, specify or indicate on Drawings where each barrier is used.

A. Nonwoven Geotextile Filter Fabric: Polypropylene or polyester fabric, 3 oz./sq. yd. (101g/sq. m) minimum, composed of fibers formed into a stable network so that fibers retain their relative position. Fabric shall be inert to biological degradation and resist naturally encountered chemicals, alkalis, and acids.

B. Composite Fabric: Woven, needle-punched polypropylene substrate bonded to a nonwoven polypropylene fabric, 4.8 oz./sq. yd. (162 g/sq. m).

2.5 PESTICIDES

A. General: Pesticide registered and approved by the EPA, acceptable to authorities having jurisdiction, and of type recommended by manufacturer for each specific problem and as required for Project conditions and application. Do not use restricted pesticides unless authorized in writing by authorities having jurisdiction.

2.6 LANDSCAPE EDGINGS

A. Wood Edging: Of sizes indicated on Drawings, and wood stakes as follows:

Retain required species in "Species" Subparagraph below.

1. Species: [Western red cedar, all heart] [Southern pine with specified wood pressure-preservative treatment].

2. Stakes: Same species as edging, 1-by-2-inch nominal (19-by-38-mm actual) by 18 inches (450 mm) long, with galvanized nails for anchoring edging.

B. Steel Edging: Standard commercial-steel edging, fabricated in sections of standard lengths, with loops stamped from or welded to face of sections to receive stakes.

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

1. Manufacturers: Subject to compliance with requirements, [provide products by the following] [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to identify a specific product or a comparable product from manufacturers listed. Retain option and delete insert note if manufacturer's name and model number are indicated on Drawings.

2. Basis-of-Design Product: Subject to compliance with requirements, provide [product indicated on Drawings] <Insert manufacturer's name; product name or designation> or comparable product by one of the following:

a. Border Concepts, Inc.

b. Collier Metal Specialties, Inc.

c. Russell, J. D. Company (The).

d. Sure-loc Edging Corporation.

e. <Insert manufacturer's name>.

3. Edging Size: [3/16 inch (4.8 mm) thick by 4 inches (100 mm) deep] [1/4 inch (6.4 mm) thick by 5 inches (125 mm) deep] [1/4 inch (6.4 mm) thick by 4 inches (100 mm) deep] [1/8 inch (3.2 mm) thick by 4 inches (100 mm) deep] [1/8 inch (3.2 mm) thick by 6 inches (150 mm) deep] [0.1 inch (2.5 mm) thick by 4 inches (100 mm) deep] <Insert dimensions>.

4. Finish: [Manufacturer's standard paint] [Zinc coated] [Unfinished] <Insert requirement>.

C. Aluminum Edging: Standard-profile extruded-aluminum edging, ASTM B 221 (ASTM B 221M), Alloy 6063-T6, fabricated in standard lengths with interlocking sections with loops stamped from face of sections to receive stakes.

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

1. Manufacturers: Subject to compliance with requirements, [provide products by the following] [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to identify a specific product or a comparable product from manufacturers listed. Retain option and delete insert note if manufacturer's name and model number are indicated on Drawings.

2. Basis-of-Design Product: Subject to compliance with requirements, provide [product indicated on Drawings] <Insert manufacturer's name; product name or designation> or comparable product by one of the following:

a. Curv-Rite, Inc.

b. Permaloc Corporation.

c. Russell, J. D. Company (The).

d. Sure-loc Edging Corporation.

e. <Insert manufacturer's name>.

3. Edging Size: [3/16 inch (4.8 mm) thick by 5-1/2 inches (140 mm) deep] [3/16 inch (4.8 mm) thick by 4 inches (100 mm) deep] [1/8 inch (3.2 mm) thick by 5-1/2 inches (140 mm) deep] [1/8 inch (3.2 mm) thick by 4 inches (100 mm) deep] <Insert dimensions>.

4. Finish: [Manufacturer's standard paint] [Powder-coat paint] [Mill (natural aluminum)] [Black anodized] <Insert requirement>.

D. Plastic Edging: Standard black polyethylene or vinyl edging, [V-lipped bottom] [horizontally grooved] <Insert configuration>, extruded in standard lengths, with 9-inch (225-mm) [steel angle] [plastic] stakes.

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

1. Manufacturers: Subject to compliance with requirements, [provide products by the following] [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to identify a specific product or a comparable product from manufacturers listed. Retain option and delete insert note if manufacturer's name and model number are indicated on Drawings.

2. Basis-of-Design Product: Subject to compliance with requirements, provide [product indicated on Drawings] <Insert manufacturer's name; product name or designation> or comparable product by one of the following:

a. Oly-Ola Edgings, Inc.

b. Permaloc Corporation.

c. Sure-loc Edging Corporation.

d. Valley View Industries.

e. Villa Root Barrier.

f. <Insert manufacturer's name>.

3. Edging Size: [0.1 inch (2.5 mm) thick by 5 inches (125 mm) deep] [0.07 inch (1.8 mm) thick by 5 inches (125 mm) deep] <Insert dimensions>.

2.7 TREE-WATERING DEVICES

Retain "Slow-Release Watering Device" Paragraph below if required.

A. Slow-Release Watering Device: Standard product manufactured for drip irrigation of plants and emptying its water contents over an extended time period; manufactured from UV-light-stabilized nylon-reinforced polyethylene sheet, PVC, or HDPE plastic.

Retain "Products" Subparagraph and list of manufacturers and products below to require specific products or a comparable product from other manufacturers.

1. Products: Subject to compliance with requirements, [provide the following] [provide one of the following] [available products that may be incorporated into the Work include, but are not limited to, the following]:

a. BIO-PLEX.

b. Engineered Watering Solutions; PQ Partners, LLC.

c. Spectrum Products, Inc.

d. <Insert manufacturer's name; product name or designation>.

PART 3 -  EXECUTION

3.1 PLANTING AREA ESTABLISHMENT

A. General: Prepare planting area for soil placement and mix planting soil according to [Section 329113 "Soil Preparation."] [Section 329115 "Soil Preparation (Performance Specification)."]

Coordinate "Placing Planting Soil" Paragraph below with Section 329113 "Soil Preparation" or Section 329115 "Soil Preparation (Performance Specification)."

B. Placing Planting Soil: [Place and mix planting soil in-place over exposed subgrade] [Place manufactured planting soil over exposed subgrade] [Blend planting soil in place] <Insert requirement>.

C. Before planting, obtain Architect's acceptance of finish grading; restore planting areas if eroded or otherwise disturbed after finish grading.

3.2 EXCAVATION FOR TREES AND SHRUBS

Revise size and cross section of tree and shrub planting pits and trenches in "Planting Pits and Trenches" Paragraph below to suit Project.

A. Planting Pits and Trenches: Excavate circular planting pits.

1. Excavate planting pits with sides sloping inward at a 45-degree angle. Excavations with vertical sides are unacceptable. Trim perimeter of bottom leaving center area of bottom raised slightly to support root ball and assist in drainage away from center. Do not further disturb base. Ensure that root ball will sit on undisturbed base soil to prevent settling. Scarify sides of planting pit smeared or smoothed during excavation.

Retain first two subparagraphs below if applicable.

2. Excavate approximately three times as wide as ball diameter.

3. Excavate at least 12 inches (300 mm) wider than root spread and deep enough to accommodate vertical roots for bare-root stock.

4. Do not excavate deeper than depth of the root ball, measured from the root flare to the bottom of the root ball.

Revise "Backfill Soil" Paragraph below to suit Project. Planting practices for trees often allow use of unamended native subsoil or topsoil as backfill soil.

B. Backfill Soil: Subsoil and topsoil removed from excavations [may] [may not] be used as backfill soil unless otherwise indicated.

3.3 TREE, SHRUB, AND VINE PLANTING

Retain applicable paragraphs in this article.

A. Inspection: At time of planting, verify that root flare is visible at top of root ball according to ANSI Z60.1. If root flare is not visible, remove soil in a level manner from the root ball to where the top-most root emerges from the trunk. After soil removal to expose the root flare, verify that root ball still meets size requirements.

B. Roots: Remove stem girdling roots and kinked roots. Remove injured roots by cutting cleanly; do not break.

C. Set each plant plumb and in center of planting pit or trench with root flare [1 inch (25 mm) above] [2 inches (50 mm) above] <Insert requirement> adjacent finish grades.

Retain "Backfill" Subparagraph below if backfill or planting-soil type is not indicated on Drawings; revise to suit Project.

1. Backfill: Planting soil <Insert drawing designation>.[ For trees, use excavated soil for backfill.]

Retain applicable stock in first four subparagraphs below.

2. Balled and Burlapped Stock: After placing some backfill around root ball to stabilize plant, carefully cut and remove burlap, rope, and wire baskets from tops of root balls and from sides, but do not remove from under root balls. Remove pallets, if any, before setting. Do not use planting stock if root ball is cracked or broken before or during planting operation.

3. [Balled and Potted] [and] [Container-Grown] Stock: Carefully remove root ball from container without damaging root ball or plant.

4. Fabric Bag-Grown Stock: Carefully remove root ball from fabric bag without damaging root ball or plant. Do not use planting stock if root ball is cracked or broken before or during planting operation.

5. Bare-Root Stock: Support stem of each plant and spread roots without tangling or turning toward surface. Plumb before backfilling, and maintain plumb while working. Carefully work backfill around roots by hand. Bring roots into close contact with the soil.

6. Backfill around root ball in layers, tamping to settle soil and eliminate voids and air pockets. When planting pit is approximately one-half filled, water thoroughly before placing remainder of backfill. Repeat watering until no more water is absorbed.

7. Place planting tablets equally distributed around each planting pit when pit is approximately one-half filled. Place tablets beside the root ball about 1 inch (25 mm) from root tips; do not place tablets in bottom of the hole.

a. Bare-Root Stock: Place tablets beside soil-covered roots; do not place tablets touching the roots.

b. Quantity: [As indicated on Drawings] [Two per plant] [Three for each caliper inch of plant] <Insert requirement>.

8. Continue backfilling process. Water again after placing and tamping final layer of soil.

D. Slopes: When planting on slopes, set the plant so the root flare on the uphill side is flush with the surrounding soil on the slope; the edge of the root ball on the downhill side will be above the surrounding soil. Apply enough soil to cover the downhill side of the root ball.

3.4 TREE, SHRUB, AND VINE PRUNING

Retain one of first three paragraphs in this article. Retain first as a minimum requirement.

A. Remove only dead, dying, or broken branches. Do not prune for shape.

B. Prune, thin, and shape trees, shrubs, and vines as directed by Architect.

C. Prune, thin, and shape trees, shrubs, and vines according to standard professional horticultural and arboricultural practices. Unless otherwise indicated by Architect, do not cut tree leaders; remove only injured, dying, or dead branches from trees and shrubs; and prune to retain natural character.

D. Do not apply pruning paint to wounds.

3.5 GROUND COVER AND PLANT PLANTING

Revise spacing in first paragraph below to suit Project. Insert spacing for each plant if preferred, or indicate spacing on Drawings.

A. Set out and space ground cover and plants other than trees, shrubs, and vines [9 inches (225 mm) apart] [12 inches (300 mm) apart] [18 inches (450 mm) apart] [24 inches (600 mm) apart] [as indicated on Drawings] in even rows with triangular spacing.

Retain first paragraph below if planting-soil type is not indicated on Drawing; revise if other types of backfill are required.

B. Use planting soil <Insert drawing designation> for backfill.

C. Dig holes large enough to allow spreading of roots.

D. Work soil around roots to eliminate air pockets and leave a slight saucer indentation around plants to hold water.

E. Water thoroughly after planting, taking care not to cover plant crowns with wet soil.

F. Protect plants from hot sun and wind; remove protection if plants show evidence of recovery from transplanting shock.

3.6 PLANTING AREA MULCHING

Retain first paragraph below if required.

A. Install weed-control barriers before mulching according to manufacturer's written instructions. Completely cover area to be mulched, overlapping edges a minimum of [6 inches (150 mm)] [12 inches (300mm)] and secure seams with galvanized pins.

B. Mulch backfilled surfaces of planting areas and other areas indicated.

Retain required mulch applications in two subparagraphs below.

1. Trees[ and Treelike Shrubs] in Turf Areas: Apply [organic] [mineral] mulch ring of [2-inch (50-mm)] [3-inch (75-mm)] <Insert dimension> average thickness, with [12-inch (300-mm)] [24-inch (600-mm)] [36-inch (900-mm)] <Insert dimension> radius around trunks or stems. Do not place mulch within [3 inches (75 mm)] [6 inches (150 mm)] <Insert distance> of trunks or stems.

2. [Organic Mulch] [and] [Mineral Mulch] in Planting Areas: Apply [2-inch (50-mm)] [3-inch (75-mm)] <Insert dimension> average thickness of organic mulch [extending 12 inches (300 mm) beyond edge of individual planting pit or trench] [and] [over whole surface of planting area], and finish level with adjacent finish grades. Do not place mulch within [3 inches (75 mm)] [6 inches (150 mm)] <Insert distance> of trunks or stems.

3.7 EDGING INSTALLATION

A. Wood Edging: Install edging where indicated.[ Mitre cut joints and connections at a 45-degree angle.] Fasten each cut joint or connection with two galvanized nails. Anchor with wood stakes spaced up to 36 inches (900 mm) apart, driven at least 1 inch (25 mm) below top elevation of edging. Use two galvanized nails per stake to fasten edging, of length as needed to penetrate both edging and stake and provide 1/2-inch (13-mm) clinch at point. Predrill stakes if needed to avoid splitting. Replace stakes that crack or split during installation process.

B. Steel Edging: Install steel edging where indicated according to manufacturer's written instructions. Anchor with steel stakes spaced approximately 30 inches (760 mm) apart, driven below top elevation of edging.

C. Aluminum Edging: Install aluminum edging where indicated according to manufacturer's written instructions. Anchor with aluminum stakes spaced approximately [36 inches (900 mm)] [48 inches (1200 mm)] apart, driven below top elevation of edging.

D. Plastic Edging: Install plastic edging where indicated according to manufacturer's written instructions. Anchor with steel stakes spaced approximately [36 inches (900 mm)] [48 inches (1200 mm)] apart, driven through upper base grooves or V-lip of edging.

E. Shovel-Cut Edging: Separate mulched areas from turf areas[, curbs, and paving] with a 45-degree, 4- to 6-inch- (100- to 150-mm-) deep, shovel-cut edge[ as indicated on Drawings].

3.8 INSTALLING SLOW-RELEASE WATERING DEVICE

A. Provide one device for each tree.

3.9 PLANT MAINTENANCE

A. Maintain plantings by pruning, cultivating, watering, weeding, fertilizing, mulching, restoring planting saucers, resetting to proper grades or vertical position, and performing other operations as required to establish healthy, viable plantings.

B. Fill in, as necessary, soil subsidence that may occur because of settling or other processes. Replace mulch materials damaged or lost in areas of subsidence.

C. Apply treatments as required to keep plant materials, planted areas, and soils free of pests and pathogens or disease. Use integrated pest management practices when possible to minimize use of pesticides and reduce hazards. Treatments include physical controls such as hosing off foliage, mechanical controls such as traps, and biological control agents.

D. Apply pesticides and other chemical products and biological control agents according to authorities having jurisdiction and manufacturer's written recommendations. Coordinate applications with Owner's operations and others in proximity to the Work. Notify Owner before each application is performed.

E. Protect plants from damage due to landscape operations and operations of other contractors and trades. Maintain protection during installation and maintenance periods. Treat, repair, or replace damaged plantings.

Retain paragraph below if tree-watering devices are required.

F. At time of Substantial Completion, verify that tree-watering devices are in good working order and leave them in place. Replace improperly functioning devices.

3.10 MAINTENANCE SERVICE

A. Maintenance Service: Provide maintenance by skilled employees of landscape Installer. Maintain as required in "Plant Maintenance" Article. Begin maintenance immediately after plants are installed and continue until plantings are acceptably healthy and well established, but for not less than maintenance period below:

Revise time periods in "Maintenance Period for Trees and Shrubs" and "Maintenance Period for Ground Cover and Other Plants" subparagraphs below to suit local customs and growing conditions.

1. Maintenance Period for Trees and Shrubs: [12] [Six] [Three] months from date of [planting completion] [Substantial Completion] <Insert starting time>.

2. Maintenance Period for Ground Cover and Other Plants: [Six] [Three] months from date of [planting completion] [Substantial Completion] <Insert starting time>.

END OF SECTION 329300

	PLANTS
	329300 - 1



