SECTION 06193PRIVATE 

PLATE CONNECTED WOOD TRUSSES
1 PART 1 GENERAL


.1

SECTION INCLUDES

A.
Shop fabricated wood trusses for roof and floor framing.



B.
Bridging, bracing, and anchorage.

.2

seq level2 \h \r0 

PRODUCTS FURNISHED BUT NOT INSTALLED UNDER THIS SECTION




Not Used


.3
seq level2 \h \r0 
RELATED SECTIONS



A.
Section:   Setting anchors in concrete masonry. 



B.
Section:   Prefabricated steel structural supports. 



C.
Section 06181 ‑ Glued Laminated Structural Units.


.4

seq level2 \h \r0 

REFERENCES

A.
ALSC ‑ American Lumber Standards Committee:  Softwood Lumber Standards.

B.
ASTM A167 ‑ Stainless and Heat Resisting Chromium ‑ Nickel Steel Plate, Sheet, and Strip.

C.
ASTM A446 ‑ Steel Sheet, Zinc Coated (Galvanized) by the Hot‑Dip Process, Structural (Physical) Quality.



D.
NFPA:  National Forest Products Association.



E.
SPIB:  Southern Pine Inspection Bureau.

F.
TPI (Truss Plate Institute) BWT‑76 ‑ Bracing Wood Trusses.

G.
TPI (Truss Plate Institute) HET‑80 ‑ Handling and Erecting Wood Trusses.

H.
TPI (Truss Plate Institute) PCT‑80 ‑ Metal Plate Connected Parallel Chord Wood Trusses.

I.
TPI (Truss Plate Institute) TPI‑85 ‑ Metal Plate Connected Wood Trusses.

J.
TPI (Truss Plate Institute) QST‑88 ‑ Metal Plate Connected Wood Trusses.

.5

seq level2 \h \r0 

SYSTEM DESCRIPTION

A.
Design floor live load: 50-lbs./sq. ft with deflection limited to 1/360.

B.
Design roof live load: 15-lbs./sq. ft with deflection limited to 1/240.


.6

seq level2 \h \r0 

SUBMITTALS



A.
Submit under provisions of Section 01300.

B.
Shop Drawings:  Indicate sizes and spacing of trusses, loads and truss cambers, framed openings. 

C.
Product Data:  Provide truss configurations, bearing and anchor details, bridging and bracing. 


.7
seq level2 \h \r0 
QUALITY ASSURANCE

A.
Perform Work in accordance with the following agencies:




1.
Lumber Grading Agency:
Certified by ALSC.




2.
Plywood Grading Agency:  Certified by APA. 

B.seq level3 \h \r0 
Truss Design, Fabrication, and Installation:
In accordance with Truss Plate Institute BWT‑76, HET‑80, PCT‑80 including Supplement, TPI‑85 including Supplement, QST‑88.



C.
Maintain one copy of each document on site.


.8
seq level2 \h \r0 
QUALIFICATIONS

A.
Manufacturer:  Company specializing in manufacturing the Products specified in this section with minimum five years documented experience.

B.
Design trusses under direct supervision of a Professional Structural Engineer experienced in design of this Work and licensed of Louisiana.


.9

seq level2 \h \r0 

REGULATORY REQUIREMENTS

A.
Conform to Southern Building codes for loads, seismic zoning, other governing load criteria, and fire retardant requirements.


.10

seq level2 \h \r0 
DELIVERY, STORAGE, AND HANDLING

A.
Deliver, store, protect, and handle products to site in good condition.

B.
Handle and erect trusses in accordance with TPI HET‑80.

C.
Store trusses in vertical position resting on bearing ends.


.11

seq level2 \h \r0 
FIELD MEASUREMENTS



A.
Verify that field measurements are as indicated. 

2

seq level1 \h \r0 

seq level2 \h \r0  PART 2 PRODUCTS


.1

MANUFACTURERS




Not Used


.2
seq level2 \h \r0 
MATERIALS



A.
Lumber Grading Rules: SPIB.

B.
Wood Members: Single top and bottom chord, Stress Group species grade size classification, 19 percent maximum and 7 percent minimum moisture content.  Finger scarifying not permitted.



C.
Steel Connectors: ASTM A446 steel, Grade B hot dip galvanized; die stamped with integral teeth inch m) thick.
D.
Truss Bridging: Type, size and spacing recommended by truss manufacturer.


.3
seq level2 \h \r0 
ACCESSORIES

A.
Fasteners:  Electro‑galvanized steel, type to suit application.



B.
Bearing Plates:
Electro‑galvanized.


.4

seq level2 \h \r0 

FABRICATION

A.
Fabricate trusses to achieve structural requirements specified.



B.
Brace wood trusses in accordance with TPI BWT‑76.


.5
seq level2 \h \r0 
WOOD TREATMENT




Not Used.

3

seq level1 \h \r0 

seq level2 \h \r0  PART 3 EXECUTION


.1

EXAMINATION

A.
Verify that supports and openings are ready to receive trusses.


.2

seq level2 \h \r0 

PREPARATION



A.
Coordinate placement of [bearing] [support] items.


.3
seq level2 \h \r0 
ERECTION

A.
Install trusses in accordance with manufacturer's instructions.


.4
seq level2 \h \r0 
SITE APPLIED WOOD TREATMENT




Not Used.


.5

seq level2 \h \r0 

TOLERANCES

A.
Framing Members:
1/2 inch maximum, from true position.


.6

seq level2 \h \r0 

SCHEDULES



A.
Sizes as indicated on drawings.







END OF SECTION
PAGE  
Section 06193 Page 1 of 4

