

SECTION 08710PRIVATE 


DOOR HARDWARE

Provisions of the contract including General and Supplementary General Conditions and the General Requirements apply to the work of this section.

1 PART 1 GENERAL


.1

SECTION INCLUDES



A.
Hardware for hollow steel doors.



B.
Thresholds.



C.
Weather-stripping, seals and door gaskets.


.2

PRODUCTS FURNISHED BUT NOT INSTALLED UNDER THIS SECTION



A.
Section 08112 – Standard Steel Framing: Furnish templates for frame preparation.



B.
Section 08111: - Standard Steel Doors: Furnish templates for door preparation.


.3 
RELATED SECTIONS



A.
Section 01019 – Contract Considerations hardware allowances.



B.
Section 08111 ‑ Standard Steel Doors.



C.
Section 08112 ‑ Standard Steel Frames.



.4

REFERENCES



A.
ANSI A117.1 ‑ Specifications for Making Buildings and Facilities Accessible to and Usable by Physically Handicapped People.



B.
NFPA 80 ‑ Fire Doors and Windows.



C.
AWI ‑ Architectural Woodwork Institute ‑ Quality Standards.


D.
NFPA 101 ‑ Code for Safety to Life from Fire in Buildings and Structures.



E.
NFPA 252 ‑ Fire Tests of Door Assemblies.



F.
UL 10B ‑ Fire Tests of Door Assemblies.

G. UL 305 ‑ Panic Hardware.



H.
Reference to General Conditions. 


.5

SUBMITTALS



A.
Submit under provisions of Section 01300.



B.
Shop Drawings: Indicate locations and mounting heights of each type of hardware. 



C.
Submit manufacturer's parts list, templates.



D.
Manufacturer's Installation Instructions: Indicate special procedures, perimeter conditions requiring special attention.


.6

PROJECT RECORD DOCUMENTS



A.
Submit under provisions of Section 01700.



B.
Record actual locations of installed cylinders and their master key code.


.7

OPERATION AND MAINTENANCE DATA



A.
Submit under provisions of Section 01700.



B.
Maintenance Data: Include data on operating hardware, lubrication requirements, and inspection procedures related to preventative maintenance.


.8

QUALITY ASSURANCE



A.
Perform work in accordance with the following requirements:




1.
ANSI A117.1 ‑ Specifications for Making Buildings and Facilities Accessible to and Usable by Physically Handicapped People.




2.
NFPA 101.




3.
ANSI A117.1.




4.
NFPA 80.




5.
NFPA 252.


.9

QUALIFICATIONS



A.
Hardware Supplier: Company specializing in supplying commercial door hardware with five years experience.



B.
Hardware Supplier Personnel:
Employ an Architectural Hardware Consultant (AHC) to assist in the work of this section.


.10
REGULATORY REQUIREMENTS



A.
Conform to applicable code for requirements applicable to fire rated doors and frames


.11
DELIVERY, STORAGE, AND HANDLING



A.
Deliver, store, protect and handle products to site under provisions of Section 01600.



B.
Package hardware items individually; label and identify each package with door opening code to match hardware schedule.



C.
Deliver keys to Owner by security shipment direct from hardware supplier.


.12
COORDINATION



A.
Coordinate work under provisions of Section 01039.



B.
Coordinate the work with other directly affected sections involving manufacture or fabrication of internal reinforcement for door hardware.


.13
WARRANTY



A.
Provide five-year warranty under provisions of Section 01700. 



B.
Warranty:  Include coverage for door closures.


.14
MAINTENANCE MATERIALS



A.
Provide maintenance materials under provisions of 01700. 



B.
Provide special wrenches and tools applicable to each different or special hardware component.



C.
Provide maintenance tools and accessories supplied by hardware component manufacturer.

2 PART 2 PRODUCTS


.1

ACCEPTABLE MANUFACTURERS



A.
Hinges:  Hager, Masterlock Kwikset, Stanley, Shlage.



B.
Latch Sets: Masterlock, Kwikset, Stanley, Shlage.



C.
Push/Pulls: Masterlock, Kwikset, Stanley, Shlage.



D.
Cylinder Locks: Masterlock, Kwikset, Stanley, Stlage.



E.
Mortise Locks: Arrow, Corbin, and Sargent. 



F.
Exit Devices: Adamsrite, Von-Duprin, and Russwin.

G. Closers: LCN, Norton, and Rixson. 

H. Overhead Holders: Glynn Johnson, Ives, and Stanley. 



I.
Substitutions: Under provisions of Section 01600.


.2

KEYING



A 
Door Locks: Keyed in like‑groups. Keyed differently. Master keyed. Grand master keyed. Great grand master keyed. Include construction keying.  Key to existing keying system.


.3

FINISHES



A.
Finishes:  As selected to match existing.

3 PART 3 EXECUTION


.1

EXAMINATION



A.
Verify site conditions under provisions of Section 01039. 



B.
Verify that doors and frames are ready to receive work and dimensions are as indicated on shop drawings.



C.
Verify that electric power is available to power operated devices and of the correct characteristics.


.2

INSTALLATION



A.
Install hardware in accordance with manufacturer's instructions.



B.
Use templates provided by hardware item manufacturer.


.3

FIELD QUALITY CONTROL



A.
Architectural Hardware Consultant to inspect installation and certify that hardware and installation has been furnished and installed in accordance with manufacturer's instructions and as specified.


.4

ADJUSTING



A.
Adjust work under provisions of Section 01700.



B.
Adjust hardware for smooth operation.


.5

PROTECTION OF FINISHED WORK



A.
Protect finished Work under provisions of Section 01500. 



B.
Do not permit adjacent work to damage hardware or finish.


END OF SECTION
PAGE  
Error! Main Document Only.
Section 08710 Page 2 of 4

