

SECTION 10171PRIVATE 


SOLID PHENOLIC TOILET COMPARTMENTS

PART 1
GENERAL

.1

SECTION INCLUDES


A.
Solid plastic toilet compartments, floor mounted. head rail braced. 


B.
Urinal screens.

.2

RELATED SECTIONS


A.
Section 10800 ‑ Toilet and Bath Accessories.

.3

REFERENCES


A.
ANSI A117.1 ‑ Safety Standards for the Handicapped.


B.
ASTM A167 ‑ Stainless and Heat‑Resisting Chromium‑Nickel Steel Plate, Sheet, and Strip.


C.
NEMA LD‑3 ‑ High Pressure Decorative Laminates.

.4

SUBMITTALS


A.
Submit under provisions of Section 01300.


B.
Shop Drawings: Indicate partition plan, elevation views, dimensions, details of wall, floor, supports, door swings.


C.
Product Data: Provide data on panel construction, hardware, and accessories.

.5

REGULATORY REQUIREMENTS


A.
Conform to ANSI A117.1 code for access for the handicapped.

.6

FIELD MEASUREMENTS


A.
Verify that field measurements are as indicated on shop drawings and that all measurements agree with handicap requirements.

.7

COORDINATION


A.
Coordinate work under provisions of Section 01039.


B.
Coordinate the work with placement of support framing and anchors in wall.

PART 2
PRODUCTS

.1

ACCEPTABLE MANUFACTURERS


A.
Santana Product Polymar HD.


B.
Substitutions:  Under provisions of Section 01600.

.2

MATERIALS


A.
Solid Phenolic Panel: NEMA LD‑3 solid phonemic core with melamine laminate veneer material.

.3

ACCESSORIES


A.
Pilaster Shoe: Formed ASTM A167 type 304 stainless steel with No. 4 finish, 3 inch (175 mm) high, with adjustable screw jack.

B. Head Rails: Heavy aluminum extrusion (6063T6 Alloy), with bright dip finish , in anti-grip configuration, weighing not less than 1.199 lbs/1.f. equal to Santana # 58993. 


C.
Attachments, Screws, and Bolts: Stainless steel; tamper proof type, heavy duty extruded aluminum brackets.


D.
Through Bolts and Nuts: Stainless steel with tamper proof heads.


E.
Hardware:  Chrome plated non‑ferrous cast metal:



1.Pivot hinges, gravity type, adjustable for doors close

         positioning.



2.Nylon bearings.



3.Thumb turn door latch with exterior emergency access feature.



4.Door strike and keeper with rubber bumper.



5.Coat hook with rubber bumper.



6.Door pull for out-swinging doors.

.4

FABRICATION


A.
Fabricate partitions by forming solid phonemic with finished faces and edges.  Finish edges convex.


B.
Doors and Panels:



1.Thickness: 1 inch (25 mm)



2.Door Width: 24 inch (610 mm)



3.Door Width for Handicapped Use: 36 inch (915 mm), out‑swinging.



4.Height: 58 inch (1 473 mm)


C.
Thickness of Pilasters: 1 inch.

PART 3
EXECUTION

.1

EXAMINATION


A.
Verify site conditions under provisions of Section 01039. 


B.
Verify correct spacing of and between plumbing fixtures.


C.
Verify correct location of built‑in framing, anchorage, and bracing.

.2

INSTALLATION


A.
Install partitions secure, rigid, plumb, and level in accordance with manufacturer's instructions.


B.
Maintain 3/8 to 1/2  inch ([9 to 13 mm) space between wall and panels and between wall and end pilasters.


C.
Attached panel brackets securely to walls using anchor devices.


D.
Attach panels and pilasters to brackets with tamper proof through bolts and nuts. 


E.
Anchor urinal screen panels to walls with two panel brackets and vertical upright consisting of pilaster anchored to floor and ceiling.


F.
Provide adjustment for floor variations with screw jack through steel saddles integral with pilaster.  Conceal floor fastenings with pilaster shoes.


G.
Equip each door with two hinges, one door latch, one coat hook and bumper; out-swinging door with pull.


H.
Install door strike and keeper with door bumper on each pilaster in alignment with door latch.

I. Field touch‑up of scratches or damaged finish will not be     


permitted.

J. 
Replace damaged or scratched materials with new materials.

.3

ERECTION TOLERANCES


A.
Maximum Variation From True Position: 1/4 inch (6 mm).


B.
Maximum Variation From Plumb: 1/8 inch (3 mm).

.4

ADJUSTING


A.
Adjust work under provisions of Section 01700.


B.
Adjust and align hardware to uniform clearance at vertical edge of doors, not exceeding 3/16 inch (5 mm).


C.
Adjust hinges to position doors in partial opening position when unlatched.  Return out swinging doors to closed position.


D.
Adjust adjacent components for consistency of line or plane.


END OF SECTION
PAGE  
Error! Main Document Only.
Section 10171 Page 3 of 3     

