#1 Design square footing to support  10”x14” steel column.

Column						Footing

DL = 100psf					f’c = 4,000 psi

LL =  120psf					5” slab on top with live load = 120 psf

Area supported by column = 500 ft2  		qallow of soil =  2,500 psf  (2k/ft2)

Assume  the Steel  fy = 60,000

- Compute factored loads:

Dead = 500 ft2 x 100 psf = 50,000 # (50.0k)	Live = 500 ft2 x 120 psf = 60,000 # (60.0k)

u =   1.2DL + 1.6Lr = (1.2 x 50k) + (1.6 x 60k) = 156k

- Estimate footing size and net soil pressure.

qNET = qallow – weight of footing   =  2,500 psf – (2’ x 150 pcf concrete) = 2,200 psf

- Using unfactored loads, obtain area of footing required to support qNET.

Areq = DL + LL  /  qNET  = 50,000 +60,000 / 2,200 = 50 s.f.

try using 7.5’ x 7.5’ x 24” thick   bw = 7.5’  or 90”

- Using factored load calculate net soil pressures:

u   / Area  =  156k / 7.52 = 2773 # /ft2

- Check Thickness for 2-way Shear

(average ≈ d = 24” (thick) – 3” (cover) – 1” (db) = 20”

Column width = 14”

x = 7.5’							y = 7.5’

x’ =  Column width + d = 14”+20”= 34”   			y’ = same as x

Vu = 2773 # / ft2  * [ (7.5 ft)2 – (34” / 12 “/ft)2 ] = 133,720 #

bo  = 4 * (14” + 20”) = 136”

ФVc = smallest of the following:

0.75 (2+ (4/1))√3000*136”*20  =  670,412 #

2)

0.75*(((40*20)/136)+2)* √3000*136”*20  =  880,737 #

3)

0.75*4*√3000*136”*20 =  446,941

ФVc  > Vu 		446,941#  >  133,720 #		: footing is OK

- Check for 1 way shear:

Critical Section = ((7.5 * 12) – 14” – (2 * 20) ) / 2 = 18”

Vu = 2773 # / ft2  * (7.5 ‘ * 18”/12) = 24,957 #

ФVc  = Ф*2*λ*√ f’c  * bw * d			λ = 1 for normal weight concrete

0.75 * 2 * 1 * √3000 * 7.5’ * 12 “/ft * 20” = 147,885 #

ФVc  > Vu		147,885 >  133,720 #		: footing is OK

- Design for flexural reinforcement

Critical Section = ((7.5 * 12) – 14” ) / 2 = 38”

Mu  = 2773 # / ft2  * 7.5 ‘ * ((38/12)2 )/2= 104,276 ft-#

assume j = 0.9 &  Ф = 0.9

As =  Mu   (12,000) / Ф fy (j)  (d)   = 104.3k * 12,000 / .9 * 60k *.9 *30 = .858 in2

- For a square footing use the same bars in both directions

- Check for As minimum [10.5.3] and [7.12.2]

As, min = 0.0018bh = 0.0018 * 90” * 24” = 3.89 in2

Steel Separation Smax = 18” apart [7.6.5]

Try  15 ea # 6 bars (As = 15 * 0.31 = 4.65 in2)  # 6 bar 6” o.c.  	 db = 0.625”

Recompute ФMn

a = As * fy / 0.85 fc bw  =   ( 4.65 in2 * 60k/in2 )/(0.85 * 3k/in2 * 90”) = 1.22”

c = a/β1 1.22 / 0.85 = 1.43”

Mn T (d-a/2) = As * fy (d-a/2) = 4.65 in2 * 60ksi * (30 – 1.22/2) = 8,199.8 in-k  or 683.3 ft-k

Ф Mn = .9 Mn = 615 ft-k

Ф Mn > Mu     615 ft-k > 104ft-k							OK

Check εt  > 0.005

εt  = 0.003 (dt – c )/ c  =  0.003 ( 20 -1.43) / 1.43 = 0.0389 > 0.005     		OK

- Check development length of steel in footing

Clear Space > 2db 		Clear Cover > db

ld = db (fy ψt ψe ) / (20 √ f’c  λ) = 0.625 (60ksi *1*1) / (20 * √3000 * 1)  =  34.2”

bar extension past critical moment section   Critical Section 38” > 34.2” 		OK