#1 Design square footing to support  10”x14” steel column.

Column						Footing

DL = 100psf					f’c = 4,000 psi

LL =  120psf					5” slab on top with live load = 120 psf

qallow of soil =  2,500 psf  (2k/ft2)

Assume  the Steel  fy = 60,000

- Compute factored loads:

Total Ceiling Dead Loads to Beam		=  324plf (See Center beam and Column Calcs.doc)

Total Roof Dead Loads to Beam		= 236plf (See Center beam and Column Calcs.doc)

ASCE  7-10

1.2DL + 1.6RoofLive + .8W

1.2(324 + 236) + 1.6(480) + .8(134.4) = 1,547 plf or 1.547klf

The W-Flange is 40 ft in length and each column will support ½ load, 20ft

U = 1547 plf x 20ft = 30,940 #  or 31k

- Estimate footing size and net soil pressure.

qNET = qallow – weight of footing   =  1,500 psf – (3’ x 150 pcf concrete) = 1,050 psf

- Using unfactored loads, obtain area of footing required to support qNET.

Areq = DL + LL  /  qNET  = (324 * 20) + (480 * 20) + (134.4 * 20) / 1,050 = 18 s.f.

try using 4’ x 4’ x 36” thick   bw = 4’  or 48”  (The other spread footing around the Square footing will compensate for the additional 2 s.f. required.

- Using factored load calculate net soil pressures:

u   / Area  =   (324 * 20) + (480 * 20) + (134.4 * 20)/ 42 = 1173 # /ft2

- Check Thickness for 2-way Shear

average ≈ d = 12” (thick) – 3” (cover) – .5” (db) = 8.5”

Column width = 4”

x = 4’								y = 4’

x’ =  Column width + d = 4”+8.5”= 12.5”   			y’ = same as x

Vu = u * [x2 – (x’/12)2 ]

Vu = 1173 # / ft2  * [ (4 ft)2 – (12.5” / 12 “/ft)2 ] = 17,495 #

bo  = 4 * x’ = 4 * 12.5 = 50”

ФVc = smallest of the following three:

0.75 (2+ (4/1))√4000* 50” * 12.5”  =  118,586 #

2)

0.75*(((40*8.5)/50)+2)* √4000 * 50” * 12.5”  =  260,888 #

3)

0.75*4*√4000* 50” *12.5” =  118,585

ФVc  > Vu 		118,585#  >  17,495 #		: footing is OK

- Check for 1 way shear:

Critical Section = ((x * 12”/ft) – Column Width – (2d))/2

Critical Section = ((4 * 12) – 4” – (2 * 8.5) ) / 2 = 13.5”

Vu = u * [x * (Critical Section/12) ]

Vu = 1173 # / ft2  * (4 ‘ * 13.5”/12) = 5,278 #

ФVc  = Ф*2*λ*√ f’c  * bw * d			λ = 1 for normal weight concrete

0.75 * 2 * 1 * √4000 * 4’ * 12 “/ft * 8.5” = 38,706 #

ФVc  > Vu		38,706  >  5,278 #		: footing is OK

- Design for flexural reinforcement

Critical Section = ((x * 12”/ft) – Column Width)/2

Critical Section = ((4 * 12) – 4” ) / 2 = 22”

Mu  = u * x * (Critical Section /12)2 )/2

Mu  = 1173 # / ft2  * 4 ‘ * ((22/12)2 )/2= 7,885 ft-#

assume j = 0.9 &  Ф = 0.9

As =  Mu   (12,000) / Ф fy (j)  (d)   = 7.885k * 12,000 / .9 * 60k *.9 *8.5 = .229 in2

- For a square footing use the same bars in both directions

- Check for As minimum [10.5.3] and [7.12.2]

As, min = 0.0018bh = 0.0018 * 48” * 12” = 1.04 in2

Steel Separation Smax = 18” apart [7.6.5]

Try  7 ea # 4 bars (As = 1.4  in2)  # 4 bar 6” o.c.  	 db = 0.5”

Recompute ФMn

a = As * fy / 0.85 fc bw  =   ( 1.4 in2 * 60k/in2 )/(0.85 * 4k/in2 * 48”) = 0.515”

c = a/β1 = 0.515 / 0.85 = 0.61”

Mn T (d-a/2) = As * fy (d-a/2) = 1.4 in2 * 60ksi * (8.5 –.515/2) = 692 in-k  or 8,308 ft-k

Ф Mn = .9 Mn = 7,477 ft-k

Ф Mn > Mu  7,477 ft-k > 7.885 ft-k							OK

Check εt  > 0.005

εt  = 0.003 (dt – c )/ c  =  0.003 ( 8.5 -.61) / .61 = 0.0388 > 0.005     		OK

- Check development length of steel in footing

Clear Space > 2db 		Clear Cover > db

[12.2.2] ld = db (fy ψt ψe ) / (25 √ f’c  λ) for #6 and smaller bars  AND  db (fy ψt ψe ) / (20 √ f’c  λ) for #7 bars and larger

ld= 0.5 (60,000 in2 *1*1) / (25 * √4000 * 1)  =  19.0”

bar extension past critical moment section   Critical Section 22” > 19.0” 		OK