Introduction of Potable Water System

The subdivision under design includes 131 lots with 3 Avenues and 3 connecting numbered streets.

The city of Gramercy has a 6” water main that runs along North Airline Ave, a 6” water main that runs from North Airline Ave down East 6th street and a 12” water main running parallel with Central Canal.   The potable water system for this neighborhood was designed as a grid system with interconnecting pipes on the numbered streets.  A tap was designed at the intersection of East 6th St and N. Montz Av., East 6th St and N. Ezidor Ave., East 6th St and N. Miller Ave., North Airline Ave and East 8th St., & the NorthEast end of East 8th St. across on the other side of Central Canal.

Assumptions: The town of Gramercy has an elevated water tower and supply water to the subdivision at a pressure of 60psi

# Reference Codes: Louisiana Plumbing Code 2000, NFPA 14 - Standard for the Installation of Standpipes and Hose Systems, NFPA 24 - Standard for the Installation of Private Fire Service Mains and Their Appurtenances, NFPA 291 - Practice for Fire Flow Testing and Marking of Hydrants