1.28
METAL TOILET PARTITIONS
1.28.1
Scope

The Contractor shall furnish all labor, materials, equipment and services necessary to  install the toilet partitions, dressing rooms, shower compartments, urinal screens, and all metal toilet accessories.

1.28.2
General
1.28.3
Referenced Publications, Codes and Standards

American National Standards Institute


ANSI A117.1
Providing Accessibility and Usability for Physically Handicapped People

1.28.4
Materials
1.28.4.1
Toilet Compartments
1.28.4.1.1
Toilet compartments shall be Texture Porcelain Academy Type, Color No. TA626 Red as manufactured by the Sanymetal Products Company, Inc..

1.28.4.1.2
Partition panels shall be one inch thick with two sheets of 18 gauge galvanized-banderized steel enclosing sound-deadening bridge core, and all formed edges shall be sealed with a locking strip.  Partitions to be vitreous porcelain enameled on all interior and exterior surfaces with color and finish coats of porcelain on exterior surfaces only.

1.28.4.1.3
Pilasters shall be 1 1/4 inch thick with two sheets of 18 gauge galvanized-banderized steel enclosing sound-deadening bridge core.  All formed edges shall be welded every 18 inches, and sealed with a surrounding locking strip, mitered, welded, and finished at the corners.  Pilaster bases shall be attached with floor stirrups and leveling bolts for secure anchorage.  Pilaster tops shall be overhead braced the full perimeter of the installation with 1" X 1 1/2" lock seam tubing secured to wall and each pilaster top.

1.28.4.1.4
Doors shall be one inch thick with two sheets of 18 gauge galvanized-banderized steel enclosing sound-deadening bridge core, and all formed edges shall be welded every 18 inches, and sealed with a locking strip, mitered, welded and finished at the corners.  Doors shall be equipped with double-duty hinge set.  Each hinge set shall be of high strength cast-alloy chrome plate.  The top hinge pin shall be secured with all door hinge fittings fully flush with face plates of the door.

1.28.4.1.5
Each door shall be equipped with chrome plated cast alloy coat hook and bumper, concealed latch with bolt of stainless steel permitting exterior access, a one piece chrome plated stop and keeper, and concealed power bearing gravity hinge.

1.28.4.1.6
Door shall be adjustable to permit rest position at any angle within a 270 degree arc, and the weight at all times shall be carried by a thrust bearing with all moving parts concealed within the door thickness.

1.28.4.1.7
Pilaster hinge brackets shall be factory mounted flush to the pilaster.

1.28.4.1.8
Floor connects shall be concealed with a one piece stainless steel theft proof plinth.

1.28.4.1.9
Heat treated, polished, anodized aluminum stirrup brackets shall be used to attach panels and pilasters.

1.28.4.2
Urinal Screens

Urinal screens shall be floor supported as manufactured by The Sanymetal Products Company, Inc., Cleveland, OH.  The product construction and hardware shall be the same as toilet compartments.

1.28.4.3
Dressing Rooms

Dressing rooms shall be as manufactured by the Sanymetal Products Company, Inc.  The partitions shall be 1/2 inch thick solid phenolic core (SPC) panels and one inch SPC pilasters.  All hardware, latches, and pilaster base assemblies shall be of stainless steel.  The compartment seats shall be 1/2 inch SPC with steel brackets.  Overhead bracing shall feature aluminum handrail and integral curtain track with hooks.

1.28.4.4
Shower Compartments

Shower compartments shall be baked acrylic as manufactured by The Sanymetal Products Company, Inc., Cleveland, OH.

1.28.4.5
Metal Toilet Accessories

Metal toilet accessories specified are by Bobrick.  



Soap dispenser
Bobrick No. B-111, surface mounted, one above each lavatory



Paper towel dispenser
Bobrick No. B-3947, semi-recessed



and waste receptacle



Toilet tissue dispenser
Bobrick No. B-388, double roll type, surface mounted in each toilet compartment



Feminine napkin
Bobrick No. B-352, without coin mechanism



dispenser



Feminine napkin disposal
Bobrick No. 354, surface mounted



receptacle



Mirror
Bobrick No. B-294, 16" X 30" stainless steel frame, concealed hanging, one above each lavatory



Grab bar
Bobrick No. 593.



Shower curtain rod,
Bobrick No. 207, 204-1, 204-2



hooks, and curtain

1.28.5
Delivery and Storage

The materials shall be delivered and stored in original unopened containers with labels intact until time of use.  Materials shall be stored and handled in a manner to prevent damage or contamination by water, freezing, or foreign matter.

1.28.6
Performance Requirements (Not Used)
1.28.7
Workmanship
1.28.7.1
Installation shall be in accordance with shop drawings and the manufacturer's instructions.  Partitions and screens shall be installed straight and plumb with all horizontal lines level.  Partition panels shall be installed with a clearance of one inch at walls.  Wall-end pilasters shall be installed with a clearance for cove base.  Panels and pilasters shall be fastened to walls with brackets located near top and bottom.  Doors shall be out of wind.  The clearance of vertical edges of doors shall be uniform from top to bottom and shall not exceed 3/16 inch.  Hardware shall be carefully adjusted and left in perfect working order.  Units or sections that are dented, punctured, deeply scratched to metal, or otherwise damaged shall be removed and replaced with undamaged units or sections.

1.28.7.2
Except as hereinafter specified, surface mounted accessories shall be installed with machine or wood screws in metal or fiber shields or sleeves, or with toggle bolts, as required by the construction.  Back plates shall be installed in the same manner or shall be provided with lugs or anchors and set in mortar as required by the construction.  All accessories shall be installed at the proper height and location except as indicated on drawings.  Grab bars and accessories shall be mounted as required for handicapped access according to ANSI A117.1.

1.28.7.3
After installation, all surfaces shall be thoroughly cleaned, and pencil marks used in laying out shall be removed.

1.28.7.4
The Contractor shall protect work from damage and staining at all times.  Cracked, chipped, or defective work will not be accepted.  Areas in which work is in progress shall be closed to traffic until completion.

1.28.8
Inspection
1.28.9
Special Conditions
STD SPEC
1.28 - 3

