1.32
CRIB FENCING
1.32.1
Scope

This section includes all labor, materials, and services necessary for the complete installation of crib fencing (wire mesh partitions) shown on the design drawings and specified herein.

1.32.2
General
1.32.3
Reference Publications, Codes and Standards 

The latest catalog of the manufacturer of the crib fencing used for this project shall be deemed part of this specification.  The catalogs shall be one of, but not limited to, the following:



"Stock Size Wire Partitions, Folding Gates, Spiral Stairs, Wire Window Guards"; Logan Co., General Products Division; Louisville, KY  40206



"Wire Mesh Partitions"; Acorn Wire and Iron Works; 4940 South Kilbourne Avenue; Chicago, IL  60632



Crib Fencing Catalog; Kentucky Metal Products Co.; 3104 Preston Street; Box 36304; Louisville, KY  40233



"Folding Gates and Mesh Partitions"; Miller Wire Works, Inc.; 7429 Georgia Road; P.O. Box 4155; Birmingham, AL  35206

1.32.4
Materials
1.32.4.1
Wire mesh partitions shall be 8'-0" high and shall be "standard" width as provided by the manufacturer and as indicated on the drawings.  Adjustable width panels shall be used as necessary where standard width panels will not completely close in an area.

1.32.4.2
The following materials are for crib fencing manufactured by the Logan Co. whose catalog is referenced in section 1.32.3.:



Fabric shall be triple-crimped ten gauge steel wire, woven into 1 1/2 inch diamond mesh with wire securely clinched to the channel frame.



Panel frames shall be 1 inch by 1/2 inch by 1/8 inch standard mill channel with corners mortised and tenoned for extra strength.  Vertical frames shall be provided with holes for bolting to adjacent panels and shall extend below the bottom frame member to provide sweeping space.



Horizontal stiffeners shall be 1 inch by 3/8 inch by 1/8 inch standard mill channels with one on each side of the wire mesh, 3'-6" above the floor line, and shall be riveted together through mesh on about 13 inch centers.



Top capping members shall be 1 1/2 inch by 1/2 inch by 1/8 inch standard mill channels pre-drilled on about 12 inch centers for adjustment in spacing machine bolt fasteners on about 24 inch centers.  Top capping members shall be furnished to span more than one panel.



Floor sockets shall be high grade gray iron castings, equipped with set screw and provided with two slotted holes for floor attachment.



Hinged door panels shall be of the same frame and fabric construction as the fencing panels, shall be finish-banded with 1 inch by 1/8 inch flat bars, and shall swing smoothly on three 2 1/2 inch by 3 1/2 inch tight-pin butt hinges.  Each door panel shall be equipped to receive a lockset and furnished without the mortise as described herein.



The locksets for the hinged door panels shall be passage-type, shall be furnished and installed with cylinders, and shall be bored and tapped to receive mortise lock cylinders having the following dimensions:



Outside diameter
1.25 inch



Overall depth
1.250 inch



Threads
32 per inch, 0.027 inch depth



Lockwood (Lloyd Matheson, Inc.) Catalog No. 951 cylinders will be furnished and installed by others.  No substitutes for the Lockwood cylinders are permitted.



Each hinged door panel shall be mounted to and shall close against a standard channel post which shall be anchored to the floor with concrete anchors of the size recommended by the manufacturer.



All bolts, clips, and other accessories required for complete installation of the crib fencing shall be compatible with the panels used and shall be furnished by the Contractor.

1.32.4.3
Crib fencing manufactured by the Logan Co., Acorn Wire and Iron Works, Kentucky Metal Products, Inc., and Miller Wire Works, Inc. are considered to satisfy the specification requirements for crib fencing.  No further approval is required for use of these products.

1.32.5
Installation
1.32.5.1
Horizontal runs of crib fencing shall terminate in channel posts.  Panel frames shall be set in floor sockets.  Both channel posts and floor sockets shall be fastened to the floor with concrete anchors of the size recommended by the manufacturer.

1.32.5.2
Crib fencing (mesh partitions and doors) shall be installed plumb, level, and in accordance with the manufacturer's installation instructions.  The panels shall be anchored to the walls when so indicated on the design drawings in addition to being anchored to the floor.

1.32.6
Cleaning

At the completion of this work, all excess materials and debris shall be removed from the site, and the entire area shall be left in a neat condition.

1.32.7
Damaged Fencing

All fencing installed in accordance with this specification that is damaged prior to the Contractor completing the work under this contract shall be repaired or replaced at the Contractor's expense.

STD SPEC
1.32 - 2

