1.36
Flexible Plastic Strips
1.36.1
Scope

Furnish and install complete with all accessories, fasteners, closures, and flashings as indicated on the drawings and herein specified.

1.36.2
General

Flexible plastic strips shall provide a see-through barrier which aids in insulation and at the same time allows for quick and easy passage for personnel.

1.36.3
Materials

Flexible strip closures shall consist of double overlapping, transparent vitran strips fitted with prepunched galvanized hanger brackets which made with formed metal arms on the universal hardware.  The shape of the galvanized hardware shall provide full swivel action to lessen stress on the strips from passing high loads.  A snap-on cover plate shall be provided to form a box section and prevent accidental removal of the strips of two inch centers.  End strips shall be orange to outline the opening.  Strips shall have rounded edges and overlap shall form a seal.

1.36.4
Performance Requirements

1.36.4.1
Use eight inch wide by 0.80 inch thick strips for interior areas.

1.36.4.2
Strips shall overlap two inches on each side with a four inch spacing in between.

1.36.4.3
Wall mounts should be used wherever possible.  They require five inches of wall space above the door for mounting.  Wall mounts provided an overlap at each edge of the door.

1.36.4.4
Header mounts are used where a permanent door or other obstacle prohibits the use of a wall mount.

1.36.5
Work To Be Performed
1.36.5.1
Erection shall be in accordance with the manufacturer's instructions.

1.36.5.2
Any closures that are abraded, scarred, bent, scratched, deformed any way physically, or have any defect shall be replaced prior to acceptance of the job.

1.36.6
Submittals

For required submittals, see Division VI of this specification.



�page \# "'Page: '#'�'"  ���





STD SPEC
1.36 - 1

