Copyright 2005 AIA
MASTERSPEC Full Length
05/05 (R 05/06)

Copyright 2005 by The American Institute of Architects (AIA)

Exclusively published and distributed by Architectural Computer Services, Inc. (ARCOM) for the AIA

SECTION 096513 - RESILIENT BASE AND ACCESSORIES
Revise this Section by deleting and inserting text to meet Project-specific requirements.

This Section uses the term "Architect."  Change this term to match that used to identify the design professional as defined in the General and Supplementary Conditions.

Verify that Section titles referenced in this Section are correct for this Project's Specifications; Section titles may have changed.

PART 1 -  GENERAL

1.1 RELATED DOCUMENTS

Retain or delete this article in all Sections of Project Manual.

A. Drawings and general provisions of the Contract, including General and Supplementary Conditions and Division 01 Specification Sections, apply to this Section.

1.2 SUMMARY

A. Section Includes:

1. Resilient base.

2. Resilient stair accessories.

3. Resilient molding accessories.

B. Related Sections:

Retain Sections in subparagraphs below that contain requirements Contractor might expect to find in this Section but are specified in other Sections.

1. Division 09 Section "Resilient Sheet Flooring" for resilient sheet floor coverings.

2. Division 09 Section "Linoleum Flooring" for linoleum floor coverings.

3. Division 09 Section "Resilient Tile Flooring" for resilient floor tile.

4. Division 09 Section "Static-Control Resilient Flooring" for resilient floor coverings designed to control electrostatic discharge.

5. Division 09 Section "Resilient Athletic Flooring" for resilient floor coverings for use in athletic-activity or support areas.

1.3 SUBMITTALS

Five paragraphs in this article are defined in Division 01 Section "Submittal Procedures" as "Action Submittals."

A. Product Data:  For each type of product indicated.

B. LEED Submittals:

Retain subparagraph below if low-emitting materials are required for LEED-NC or LEED-CI Credit EQ 4.1; coordinate with requirements selected in Part 2 for adhesives.

1. Product Data for Credit EQ 4.1:  For adhesives, including printed statement of VOC content.

C. Samples for Initial Selection:  For each type of product indicated.

Delete "Samples for Initial Selection" Paragraph above if colors and other characteristics are preselected and specified or scheduled.  Retain first paragraph below with or without above.

D. Samples for Verification:  For each type of product indicated, in manufacturer's standard-size Samples but not less than 12 inches (300 mm) long, of each resilient product color, texture, and pattern required.

E. Product Schedule:  For resilient products.[  Use same designations indicated on Drawings.]

1.4 QUALITY ASSURANCE

Delete first paragraph below if fire-test-response testing is not required by authorities having jurisdiction.  In most building codes, resilient products are exempt from fire-test-response requirements.  See "Fire-Test-Response Characteristics" Article in the Evaluations for additional information.

A. Fire-Test-Response Characteristics:  As determined by testing identical products according to ASTM E 648 or NFPA 253 by a qualified testing agency.

1. Critical Radiant Flux Classification:  Class I, not less than 0.45 W/sq. cm.

B. Mockups:  Provide resilient products with mockups specified in other Sections.

1.5 DELIVERY, STORAGE, AND HANDLING

A. Store resilient products and installation materials in dry spaces protected from the weather, with ambient temperatures maintained within range recommended by manufacturer, but not less than 50 deg F (10 deg C) or more than 90 deg F (32 deg C).

1.6 PROJECT CONDITIONS

Ambient temperature range for installation varies among manufacturers.  Consult manufacturers for recommendations and revise first paragraph below to suit Project.

A. Maintain ambient temperatures within range recommended by manufacturer, but not less than [70 deg F (21 deg C)] <Insert temperature> or more than [95 deg F (35 deg C)] <Insert temperature>, in spaces to receive resilient products during the following time periods:

1. 48 hours before installation.

2. During installation.

3. 48 hours after installation.

B. Until Substantial Completion, maintain ambient temperatures within range recommended by manufacturer, but not less than [55 deg F (13 deg C)] <Insert temperature> or more than [95 deg F (35 deg C)] <Insert temperature>.

C. Install resilient products after other finishing operations, including painting, have been completed.

1.7 EXTRA MATERIALS

A. Furnish extra materials that match products installed and that are packaged with protective covering for storage and identified with labels describing contents.

1. Furnish not less than [10 linear feet (3 linear m)] <Insert length> for every [500 linear feet (150 linear m)] <Insert length> or fraction thereof, of each type, color, pattern, and size of resilient product installed.

PART 2 -  PRODUCTS

2.1 RESILIENT BASE <Insert drawing designation>

Refer to the tables at the end of the Evaluations for a list of manufacturers' products.  Use these tables in combination with manufacturers' catalogs or product data to insert series, type, model, and designations of other characteristics.

Copy this article and re-edit for each product.

Insert number to complete drawing designation.  Use these designations on Drawings to identify each product.

A. Resilient Base:

See Editing Instruction No. 1 in the Evaluations for cautions about naming manufacturers.  Retain subparagraph and list of manufacturers below.  See Division 01 Section "Product Requirements."

1. Manufacturers:  Subject to compliance with requirements, [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

a. Allstate Rubber Corp.; Stoler Industries.

b. Armstrong World Industries, Inc.

c. Burke Mercer Flooring Products; Division of Burke Industries, Inc.

d. Endura Rubber Flooring; Division of Burke Industries, Inc.

e. Estrie Products International; American Biltrite (Canada) Ltd.

f. Flexco, Inc.

g. Johnsonite.

h. Mondo Rubber International, Inc.

i. Musson, R. C. Rubber Co.

j. Nora Rubber Flooring; Freudenberg Building Systems, Inc.

k. PRF USA, Inc.

l. Roppe Corporation, USA.

m. VPI, LLC; Floor Products Division.

n. <Insert manufacturer's name>.

B. Resilient Base Standard:  ASTM F 1861.

See "Resilient Base" Article in the Evaluations for a discussion of material requirement, manufacturing method, and style.

1. Material Requirement:  [Type TV (vinyl, thermoplastic)] [Type TS (rubber, vulcanized thermoset)] [Type TP (rubber, thermoplastic)] [Type TS (rubber, vulcanized thermoset) or Type TP (rubber, thermoplastic)].

2. Manufacturing Method:  [Group I (solid, homogeneous)] [Group II (layered)] [Group I (solid, homogeneous) or Group II (layered)].

3. Style:  [Cove (base with toe)] [Straight (flat or toeless)] [Butt to (fit-to-floor)] <Insert special style>.

Both types of rubber base are generally 0.125 inch (3.2 mm) thick; however, some manufacturers offer various thicknesses.  Vinyl base is available in both thickness options in first paragraph below but not from every manufacturer.

C. Minimum Thickness:  [0.125 inch (3.2 mm)] [0.080 inch (2.0 mm)] <Insert thickness>.

D. Height:  [2-1/2 inches (64 mm)] [4 inches (102 mm)] [6 inches (152 mm)] [As indicated on Drawings].

Coordinate height selected in paragraph above with length selected in first paragraph below; some manufacturers do not offer coils for every height.

E. Lengths:  [Cut lengths, 48 inches (1219 mm) long] [Coils in manufacturer's standard length] [Cut lengths 48 inches (1219 mm) long or coils in manufacturer's standard length].

F. Outside Corners:  [Job formed] [Preformed] [Job formed or preformed].

Coordinate first paragraph below with "Resilient Base Installation" Article.

G. Inside Corners:  [Job formed] [Preformed] [Job formed or preformed].

H. Finish:  [Satin] [Matte] [Low luster] [As selected by Architect from manufacturer's full range].

Retain paragraph below if colors and patterns are not indicated in a separate schedule.

I. Colors and Patterns:  [As indicated by manufacturer's designations] [Match Architect's sample] [As selected by Architect from full range of industry colors].

Insert characteristics for resilient terrazzo tile base if required for Project.

2.2 RESILIENT STAIR ACCESSORIES <Insert drawing designation>

Refer to the tables at the end of the Evaluations for a list of manufacturers' products.  Use these tables in combination with manufacturers' catalogs or product data to insert series, type, model, and designations of other characteristics.

Copy this article and re-edit for each product.

Insert number to complete drawing designation.  Use these designations on Drawings to identify each product.

A. Resilient Stair Treads:

See Editing Instruction No. 1 in the Evaluations for cautions about naming manufacturers.  Retain subparagraph and list of manufacturers below.  See Division 01 Section "Product Requirements."

1. Manufacturers:  Subject to compliance with requirements, [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

a. Burke Mercer Flooring Products; Division of Burke Industries, Inc.

b. Endura Rubber Flooring; Division of Burke Industries, Inc.

c. Estrie Products International; American Biltrite (Canada) Ltd.

d. Flexco, Inc.

e. Johnsonite.

f. Mondo Rubber International, Inc.

g. Musson, R. C. Rubber Co.

h. Nora Rubber Flooring; Freudenberg Building Systems, Inc.

i. PRF USA, Inc.

j. R.C.A. Rubber Company (The).

k. Roppe Corporation, USA.

l. VPI, LLC; Floor Products Division.

m. <Insert manufacturer's name>.

B. Resilient Stair Treads Standard:  ASTM F 2169.

See "Resilient Base" Article in the Evaluations for a discussion of type, class, and group.

1. Material Requirement:  [Type TV (vinyl, thermoplastic)] [Type TS (rubber, vulcanized thermoset)] [Type TP (rubber, thermoplastic)] [Type TS (rubber, vulcanized thermoset) or Type TP (rubber, thermoplastic)].

2. Surface Design:

Retain first subparagraph below for smooth (flat) tread surface.  Retain second subparagraph for tread surface with pattern.

a. Class 1, Smooth (flat).

b. Class 2, Pattern:  [Raised-disc design] [Raised-square design] [Raised-chevron design] [Raised-diamond design] [Raised-rib design] [Raised-rib design with abrasive strips] <Insert pattern>.

Retain subparagraph below for both Class 1 and 2 treads.

3. Manufacturing Method:  [Group 1, tread with embedded abrasive strips] [Group 2, tread with contrasting color for the visually impaired].

C. Nosing Style:  [Square, adjustable to cover angles between 60 and 90 degrees] [Square] [Round].

D. Nosing Height:  [1-1/2 inches (38 mm)] [2 inches (51 mm)] [2-3/16 inches (56 mm)] <Insert dimension>.

E. Thickness:  [1/4 inch (6 mm) and tapered to back edge] <Insert thickness>.

F. Size:  Lengths and depths to fit each stair tread in [one piece] [one piece or, for treads exceeding maximum lengths manufactured, in equal-length units].

Retain first paragraph below for separate riser units or delete and insert requirements for integral stair and riser units as required by Project.

G. Risers:  Smooth, flat, [coved-toe, 7 inches (178 mm) high by length matching treads] [toeless, height and length to cover risers]; produced by same manufacturer as treads and recommended by manufacturer for installation with treads.

1. Thickness:  [0.125 inch (3.2 mm)] [0.080 inch (2.0 mm)] <Insert thickness>.

H. Stringers:  Of same thickness as risers, height and length after cutting to fit risers and treads and to cover stair stringers; produced by same manufacturer as treads and recommended by manufacturer for installation with treads.

Retain paragraph below if colors and patterns are not indicated in a separate schedule.

I. Colors and Patterns:  [As indicated by manufacturer's designations] [Match Architect's sample] [As selected by Architect from full range of industry colors].

2.3 RESILIENT MOLDING ACCESSORY <Insert drawing designation>

Refer to the tables at the end of the Evaluations for a list of manufacturers' products.  Use these tables in combination with manufacturers' catalogs or product data to insert series, type, model, and designations of other characteristics.

Copy this article and re-edit for each product.

Insert number to complete drawing designation.  Use these designations on Drawings to identify each product.

A. Resilient Molding Accessory:

See Editing Instruction No. 1 in the Evaluations for cautions about naming manufacturers.  Retain subparagraph and list of manufacturers below.  See Division 01 Section "Product Requirements."

1. Manufacturers:  Subject to compliance with requirements, [provide products by one of the following] [available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following]:

a. Burke Mercer Flooring Products; Division of Burke Industries, Inc.

b. Flexco, Inc.

c. Johnsonite.

d. R.C.A. Rubber Company (The).

e. Roppe Corporation, USA.

f. VPI, LLC; Floor Products Division.

g. <Insert manufacturer's name>.

B. Description:  [Cap for cove carpet] [Cap for cove resilient floor covering] [Carpet bar for tackless installations] [Carpet edge for glue-down applications] [Nosing for carpet] [Nosing for resilient floor covering] [Reducer strip for resilient floor covering] [Joiner for tile and carpet] [Transition strips] <Insert description>.

C. Material:  [Vinyl] [Rubber].

D. Profile and Dimensions:  [As indicated] <Insert profile and dimensions>.

Retain paragraph below if colors and patterns are not indicated in a separate schedule.

E. Colors and Patterns:  [As indicated by manufacturer's designations] [Match Architect's sample] [As selected by Architect from full range of industry colors].

2.4 INSTALLATION MATERIALS

A. Trowelable Leveling and Patching Compounds:  Latex-modified, portland cement based or blended hydraulic-cement-based formulation provided or approved by manufacturer for applications indicated.

B. Adhesives:  Water-resistant type recommended by manufacturer to suit resilient products and substrate conditions indicated.

Retain subparagraph below if low-emitting materials are required for LEED-NC or LEED-CI Credit EQ 4.1.

1. Use adhesives that comply with the following limits for VOC content when calculated according to 40 CFR 59, Subpart D (EPA Method 24):

a. Cove Base Adhesives:  Not more than 50 g/L.

Retain subparagraph below if rubber stair treads or molding accessories are required.

b. Rubber Floor Adhesives:  Not more than 60 g/L.

Retain first paragraph below if resilient stair treads are specified in this Section.

C. Stair-Tread-Nose Filler:  Two-part epoxy compound recommended by resilient tread manufacturer to fill nosing substrates that do not conform to tread contours.

Retain first paragraph below if metal edge strips are required.

D. Metal Edge Strips:  Extruded aluminum with mill finish of width shown, of height required to protect exposed edges of tiles, and in maximum available lengths to minimize running joints.

Before retaining paragraph below, verify manufacturers' written instructions for floor polish products.  Delete paragraph if this Section is used with resilient floor coverings at stair landings (with floor polish) specified in other Sections.

E. Floor Polish:  Provide protective liquid floor polish products as recommended by resilient stair tread manufacturer.

PART 3 -  EXECUTION

3.1 EXAMINATION

Coordinate requirements specified in other Sections for substrate construction and tolerances to ensure that they are appropriate for resilient products selected.

A. Examine substrates, with Installer present, for compliance with requirements for maximum moisture content and other conditions affecting performance of the Work.

B. Verify that finishes of substrates comply with tolerances and other requirements specified in other Sections and that substrates are free of cracks, ridges, depressions, scale, and foreign deposits that might interfere with adhesion of resilient products.

C. Proceed with installation only after unsatisfactory conditions have been corrected.

3.2 PREPARATION

Extensive surface preparation is required over substrates from which existing products have been removed.  Requirements vary among manufacturers.  Insert requirements to suit Project.

A. Prepare substrates according to manufacturer's written instructions to ensure adhesion of resilient products.

B. Concrete Substrates for Resilient Stair Treads and Accessories:  Prepare according to ASTM F 710.

1. Verify that substrates are dry and free of curing compounds, sealers, and hardeners.

2. Remove substrate coatings and other substances that are incompatible with adhesives and that contain soap, wax, oil, or silicone, using mechanical methods recommended by manufacturer.  Do not use solvents.

3. Alkalinity and Adhesion Testing:  Perform tests recommended by manufacturer.

4. Moisture Testing:  Perform tests recommended by manufacturer[ and as follows].  Proceed with installation only after substrates pass testing.

Follow manufacturer's requirements for moisture testing.  Retain one or both subparagraphs below.

a. Perform anhydrous calcium chloride test, ASTM F 1869.  Proceed with installation only after substrates have maximum moisture-vapor-emission rate of [3 lb of water/1000 sq. ft. (1.36 kg of water/92.9 sq. m)] <Insert emission> in 24 hours.

b. Perform relative humidity test using in situ probes, ASTM F 2170.  Proceed with installation only after substrates have maximum [75 percent] <Insert acceptable percentage> relative humidity level measurement.

C. Fill cracks, holes, and depressions in substrates with trowelable leveling and patching compound and remove bumps and ridges to produce a uniform and smooth substrate.

D. Do not install resilient products until they are same temperature as the space where they are to be installed.

1. Move resilient products and installation materials into spaces where they will be installed at least 48 hours in advance of installation.

E. Sweep and vacuum clean substrates to be covered by resilient products immediately before installation.

3.3 RESILIENT BASE INSTALLATION

A. Comply with manufacturer's written instructions for installing resilient base.

B. Apply resilient base to walls, columns, pilasters, casework and cabinets in toe spaces, and other permanent fixtures in rooms and areas where base is required.

C. Install resilient base in lengths as long as practicable without gaps at seams and with tops of adjacent pieces aligned.

D. Tightly adhere resilient base to substrate throughout length of each piece, with base in continuous contact with horizontal and vertical substrates.

E. Do not stretch resilient base during installation.

Retain first paragraph below if required or revise to suit Project.

F. On masonry surfaces or other similar irregular substrates, fill voids along top edge of resilient base with manufacturer's recommended adhesive filler material.

Retain first paragraph below for resilient base with preformed corners; retain second paragraph for resilient base with job-formed corners.

G. Preformed Corners:  Install preformed corners before installing straight pieces.

H. Job-Formed Corners:

1. Outside Corners:  Use straight pieces of maximum lengths possible.  Form without producing discoloration (whitening) at bends.

2. Inside Corners:  Use straight pieces of maximum lengths possible.

3.4 RESILIENT ACCESSORY INSTALLATION

A. Comply with manufacturer's written instructions for installing resilient accessories.

B. Resilient Stair Accessories:

1. Use stair-tread-nose filler to fill nosing substrates that do not conform to tread contours.

2. Tightly adhere to substrates throughout length of each piece.

Retain subparagraph below if applicable.

3. For treads installed as separate, equal-length units, install to produce a flush joint between units.

C. Resilient Molding Accessories:  Butt to adjacent materials and tightly adhere to substrates throughout length of each piece.  Install reducer strips at edges of [carpet] [resilient floor covering] that would otherwise be exposed.

3.5 CLEANING AND PROTECTION

A. Comply with manufacturer's written instructions for cleaning and protection of resilient products.

B. Perform the following operations immediately after completing resilient product installation:

1. Remove adhesive and other blemishes from exposed surfaces.

Delete first subparagraph below if only resilient base is included in this Section.

2. Sweep and vacuum surfaces thoroughly.

3. Damp-mop surfaces to remove marks and soil.

C. Protect resilient products from mars, marks, indentations, and other damage from construction operations and placement of equipment and fixtures during remainder of construction period.

Before retaining first paragraph below, verify resilient stair tread manufacturer's written instructions for floor polish.  Generally, floor polish is used with most vinyl products.

D. Floor Polish:  Remove soil, visible adhesive, and surface blemishes from resilient stair treads before applying liquid floor polish.

Revise subparagraph below to suit products selected.

1. Apply [one] [two] [three] <Insert requirement> coat(s).

E. Cover resilient products until Substantial Completion.

END OF SECTION 096513

RESILIENT BASE AND ACCESSORIES
096513 - 1

