Copyright 2001 and 2005 AIA
MASTERSPEC Full Length
05/01 (R 05/06)

Copyright 2001 and 2005 by The American Institute of Architects (AIA)

Exclusively published and distributed by Architectural Computer Services, Inc. (ARCOM) for the AIA

SECTION 102113 - TOILET COMPARTMENTS
This Section uses the term "Architect."  Change this term to match that used to identify the design professional as defined in the General and Supplementary Conditions.

Verify that Section titles referenced in this Section are correct for this Project's Specifications; Section titles may have changed.

PART 1 -  GENERAL

1.1 RELATED DOCUMENTS

A. Drawings and general provisions of the Contract, including General and Supplementary Conditions and Division 01 Specification Sections, apply to this Section.

1.2 SUMMARY

See "Unit Classifications" Article in the Evaluations for descriptions of other, less frequently specified types of units, which can be added to this Section and inserted in first paragraph below.

A. This Section includes [baked-enamel] [stainless-steel] [plastic-laminate] [phenolic-core] [solid-polymer] units as follows:

Adjust list below to suit Project.  Consult manufacturers to verify that mounting styles selected in subparagraphs below are available for type of unit selected in paragraph above.

1. Toilet Enclosures:  [Overhead braced] [Floor anchored] [Ceiling hung] [Floor and ceiling anchored].

2. Entrance Screens:  [Overhead braced] [Floor anchored] [Ceiling hung] [Floor and ceiling anchored].

3. Urinal Screens:  [Wall hung] [Floor anchored] [Post supported] [Overhead braced].

B. Related Sections include the following:

List below only products, construction, and equipment that the reader might expect to find in this Section but are specified elsewhere.

Steel overhead support is generally required for ceiling-hung units; wood construction does not provide adequate support.

1. Division 05 Section "Metal Fabrications" for supports that attach [ceiling-hung] [floor-and-ceiling-anchored] units to overhead structural system.

Wood may provide adequate overhead support for floor-and-ceiling-anchored units but is inadequate for supporting ceiling-hung units.  Consult manufacturers for recommendations.

2. Division 06 Section "[Rough Carpentry] [Miscellaneous Carpentry]" for [blocking] [and] [overhead support of floor-and-ceiling-anchored units].

Delete below if accessories are inserted in this Section.  Some manufacturers offer a limited number of standard accessory items.  Verify availability with manufacturers selected.

3. Division 10 "Toilet, Bath, and Laundry Accessories" for toilet tissue dispensers, grab bars, purse shelves, and similar accessories.

1.3 SUBMITTALS

A. Product Data:  For each type of product indicated.  Include construction details, material descriptions, dimensions of individual components and profiles, and finishes.

B. LEED Submittal:

Retain subparagraph below if plastic-laminate units are used and low-emitting materials are required for LEED-NC or LEED-CI Credit EQ 4.4.

1. Product Data for Credit EQ 4.4:  For particleboard cores of plastic-laminate units, documentation indicating that product contains no urea formaldehyde.

C. Shop Drawings:  Include plans, elevations, sections, details, and attachments to other work.

Delete subparagraph below if not required or revise to suit Project.

1. Show locations of cutouts for compartment-mounted toilet accessories.

Retain subparagraph below if required for metal units; see "Metal Units, General" Article in the Evaluations.

2. Show locations of reinforcements for compartment-mounted grab bars.

D. Samples for Initial Selection:  For each type of unit indicated.

Delete paragraph above if colors and other characteristics are preselected and specified or scheduled.  Retain below with or without above.

E. Samples for Verification:  Of each type of color and finish required for units, prepared on 6-inch- (150-mm-) square Samples of same thickness and material indicated for Work.

1.4 QUALITY ASSURANCE

Consider retaining paragraph below for Federal projects.

A. Comply with requirements in CID-A-A-60003, "Partitions, Toilets, Complete."

If required, insert fire-test-response requirements for phenolic-core or solid-polymer units.  See "Fire-Test-Response Characteristics" Article in the Evaluations.

1.5 PROJECT CONDITIONS

A. Field Measurements:  Verify actual locations of walls, columns, ceilings, and other construction contiguous with toilet compartments by field measurements before fabrication and indicate measurements on Shop Drawings.

Delete below if not allowed.

1. Established Dimensions:  Where field measurements cannot be made without delaying the Work, establish dimensions and proceed with fabricating toilet compartments without field measurements.  Coordinate wall, floor, ceilings, and other contiguous construction to ensure that actual dimensions correspond to established dimensions.

PART 2 -  PRODUCTS

2.1 METAL UNITS

See Editing Instruction No. 1 in the Evaluations for cautions about naming manufacturers and products.

A. Available Manufacturers:  Subject to compliance with requirements, manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following:

Retain above for nonproprietary or below for semiproprietary specification.  Refer to Division 01 Section "Product Requirements."

B. Manufacturers:  Subject to compliance with requirements, provide products by one of the following:

1. Accurate Partitions Corporation.

2. All American Metal Corp.

3. American Sanitary Partition Corporation.

4. Ampco.

5. Bradley Corporation; Mills Partitions.

6. Flush Metal Partition Corp.

7. General Partitions Mfg. Corp.

8. Global Steel Products Corp.

9. Hadrian Inc.

10. Knickerbocker Partitions Corp.

11. Lambaton Universal.

12. Metpar Corp.

13. Sanymetal; a Crane Plumbing Company.

14. Weis-Robart Partitions, Inc.

15. <Insert manufacturer's name.>

If specific coating designation for hot-dip-galvanized or galvannealed steel sheet is required, insert requirements in first paragraph below.

C. Baked-Enamel Units:  Facing sheets and closures fabricated from [ASTM A 591/A 591M, 80Z (24G) (electrolytically zinc-coated)] [or] [ASTM A 653/A 653M (hot-dip galvanized or galvannealed)], commercial steel sheet for exposed applications, that is mill phosphatized, and selected for smoothness.

1. Facing Sheet Thicknesses:  Minimum base-metal (uncoated) thicknesses as follows:

Before editing first six subparagraphs below, see "Metal Units, General" Article, and its accompanying table, in the Evaluations for discussion of steel sheet thicknesses; the table correlates traditional gage numbers with decimal thicknesses.

a. Pilasters, Braced at Both Ends:  Manufacturer's standard thickness, but not less than 0.0329 inch (0.85 mm).

Retain subparagraph above for overhead-braced and floor-and-ceiling-anchored mounting styles.  Retain subparagraph below for floor-anchored and ceiling-hung mounting styles.

b. Pilasters, Unbraced at One End:  Manufacturer's standard thickness, but not less than 0.0438 inch (1.1 mm).

Retain first option in first subparagraph below to maximize competition among listed manufacturers.  Second option will eliminate some manufacturers or require custom fabrications.

c. Panels:  [Manufacturer's standard thickness, but not less than 0.0269 inch (0.7 mm)] [0.0329 inch (0.0.85 mm)].

d. Doors:  Manufacturer's standard thickness, but not less than 0.0269 inch (0.7 mm).

e. Integral-Flange, Wall-Hung Urinal Screens:  Manufacturer's standard thickness, but not less than 0.0269 inch (0.7 mm).

f. Wedge-Shaped, Wall-Hung Urinal Screens:  Manufacturer's standard thickness, but not less than 0.0329 inch (0.85 mm).

Revise first subparagraph below to suit Project.  See Evaluations.

2. Finish:  Manufacturer's standard pigmented, organic coating, including thermosetting, electrostatically applied, and powder coatings.  Provide coating system that complies with coating manufacturer's written instructions for pretreatment, application, baking, and minimum dry film thickness.

a. Color:  [One color] [Two colors] in each room as selected by Architect from manufacturer's full range of colors.

D. Stainless-Steel Units:  Facing sheets and closures fabricated from ASTM A 666, Type 302 or 304, stainless-steel sheet, leveled to stretcher-leveled flatness.

1. Stainless-Steel Facing Sheet Thicknesses:  Specified thicknesses as follows:

Before editing first six subparagraphs below, see "Metal Units, General" Article, and its accompanying table, in the Evaluations for discussion of steel sheet thicknesses; the table correlates traditional gage numbers with specified decimal thicknesses.

a. Pilasters, Braced at Both Ends:  Manufacturer's standard thickness, but not less than 0.0375 inch (0.95 mm).

Retain subparagraph above for overhead-braced and floor-and-ceiling-anchored mounting styles.  Retain subparagraph below for floor-anchored and ceiling-hung mounting styles.

b. Pilasters, Unbraced at One End:  Manufacturer's standard thickness, but not less than 0.050 inch (1.3 mm).

Retain first option in first subparagraph below to maximize competition among listed manufacturers.  Second option will eliminate some manufacturers or require custom fabrications.

c. Panels:  [Manufacturer's standard thickness, but not less than 0.0312 inch (0.8 mm)] [0.0375 inch (0.95 mm)].

d. Doors:  Manufacturer's standard thickness, but not less than 0.0312 inch (0.8 mm).

Retain first subparagraph below for government-style metal screens; coordinate with manufacturers selected.

e. Integral-Flange, Wall-Hung Urinal Screens:  Manufacturer's standard thickness, but not less than 0.0312 inch (0.8 mm).

f. Wedge-Shaped, Wall-Hung Urinal Screens:  Manufacturer's standard thickness, but not less than 0.0375 inch (0.95 mm).

Textured finishes are available from some manufacturers, but textures vary.

2. Finish:  [No. 4 bright, directional polish] [Manufacturer's standard textured finish] on exposed faces.  Exposed surfaces are protected from damage by application of strippable, temporary protective covering before shipment.

Some manufacturers weld face sheets together; others do not.  Revise first paragraph below to suit Project.

E. Door, Panel, and Pilaster Construction:  Seamless, metal facing sheets are pressure laminated to core material.  Units have continuous, interlocking molding strip or lapped and formed edge closures.  Exposed surfaces are free of pitting, seam marks, roller marks, stains, discolorations, telegraphing of core material, or other imperfections.  Corners are sealed by welding or clips.  Exposed welds are ground smooth.

Revise subparagraphs below to suit Project.  See Evaluations.

1. Core Material:  Manufacturer's standard sound-deadening honeycomb of resin-impregnated kraft paper in thickness required to provide finished thickness of 1 inch (25 mm) for doors and panels and 1-1/4 inches (32 mm) for pilasters.

2. Grab-Bar Reinforcement:  Provide concealed internal reinforcement for grab bars mounted on units.

If necessary, insert requirements for internal reinforcement where small compartment-mounted accessories, such as toilet tissue dispensers, are attached; see "Metal Units, General" Article in the Evaluations.

3. Tapping Reinforcement:  Provide concealed reinforcement for tapping (threading) at locations where machine screws are used for attaching items to units.

Retain second option in subparagraph below for government-style metal screens; coordinate with manufacturers selected.

4. Urinal-Screen Construction:  [Matching panels] [Similar to panels, with integral full-height flanges for wall attachment, and maximum 1-1/4 inches (32 mm) thick] [Similar to panels, V-shaped, fabricated for concealed wall attachment, and maximum 6 inches (150 mm) wide at wall and minimum 1 inch (25 mm) wide at protruding end].

F. Pilaster [Shoes] [and] [Sleeves (Caps)]:  Stainless steel, ASTM A 666, Type 302 or 304, not less than 0.0312 inch (0.8 mm) specified thickness and 3 inches (75 mm) high, finished to match hardware.

G. Brackets (Fittings):

Retain one or both subparagraphs below; if retaining both, indicate locations of each type of fitting on Drawings or by inserts.  If necessary, revise to indicate fitting requirements for specific applications, e.g., heavy-duty urinal-screen brackets.

1. Stirrup Type:  Ear or U-brackets, [chrome-plated, nonferrous, cast zinc alloy (zamac) or clear anodized aluminum] [stainless steel] [chrome-plated brass].

2. Full-Height (Continuous) Type:  Manufacturer's standard design; [stainless steel] [aluminum].

2.2 PLASTIC-LAMINATE UNITS

See Editing Instruction No. 1 in the Evaluations for cautions about naming manufacturers and products.

A. Available Manufacturers:  Subject to compliance with requirements, manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following:

Retain above for nonproprietary or below for semiproprietary specification.  Refer to Division 01 Section "Product Requirements."

B. Manufacturers:  Subject to compliance with requirements, provide products by one of the following:

1. Accurate Partitions Corporation.

2. All American Metal Corp.

3. American Sanitary Partition Corporation.

4. Ampco.

5. Bobrick Washroom Equipment, Inc.

6. Bradley Corporation; Mills Partitions.

7. Flush Metal Partition Corp.

8. General Partitions Mfg. Corp.

9. Global Steel Products Corp.

10. Knickerbocker Partitions Corp.

11. Lambaton Universal.

12. Metpar Corp.

13. Partition Systems, Inc.

14. Sanymetal; a Crane Plumbing Company.

15. Tex-Lam Manufacturing, Inc.

16. Weis-Robart Partitions, Inc.

17. <Insert manufacturer's name.>

C. Plastic Laminate:  NEMA LD 3, HGS, 0.048-inch (1.2-mm) nominal thickness.

1. Color and Pattern:  [One color and pattern] [Two colors and patterns] in each room as selected by Architect from manufacturer's full range of colors and patterns.

D. Door, Panel, and Pilaster Construction:  Plastic-laminate facing sheets are pressure laminated to core material without splices or joints in facings or cores.  Laminate is applied to edges before broad surfaces to seal edges and prevent laminate from being pried loose.  Exposed core material is sealed at cutouts to protect core from moisture.

Revise three subparagraphs and associated subparagraphs below to suit Project.  See Evaluations.  Retain option in first subparagraph if low-emitting materials are required for LEED-NC or LEED-CI Credit EQ 4.4.

1. Core Material:  ANSI A208.1, Grade M-2 particleboard with 45-lb (20.4-kg) density[; made with binder containing no urea formaldehyde].

1-inch- (25-mm-) thick doors and panels are the predominant standard offering of listed manufacturers.  However, some offer 7/8-inch- (22-mm-) thick doors and panels as standard but will provide 1-inch- (25-mm-) thick doors and panels if specified.  Manufacturers may offer panels that are thinner than doors as standard but will provide panels that match door thickness if specified.

2. Doors and Panels:  Finished to not less than [7/8 inch (22 mm)] [1 inch (25 mm)] thick.

Retain option in subparagraph below if more than one type of pilaster construction is acceptable; coordinate with manufacturers selected.

3. Pilasters:  Provide construction to comply with[ one of] the following:

1-1/4-inch- (32-mm-) thick pilasters with particleboard cores are the predominant standard offering from manufacturers.  Steel reinforcement is usually an upgrade; before retaining option in subparagraph below, verify availability with manufacturers selected.

a. Finished to not less than 1-1/4 inches (32 mm) thick[ and with internal, 0.1196-inch- (3.0-mm-) thick steel-sheet reinforcement].

American Sanitary provides construction described in subparagraph below as standard.

b. Finished to 1-1/4 inches (32 mm) thick and with manufacturer's standard core of corrosion-resistant steel sheet laminated to both sides of honeycomb of resin-impregnated kraft paper in lieu of particleboard core.

Bobrick provides construction described in subparagraph below as standard.

c. Finished to not less than 1 inch (25 mm) thick and with internal, 0.1196-inch- (3.0-mm-) thick steel-sheet reinforcement.

E. Pilaster [Shoes] [and] [Sleeves (Caps)]:  Stainless steel, ASTM A 666, Type 302 or 304, not less than 0.0312 inch (0.8 mm) specified thickness and 3 inches (75 mm) high, finished to match hardware.

F. Brackets (Fittings):

Retain one or both subparagraphs below; if retaining both, indicate locations of each type of fitting on Drawings or by inserts.  If necessary, revise to indicate fitting requirements for specific applications, e.g., heavy-duty urinal-screen brackets.

1. Stirrup Type:  Ear or U-brackets, [chrome-plated, nonferrous, cast zinc alloy (zamac) or clear anodized aluminum] [stainless steel] [chrome-plated brass].

2. Full-Height (Continuous) Type:  Manufacturer's standard design; [stainless steel] [aluminum].

2.3 PHENOLIC-CORE UNITS

See Editing Instruction No. 1 in the Evaluations for cautions about naming manufacturers and products.

A. Available Manufacturers:  Subject to compliance with requirements, manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following:

Retain above for nonproprietary or below for semiproprietary specification.  Refer to Division 01 Section "Product Requirements."

B. Manufacturers:  Subject to compliance with requirements, provide products by one of the following:

1. Accurate Partitions Corporation.

2. American Sanitary Partition Corporation.

3. Ampco.

4. Bobrick Washroom Equipment, Inc.

5. Bradley Corporation; Mills Partitions.

6. Capitol Partitions, Inc.

7. Flush Metal Partition Corp.

8. General Partitions Mfg. Corp.

9. Global Steel Products Corp.

10. Knickerbocker Partitions Corp.

11. Lambaton Universal.

12. Metpar Corp.

13. Partition Systems, Inc.

Sanymetal does not recommend ceiling-hung and floor-anchored mounting styles for phenolic-core units.

14. Sanymetal; a Crane Plumbing Company.

15. Tex-Lam Manufacturing, Inc.

16. Weis-Robart Partitions, Inc.

17. Young Group, Ltd. (The); DesignRite Partitions.

18. <Insert manufacturer's name.>

Systems are available that incorporate a lap joint at doors and pilasters to eliminate vertical sightlines into compartments.  If required, verify availability with manufacturers selected and revise first paragraph below.

C. Door, Panel and Pilaster Construction:  Solid phenolic-core panel material with melamine facing on both sides fused to substrate during panel manufacture (not separately laminated), and with eased and polished edges.  Provide minimum 3/4-inch- (19-mm-) thick doors and pilasters and minimum 1/2-inch- (13-mm-) thick panels.

1. Facing Sheet Color:  [One color] [Two colors] in each room as selected by Architect from manufacturer's full range of colors.

2. Core Color:  Manufacturer's standard dark color.

D. Pilaster [Shoes] [and] [Sleeves (Caps)]:  Stainless steel, ASTM A 666, Type 302 or 304, not less than 0.0312 inch (0.8 mm) specified thickness and 3 inches (75 mm) high, finished to match hardware.

E. Brackets (Fittings):

Retain one or both subparagraphs below; if retaining both, indicate locations of each type of fitting on Drawings or by inserts.  If necessary, revise to indicate fitting requirements for specific applications, e.g., heavy-duty urinal-screen brackets.

1. Stirrup Type:  Ear or U-brackets, [stainless steel] [chrome-plated, nonferrous, cast zinc alloy (zamac) or clear anodized aluminum] [chrome-plated brass].

2. Full-Height (Continuous) Type:  Manufacturer's standard design; [stainless steel] [aluminum].

2.4 SOLID-POLYMER UNITS

Solid-polymer units are generally available in overhead-braced and floor-and-ceiling-anchored mounting styles.  If ceiling-hung units are required, consult manufactures for recommendations and to verify availability.  Floor-anchored mounting style is generally not recommended for solid-polymer units.

See Editing Instruction No. 1 in the Evaluations for cautions about naming manufacturers and products.

A. Available Manufacturers:  Subject to compliance with requirements, manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following:

Retain above for nonproprietary or below for semiproprietary specification.  Refer to Division 01 Section "Product Requirements."

B. Manufacturers:  Subject to compliance with requirements, provide products by one of the following:

Standard height for doors and panels offered by most manufacturers is 55 inches (1397 mm); few offer 58-inch- (1473-mm-) high doors and panels.  If this dimension is critical, verify the height of standard units with manufacturers selected.

1. Accurate Partitions Corporation.

2. Ampco.

3. Bradley Corporation; Mills Partitions.

4. Capitol Partitions, Inc.

5. Comtec Industries.

6. General Partitions Mfg. Corp.

7. Global Steel Products Corp.

8. Metpar Corp.

9. Santana Products, Inc.

10. Sanymetal; a Crane Plumbing Company.

11. Weis-Robart Partitions, Inc.

12. <Insert manufacturer's name.>

Retain options for HDPE and PP in first paragraph below to maximize competition among listed manufacturers.  Currently, PP is available only from Capitol and Comtec.  Costs for HDPE and PP units are essentially the same; however, PP has greater scratch and dent resistance than HDPE and is virtually graffiti proof.

Systems are available that incorporate a lap joint at doors and pilasters to eliminate vertical sightlines into compartments.  If required, verify availability with manufacturers selected and revise first paragraph below.

C. Door, Panel, and Pilaster Construction:  Solid, [high-density polyethylene (HDPE)] [or] [polypropylene (PP)] panel material, not less than 1 inch (25 mm) thick, seamless, with eased edges, and with homogenous color and pattern throughout thickness of material.

1. Color and Pattern:  [One color and pattern] [Two colors and patterns] in each room as selected by Architect from manufacturer's full range of colors and patterns.

D. Pilaster [Shoes] [and] [Sleeves (Caps)]:  Manufacturer's standard design; [polymer or stainless steel] [polymer] [stainless steel].

Retain subparagraph below if polymer material is selected in paragraph above.

1. Polymer Color and Pattern:  [Matching pilaster] [Contrasting with pilaster, as selected by Architect from manufacturer's full range of colors and patterns].

E. Brackets (Fittings):

Retain one or both of first two subparagraphs below; if retaining both, indicate locations of each type on Drawings or by inserts.  If necessary, revise to indicate fitting requirements for specific applications, e.g., heavy-duty urinal-screen brackets.

1. Stirrup Type:  Ear or U-brackets, [chrome-plated, nonferrous, cast zinc alloy (zamac) or clear anodized aluminum] [stainless steel] [chrome-plated brass].

Polymer brackets are not recommended for use with ceiling-hung or floor-anchored units.

2. Full-Height (Continuous) Type:  Manufacturer's standard design; [polymer or extruded aluminum] [polymer] [extruded aluminum] [stainless steel].

Retain subparagraph below with first or second option in subparagraph above.

a. Polymer Color and Pattern:  [Matching pilaster] [Contrasting with pilaster, as selected by Architect from manufacturer's full range of colors and patterns].

F. Heat-Sink Strip:  Manufacturer's standard continuous, extruded-aluminum strip fastened to exposed bottom edges of solid-polymer components to prevent burning.

Ceiling-hung units are available from a limited number of manufacturers; before retaining below, verify availability with manufacturers selected.

G. Overhead Cross Bracing for Ceiling-Hung Units:  As recommended by manufacturer and fabricated from solid polymer.

2.5 ACCESSORIES

A. Hardware and Accessories:  Manufacturer's standard design, heavy-duty operating hardware and accessories.

Retain subparagraph below or revise to suit Project.  See Evaluations.

1. Material:  [Chrome-plated, nonferrous, cast zinc alloy (zamac) or clear anodized aluminum] [Stainless steel] [Chrome-plated brass].

Retain paragraph below for overhead-braced units.  Revise if specific finish is required.  Generally, clear anodized finish is provided.

B. Overhead Bracing:  Manufacturer's standard continuous, extruded-aluminum head rail with antigrip profile and in manufacturer's standard finish.

Retain paragraph below for post-supported urinal screens; revise for floor-to-ceiling posts.

C. Support Posts for Urinal Screens:  Manufacturer's standard aluminum post with floor shoe for anchoring to floor construction.

Revise below to suit Project.

D. Anchorages and Fasteners:  Manufacturer's standard exposed fasteners of stainless steel or chrome-plated steel or brass, finished to match hardware, with theft-resistant-type heads.  Provide sex-type bolts for through-bolt applications.  For concealed anchors, use hot-dip galvanized or other rust-resistant, protective-coated steel.

2.6 FABRICATION

Retain applicable mounting styles in four paragraphs below.

A. Overhead-Braced Units:  Provide manufacturer's standard corrosion-resistant supports, leveling mechanism, fasteners, and anchors at pilasters to suit floor conditions.  Make provisions for setting and securing continuous head rail at top of each pilaster.  Provide shoes at pilasters to conceal supports and leveling mechanism.

B. Floor-Anchored Units:  Provide manufacturer's standard corrosion-resistant anchoring assemblies complete with threaded rods, lock washers, and leveling adjustment nuts at pilasters for structural connection to floor.  Provide shoes at pilasters to conceal anchorage.

C. Ceiling-Hung Units:  Provide manufacturer's standard corrosion-resistant anchoring assemblies complete with threaded rods, lock washers, and leveling adjustment nuts at pilasters for connection to structural support above finished ceiling.  Provide assemblies that support pilasters from structure without transmitting load to finished ceiling.  Provide sleeves (caps) at tops of pilasters to conceal anchorage.

D. Floor-and-Ceiling-Anchored Units:  Provide manufacturer's standard corrosion-resistant anchoring assemblies complete with leveling adjustment at tops and bottoms of pilasters.  Provide shoes and sleeves (caps) at pilasters to conceal anchorage.

Verify requirements of authorities having jurisdiction and revise paragraph and subparagraphs below to suit Project.

E. Doors:  Unless otherwise indicated, provide 24-inch- (610-mm-) wide in-swinging doors for standard toilet compartments and 36-inch- (914-mm-) wide out-swinging doors with a minimum 32-inch- (813-mm-) wide clear opening for compartments indicated to be accessible to people with disabilities.

If desired, insert specific hinge type required in subparagraph below; see Evaluations.  For solid-polymer units, manufacturers' hinges are either preset or adjustable.  If preset hinges are acceptable, insert requirements for hold-open angle.

1. Hinges:  Manufacturer's standard self-closing type that can be adjusted to hold doors open at any angle up to 90 degrees.

Recessed latch-operating devices that are accessible to people with disabilities are available.  Verify requirements of authorities having jurisdiction.

2. Latch and Keeper:  Manufacturer's standard [recessed] [surface-mounted] latch unit designed for emergency access and with combination rubber-faced door strike and keeper.  Provide units that comply with accessibility requirements of authorities having jurisdiction at compartments indicated to be accessible to people with disabilities.

3. Coat Hook:  Manufacturer's standard combination hook and rubber-tipped bumper, sized to prevent door from hitting compartment-mounted accessories.

4. Door Bumper:  Manufacturer's standard rubber-tipped bumper at out-swinging doors[ and entrance screen doors].

5. Door Pull:  Manufacturer's standard unit at out-swinging doors that complies with accessibility requirements of authorities having jurisdiction.  Provide units on both sides of doors at compartments indicated to be accessible to people with disabilities.

PART 3 -  EXECUTION

3.1 INSTALLATION

A. General:  Comply with manufacturer's written installation instructions.  Install units rigid, straight, level, and plumb.  Secure units in position with manufacturer's recommended anchoring devices.

1. Maximum Clearances:

a. Pilasters and Panels:  1/2 inch (13 mm).

b. Panels and Walls:  1 inch (25 mm).

Delete subparagraph and associated subparagraphs below if no stirrup brackets.  For solid-polymer units, three brackets are recommended for wall-to-panel and panel-to-pilaster connections to prevent sagging of panel.

2. Stirrup Brackets:  Secure panels to walls and to pilasters with not less than [two brackets attached] [three brackets attached at midpoint and] near top and bottom of panel.

a. Locate wall brackets so holes for wall anchors occur in masonry or tile joints.

b. Align brackets at pilasters with brackets at walls.

Retain applicable mounting styles from five paragraphs below.

B. Overhead-Braced Units:  Secure pilasters to floor and level, plumb, and tighten.  Secure continuous head rail to each pilaster with not less than two fasteners.  Hang doors to align tops of doors with tops of panels and adjust so tops of doors are parallel with overhead brace when doors are in closed position.

C. Floor-Anchored Units:  Set pilasters with anchors penetrating not less than 2 inches (50 mm) into structural floor, unless otherwise indicated in manufacturer's written instructions.  Level, plumb, and tighten pilasters.  Hang doors and adjust so tops of doors are level with tops of pilasters when doors are in closed position.

D. Ceiling-Hung Units:  Secure pilasters to supporting structure and level, plumb, and tighten.  Hang doors and adjust so bottoms of doors are level with bottoms of pilasters when doors are in closed position.

E. Floor-and-Ceiling-Anchored Units:  Secure pilasters to supporting construction and level, plumb, and tighten.  Hang doors and adjust so doors are level and aligned with panels when doors are in closed position.

F. [Wall-Hung] [and] [Post-Supported] Urinal Screens:  Attach with anchoring devices to suit supporting structure.  Set units level and plumb and to resist lateral impact.

3.2 ADJUSTING

Revise hinge adjustment requirements below to suit Project.  Verify accessibility requirements of authorities having jurisdiction.

A. Hardware Adjustment:  Adjust and lubricate hardware according to manufacturer's written instructions for proper operation.  Set hinges on in-swinging doors to hold doors open approximately 30 degrees from closed position when unlatched.  Set hinges on out-swinging doors[ and doors in entrance screens] to return doors to fully closed position.

END OF SECTION 102113

TOILET COMPARTMENTS
102113 - 1

