Copyright 2003 and 2005 AIA
MASTERSPEC Full Length
05/03 (R 05/06)

Copyright 2003 and 2005 by The American Institute of Architects (AIA)

Exclusively published and distributed by Architectural Computer Services, Inc. (ARCOM) for the AIA

SECTION 102226.13 - ACCORDION FOLDING PARTITIONS
This Section uses the term "Architect."  Change this term to match that used to identify the design professional as defined in the General and Supplementary Conditions.

Verify that Section titles referenced in this Section are correct for this Project's Specifications; Section titles may have changed.

PART 1 -  GENERAL

1.1 RELATED DOCUMENTS

A. Drawings and general provisions of the Contract, including General and Supplementary Conditions and Division 01 Specification Sections, apply to this Section.

1.2 SUMMARY

A. This Section includes the following:

Adjust list below to suit Project.

1. Manually operated, accordion folding partitions.

2. Electrically operated, accordion folding partitions.

3. [Manually] [Electrically] operated, fire-rated accordion folding partitions.

B. Related Sections include the following:

List below only products that the reader might expect to find in this Section but are specified elsewhere.

1. Division 09 Section "Gypsum Board " for sound barrier[ or fire-rated] construction above ceiling at track.

Delete subparagraph below if no electrically operated partitions, or revise if electrical connections and wiring are inserted into this Section.  If inserting connections and wiring, add "Electrical connections and wiring to building service" Subparagraph to list in "This Section includes" Paragraph above.

2. Division 26 Sections for electrical service and connections for motor operators, controls, and limit switches; and for system disconnect switches.

1.3 DEFINITIONS

Retain abbreviations that remain after this Section has been edited.

A. NIC:  Noise Isolation Class.

B. NRC:  Noise Reduction Coefficient.

C. NVLAP:  National Voluntary Laboratory Accreditation Program.

D. STC:  Sound Transmission Class.

1.4 PERFORMANCE REQUIREMENTS

Revise first paragraph below to indicate specific loads determined by Project's structural engineer or refer to loads indicated on Drawings.  Model building codes and ASCE 7 establish criteria for buildings subject to earthquake motions.  Verify requirements of authorities having jurisdiction.

A. Seismic Performance:  Provide accordion folding partitions capable of withstanding the effects of earthquake motions determined according to [ASCE 7, "Minimum Design Loads for Buildings and Other Structures."] <Insert applicable code requirement.>

B. Acoustical Performance:  Provide accordion folding partitions tested by a qualified testing agency for the following acoustical properties according to test methods indicated:

If retaining either of first two subparagraphs below, indicate ratings in Part 2 or on Drawings.

1. Sound Transmission Requirements:  Accordion folding partition assembly tested in a laboratory for sound transmission loss performance according to ASTM E 90, determined by ASTM E 413, and rated for not less than the STC indicated.

Retain subparagraph below only if NRC rating is required.  Verify with manufacturer that values retained for NRC and STC are possible and practical for products selected.

2. Noise Reduction Requirements:  Accordion folding partition assembly, identical to partition tested for STC, tested for sound-absorption performance according to ASTM C 423, and rated for not less than the NRC indicated.

Retain subparagraph below only if NIC rating is required for installed accordion folding partitions.  Verify with manufacturer that values retained for NIC, NRC, and STC are possible and practical for products selected and as installed.  Testing is required in Part 3 "Field Quality Control" Article.  See Evaluations.

3. Acoustical Performance Requirements:  Installed accordion folding partition assembly, identical to partition tested for STC, tested for NIC according to ASTM E 336, determined by ASTM E 413, and rated for [10 points less than STC value indicated] [not less than the NIC value indicated] <Insert value>.

Retain paragraph below for fire-rated accordion folding partition.

C. Fire Resistance:  Provide fire-rated accordion folding partition assemblies with fire-resistance ratings indicated.

1.5 SUBMITTALS

A. Product Data:  For each type of product indicated.

Revise paragraph below to clarify special conditions that deviate from standard installations.

B. Shop Drawings:  Include plans, elevations, sections, details, and attachments to other work.

Retain first subparagraph below if products are required to withstand specific design loads and design responsibilities have been delegated to Contractor or if structural data are required as another way to verify product's compliance with performance requirements.  Professional engineer qualifications are specified in Division 01 Section "Quality Requirements."

1. For installed products indicated to comply with design loads, include structural analysis data signed and sealed by the qualified professional engineer responsible for their preparation.

2. Indicate storage and operating clearances.  Indicate location and installation requirements for hardware and track, blocking, and direction of travel.

Retain subparagraph below if equipment includes wiring.

3. Wiring Diagrams:  Power, signal, and control wiring.

Retain paragraph and subparagraphs below if Drawings do not include detailed plans or if Project involves unusual coordination requirements.

C. Coordination Drawings:  Reflected ceiling plans, drawn to scale, on which the following items are shown and coordinated with each other, based on input from installers of the items involved:

Edit subparagraphs below to suit Project.

1. Suspended ceiling components.

2. Structural members to which suspension systems will be attached.

3. Size and location of initial access modules for acoustical tile.

4. Items penetrating finished ceiling, including the following:

a. Lighting fixtures.

b. Air outlets and inlets.

c. Speakers.

d. Sprinklers.

e. Access panels.

f. <Insert item.>

Edit first paragraph below to suit Project.

D. Setting Drawings:  For embedded items and cutouts required in other work[, including support-beam punching template].

E. Samples for Initial Selection:  For each type of finish, covering, or facing indicated.

1. Include similar Samples of accessories involving color selection.

Delete paragraph and subparagraph above if colors and other characteristics are preselected and specified or scheduled.  Retain first paragraph and subparagraphs below with or without above.

F. Samples for Verification:  For each type of finish, covering, or facing indicated, prepared on Sample of size indicated below.

1. Fabric:  Full width by not less than 36-inch- (914-mm-) long section of fabric from dye lot to be used for the Work, with specified treatments applied.  Show complete pattern repeat.

Retain subparagraph above for textile wall coverings; retain first subparagraph below for other type of facing material.

2. Facing Material:  Manufacturer's standard-size unit, not less than 3 inches (75 mm) square.

3. Edge Material:  Not less than full width by 3 inches (75 mm) long.

4. Hardware:  Manufacturer's standard exposed door-operating device.

G. LEED Submittals:

Retain subparagraph below if low-emitting materials are required for LEED-NC or LEED-CI Credit EQ 4.4.

1. Product Data for Credit EQ 4.4:  For composite wood products, documentation indicating that product contains no urea formaldehyde.

Retain subparagraph below and "Forest Certification" Paragraph in "Quality Assurance" Article if partitions contain wood veneers or wood-based panel products and wood is required to be certified for LEED-NC or LEED-CI Credit MR 7, which requires that a minimum of 50 percent of wood-based materials be certified.

2. Certificates for Credit MR 7:  Chain-of-custody certificates certifying that accordion folding partitions comply with forest certification requirements.  Include evidence that manufacturer is certified for chain of custody by an FSC-accredited certification body.

a. Include statement indicating costs for each certified wood product.

H. Product Certificates:  For each type of accordion folding partition, signed by product manufacturer.

Retain paragraph below if matching fabric for several accordion folding partitions.

I. Material Certificates:  For each fabric dye lot, signed by manufacturers.

Coordinate paragraph below with qualification requirements in Division 01 Section "Quality Requirements" and as supplemented in "Quality Assurance" Article.

J. Qualification Data:  For [Installer] [and] [testing agency].

Retain first paragraph below if Contractor is responsible for field quality-control testing.

K. Field quality-control test reports.

L. Product Test Reports:  Based on evaluation of comprehensive tests performed by a qualified testing agency, for each accordion folding partition.

M. Maintenance Data:  For accordion folding partitions to include in maintenance manuals.

1. Finish facings and finishes for exposed trim and accessories.  Include precautions for cleaning materials and methods that could be detrimental to finishes and performance.

2. Seals, hardware, track, carriers, and other operating components.

Retain subparagraph below if electric operation is specified.

3. For electric operator.

N. Warranty:  Special warranty specified in this Section.

1.6 QUALITY ASSURANCE

Retain first paragraph below if required for LEED-NC or LEED-CI Credit MR 7.  Before retaining, verify that manufacturers selected comply.

A. Manufacturer Qualifications:  A qualified manufacturer that is certified for chain of custody by an FSC-accredited certification body.

B. Installer Qualifications:  An employer of workers trained and approved by manufacturer.

Retain paragraph below if Contractor or manufacturer selects testing agency.

C. Testing Agency Qualifications:  An independent agency or an NVLAP-accredited laboratory.  NVLAP-accredited laboratories must document accreditation, based on a "Certificate of Accreditation" and a "Scope of Accreditation" listing the test methods specified.

Delete paragraph and subparagraphs below if no fire-rated units are required.

D. Fire-Test-Response Characteristics:  Provide accordion folding partitions with the following fire-test-response characteristics as determined by testing identical products by UL or another testing and inspecting agency acceptable to authorities having jurisdiction:

1. Surface-Burning Characteristics:  ASTM E 84.

a. Flame-Spread Index:  25 or less.

b. Smoke-Developed Index:  450 or less.

Retain subparagraph below for fire-rated accordion folding partitions.

2. Fire-Rated Door Assemblies:  Comply with NFPA 80, based on testing according to UL 10B.

Revise subparagraph above and below if not acceptable to authorities having jurisdiction.  Currently, manufacturers test accordion folding partitions as "Special-Purpose Fire Doors" per UL 10B.

3. Oversize Fire-Rated Door Assemblies:  For units exceeding sizes of tested assemblies, provide certification or label certification by a testing agency acceptable to authorities having jurisdiction that doors comply with standard construction requirements for tested and labeled fire-rated door assemblies except for size.

Delete subparagraph below if no textile or carpet wall covering or if protection by automatic sprinkler system is an acceptable alternative meeting requirements of authorities having jurisdiction.  Example below is relevant for textile wall covering under the UBC.  BOCA and SBCCI have requirements for carpet and carpetlike wall coverings.  SBCCI references NFPA 265; SBCCI and BOCA have prescriptive testing requirements.  Verify and insert requirements of authorities having jurisdiction.

4. Fire Growth Contribution:  Textile wall coverings complying with acceptance criteria in UBC Standard 8-2.

Retain paragraph below if accordion folding partitions contain wood veneers or wood-based panel products and wood is required to be certified for LEED-NC or LEED-CI Credit MR 7, which requires that a minimum of 50 percent of wood-based materials be certified.

E. Forest Certification:  Fabricate products with wood, wood veneers, and wood-based panel products produced from wood obtained from forests certified by an FSC-accredited certification body to comply with FSC STD-01-001, "FSC Principles and Criteria for Forest Stewardship."

Retain paragraph below if low-emitting materials are required for LEED-NC or LEED-CI Credit EQ 4.4.  Verify availability before retaining.

F. Low-Emitting Materials:  Provide accordion folding partitions made with adhesives and composite wood products that do not contain urea-formaldehyde resins.

Retain paragraph below if electric operation is specified.

G. Electrical Components, Devices, and Accessories:  Listed and labeled as defined in NFPA 70, Article 100, by a testing agency acceptable to authorities having jurisdiction, and marked for intended use.

Delete paragraph below if Work of this Section is not extensive or complex enough to justify a preinstallation conference.  If retaining, coordinate with Division 01.

H. Preinstallation Conference:  Conduct conference at Project site to comply with requirements in Division 01 Section "Project Management and Coordination."

1.7 DELIVERY, STORAGE, AND HANDLING

Delete this Article if sequencing facing materials is not required, e.g., for matching pattern or grain.

A. Protectively package and sequence accordion folding partitions in order for installation.  Clearly mark packages with numbering system used on Shop Drawings.  Do not use permanent markings on partitions.

1.8 PROJECT CONDITIONS

A. Field Measurements:  Verify accordion folding partition openings by field measurements before fabrication and indicate measurements on Shop Drawings.

1.9 WARRANTY

When warranties are required, verify with Owner's counsel that special warranties stated in this Article are not less than remedies available to Owner under prevailing local laws.  Coordinate with Division 01 Section "Product Requirements."

A. Special Warranty:  Manufacturer's standard form in which manufacturer agrees to repair or replace components of accordion folding partitions that fail in materials or workmanship within specified warranty period.

1. Failures include, but are not limited to, the following:

a. Faulty operation of accordion folding partitions.

b. Deterioration of metals, metal finishes, and other materials beyond normal wear.

Verify available warranties for units and components and insert number in subparagraph below.

2. Warranty Period:  [Two] [Five] <Insert number> years from date of Substantial Completion.

PART 2 -  PRODUCTS

2.1 MANUFACTURERS

See Editing Instruction No. 1 in the Evaluations for cautions about naming manufacturers and products.

A. Available Products:  Subject to compliance with requirements, manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following:

Retain above for nonproprietary or below for semiproprietary specification.  Refer to Division 01 Section "Product Requirements."

B. Products:  Subject to compliance with requirements, provide one of the following:

1. Curtition, Inc.; <Insert product name or designation>.

2. FolDoor, Holcomb & Hoke Mfg. Co., Inc.; <Insert product name or designation>.

3. Hufcor; <Insert product name or designation>.

4. KWIK-WALL Company; <Insert product name or designation>.

5. Moderco Inc.; <Insert product name or designation>.

6. Modernfold, Inc.; <Insert product name or designation>.

7. Panelfold Inc.; <Insert product name or designation>.

8. Won-Door Corporation; <Insert product name or designation>.

9. <Insert manufacturer's name; product name or designation.>

C. Basis-of-Design Product:  The design for accordion folding partitions is based on <Insert manufacturer's name and product>.  Subject to compliance with requirements, provide the named product or a comparable product by one of the following:

1. <Insert, in separate subparagraphs, manufacturer's name.>

2.2 ACCORDION FOLDING PARTITIONS

Verify whether internal sound insulation is included in products selected; if so, retain option in first paragraph below.

A. Construction:  Provide pantograph or hinged-section, accordion folding frame supporting acoustical core and decorative facing/cover, supported by overhead suspension system, designed for horizontal extension and retraction, and reinforced for hardware attachment.[  Securely attach sound-insulating core and facing/cover to frame.] Fabricate partitions rigid; level; plumb; aligned, with tight joints and uniform appearance; and free of deformation and surface and finish irregularities.

1. STC:  Not less than [36] [38] [40] [44] <Insert STC rating>.

Coordinate STC rating in subparagraph above with NRC rating in subparagraph below if retaining both subparagraphs.

2. NRC:  Not less than <Insert NRC rating>.

B. Dimensions:  Fabricate accordion folding partitions, from manufacturer's standard sizes, to form an assembled system of dimensions indicated on Drawings and verified by field measurements.

1. Stacked Thickness:  Not more than [10-1/2 inches (267 mm)] [13 inches (330 mm)] <Insert dimension>.

Retain subparagraph above and below if dimensions are critical.  Thicknesses listed are examples only.

2. Extended Thickness:  Not more than [5-1/2 inches (140 mm)] [7-1/4 inches (184 mm)] <Insert dimension>.

C. Trim:  Manufacturer's standard trim with decorative, protective finish.

D. Tiebacks:  As required to maintain accordion folding partitions in stacked position.

2.3 FIRE-RATED ACCORDION FOLDING PARTITIONS

A. Construction:  Provide fire-rated, hinged-section, accordion folding steel panels supporting acoustical core and decorative vinyl-clad facing/cover, supported by overhead suspension system, designed for horizontal extension and retraction, and reinforced for hardware attachment.  Fabricate partitions rigid; level; plumb; aligned, with tight joints and uniform appearance; and free of deformation and surface and finish irregularities.

1. Fire Rating:  1 hour.

2. STC:  Not less than [44] <Insert STC rating>.

3. Dimensions:  Fabricate accordion folding partitions, from manufacturer's standard sizes, to form an assembled system of dimensions indicated on Drawings and verified by field measurements.

4. Stacked Thickness:  Not more than [16 inches (406 mm)] <Insert dimension>.

Retain subparagraph above and below if dimensions are critical.  Thicknesses listed are examples only.

5. Extended Thickness:  Not more than [7 inches (178 mm)] <Insert dimension>.

B. Trim:  Manufacturer's standard trim with decorative, protective finish.

C. Tiebacks:  As required to maintain accordion folding partitions in stacked position.

2.4 SEALS AND POSTS

A. General:  Provide types of acoustical seals and posts indicated that produce accordion folding partitions complying with acoustical performance requirements and the following:

1. Seals and posts made from materials and in profiles that minimize sound leakage.

2. Seals and posts fitting tight at contact surfaces and sealing continuously between adjacent accordion folding partitions and between partition perimeter and adjacent surfaces, when accordion folding partition is extended and closed.

B. Perimeter Seals and Closures:  Manufacturer's standard vinyl or neoprene vertical seals, horizontal top and bottom seals, and closures for lead posts and jambs.

Retain paragraph below for fire-rated partitions.

C. Perimeter Seals and Closures:  Provide manufacturer's standard vinyl, or neoprene, or woven silica vertical seals, horizontal top and bottom seals, and closures identical to products tested for fire rating indicated and forming an effective smoke and draft seal.

Retain option in paragraph below for posts with air-release vents.  This feature, which allows entrapped air to escape from within accordion folding partition, is standard with most manufacturers.  FolDoor provides for air release at the top of partition through the track assembly.  KWIK-WALL provides for air release through the facing/cover.

D. Posts and Jambs:  Steel or aluminum; formed with deep-nesting and interlocking interfaces[ and air-release vents] and fabricated to ensure rigidity for accordion folding partition.

Retain one or more of five subparagraphs below to suit Project.  Show location(s) of posts and indicate type on Drawings.

1. Post:  [Floating] [Attached] multiple meeting post with [two-way, L-configured] [two-way, centered] [three-way] [four-way] meeting/closing arrangements[ as indicated].

2. Lead Posts:  For single partition, [one post, latching at leading edge] [two posts, latching at both edges].

3. Intermediate Post:  For latching a partition to another partition, latching on [one side] [two opposite sides] of post.

Post types in two subparagraphs below are alternative examples for end partitions stored in pocket storage spaces.

4. Rolling Post:  For moving trailing edge of partition from storage pocket and latching to fixed jamb on pocket door.

5. Sliding Pocket-Door Post:  With partition attached to fixed jamb on pocket door, for moving partition from storage pocket.

2.5 HARDWARE

Revise paragraph below to specify another hardware finish if required to suit Project.

A. Hardware:  Manufacturer's standard manually operated pulls and latches as required to operate accordion folding partitions and as follows; with decorative, protective finish:

Retain one option in first subparagraph below.

1. Latch:  Operable from [both sides] [one side] [one side with coin-slot release on opposite side] of closed partitions.

2. Lock:  [Key-operated lock cylinder] [Lock with deadlock][; keyed to master key system].

Retain subparagraph above or first subparagraph below or, if more than one type, retain both and indicate types in an Accordion Folding Partition Schedule.  If retaining below, coordinate with "Related Sections include" Paragraph in Part 1 "Summary" Article.

3. Locks:  Refer to Division 08 door hardware Sections for lock cylinder and keying requirements.

4. Pendant Pull:  Near top of lead in addition to standard pull handle/latch[ for units more than 10 feet (3 m) high or 20 feet (6 m) wide, or both].

Retain subparagraph below to anchor accordion folding partition in partially open position or to secure intersection with other partition types.

5. Foot Bolt:  On lead post where indicated; secure to post to avoid interference with seals.

2.6 FINISH FACING

Retain or revise this Article to suit Project.  Insert requirements to suit Owner-furnished material.  If needed, insert backing requirements, which vary depending on facing material and manufacturer.

A. General:  Provide finish facings that comply with indicated fire-test-response characteristics; factory attached or applied to accordion folding partitions over acoustical core[ with appropriate backing], using concealed fasteners; designed to be field replaceable.

First two subparagraphs below include options suitable for carpet, fabric, and vinyl-coated fabric.  Revise to suit type of facing retained.  Manufacturers' facings are usually seamless, depending on partition dimensions Owner-furnished material may require joint or seams.

1. Apply facings free of air bubbles, wrinkles, blisters, and other defects, [with vertical edges tightly butted,] [with vertical invisible seams, and] with no gaps or overlaps.  Horizontal [butted edges] [seams] are not permitted.  Tightly secure and conceal raw and selvage edges of facing for finished appearance.

2. Where facings with [directional or repeating patterns or directional weave] [directional, repeating, or matching grain] are indicated, mark facing top and attach facing in same direction.

3. Color/Pattern:  [Match Architect's samples] [As selected by Architect from manufacturer's full range] <Insert manufacturer's name and designation for color and pattern>.

B. Vinyl-Coated Fabric Wall Covering:  Manufacturer's standard mildew-resistant, washable, vinyl-coated fabric wall covering; complying with CFFA-W-101-B for Type [II] [III]; Class A.

1. Total Weight:  <Insert weight.>

Treatment in subparagraph below is integral to standard and upgraded vinyl-coated fabric wall coverings available from most manufacturers and may be required to suit Project.  If needed, insert additional requirements for other treatments after verifying availability with manufacturers.

2. Antimicrobial Treatment:  Additives capable of inhibiting growth of bacteria, fungi, and yeasts.

C. Vinyl Film:  Manufacturer's standard.

Revise first paragraph below to suit Project.  Option is an example of standard carpet wall covering, with total weight not less than 23 oz./linear yd. (713 g/linear m), available from some manufacturers.

D. Carpet Wall Covering:  Manufacturer's standard[ nonwoven, needle-punched carpet with fibers fused to backing].

E. Fabric Wall Covering:  Manufacturer's standard fabric, from same dye lot, treated to resist stains.

F. High-Pressure Decorative Laminate:  NEMA LD 3, Grade HGS.

Retain paragraph below only if wood-veneer-faced panels comply with requirements of authorities having jurisdiction.  Edit to suit Project.  See the Evaluations in this Section and those in Division 06 Section "Interior Architectural Woodwork" for further discussion.

G. Wood Veneer:  Genuine wood veneer, laminated to[ fire-retardant-treated wood] core with moisture-resistant adhesive, of wood species indicated.

Delete paragraph below if not required.  Materials must be acceptable to accordion folding partition manufacturer for fabrication and acoustical performance characteristics and to authorities having jurisdiction for materials' fire-test-response characteristics.

H. Facing Materials:  Owner furnished.

2.7 SUSPENSION SYSTEMS

A. Suspension Tracks:  Steel or aluminum [mounted directly to overhead structural support,] [with adjustable steel hanger rods for overhead support,] designed for type of operation, size, and weight of accordion folding partition indicated.  Size track to support partition operation and storage without damage to suspension system, accordion folding partitions, or adjacent construction.  Limit track deflection, independent of structural support, to no more than 80 percent of bottom clearance.  Provide a continuous system of track sections and accessories to accommodate configuration and layout indicated for partition operation and storage.

Retain first subparagraph below or, if more than one track type is required for Project, retain both "Track" subparagraphs and indicate locations on Drawings.  Won-Door uses parallel tracks.

1. Track:  [Surface mounted] [Surface-mounted parallel tracks].

2. Track:  Recessed[ parallel tracks].

a. Head Closure Trim:  Integral with track for protecting overhead surfaces; [with factory-applied, decorative, protective finish] [primed for field finish].

Retain subparagraph above for surface track or subparagraph below for recessed track.  Track channel pocket below is recommended by some manufacturers for easier track installation.

b. Head Closure Trim and Track Channel Pocket:  For protecting overhead surfaces and enclosing overhead track opening; [with factory-applied, decorative, protective finish] [primed for field finish].

Four-wheel carriers in paragraph below are most common.  Some manufacturers offer 12- or 6-wheel variations for heavier partitions and lead posts.

B. Carriers:  Trolley system as required for size and weight of partition and for easy, quiet operation; with [four-wheel] [or] [six-wheel] [12-wheel] [manufacturer's standard] ball-bearing carriers at lead post and [two-wheel] [manufacturer's standard] ball-bearing carriers at intermediate panel supports.

1. Wheels:  [Nylon] [Steel] [Manufacturer's standard].

Retain first paragraph and subparagraphs below for biparting partitions or to suit Project.  Retain applicable track switch type(s) and accessories below and indicate location on Drawings; delete if not required.

C. Track Switches and Accessories:  As required for type of operation, storage, track configuration, and layout indicated for accordion folding partition, and compatible with partition assembly specified.  Fabricate track switches from steel or aluminum.

1. Center carrier stop.

2. Glide Switch:  Allowing radius turns to divert partition to an auxiliary track.

3. Pivot Switch:  Allowing partition to change direction of travel.

4. Cross-Track Switch:  Allowing one partition to cross track of another.

5. <Insert switch type.>

D. Aluminum Finish:  Mill finish or manufacturer's standard, factory-applied, decorative finish, unless otherwise indicated.

E. Steel Finish:  Factory-applied, corrosion-resistant, protective coating, unless otherwise indicated.

2.8 ELECTRIC OPERATORS

Retain this Article if partition is electrically operated.

A. General:  Provide factory-assembled electric operation system of size and capacity recommended and provided by accordion folding partition manufacturer for partition specified; with electric motor and factory-prewired motor controls, speed reducer, chain drive, remote-control stations, control devices, and accessories required for proper operation.  Include wiring from motor control to motor.  Coordinate operator wiring requirements and electrical characteristics with building electrical system.

B. Comply with NFPA 70.

C. Motor Characteristics:  Sufficient to start, accelerate, and operate connected loads at designated speeds, within installed environment, with indicated operating sequence, and without exceeding nameplate rating or considering service factor.  Comply with NEMA MG 1[, and the following:][.]

Motor characteristics such as NEMA designation, temperature rating, service factor, enclosure type, and efficiency are specified in Division 11 Section "Common Motor Requirements for Equipment." If manufacturer's standard motor is acceptable, delete six subparagraphs below and first option in paragraph above.  If different characteristics are required, add subparagraphs to suit Project.

1. Voltage:  [120 V] [208-220 V] [NEMA standard voltage selected to operate on nominal circuit voltage to which motor is connected] <Insert voltage>.

2. Horsepower:  [1/4] [1/3] [3/4] [Manufacturer's standard] <Insert number>.

3. Enclosure:  [Open dripproof] [Totally enclosed] [Manufacturer's standard].

4. Duty:  Continuous duty at ambient temperature of 105 deg F (40 deg C) and at altitude of 3300 feet (1005 m) above sea level.

5. Service Factor:  1.15 for open dripproof motors; 1.0 for totally enclosed motors.

6. Phase:  [One] [Polyphase].

If partitions are very large and heavy, consult manufacturers and insert requirements for electric interlocks for merging partitions, for partitions more than 18 feet (5.5 m) high, or for all-steel-fabricated accordion folding partitions.

D. Control Equipment:  Complying with NEMA ICS 1, NEMA ICS 2, and NEMA ICS 6.

1. Remote-Control Station:  Manufacturer's standard control station labeled "Open," "Close," and "[Off] [Stop]." Provide two keys per station.

2. Limit Switches:  Adjustable switches, interlocked with motor controls and set to automatically stop accordion folding partition at fully extended and fully stacked positions.

3. Emergency Release Mechanism:  Quick disconnect-release of electric-motor drive system, permitting manual operation in event of operating failure.

PART 3 -  EXECUTION

3.1 EXAMINATION

A. Examine flooring, structural support, and opening, with Installer present, for compliance with requirements for installation tolerances and other conditions affecting performance of accordion folding partitions.

1. Proceed with installation only after unsatisfactory conditions have been corrected.

3.2 INSTALLATION

Verify that manufacturer's written instructions do not conflict with ASTM E 557.

A. General:  Comply with accordion folding partition manufacturer's written installation instructions.

B. Install accordion folding partitions and accessories after other finishing operations, including painting, have been completed.

3.3 ADJUSTING

A. Adjust accordion folding partitions to operate smoothly, without warping or binding.  Lubricate hardware[, electric operator,] and other moving parts.

Retain subparagraph below for fire-rated partitions.

1. Fire-Rated Accordion Folding Partitions:  Verify that operations are functional and comply with requirements of authorities having jurisdiction.

3.4 FIELD QUALITY CONTROL

Retain this Article if sound control is critical.  NIC testing may be cost prohibitive or not practical for panels with an STC rating less than 48.  If retaining, review with Owner and edit to suit Project.  Coordinate with testing requirements in Division 01 to avoid conflicts.

NIC is an in-place, field-test value and does not directly coordinate with laboratory STC values, generally being 5 to 10 points lower than laboratory STC values.  A measure of the relative acoustical isolation of spaces separated by partitions, NIC is affected by opening width and height, room finishes, floor finishes, and flanking paths.  NIC is appropriate for partitions with an STC rating of 48 or higher.

Retain first option in first paragraph below if Owner engages agency; retain second option if Contractor engages agency.  If retaining second option, retain requirement for field quality-control test reports in Part 1 "Submittals" Article.

A. NIC Testing:  [Owner will engage] [Engage] a qualified independent testing and inspecting agency to perform field tests and inspections and prepare test reports.

B. Testing Methodology:  Perform testing of installed accordion folding partition for noise isolation according to ASTM E 336, determined by ASTM E 413, and rated for not less than the NIC indicated.  Adjust and fit partitions to comply with NIC test method requirements.

Revise three paragraphs below to suit Project.  Revise first paragraph to list actual panels to be tested.

C. Testing Extent:  Testing agency shall randomly select [one] <Insert number> accordion folding partition installation(s) for testing.

D. Repair or replace accordion folding partitions within areas where test results indicate partitions do not comply with requirements, and retest partitions.

E. Additional testing and inspecting, at Contractor's expense, will be performed to determine compliance of repaired, replaced, or additional work with specified requirements.

3.5 CLEANING

A. Clean soiled surfaces of accordion folding partitions, to remove dust, loose fibers, fingerprints, adhesives, and other foreign materials according to manufacturer's written instructions.

3.6 DEMONSTRATION

A. Engage a factory-authorized service representative to train Owner's maintenance personnel to adjust, operate, and maintain accordion folding partitions.  Refer to Division 01 Section "Demonstration and Training."

END OF SECTION 102226.13

ACCORDION FOLDING PARTITIONS
102226.13 - 1

