Copyright 2003 and 2005 AIA
MASTERSPEC Full Length
02/03 (R 05/05)

Copyright 2003 and 2005 by The American Institute of Architects (AIA)

Exclusively published and distributed by Architectural Computer Services, Inc. (ARCOM) for the AIA

SECTION 102800 - TOILET, BATH, AND LAUNDRY ACCESSORIES
This Section uses the term "Architect."  Change this term to match that used to identify the design professional as defined in the General and Supplementary Conditions.

Verify that Section titles referenced in this Section are correct for this Project's Specifications; Section titles may have changed.

PART 1 -  GENERAL

1.1 RELATED DOCUMENTS

A. Drawings and general provisions of the Contract, including General and Supplementary Conditions and Division 01 Specification Sections, apply to this Section.

1.2 SUMMARY

A. This Section includes the following:

Adjust list below to suit Project.

1. Public-use washroom accessories.

2. Public-use shower room accessories.

3. Private-use bathroom accessories.

4. Healthcare accessories.

5. Warm-air dryers.

6. Childcare accessories.

7. Underlavatory guards.

8. Custodial accessories.

For large industrial projects, campus-type projects with multiple buildings, or similar situations, Owner may have separate service contracts for furnishing various accessories and corresponding supplies.  If Owner will furnish accessories to Contractor for installation, insert requirements in first paragraph below.

B. Owner-Furnished Material:  <Insert product.>

C. Related Sections include the following:

List below only products and construction that the reader might expect to find in this Section but are specified elsewhere.

1. Division 08 Section "Mirrors" for frameless mirrors.

2. Division 09 Section "Tiling" for ceramic toilet and bath accessories.

3. Division 10 Section "Detention Toilet Accessories" for accessories designed for installation in detention facilities.

1.3 SUBMITTALS

A. Product Data:  For each type of product indicated.  Include the following:

1. Construction details and dimensions.

2. Anchoring and mounting requirements, including requirements for cutouts in other work and substrate preparation.

3. Material and finish descriptions.

4. Features that will be included for Project.

5. Manufacturer's warranty.

Normally delete paragraph below or revise to limit sample requirements to specific items.

B. Samples:  Full size, for each accessory item to verify design, operation, and finish requirements.

Delete subparagraph below if not applicable.

1. Approved full-size Samples will be returned and may be used in the Work.

C. Product Schedule:  Indicating types, quantities, sizes, and installation locations by room of each accessory required.

1. Identify locations using room designations indicated on Drawings.

Delete subparagraph below if product designations are not indicated on Drawings.  Coordinate with Part 2 Articles that specify products.

2. Identify products using designations indicated on Drawings.

D. Maintenance Data:  For toilet and bath accessories to include in maintenance manuals.

1.4 QUALITY ASSURANCE

Delete paragraph below if products are specified by proprietary method alone in Part 2.

A. Source Limitations:  For products listed together in the same articles in Part 2, provide products of same manufacturer unless otherwise approved by Architect.

Retain paragraph below if warm-air dryers are required.

B. Electrical Components, Devices, and Accessories:  Listed and labeled as defined in NFPA 70, Article 100, by a testing agency acceptable to authorities having jurisdiction, and marked for intended use.

1.5 COORDINATION

A. Coordinate accessory locations with other work to prevent interference with clearances required for access by people with disabilities, and for proper installation, adjustment, operation, cleaning, and servicing of accessories.

B. Deliver inserts and anchoring devices set into concrete or masonry as required to prevent delaying the Work.

1.6 WARRANTY

When warranties are required, verify with Owner's counsel that special warranties stated in this Article are not less than remedies available to Owner under prevailing local laws.  Coordinate with Division 01 Section "Product Requirements."

A. Special Mirror Warranty:  Manufacturer's standard form in which manufacturer agrees to replace mirrors that develop visible silver spoilage defects and that fail in materials or workmanship within specified warranty period.

Not all manufacturers offer a special mirror warranty.  Verify available warranties and insert number in subparagraph below.

1. Warranty Period:  [15] <Insert number> years from date of Substantial Completion.

PART 2 -  PRODUCTS

2.1 MATERIALS

For proprietary, semiproprietary, or basis-of-design specification, delete material descriptions below that are determined by manufacturers' product designations.

A. Stainless Steel:  ASTM A 666, Type 304, 0.0312-inch (0.8-mm) minimum nominal thickness, unless otherwise indicated.

B. Brass:  ASTM B 19 flat products; ASTM B 16 (ASTM B 16M), rods, shapes, forgings, and flat products with finished edges; or ASTM B 30, castings.

C. Steel Sheet:  ASTM A 1008/A 1008M, Designation CS (cold rolled, commercial steel), 0.0359-inch (0.9-mm) minimum nominal thickness.

D. Galvanized Steel Sheet:  ASTM A 653/A 653M, with G60 (Z180) hot-dip zinc coating.

E. Galvanized Steel Mounting Devices:  ASTM A 153/A 153M, hot-dip galvanized after fabrication.

F. Fasteners:  Screws, bolts, and other devices of same material as accessory unit and tamper-and-theft resistant where exposed, and of galvanized steel where concealed.

Retain paragraph below for chrome finish.  Brass, steel, and cast-zinc alloy (zamac) are available with chrome finish.

G. Chrome Plating:  ASTM B 456, Service Condition Number SC 2 (moderate service).

Revise first paragraph below if tempered or safety glass is required; ASTM C 1503 does not cover tempered or safety glass mirrors.  Note that reflected images with tempered or safety glass may be distorted.

H. Mirrors:  ASTM C 1503, Mirror Glazing Quality, clear-glass mirrors, nominal 6.0 mm thick.

I. ABS Plastic:  Acrylonitrile-butadiene-styrene resin formulation.

Articles that specify products in the remainder of Part 2 include sample paragraphs and subparagraphs as examples only.  Use samples as guides for developing paragraphs and subparagraphs for other types of accessories.  For nonproprietary specifications, expand product descriptions to fully describe requirements; see "Specification Methods" Article in the Evaluations.

Products offered by manufacturers vary.  Verify products offered by manufacturers selected in each Article in the remainder of Part 2, and revise product characteristics accordingly.

2.2 PUBLIC-USE WASHROOM ACCESSORIES

See Editing Instruction No. 1 in the Evaluations for cautions about naming manufacturers and products.

A. Available Manufacturers:  Subject to compliance with requirements, manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following:

Retain above for nonproprietary or first paragraph below for semiproprietary specification, or delete both for basis-of-design specification.  Refer to Division 01 Section "Product Requirements."

B. Manufacturers:  Subject to compliance with requirements, provide products by one of the following:

C. Basis-of-Design Product:  The design for accessories is based on products indicated.  Subject to compliance with requirements, provide the named product or a comparable product by one of the following:

1. A & J Washroom Accessories, Inc.

2. American Specialties, Inc.

3. Bobrick Washroom Equipment, Inc.

4. Bradley Corporation.

5. General Accessory Manufacturing Co. (GAMCO).

6. <Insert manufacturer's name.>

Delete "Basis-of-Design Product" subparagraphs in remainder of this Article if not applicable.  Coordinate with specification method selected above.

Copy and re-edit paragraphs and subparagraphs in remainder of this Article if more than one of each type of accessory is required.  Insert a unique drawing designation for each product.

D. Toilet Tissue (Roll) Dispenser <Insert drawing designation>:

1. Basis-of-Design Product:  <Insert manufacturer; product.>

2. Description:  [Roll-in-reserve dispenser with hinged front secured with tumbler lockset] [Single-roll dispenser] [Double-roll dispenser] [Double-roll dispenser with shelf] <Insert description>.

3. Mounting:  [Recessed] [Partition mounted serving two adjacent toilet compartments] [Surface mounted].

Controlled-delivery units cannot be used at accessible toilets.

4. Operation:  [Noncontrol delivery with standard spindle] [Noncontrol delivery with theft-resistant spindle] [Spindleless with tension-spring controlled delivery] [Spindleless with tension-spring controlled delivery and self-locking device extending through core that prevents core removal until roll is empty] [Eccentric-shaped, molded-plastic spindle revolves one-half revolution per dispensing operation for controlled delivery; core cannot be removed until roll is empty] <Insert description>.

5. Capacity:  Designed for [4-1/2- or 5-inch- (114- or 127-mm-)] [5-inch- (127-mm-)] <Insert dimension> diameter tissue rolls.

6. Material and Finish:  [Stainless steel, No. 4 finish (satin)] [Chrome-plated zinc alloy (zamac) or steel] [Satin-finish aluminum bracket with plastic spindle] [ABS plastic, gray] <Insert material and finish>.

Combination toilet tissue dispenser units are available with sanitary-napkin disposal units, seat-cover dispensers, or both.

E. Combination Toilet Tissue Dispenser <Insert drawing designation>:

1. Basis-of-Design Product:  <Insert manufacturer; product.>

2. Description:  Combination unit with double-roll toilet tissue dispenser and the following:

a. Removable sanitary-napkin waste receptacle with self-closing disposal-opening cover.

b. Seat-cover dispenser with minimum capacity of [500] [1000] <Insert number> single or half-fold seat covers.

3. Mounting:  [Recessed] [Surface mounted] [Partition mounted, dual access with two tissue rolls per compartment] [Partition mounted, dual access with two tissue rolls per compartment and with one side that mounts flush with partition of accessible compartment].

4. Toilet Tissue Dispenser Capacity:  4-1/2- or 5-inch- (114- or 127-mm-) diameter tissue rolls.

5. Toilet Tissue Dispenser Operation:  [Noncontrol delivery with theft-resistant spindles] <Insert description>.

6. Material and Finish:  [Stainless steel, No. 4 finish (satin)] <Insert material and finish>.

7. Lockset:  Tumbler type.

F. Toilet Tissue (Folded) Dispenser <Insert drawing designation>:

1. Basis-of-Design Product:  <Insert manufacturer; product.>

2. Description:  Folded-tissue dispenser with cover hinged at bottom.

3. Mounting:  Surface mounted.

4. Minimum Capacity:  [1250] <Insert number> single-fold tissues.

5. Material and Finish:  [Stainless steel, No. 4 finish (satin)] <Insert material and finish>.

6. Lockset:  Tumbler type.

Delete subparagraph below if not required.

7. Refill Indicators:  Pierced slots at front.

G. Toilet Tissue (Jumbo-Roll) Dispenser <Insert drawing designation>:

1. Basis-of-Design Product:  <Insert manufacturer; product.>

2. Description:  [One-roll unit] [Two-roll unit with sliding panel to expose other roll].

3. Mounting:  Surface mounted.

4. Capacity:  [9- or 10-inch- (228- or 254-mm-)] <Insert dimension> diameter rolls.

5. Material and Finish:  [Stainless steel, No. 4 finish (satin)] [ABS plastic, gray] <Insert material and finish>.

6. Lockset:  Tumbler type.

7. Refill Indicator:  Pierced slots at front.

H. Paper Towel (Folded) Dispenser <Insert drawing designation>:

1. Basis-of-Design Product:  <Insert manufacturer; product.>

2. Mounting:  [Recessed] [Semirecessed] [Deck mounted, recessed] [Surface mounted].

Single-fold towels usually require 6-inch- (152.4-mm-) deep dispensers.  Dispensers 4 inches (101.6 mm) in depth accommodate C-fold or multifold towels.  C-fold towels are the highest quality and softest folded towels; single-fold towels are the least expensive.

3. Minimum Capacity:  [400 C-fold or 525 multifold towels] [600 C-fold or 800 multifold towels] [400 single-fold towels] <Insert capacity>.

4. Material and Finish:  [Stainless steel, No. 4 finish (satin)] [ABS plastic, gray] <Insert material and finish>.

5. Lockset:  Tumbler type.

Delete subparagraph below if not required.

6. Refill Indicators:  Pierced slots at sides or front.

Roll towel dispensers extend farther from the wall than folded towel dispensers.

I. Paper Towel (Roll) Dispenser <Insert drawing designation>:

1. Basis-of-Design Product:  <Insert manufacturer; product.>

2. Description:  Lever-actuated mechanism permits controlled delivery of paper rolls in preset lengths per stroke.

3. Mounting:  [Recessed] [Semirecessed] [Surface mounted].

4. Minimum Capacity:  [8-inch (203-mm) wide, 800-foot (244-m) long roll] <Insert capacity>.

5. Material and Finish:  [Stainless steel, No. 4 finish (satin)] [ABS plastic, gray] <Insert material and finish>.

6. Lockset:  Tumbler type.

J. Waste Receptacle <Insert drawing designation>:

1. Basis-of-Design Product:  <Insert manufacturer; product.>

2. Mounting:  [Open top, recessed] [Self-closing disposal-opening cover, recessed] [Semirecessed] [Surface mounted] [Wall mounted for corner installation] [Freestanding] [Undercounter] <Insert description>.

3. Minimum Capacity:  <Insert capacity in gal. (L).>

4. Material and Finish:  [Stainless steel, No. 4 finish (satin)] <Insert material and finish>.

5. Liner:  [Reusable vinyl liner] <Insert liner description>.

6. Lockset:  Tumbler type for waste-receptacle.

K. Combination Towel (Folded) Dispenser/Waste Receptacle <Insert drawing designation>:

1. Basis-of-Design Product:  <Insert manufacturer; product.>

2. Description:  Combination unit for dispensing C-fold or multifold towels, with removable waste receptacle.

3. Mounting:  [Surface mounted] [Surface mounted with stainless-steel collar] [Recessed] [Recessed with projecting receptacle] [Semirecessed].

Delete first subparagraph below for surface-mounted unit.

a. Designed for nominal [4-inch (100-mm)] [6-inch (150-mm)] wall depth.

4. Minimum Towel-Dispenser Capacity:  [600 C-fold or 800 multifold paper towels] <Insert capacity>.

5. Minimum Waste-Receptacle Capacity:  [4 gal. (15 L)] [12 gal. (45.4 L)] <Insert capacity>.

6. Material and Finish:  [Stainless steel, No. 4 finish (satin)] <Insert material and finish>.

7. Liner:  [Reusable, vinyl waste-receptacle liner] <Insert liner description>.

8. Lockset:  Tumbler type for towel-dispenser compartment[ and waste receptacle].

L. Combination Towel (Roll) Dispenser/Waste Receptacle <Insert drawing designation>:

1. Basis-of-Design Product:  <Insert manufacturer; product.>

2. Description:  Combination unit for dispensing preset length of roll paper towels, with removable waste receptacle.

3. Mounting:  [Recessed] [Semirecessed] [Surface mounted].

4. Minimum Towel-Dispenser Capacity:  [8-inch (203-mm) wide, 800-foot (244-m) long roll] <Insert capacity>.

5. Minimum Waste Receptacle Capacity:  [8 gal. (30 L)] [12 gal. (45.4 L)] [15 gal. (56.8 L)] <Insert capacity>.

6. Material and Finish:  [Stainless steel, No. 4 finish (satin)] <Insert material and finish>.

7. Liner:  [Reusable, vinyl waste-receptacle liner] <Insert liner description>.

8. Lockset:  Tumbler type for towel dispenser compartment[ and waste receptacle].

M. Multipurpose Soap/Towel Dispenser Unit <Insert drawing designation>:

1. Basis-of-Design Product:  <Insert manufacturer; product.>

2. Description:  Combination unit for dispensing soap in [liquid or lotion] [lather] form and folded towels.

3. Mounting:  [Recessed, designed for nominal 4-inch (100-mm) wall depth] [Surface mounted with stainless-steel collar].

4. Minimum Soap-Dispenser Capacity:  [80 oz. (2.36 L)] <Insert capacity>.

5. Minimum Towel-Dispenser Capacity:  [600 C-fold or 800 multifold] [1000 single-fold] <Insert capacity> towels.

6. Material and Finish:  [Stainless steel, No. 4 finish (satin)] <Insert material and finish> for unit body and soap valve.

7. Lockset:  Tumbler type.

Coordinate requirements for soap dispensers with other Sections; dispensers are often specified in Division 22, especially if a central soap storage tank is included.

N. Liquid-Soap Dispenser <Insert drawing designation>:

1. Basis-of-Design Product:  <Insert manufacturer; product.>

2. Description:  Designed for dispensing soap in [liquid or lotion] [lather] form.

3. Mounting:  [Deck mounted on vanity] [Deck mounted on lavatory] [Horizontally oriented, recessed] [Horizontally oriented, surface mounted] [Vertically oriented, surface mounted].

4. Capacity:  <Insert capacity in oz. (mL).>

5. Materials:  <Insert requirements for valve and reservoir materials and finishes.>

6. Lockset:  Tumbler type.

Delete subparagraph above and below if not required.

7. Refill Indicator:  Window type.

O. Grab Bar <Insert drawing designation>:

1. Basis-of-Design Product:  <Insert manufacturer; product.>

2. Mounting:  Flanges with [concealed] [exposed] fasteners.

3. Material:  Stainless steel, 0.05 inch (1.3 mm) thick.

a. Finish:  [Smooth, No. 4, satin finish] [Smooth, No. 4, satin finish on ends and slip-resistant texture in grip area].

Select first option in first subparagraph below for medium-duty applications, second option for heavy-duty.

4. Outside Diameter:  [1-1/4 inches (32 mm)] [1-1/2 inches (38 mm)].

5. Configuration and Length:  [As indicated on Drawings] [Straight, 36 inches (914 mm) long] <Insert configuration and length>.

P. Vendor <Insert drawing designation>:

1. Basis-of-Design Product:  <Insert manufacturer; product.>

2. Type:  [Sanitary napkin] [Sanitary napkin and tampon] [Condom].

3. Mounting:  [Fully recessed, designed for 4-inch (100-mm) wall depth] [Semirecessed] [Surface mounted].

4. Capacity:  <Insert description.>

5. Operation:  [No coin (free)] [Single coin (25 cents)] [Two coin (50 cents)] <Insert description>.

6. Exposed Material and Finish:  [Stainless steel, No. 4 finish (satin)] <Insert material and finish>

7. Lockset:  Tumbler type with separate lock and key for coin box.

Q. Sanitary-Napkin Disposal Unit <Insert drawing designation>:

1. Basis-of-Design Product:  <Insert manufacturer; product.>

2. Mounting:  [Recessed] [Partition mounted, dual access] [Surface mounted].

3. Door or Cover:  Self-closing disposal-opening cover[ and hinged face panel with tumbler lockset].

4. Receptacle:  Removable.

5. Material and Finish:  [Stainless steel, No. 4 finish (satin)] [ABS plastic, gray] <Insert material and finish>.

R. Seat-Cover Dispenser <Insert drawing designation>:

1. Basis-of-Design Product:  <Insert manufacturer; product.>

2. Mounting:  [Surface mounted] [Recessed] [Partition mounted, dual access].

3. Minimum Capacity:  [250] [500] <Insert number> seat covers.

4. Exposed Material and Finish:  [Stainless steel, No. 4 finish (satin)] [ABS plastic, gray] <Insert material and finish>.

5. Lockset:  Tumbler type.

S. Fold-Down Purse Shelf <Insert drawing designation>:

1. Basis-of-Design Product:  <Insert manufacturer; product.>

2. Description:  Hinged unit with spring-loaded shelf that automatically returns to vertical position.

3. Nominal Size:  [15 inches (381 mm) long by 5-1/2 inches (140 mm) wide] <Insert dimensions>.

4. Material and Finish:  [Chrome-plated cast-zinc alloy (zamac) with stippled finish on tray or stainless steel, No. 4 finish (satin)] [Chrome-plated cast-zinc alloy (zamac) with stippled finish on tray and bright chrome finish on edges] [Stainless steel, No. 4 finish (satin)].

T. Mirror Unit <Insert drawing designation>:

1. Basis-of-Design Product:  <Insert manufacturer; product.>

Tilted and adjustable tilting mirrors are for use by persons in wheelchairs.  Adjustable tilting mirrors are prone to vandalism.  In lieu of tilted mirrors, standard flat mirrors can be mounted at heights to accommodate users in wheelchairs.

2. Frame:  [Stainless-steel angle, 0.05 inch (1.3 mm) thick] [Stainless-steel channel] [Stainless steel, fixed tilt] [Stainless steel, adjustable tilt].

a. Corners:  [Manufacturer's standard] [Mitered and mechanically interlocked] [Welded and ground smooth].

Delete first subparagraph below if not required.

3. Integral Shelf:  5 inches (127 mm) deep.

4. Hangers:  Produce rigid, tamper- and theft-resistant installation, using method indicated below.

a. One-piece, galvanized steel, wall-hanger device with spring-action locking mechanism to hold mirror unit in position with no exposed screws or bolts.

Delete subparagraph above or first subparagraph below, or retain both and indicate as Contractor's option.  Delete below if tilting mirrors are required.

b. Wall bracket of galvanized steel, equipped with concealed locking devices requiring a special tool to remove.

5. Size:  [As indicated on Drawings] <Insert dimensions>.

U. Facial Tissue Dispenser <Insert drawing designation>:

1. Basis-of-Design Product:  <Insert manufacturer; product.>

2. Mounting:  [Wall mounted, recessed] [Surface mounted].

3. Nominal Depth:  [2-1/4 inches (57 mm)] [4 inches (102 mm)] <Insert dimension>.

4. Capacity:  [150] <Insert number> double-ply tissues.

5. Material and Finish:

a. Dispenser Face:  [Stainless steel, No. 4 finish (satin)] [Stainless steel, No. 7 finish (polished)] <Insert material and finish>.

b. Cabinet:  [Steel with corrosion-resistant finish] <Insert material and finish>.

2.3 PUBLIC-USE SHOWER ROOM ACCESSORIES

See Editing Instruction No. 1 in the Evaluations for cautions about naming manufacturers and products.

A. Available Manufacturers:  Subject to compliance with requirements, manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following:

Retain above for nonproprietary or first subparagraph below for semiproprietary specification, or delete both for basis-of-design specification.  Refer to Division 01 Section "Product Requirements."

B. Manufacturers:  Subject to compliance with requirements, provide products by one of the following:

C. Basis-of-Design Product:  The design for accessories is based on products indicated.  Subject to compliance with requirements, provide the named product or a comparable product by one of the following:

1. A & J Washroom Accessories, Inc.

2. American Specialties, Inc.

3. Bobrick Washroom Equipment, Inc.

4. Bradley Corporation.

5. General Accessory Manufacturing Co. (GAMCO).

6. <Insert manufacturer's name.>

Delete "Basis-of-Design Product" subparagraphs in remainder of this Article if not applicable.  Coordinate with specification method selected above.

Copy and re-edit paragraphs and subparagraphs in remainder of this Article if more than one of each type of accessory is required.  Insert a unique drawing designation for each product.

D. Shower Curtain Rod <Insert drawing designation>:

1. Basis-of-Design Product:  <Insert manufacturer; product.>

Select first option in first subparagraph below for normal-duty applications, second option for heavy-duty.

2. Description:  [1-inch (25.4-mm) OD; fabricated from nominal 0.0375-inch- (0.95-mm-) thick stainless steel] [1-1/4-inch (32-mm) OD; fabricated from nominal 0.05-inch- (1.3-mm-) thick stainless steel].

3. Mounting Flanges:  [Stainless-steel flanges designed for exposed fasteners] <Insert requirements>.

4. Finish:  [No. 4 (satin)] <Insert finish>.

E. Shower Curtain <Insert drawing designation>:

1. Basis-of-Design Product:  <Insert manufacturer; product.>

Curtains that are 12 inches (305 mm) wider than opening are recommended for shower openings 48 inches (1219 mm) wide and wider.

2. Size:  Minimum [6 inches (152 mm)] [12 inches (305 mm)] wider than opening by 72 inches (1828 mm) high.

3. Material:  [Vinyl, minimum 0.006-inch- (0.15-mm-) thick, opaque, matte] [Duck, minimum 8 oz. (227-g), white, 100 percent cotton] [Nylon-reinforced vinyl, minimum 10-oz. (284-g) or 0.008-inch- (0.2-mm-) thick vinyl, with integral antibacterial agent] <Insert material>.

Antibacterial shower curtains are available in green.

4. Color:  [White] [Green] [As selected from manufacturer's full range] <Insert color>.

5. Grommets:  Corrosion resistant at minimum 6 inches (152 mm) o.c. through top hem.

6. Shower Curtain Hooks:  Chrome-plated or stainless-steel, spring wire curtain hooks with snap fasteners, sized to accommodate specified curtain rod.  Provide one hook per curtain grommet.

F. Folding Shower Seat <Insert drawing designation>:

1. Basis-of-Design Product:  <Insert manufacturer; product.>

2. Configuration:  [L-shaped seat, designed for wheelchair access] [Rectangular seat] [Triangular, corner-type seat] [Stainless-steel seat designed to fold into recessed-mounted, stainless-steel wall box].

3. Seat:  [Phenolic or polymeric composite of slat-type or one-piece construction in color as selected by Architect] [White vinyl padded seat] [Stainless steel, No. 4 finish (satin); 0.05-inch (1.3-mm) minimum nominal thickness; with single-piece, pan-type construction and edge seams welded and ground smooth] <Insert description>.

4. Mounting Mechanism:  [Stainless steel, No. 4 finish (satin)] <Insert description>.

5. Dimensions:  <Insert dimensions.>

G. Soap Dish <Insert drawing designation>:

1. Basis-of-Design Product:  <Insert manufacturer; product.>

2. Description:  [With] [Without] washcloth bar.

3. Mounting:  [Recessed] [Surface mounted].

4. Material and Finish:  [Stainless steel, No. 4 finish (satin)] <Insert material and finish>.

2.4 PRIVATE-USE BATHROOM ACCESSORIES

See Editing Instruction No. 1 in the Evaluations for cautions about naming manufacturers and products.

A. Available Manufacturers:  Subject to compliance with requirements, manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following:

Retain above for nonproprietary or first paragraph below for semiproprietary specification, or delete both for basis-of-design specification.  Refer to Division 01 Section "Product Requirements."

B. Manufacturers:  Subject to compliance with requirements, provide products by one of the following:

C. Basis-of-Design Product:  The design for accessories is based on products indicated.  Subject to compliance with requirements, provide the named product or a comparable product by one of the following:

Types of accessories offered by manufactures listed below vary among commercial, hospitality, and residential grades.  Verify products offered by each manufacturer, and revise list to suit Project.

1. American Bathroom Accessories, Inc.

2. Basco, Inc.

3. Bobrick Washroom Equipment, Inc.

4. Franklin Brass Manufacturing Co.

5. General Accessory Manufacturing Co. (GAMCO)

6. Ginger; GUSA, Inc.

7. Seachrome Corporation.

8. <Insert manufacturer's name.>

Delete "Basis-of-Design Product" subparagraphs in remainder of this Article if not applicable.  Coordinate with specification method selected above.

Copy and re-edit paragraphs and subparagraphs in remainder of this Article if more than one of each type of accessory is required.  Insert a unique drawing designation for each product.

D. Toilet Tissue Dispenser <Insert drawing designation>:

1. Basis-of-Design Product:  <Insert manufacturer; product.>

2. Description:  [Single-roll dispenser] [Double-roll dispenser] [Single-roll dispenser with hood] [Double-roll dispenser with hood] <Insert description>.

3. Mounting:  [Recessed] [Surface mounted].

4. Capacity:  Designed for 4-1/2- or 5-inch- (114- or 127-mm-) diameter tissue rolls.

5. Material and Finish:  [Solid brass, polished] [Polished brass-plated zinc alloy (zamac)] [Polished chrome-plated brass] [Polished chrome-plated zinc alloy (zamac)] [Stainless steel, No. 4 finish (satin)] [Stainless steel, No. 7 finish (polished)] <Insert material and finish>.

E. Shower Curtain Rod <Insert drawing designation>:

1. Basis-of-Design Product:  <Insert manufacturer; product.>

2. Outside Diameter:  [1 inch (25.4 mm)] [1-1/4 inch (32 mm)].

3. Mounting:  Flanges with [exposed] [concealed] fasteners.

4. Rod Material and Finish:  [Solid brass, polished] [Polished chrome-plated brass] [Stainless steel, No. 4 finish (satin)] [Stainless steel, No. 7 finish (polished)] <Insert material and finish>.

5. Flange Material and Finish:  [Polished brass-plated zinc alloy (zamac)] [Polished chrome-plated brass] [Polished chrome-plated zinc alloy (zamac)] [Stainless steel, No. 4 finish (satin)] [Stainless steel, No. 7 finish (polished)] <Insert material and finish>.

Delete subparagraph below if not required.

6. Accessories:  Integral chrome-plated brass glide hooks.

F. Soap Dish <Insert drawing designation>:

1. Basis-of-Design Product:  <Insert manufacturer; product.>

2. Description:  <Insert description>.

3. Mounting:  [Recessed] [Surface mounted].

4. Material and Finish:  [Solid brass, polished] [Polished brass-plated zinc alloy (zamac)] [Polished chrome-plated brass] [Polished chrome-plated zinc alloy (zamac)] [Stainless steel, No. 4 finish (satin)] [Stainless steel, No. 7 finish (polished)] <Insert material and finish>.

G. Medicine Cabinet <Insert drawing designation>:

1. Basis-of-Design Product:  <Insert manufacturer; product.>

2. Mounting:  [Recessed, for nominal 4-inch (100-mm) wall depth] [Surface mounted].

3. Size:  [18 by 24 inches (460 by 610 mm)] <Insert size>.

4. Door:  [Framed mirror door concealing storage cabinet equipped with continuous hinge and spring-buffered, rod-type stop and magnetic door catch] <Insert description>.

5. Shelves:  [Three, adjustable] <Insert requirements>.

6. Material and Finish:

a. Cabinet:  [Stainless steel, No. 4 finish (satin)] [Steel with corrosion resistant finish].

b. Mirror Frame:  <Insert material and finish.>

c. Door:  <Insert material and finish.>

d. Hinge:  <Insert material and finish.>

e. Shelves:  <Insert material and finish.>

H. Facial Tissue Dispenser <Insert drawing designation>:

1. Basis-of-Design Product:  <Insert manufacturer; product.>

2. Mounting:  [Wall mounted, recessed] [Surface mounted].

3. Depth:  [2-5/8 inches (67 mm)] [4 inches (102 mm)] <Insert dimension>.

4. Material and Finish:

a. Dispenser Face:  [Polished chrome-plated brass] [Polished brass-plated zinc alloy (zamac)] [Polished chrome-plated steel] [Stainless steel, No. 4 finish (satin)] [Stainless steel, No. 7 finish (polished)] <Insert material and finish>.

b. Cabinet:  [Steel with corrosion-resistant finish] <Insert material and finish>.

I. Robe Hook <Insert drawing designation>:

1. Basis-of-Design Product:  <Insert manufacturer; product.>

2. Description:  [Double] [Single]-prong unit.

3. Material and Finish:  [Solid brass, polished] [Polished brass-plated zinc alloy (zamac)] [Polished chrome-plated brass] [Polished chrome-plated zinc alloy (zamac)] [Stainless steel, No. 4 finish (satin)] [Stainless steel, No. 7 finish (polished)] <Insert material and finish>.

J. Toothbrush and Tumbler Holder <Insert drawing designation>:

1. Basis-of-Design Product:  <Insert manufacturer; product.>

2. Description:  <Insert description>.

3. Material and Finish:  [Solid brass, polished] [Polished brass-plated zinc alloy (zamac)] [Polished chrome-plated brass] [Polished chrome-plated zinc alloy (zamac)] [Stainless steel, No. 4 finish (satin)] [Stainless steel, No. 7 finish (polished)] <Insert material and finish>.

K. Towel Bar <Insert drawing designation>:

1. Basis-of-Design Product:  <Insert manufacturer; product.>

2. Description:  [3/4-inch- (19-mm-) square tube with rectangular end brackets] [3/4-inch- (19-mm-) round tube with circular end brackets] <Insert description>.

3. Mounting:  Flanges with [concealed] [exposed] fasteners.

4. Length:  [18 inches (457 mm)] [24 inches (610 mm)] [30 inches (762 mm)] <Insert dimension>.

5. Material and Finish:  [Stainless steel, No. 4 finish (satin)] [Stainless steel, No. 7 finish (polished)] [Polished aluminum] <Insert material and finish>.

L. Towel Pin <Insert drawing designation>:

1. Basis-of-Design Product:  <Insert manufacturer; product.>

2. Description:  Projecting minimum of [3 inches (75 mm)] [5 inches (127 mm)] from wall surface.

3. Material and Finish:  [Polished brass-plated zinc alloy (zamac)] [Polished chrome-plated brass] [Stainless steel, No. 4 finish (satin)] [Stainless steel, No. 7 finish (polished)] <Insert material and finish>.

M. Towel Ring <Insert drawing designation>:

1. Basis-of-Design Product:  <Insert manufacturer; product.>

2. Description:  Pin projecting approximately 2-1/2 inches (63 mm) from wall with [square] [circular] [oval] [trapezoidal] ring.

3. Pin Material and Finish:  [Solid brass, polished] [Polished brass-plated zinc alloy (zamac)] [Polished chrome-plated brass] [Polished chrome-plated zinc alloy (zamac)] [Stainless steel, No. 4 finish (satin)] [Stainless steel, No. 7 finish (polished)] <Insert material and finish>.

4. Ring Material and Finish:  [Matching pin] [Clear plastic].

N. Towel Shelf <Insert drawing designation>:

1. Basis-of-Design Product:  <Insert manufacturer; product.>

2. Description:  Surface-mounted, guest-towel shelf with four [3/8-inch- (9-mm-) diameter] [5/16-inch- (8-mm-) square] stainless steel tubes mounted in support arms <Insert description>.

Delete first subparagraph below if not required; if retaining, revise to suit Project.

a. Towel Bar:  [1/4-inch (6-mm-) diameter] [5/16-inch- (8-mm-) square] stainless-steel towel bar below shelf.

3. Length:  [18 inches (457 mm)] [24 inches (610 mm)] <Insert dimension>.

4. Material and Finish:  [Polished brass-plated stainless steel tubes mounted in zinc alloy (zamac) support arms] [Polished chrome-plated stainless steel tubes mounted in zinc alloy (zamac) support arms] [Stainless steel, No. 7 finish (polished)] <Insert material and finish>.

O. Towel Rack <Insert drawing designation>:

1. Basis-of-Design Product:  <Insert manufacturer; product.>

2. Description:  [Surface-mounted, guest-towel unit with approximately 1/4-inch- (6-mm-) diameter wire rings welded to upright wire bracket] <Insert description>.

3. Capacity:  [2] [3] [4] <Insert number> sets of bath towels, hand towels, and washcloths.

4. Nominal Height:  [11 inches (279 mm)] [17 inches (432 mm)] [21 inches (533 mm)] <Insert dimension>.

5. Material and Finish:  [Polished brass-plated zinc alloy (zamac)] [Polished chrome-plated zinc alloy (zamac)] <Insert material and finish>.

P. Retractable Clothesline <Insert drawing designation>:

1. Basis-of-Design Product:  <Insert manufacturer; product.>

2. Description:  Surface-mounted [rectangular] [circular] housing with minimum 72-inch- (1829-mm-) long, retractable, spring-actuated, synthetic clothesline and remote retention bracket.

3. Material and Finish [Chrome-plated brass] [Stainless steel, No. 7 finish (polished)].

Q. Bottle Opener <Insert drawing designation>:

1. Basis-of-Design Product:  <Insert manufacturer; product.>

2. Description:  Surface-mounted unit with [standard] [vandal-resistant] fasteners.

3. Material and Finish:  [Stainless steel, No. 4 finish (satin)] [Stainless steel, No. 7 finish (polished)] [Chrome-plated steel].

2.5 HEALTHCARE ACCESSORIES

See Editing Instruction No. 1 in the Evaluations for cautions about naming manufacturers and products.

A. Available Manufacturers:  Subject to compliance with requirements, manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following:

Retain above for nonproprietary or first paragraph below for semiproprietary specification, or delete both for basis-of-design specification.  Refer to Division 01 Section "Product Requirements."

B. Manufacturers:  Subject to compliance with requirements, provide products by one of the following:

C. Basis-of-Design Product:  The design for accessories is based on products indicated.  Subject to compliance with requirements, provide the named product or a comparable product by one of the following:

1. A & J Washroom Accessories, Inc.

2. American Specialties, Inc.

3. Bobrick Washroom Equipment, Inc.

4. Bradley Corporation.

5. General Accessory Manufacturing Co. (GAMCO).

6. <Insert manufacturer's name.>

Delete "Basis-of-Design Product" subparagraphs in remainder of this Article if not applicable.  Coordinate with specification method selected above.

Copy and re-edit paragraphs and subparagraphs in remainder of this Article if more than one of each type of accessory is required.  Insert a unique drawing designation for each product.

D. Specimen Pass-Through Cabinet <Insert drawing designation>:

1. Basis-of-Design Product:  <Insert manufacturer; product.>

2. Description:  With self-closing doors on both sides, lock that prevents doors from both being opened at the same time, and removable stainless-steel tray.

3. Nominal Wall Opening:  [12 by 11-1/4 inches (305 by 285 mm)] <Insert dimension>, width times height.

4. Material and Finish:  Stainless steel, No. 4 finish (satin).

E. Specimen Pass-Through Box <Insert drawing designation>:

1. Basis-of-Design Product:  <Insert manufacturer; product.>

2. Description:  With minimum [12-inch (305-mm)] <Insert dimension> diameter turntable removable cylinder that revolves on stainless-steel self-lubricating ball bearing plates, and with mechanism to prevent over rotation of cylinder.

3. Nominal Wall Opening:  [13-1/4 by 14 inches (335 by 355 mm)] <Insert dimension>, width times height.

4. Material and Finish:  Stainless steel, No. 4 finish (satin).

5. Lockset:  Tumbler type.

F. Bedpan and Urinal Cabinet <Insert drawing designation>:

1. Basis-of-Design Product:  <Insert manufacturer; product.>

2. Description:  For storing one conventional size bedpan and one urinal bottle; with door that produces 1/2-inch (13-mm) opening at top and bottom of cabinet to allow air circulation.

3. Mounting:  Recessed.

4. Nominal Wall Opening:  [13-1/2 by 26-1/2 by 5 inches (340 by 670 by 130 mm)] <Insert dimension>, width times height times depth.

5. Material and Finish:  Stainless steel, No. 4 finish (satin).

G. Bedpan and Urinal Rack <Insert drawing designation>:

1. Basis-of-Design Product:  <Insert manufacturer; product.>

2. Description:  For storing one conventional size bedpan and one urinal bottle.

3. Mounting:  Surface mounted.

4. Size:  [12 by 27 inches (300 by 685 mm)] <Insert dimension>, width times height.

5. Material and Finish:  Stainless steel, No. 4 finish (satin).

2.6 WARM-AIR DRYERS

See Editing Instruction No. 1 in the Evaluations for cautions about naming manufacturers and products.

A. Available Manufacturers:  Subject to compliance with requirements, manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following:

Retain above for nonproprietary or first paragraph below for semiproprietary specification, or delete both for basis-of-design specification.  Refer to Division 01 Section "Product Requirements."

B. Manufacturers:  Subject to compliance with requirements, provide products by one of the following:

C. Basis-of-Design Product:  The design for accessories is based on products indicated.  Subject to compliance with requirements, provide the named product or a comparable product by one of the following:

1. A & J Washroom Accessories, Inc.

2. American Dryer, Inc.

3. American Specialties, Inc.

4. Bobrick Washroom Equipment, Inc.

5. Bradley Corporation.

6. Excel Dryer Corporation.

7. General Accessory Manufacturing Co. (GAMCO).

8. World Dryer Corporation.

9. <Insert manufacturer's name.>

Copy and re-edit paragraph and subparagraphs below if more than one type of warm-air dryer is required.  Insert a unique drawing designation for each product.

D. Warm-Air Dryer <Insert drawing designation>:

Delete first subparagraph below if not applicable.  Coordinate with specification method selected at the beginning of this Article.

1. Basis-of-Design Product:  <Insert manufacturer; product.>

2. Mounting:  [Recessed] [Semirecessed] [Surface mounted].

3. Operation:  [Touch-button] [Electronic-sensor] activated with timed power cut-off switch.

a. Operation Time:  [30 to 40] [80] <Insert number> seconds.

4. Cover Material and Finish:  [Steel, with white enamel finish] [Cast iron, with enamel finish in color selected by Architect] [Chrome-plated steel] [Stainless steel, No. 4 finish (satin)] [Molded plastic, gray] [Molded plastic, white] <Insert material and finish>.

5. Electrical Requirements:  [115 V, 13 A, 1500 W] [115 V, 15 A, 1725 W] [115 V, 20 A, 2300 W] [208-240 V, 9-10 A, 1900-2300 W] <Insert electrical requirements>.

2.7 CHILDCARE ACCESSORIES

See Editing Instruction No. 1 in the Evaluations for cautions about naming manufacturers and products.

A. Available Manufacturers:  Subject to compliance with requirements, manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following:

Retain above for nonproprietary or first paragraph below for semiproprietary specification, or delete both for basis-of-design specification.  Refer to Division 01 Section "Product Requirements."

B. Manufacturers:  Subject to compliance with requirements, provide products by one of the following:

C. Basis-of-Design Product:  The design for accessories is based on products indicated.  Subject to compliance with requirements, provide the named product or a comparable product by one of the following:

1. American Infant Care Products Inc.

2. American Specialties, Inc.

3. Brocar Products, Inc.

4. General Accessory Manufacturing Co. (GAMCO).

5. Koala Corporation.

6. Safe-Strap Company, Inc.

7. <Insert manufacturer's name.>

Delete "Basis-of-Design Product" subparagraphs in remainder of this Article if not applicable.  Coordinate with specification method selected above.

Copy and re-edit paragraphs and subparagraphs in remainder of this Article if more than one of each type of accessory is required.  Insert a unique drawing designation for each product.

D. Diaper-Changing Station <Insert drawing designation>:

1. Basis-of-Design Product:  <Insert manufacturer; product.>

2. Description:  [Horizontal] [Vertical] unit that opens by folding down from stored position and with child-protection strap.

a. Engineered to support a minimum of [250-lb (113-kg)] <Insert weight> static load when opened.

3. Mounting:  [Surface mounted, with unit projecting not more than 4 inches (100 mm) from wall when closed] [Semirecessed, with unit projecting not more than 1 inches (25 mm) from wall when closed].

4. Operation:  By pneumatic shock-absorbing mechanism.

5. Material and Finish:  [High-density polyethylene in manufacturer's standard color] [High-density polyethylene with plastic laminate insert in color selected by Architect] [Stainless steel, No. 4 finish (satin), with replaceable insulated polystyrene tray liner and rounded plastic corners] [Stainless steel, No. 4 finish (satin), exterior shell with rounded plastic corners; high-density polyethylene interior in manufacturer's standard color] <Insert requirements>.

6. Liner Dispenser:  Built in.

E. Diaper-Pack Vendor <Insert drawing designation>:

1. Basis-of-Design Product:  <Insert manufacturer; product.>

2. Mounting:  [Surface mounted] [Recessed].

3. Minimum Capacity:  [100] <Insert number> diaper packs.

4. Coin Operation:  Coin slot preset for [1 U.S. dollar, adjustable up in 25-cent increments] <Insert requirements>.

5. Material and Finish:  [Stainless steel, No. 4 finish (satin)] <Insert material and finish>.

F. Child-Protection Seat <Insert drawing designation>:

1. Basis-of-Design Product:  <Insert manufacturer; product.>

2. Description:  Unit that opens by folding down from stored position and with child-protection strap.

a. Engineered to support a minimum of [80-lb (36-kg)] [150-lb (68-kg)] <Insert weight> static load when opened.

3. Mounting:  Surface mounted, with unit projecting not more than [4-1/2 inches (114 mm)] [6 inches (152 mm)] from wall when closed.

4. Material and Finish:  [High-density polyethylene in manufacturer's standard color] <Insert requirements>.

2.8 UNDERLAVATORY GUARDS

Delete this Article if piping insulation is specified in Division 22 or architectural woodwork is used to prevent direct contact with and burns from piping.

See Editing Instruction No. 1 in the Evaluations for cautions about naming manufacturers and products.

A. Available Manufacturers:  Subject to compliance with requirements, manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following:

Retain above for nonproprietary or first paragraph below for semiproprietary specification, or delete both for basis-of-design specification.  Refer to Division 01 Section "Product Requirements."

B. Manufacturers:  Subject to compliance with requirements, provide products by one of the following:

C. Basis-of-Design Product:  The design for accessories is based on products indicated.  Subject to compliance with requirements, provide the named product or a comparable product by one of the following:

1. Plumberex Specialty Products, Inc.

2. TCI Products.

3. Truebro, Inc.

4. <Insert manufacturer's name.>

Copy and re-edit paragraph and subparagraphs below if more than one type of underlavatory guard is required.  Insert a unique drawing designation for each product.

D. Underlavatory Guard <Insert drawing designation>:

Delete first subparagraph below if not applicable.  Coordinate with specification method selected at the beginning of this Article.

1. Basis-of-Design Product:  <Insert manufacturer; product.>

2. Description:  Insulating pipe covering for supply and drain piping assemblies, that prevent direct contact with and burns from piping, and allow service access without removing coverings.

3. Material and Finish:  Antimicrobial, molded-plastic, white.

2.9 CUSTODIAL ACCESSORIES

See Editing Instruction No. 1 in the Evaluations for cautions about naming manufacturers and products.

A. Available Manufacturers:  Subject to compliance with requirements, manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following:

Retain above for nonproprietary or first paragraph below for semiproprietary specification, or delete both for basis-of-design specification.  Refer to Division 01 Section "Product Requirements."

B. Manufacturers:  Subject to compliance with requirements, provide products by one of the following:

C. Basis-of-Design Product:  The design for accessories is based on products indicated.  Subject to compliance with requirements, provide the named product or a comparable product by one of the following:

1. A & J Washroom Accessories, Inc.

2. American Specialties, Inc.

3. Bobrick Washroom Equipment, Inc.

4. Bradley Corporation.

5. General Accessory Manufacturing Co. (GAMCO).

6. <Insert manufacturer's name.>

Delete "Basis-of-Design Product" subparagraphs in remainder of this Article if not applicable.  Coordinate with specification method selected above.

Copy and re-edit paragraphs and subparagraphs in remainder of this Article if more than one of each type of accessory is required.  Insert a unique drawing designation for each product.

D. Utility Shelf <Insert drawing designation>:

1. Basis-of-Design Product:  <Insert manufacturer; product.>

2. Description:  With exposed edges turned down not less than 1/2 inch (12.7 mm) and supported by two triangular brackets welded to shelf underside.

3. Size:  [16 inches (406 mm) long by 6 inches (152 mm) deep] <Insert size>.

4. Material and Finish:  Not less than nominal 0.05-inch- (1.3-mm-) thick stainless steel, No. 4 finish (satin).

E. Mop and Broom Holder <Insert drawing designation>:

1. Basis-of-Design Product:  <Insert manufacturer; product.>

2. Description:  [Unit with shelf, hooks, holders, and rod suspended beneath shelf] <Insert description>.

3. Length:  [36 inches (914 mm)] <Insert length>.

4. Hooks:  [Three] <Insert number>.

5. Mop/Broom Holders:  [Four] <Insert number>, spring-loaded, rubber hat, cam type.

6. Material and Finish:  Stainless steel, No. 4 finish (satin).

a. Shelf:  Not less than nominal 0.05-inch- (1.3-mm-) thick stainless steel.

b. Rod:  Approximately 1/4-inch- (6-mm-) diameter stainless steel.

If the same paper towel dispensers and soap dispensers that are used in washrooms are also used in custodial areas, delete three paragraphs and associated subparagraphs below.

F. Paper Towel (Folded) Dispenser <Insert drawing designation>:

1. Basis-of-Design Product:  <Insert manufacturer; product.>

2. Mounting:  [Recessed] [Semirecessed] [Surface mounted].

Single-fold towels usually require 6-inch- (152.4-mm-) deep dispensers.  Dispensers 4 inches (101.6 mm) in depth accommodate C-fold or multifold towels.  C-fold towels are the highest quality and softest folded towels; single-fold towels are the least expensive.

3. Minimum Capacity:  [400 C-fold or 525 multifold towels] [600 C-fold or 800 multifold towels] [400 single-fold towels] <Insert capacity>.

4. Material and Finish:  [Stainless steel, No. 4 finish (satin)] [ABS plastic, gray] <Insert material and finish>.

5. Lockset:  Tumbler type.

Delete subparagraph below if not required.

6. Refill Indicators:  Pierced slots at sides or front.

Roll towel dispensers extend farther from the wall than folded towel dispensers.

G. Paper Towel (Roll) Dispenser <Insert drawing designation>:

1. Basis-of-Design Product:  <Insert manufacturer; product.>

2. Description:  Lever-actuated mechanism permits controlled delivery of paper rolls in preset lengths per stroke.

3. Mounting:  [Recessed] [Semirecessed] [Surface mounted].

4. Minimum Capacity:  [8-inch (203-mm) wide, 800-foot (244-m) long roll] <Insert capacity>.

5. Material and Finish:  [Stainless steel, No. 4 finish (satin)] [ABS plastic, gray] <Insert material and finish>.

6. Lockset:  Tumbler type.

H. Liquid-Soap Dispenser <Insert drawing designation>:

1. Basis-of-Design Product:  <Insert manufacturer; product.>

2. Description:  Designed for dispensing soap in [liquid or lotion] [lather] form.

3. Mounting:  [Deck mounted on vanity] [Deck mounted on lavatory] [Horizontally oriented, recessed] [Horizontally oriented, surface mounted] [Vertically oriented, surface mounted].

4. Capacity:  <Insert capacity in oz. (mL).>

5. Materials:  <Insert requirements for valve and reservoir materials and finishes.>

6. Lockset:  Tumbler type.

Delete subparagraph above and below if not required.

7. Refill Indicator:  Window type.

2.10 FABRICATION

A. General:  Fabricate units with tight seams and joints, and exposed edges rolled.  Hang doors and access panels with full-length, continuous hinges.  Equip units for concealed anchorage and with corrosion-resistant backing plates.

B. Keys:  Provide universal keys for internal access to accessories for servicing and resupplying.  Provide minimum of [six] <Insert number> keys to Owner's representative.

PART 3 -  EXECUTION

3.1 INSTALLATION

A. Install accessories according to manufacturers' written instructions, using fasteners appropriate to substrate indicated and recommended by unit manufacturer.  Install units level, plumb, and firmly anchored in locations and at heights indicated.

B. Grab Bars:  Install to withstand a downward load of at least 250 lbf (1112 N), when tested according to method in ASTM F 446.

3.2 ADJUSTING AND CLEANING

A. Adjust accessories for unencumbered, smooth operation.  Replace damaged or defective items.

B. Remove temporary labels and protective coatings.

C. Clean and polish exposed surfaces according to manufacturer's written recommendations.

END OF SECTION 102800

	TOILET, BATH, AND LAUNDRY ACCESSORIES
	102800 - 1



