Copyright 2002 and 2005 AIA
MASTERSPEC Full Length
05/02 (R 05/05)

Copyright 2002 and 2005 by The American Institute of Architects (AIA)

Exclusively published and distributed by Architectural Computer Services, Inc. (ARCOM) for the AIA

SECTION 313116 - TERMITE CONTROL
Verify that Section titles referenced in this Section are correct for this Project's Specifications; Section titles may have changed.

PART 1 -  GENERAL

1.1 RELATED DOCUMENTS

A. Drawings and general provisions of the Contract, including General and Supplementary Conditions and Division 01 Specification Sections, apply to this Section.

1.2 SUMMARY

A. This Section includes the following:

Adjust list below to suit Project.

1. Soil treatment with termiticide.

2. Wood treatment with borate.

3. Bait-station system.

4. Metal mesh barrier system.

B. Related Sections include the following:

List below only products that the reader might expect to find in this Section but are specified elsewhere.

1. Division 06 Section "Rough Carpentry" for wood preservative treatment by pressure process.

Retain subparagraph below only if requirements for termite shields have been added to Division 07 Section "Sheet Metal Flashing and Trim" and details are included on Drawings.

2. Division 07 Section "Sheet Metal Flashing and Trim" for custom-fabricated metal termite shields.

If unit prices apply to Work of this Section, insert brief paragraph here to alert Contractor and reference appropriate Division 01 Section for specific details.

1.3 UNIT PRICES

A. Basis of Bids:  Unit price for [each] <Insert number> termite bait station(s) provided.

1. See Division 01 Section "Unit Prices" for list of unit prices.

1.4 PERFORMANCE REQUIREMENTS

Delete this Article if no soil treatment.

A. Service Life of Soil Treatment:  Soil treatment by use of a termiticide that is effective for not less than [five] <Insert number> years against infestation of subterranean termites.

1.5 SUBMITTALS

A. Product Data:  For [termiticide] [borate] [bait-station system] [and] [metal mesh barrier system].

1. Include the EPA-Registered Label for [termiticide] [and] [borate] products.

B. Product Certificates:  For termite control products, signed by product manufacturer.

Coordinate paragraph below with qualification requirements in Division 01 Section "Quality Requirements" and as supplemented in "Quality Assurance" Article.

C. Qualification Data:  For Installer of termite control products.

Retain paragraph and subparagraphs below if applicable and coordinate with Division 01 Section "Closeout Procedures."

D. Soil Treatment Application Report:  After application of termiticide is completed, submit report for Owner's record information, including the following:

1. Date and time of application.

2. Moisture content of soil before application.

3. Brand name and manufacturer of termiticide.

4. Quantity of undiluted termiticide used.

5. Dilutions, methods, volumes, and rates of application used.

6. Areas of application.

7. Water source for application.

Delete paragraph and subparagraphs below if no wood borate treatment.  Coordinate below with Division 01 Section "Closeout Procedures."

E. Wood Treatment Application Report:  After application of borate is completed, submit report for Owner's record information, including the following:

1. Date and time of application.

2. Brand name and manufacturer of borate.

3. Quantity of undiluted borate used.

4. Dilutions, methods, volumes, and rates of application used.

5. Areas of application.

Delete first paragraph and subparagraphs below if no bait-station system.  Coordinate below with Division 01 Section "Closeout Procedures."

F. Bait-Station System Application Report:  After installation of bait-station system is completed, submit report for Owner's records information, including the following:

1. Location of areas and sites conducive to termite feeding and activity.

Revise first subparagraph below if monitoring stations are to be installed without bait stations.  See Editing Instruction No. 2 in the Evaluations.

2. Plan drawing showing number and locations of [monitoring stations] [and] [bait stations].

3. Dated report for each monitoring and inspection occurrence indicating level of termite activity, procedure, and treatment applied before time of Substantial Completion.

4. Brand name and manufacturer of termiticide.

5. Quantities of termite bait used.

6. Schedule of inspections for one year from date of Substantial Completion.

G. Warranty:  Special warranty specified in this Section.

1.6 QUALITY ASSURANCE

Retain option in first paragraph below for bait-station system.

A. Installer Qualifications:  A specialist who is licensed according to regulations of authorities having jurisdiction to apply termite control treatment and products in jurisdiction where Project is located[, and who employs workers trained and approved by bait-station system manufacturer to install manufacturer's products].

B. Regulatory Requirements:  Formulate and apply termiticides according to the EPA-Registered Label.

Revise paragraph below if bait station, termiticide, or borates are from different manufacturers.

C. Source Limitations:  Obtain termite control products [through one source] [from a single manufacturer for each product].

Retain paragraph below to ensure treatment of critical areas and to avoid physical and chemical hazards during application according to termiticide label statements.

Delete below if Work of this Section is not extensive or complex enough to justify a preinstallation conference.  If retaining, coordinate with Division 01.

D. Preinstallation Conference:  Conduct conference at Project site to comply with requirements in Division 01 Section "Project Management and Coordination" to schedule application of termiticide products.

1.7 PROJECT CONDITIONS

Delete this Article if no soil treatment.

A. Environmental Limitations:  To ensure penetration, do not treat soil that is water saturated or frozen.  Do not treat soil while precipitation is occurring.  Comply with requirements of the EPA-Registered Label and requirements of authorities having jurisdiction.

1.8 COORDINATION

Delete paragraph below if no soil treatment.

A. Coordinate soil treatment application with excavating, filling, grading, and concreting operations.  Treat soil under footings, grade beams, and ground-supported slabs before construction.

Delete paragraph below if no borate treatment.

B. Apply borate treatment after framing, sheathing, and exterior weather protection is completed but before electrical and mechanical systems are installed.

Retain one or both paragraphs below if bait-station system concurrent with construction is required to suit Project, such as renovation or expansion work.  Monitoring stations may be used when site infestations are not known before construction.

C. Install bait-station monitoring system during construction to determine areas of termite activity.

D. Install bait-station system after construction, including landscaping, is completed.

1.9 WARRANTY

When warranties are required, verify with Owner's counsel that special warranties stated in this Article are not less than remedies available to Owner under prevailing local laws.  Coordinate with Division 01 Section "Product Requirements."  Insert bait-station system and warranty terms if applicable.  Warranty is for protection of construction from infestation and not specifically for methods used.  If required, contact HUD for new "Subterranean Termite Soil Treatment Builder's Guarantee."

A. Special Warranty:  Manufacturer's standard form, signed by Applicator and Contractor certifying that termite control work, consisting of applied soil termiticide treatment, will prevent infestation of subterranean termites.  If subterranean termite activity or damage is discovered during warranty period, re-treat soil and repair or replace damage caused by termite infestation.

Verify available warranties and insert number below.

1. Warranty Period:  [Three] [Five] <Insert number> years from date of Substantial Completion.

1.10 MAINTENANCE SERVICE

Retain or delete this Article to suit needs of Owner.  Coordinate with warranty requirements if any.  Because bait-station systems require postconstruction monitoring and treatment, retain below if bait products are specified.  Consult Owner and revise below for other terms.

A. Continuing Service:  Beginning at Substantial Completion, provide [12 months'] <Insert number> continuing service including monitoring, inspection, and re-treatment for occurrences of termite activity.  Provide a standard continuing service agreement.  State services, obligations, conditions, and terms for agreement period; and terms for future renewal options.

PART 2 -  PRODUCTS

2.1 MANUFACTURERS

See Editing Instruction No. 1 in the Evaluations for cautions about naming manufacturers and products.

A. Available Manufacturers:  Subject to compliance with requirements, manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following:

Retain above for nonproprietary or below for semiproprietary specification.  Refer to Division 01 Section "Product Requirements."

B. Manufacturers:  Subject to compliance with requirements, provide products by one of the following:

1. Termiticides:

a. Aventis Environmental Science USA LP; Termidor.

b. Bayer Corporation; Premise 75.

c. Dow AgroSciences LLC; [Dursban TC] [Equity].

d. FMC Corporation, Agricultural Products Group; [Talstar] [Prevail FT] [Torpedo].

e. Syngenta; Demon TC.

f. <Insert manufacturer's name; product name or designation.>

2. Borates:

a. Nisus Corp.; Bora-Care, Jecta.

b. NovaGuard Technologies, Inc.; Armor-Guard, Shell-Guard.

c. U.S. Borax Inc.; Tim-Bor.

d. <Insert manufacturer's name; product name or designation.>

3. Bait-Station Systems:

a. Dow AgroSciences LLC; Sentricon System.

b. Ensystex, Inc.; [Exterra] [Quarterra] System.

c. FMC Corporation, Agricultural Products Group; First Line Systems.

d. <Insert manufacturer's name; product name or designation.>

2.2 SOIL TREATMENT

Retain this Article unless another method of preconstruction treatment is permitted by authorities having jurisdiction.

A. Termiticide:  Provide an EPA-registered termiticide complying with requirements of authorities having jurisdiction, in an aqueous solution formulated to prevent termite infestation.  Provide quantity required for application at the label volume and rate for the maximum termiticide concentration allowed for each specific use, according to product's EPA-Registered Label.

2.3 WOOD TREATMENT

Retain this Article for treatment of interior wood framing.

A. Borate:  Provide an EPA-registered borate complying with requirements of authorities having jurisdiction, in an aqueous solution for spray application and a gel solution for pressure injection, formulated to prevent termite infestation in wood.  Provide quantity required for application at the label volume and rate for the maximum diffusible borate concentration allowed for each specific use, according to product's EPA-Registered Label.

2.4 BAIT-STATION SYSTEM

A. Provide bait stations[ and monitoring stations] based on the dimensions of building perimeter indicated on Drawings, according to manufacturer's EPA-Registered Label for product, manufacturer's written instructions, and the following:

Retain one of two subparagraphs below based on manufacturer's recommended minimum spacing and number of stations in a cluster.

1. Not less than 1 station per [8 linear feet (2.4 linear meters)] [20 linear feet (6.1 linear meters)] <Insert value>.

2. Not less than 1 cluster of stations per 20 linear feet (6.1 linear meters), consisting of not less than 3 stations per cluster.

2.5 METAL MESH BARRIER SYSTEM

A. Product:  Subject to compliance with requirements, provide "TERMI-MESH" by TERMI-MESH, Inc.

B. Stainless-Steel Mesh:  0.025-by-0.018-inch (0.64-by-0.45-mm) mesh of 0.08-inch- (2.0-mm-) diameter, stainless-steel wire, Type 316.

PART 3 -  EXECUTION

3.1 EXAMINATION

Revise this Article if soil treatment is not included.

A. Examine substrates, areas, and conditions, with Applicator present, for compliance with requirements for[ moisture content of soil,] interfaces with earthwork, slab and foundation work, landscaping, and other conditions affecting performance of termite control.

1. Proceed with application only after unsatisfactory conditions have been corrected.

3.2 PREPARATION

A. General:  Comply with the most stringent requirements of authorities having jurisdiction and with manufacturer's written instructions for preparation before beginning application of termite control treatment.  Remove all extraneous sources of wood cellulose and other edible materials such as wood debris, tree stumps and roots, stakes, formwork, and construction waste wood from soil within and around foundations.

Delete paragraph and subparagraph below if no soil treatment.

B. Soil Treatment Preparation:  Remove foreign matter and impermeable soil materials that could decrease treatment effectiveness on areas to be treated.  Loosen, rake, and level soil to be treated except previously compacted areas under slabs and footings.  Termiticides may be applied before placing compacted fill under slabs if recommended in writing by termiticide manufacturer.

1. Fit filling hose connected to water source at the site with a backflow preventer, complying with requirements of authorities having jurisdiction.

3.3 APPLICATION, GENERAL

A. General:  Comply with the most stringent requirements of authorities having jurisdiction and with manufacturer's EPA-Registered Label for products.

3.4 APPLYING SOIL TREATMENT

Retain this Article for soil treatment, delete if not applicable, or revise to suit Project.  Products must be applied according to manufacturer's EPA-Registered Label for maximum performance.

A. Application:  Mix soil treatment termiticide solution to a uniform consistency.  Provide quantity required for application at the label volume and rate for the maximum specified concentration of termiticide, according to manufacturer's EPA-Registered Label, to the following so that a continuous horizontal and vertical termiticidal barrier or treated zone is established around and under building construction.  Distribute treatment evenly.

Revise five subparagraphs below to suit Project.

1. Slabs-on-Grade and Basement Slabs:  Under ground-supported slab construction, including footings, building slabs, and attached slabs as an overall treatment.  Treat soil materials before concrete footings and slabs are placed.

2. Foundations:  Adjacent soil including soil along the entire inside perimeter of foundation walls, along both sides of interior partition walls, around plumbing pipes and electric conduit penetrating the slab, and around interior column footers, piers, and chimney bases; also along the entire outside perimeter, from grade to bottom of footing.  Avoid soil washout around footings.

3. Crawlspaces:  Soil under and adjacent to foundations as previously indicated.  Treat adjacent areas including around entrance platform, porches, and equipment bases.  Apply overall treatment only where attached concrete platform and porches are on fill or ground.

Examples of masonry voids are the insides of hollow masonry units and behind masonry veneer.

4. Masonry:  Treat voids.

5. Penetrations:  At expansion joints, control joints, and areas where slabs will be penetrated.

Insert requirements here for crawlspaces used as plenum spaces only after seeing Evaluations for cautionary information.

B. Avoid disturbance of treated soil after application.  Keep off treated areas until completely dry.

C. Protect termiticide solution, dispersed in treated soils and fills, from being diluted until ground-supported slabs are installed.  Use waterproof barrier according to EPA-Registered Label instructions.

D. Post warning signs in areas of application.

E. Reapply soil treatment solution to areas disturbed by subsequent excavation, grading, landscaping, or other construction activities following application.

3.5 APPLYING BORATE TREATMENT

Retain this Article for wood treatment, delete if not applicable, or revise to suit Project.  Products must be applied according to manufacturer's EPA-Registered Label for maximum performance.

A. Application:  Mix wood treatment borate solution to a uniform consistency.  Provide quantity required for application at the label volume and rate for the maximum specified concentration of borate, according to manufacturer's EPA-Registered Label, so that wood framing, sheathing, siding, and structural members subject to infestation receive treatment.

1. Framing and Sheathing:  Apply borate solution by spray to bare wood for complete coverage.

First subparagraph below should be reviewed to ensure that holes and treatment will not reduce performance of structural members.

2. Wood Members Thicker Than 4 Inches (100 mm):  Inject borate gel solution under pressure into holes of size and spacing required by manufacturer for treatment.

3. Exterior Uncoated Wood Trim and Siding:  Apply borate solution to bare wood siding.  After 48 hours, apply a seal coat of [paint] <Insert coating> as specified in Division 09 [painting Sections] <Insert Section title>.

3.6 INSTALLING BAIT-STATION SYSTEMS

Retain this Article if retaining bait stations.  Strategies and applications of bait-station systems vary among products.  Procedures on the EPA-Registered Label, including guidelines for station placement, inspecting, and servicing, must be followed.  See Editing Instruction No. 2 in the Evaluations.

A. Place bait stations and, if applicable, monitoring stations, according to the EPA-Registered Label for the product and manufacturer's written instructions, in areas that are conducive to termite feeding and activity, as follows:

Edit examples in subparagraphs below and insert others to suit Project; if applicable, show precise locations on Drawings.

1. Conducive sites and locations indicated on Drawings.

2. In and around infested trees and stumps.

3. In mulch beds.

4. Where wood directly contacts soil.

5. Areas of high soil moisture.

6. Near irrigation sprinkler heads.

7. Each area where roof drainage system, including downspouts and scuppers, drains to soil.

8. Along driplines of roof overhangs without gutters.

9. Where condensate lines from mechanical equipment drip or drain to soil.

10. At plumbing penetrations through ground-supported slabs.

11. Other sites and locations as determined by licensed Installer.

Retain paragraph below if retaining Part 1 "Maintenance Service" Article and if Owner agrees to continuing service.

B. Inspect and service stations from time of their application until completion of time period established[ by continuing service agreement], according to the EPA-Registered Label for product and manufacturer's written instructions for termite management system and bait products.

If retaining subparagraph below, revise to suit Project.

1. Service Frequency:  Inspect monitoring stations no fewer than once every [three] <Insert number> months.

3.7 INSTALLING METAL MESH BARRIER SYSTEM

A. Metal Mesh Barrier:  Place metal mesh barrier where indicated to provide a continuous barrier to entry of subterranean termites.

If retaining both subparagraphs below, review manufacturer's written installation details.

1. Fit mesh tightly around pipe or other penetrations, and terminate at slab and foundation perimeters.

2. Install mesh under the perimeter of concrete slab edges and joints after vapor barrier and reinforcing steel are in place, and comply with manufacturer's written installation methods.

END OF SECTION 313116

	TERMITE CONTROL
	313116 - 1



