GSA SUPPLEMENT TO MASTERSPEC
GSA
9/99

SECTION 01500 - TEMPORARY FACILITIES AND CONTROLS
PART 1 -  GENERAL

1.1 SUMMARY

A. This section includes requirements for temporary utilities, support facilities and protection.

1. Temporary utilities include but are not limited to the following:

Modify the following list to suit project:

a. Temporary water service and distribution.

b. Temporary electric power and lighting.

c. Temporary heat and ventilation.

d. Temporary telephone service.

e. Temporary sanitary facilities, including drinking water.

f. Storm and sanitary sewer.

2. Support facilities include but are not limited to the following:

Modify the following list to suit project.

a. Field offices, storage sheds and fabrication shops.

b. Temporary paving.

c. Dewatering facilities and drains.

d. Temporary enclosures.

e. Temporary lifts and hoists.

f. Project identification and other temporary signs.

g. Waste disposal services.

h. Rodent and pest control.

i. Other construction aids and miscellaneous services and facilities.

3. Protection includes but is not limited to the following:

Modify the following list to suit project.

a. Temporary fire protection.

b. Barricades, warning signs, and lights.

c. Enclosure fence.

d. Environmental protection.

B. Provide temporary facilities and controls required for construction activities except, if any, for facilities and controls indicated as existing or provided by the Government or others.

1.2 UTILITY USE CHARGES

A. Unless otherwise specified, Contractor shall pay utility service use charges, whether metered or otherwise, for temporary utilities used by all entities engaged in construction activities at the Project site.  Costs for these services are included in the Contract price.

Retain the following paragraphs for available utility services provided at no cost to the Contractor, if any.  Modify if required to suit project.

B. Water Service:  The Contractor may use water from the Government's existing water system, without metering and without payment of use charges.

C. Sewer Service:  Contractor may use the Government's existing sewer system, without payment of use charges.

D. Electric Power Service:  Contractor may use electric power from the Government's existing electric power system, without metering and without payment of use charges.

If another utility is available at no cost to the Contractor, insert applicable utility in the following paragraph.  Repeat paragraph as required.  Modify if required to suit project.

E. <Insert Information Here> Service:  Contractor may use <Insert Information Here> from the Government's existing <Insert Information Here> system, without metering and without payment of use charges.

1.3 SUBMITTALS

A. Reports:  Submit reports of tests, inspections, meter readings and similar procedures for temporary utilities.

Retain the following paragraph for projects of long duration and where required for coordination with other work or conditions.  Revise time, if required.

B. Implementation and Termination Schedule:  Within 15 calendar days after the date established for the submittal of the Contractor's Construction Schedule, submit a schedule indicating implementation and termination of each temporary utility.

1.4 QUALITY ASSURANCE

A. Standards and Regulations:  Comply with industry standards and with applicable laws and regulations of authorities having jurisdiction, including but not limited to the following:

Edit the following paragraphs to suit project.  Add and delete standards and regulations as required.

1. Building code requirements.

2. Health and Safety regulations.

3. Utility company regulations.

4. Police, fire department and rescue squad rules.

5. Environmental protection regulations.

6. NFPA 241 "Standards for Safeguarding Construction, Alterations and Demolition Operations".

7. ANSI-A10 Series standards for "Safety Requirements for Construction and Demolition".

8. NECA Electrical Design Library "Temporary Electrical Facilities", NFPA 70, and NEMA, NECA and UL standards and regulations for temporary electric service.

B. Inspections:  Arrange for authorities having jurisdiction to inspect and test each temporary utility before use.  Obtain required certifications and permits.

1.5 PROJECT CONDITIONS

A. Install, operate, maintain and protect temporary facilities and controls.

1. Keep temporary services and facilities clean and neat in appearance.

2. Operate temporary services in a safe and efficient manner.

3. Relocate temporary services and facilities as needed as Work progresses.

4. Do not overload temporary services and facilities or permit them to interfere with progress.

5. Provide necessary fire prevention measures.

6. Do not allow hazardous, dangerous or unsanitary conditions, or public nuisances to develop or persist on-site.

B. At the earliest feasible time, when acceptable to the Contracting Officer's Representative, change over from temporary services to use of permanent services and remove temporary facilities when no longer needed.

C. Temporary Use of Permanent Facilities:  Contractor shall assume responsibility for the operation, maintenance and protection of each permanent service during its use as a construction facility prior to the Government's acceptance.

Retain the following paragraph with subparagraphs if project involves existing equipment to remain.  Edit for project.

D. Existing Equipment and Items:  Cover or otherwise protect and provide security for existing equipment and other items that are to remain in place, to prevent soiling, damage and loss.

1. Temporarily move equipment and other items that interfere with the performance of required work.  Upon completion of the work, return the equipment and items to their original location and installation condition.

2. Store equipment and other items that have been temporarily removed.  Upon reinstallation, clean and, if damaged, repair or replace equipment and items to match their condition prior to removal.

PART 2 -  PRODUCTS

2.1 MATERIALS

A. Provide undamaged materials in serviceable conditions and suitable for use intended.

B. Tarpaulins:  Waterproof, fire-resistant UL labeled with flame spread rating of 15 or less.  For temporary enclosures, provide translucent, nylon-reinforced, laminated polyethylene or polyvinyl chloride, fire-retardant tarpaulins.

C. Water:  Potable and approved by local health authorities.

D. Wood:  Lumber complying with DOC PS 20 and applicable grading rules of an inspection agency certified by ALSC's Board of Review for specific use.  Provide preservative treated lumber where partially or fully in contact with the earth, concrete or masonry.

Edit the following two paragraphs with subparagraphs to suit project.

E. Sign, Directory and Other Graphic Panel Materials:  Unless otherwise indicated, products shall comply as follows:

1. Panels:  Exterior type Grade B-B high density concrete-form-overlay plywood.

2. Paint:  Exterior primer and exterior grade alkyd gloss enamel top coat.

F. Safety Barrier and Covered Walkway Materials:  Unless otherwise indicated, products shall comply as follows.

1. Panels:  Minimum 5/8 inch (16 mm) thick exterior plywood.

2. Paint:  Exterior primer and exterior grade acrylic-latex emulsion top coat.

Retain one or both (if wood or open-mesh fencing is optional) of the following two paragraphs with corresponding subparagraphs if project requires enclosure fencing.  Edit to suit project.

G. Wood Fencing and Vision Panel Materials:  Unless otherwise indicated, products shall comply as follows:

1. Panels:  Minimum 3/8 inch (9.5 mm) thick exterior plywood.

2. Paint:  Exterior primer and exterior grade acrylic-latex emulsion top coat.

H. Open-Mesh Fencing:  Minimum 0.12 inch (3 mm) thick galvanized 2 inch (50 mm) chainlink fabric fencing with galvanized steel pipe posts, 1-1/2 inches (38 mm) inside diameter for line posts and 2-1/2 inches (64 mm) inside diameter for corner posts.

Revise height in following subparagraph if otherwise required for project.

1. Fence height:  6 feet (2 mm).

Retain, delete or revise following subparagraph to suit project.

2. Top Protection:  Galvanized barbed-wire top strand.

Retain the following paragraph with subparagraphs if temporary job-built facilities might be provided.  Edit as necessary for project.

I. Job-Built Temporary Office, Shop and Shed Materials:  Unless otherwise indicated, products shall comply as follows:

1. Framing, Sheathing and Siding:  UL labeled fire-treated lumber and plywood.

2. Roofing:  UL Class A standard weight asphalt shingles, or UL Class C mineral surfaced roll roofing.

3. Exterior Paint:  Exterior primer and exterior grade acrylic-latex emulsion top coat.

4. Interior Wall Panels for Offices:  Minimum ? inch (12.7 mm) thick by 48 inches (1219 mm) wide by maximum available lengths, regular type gypsum board with tapered edges, complying with ASTM C 36.

5. Interior Paint for Offices:  2 coats interior latex-flat wall paint.

Add additional paragraphs for other temporary materials with specific requirements for the project.

2.2 EQUIPMENT

A. Provide undamaged equipment in serviceable conditions and suitable for use intended.

Retain only those of the following paragraphs and subparagraphs applicable to temporary equipment that may be required.  Edit retained paragraphs and subparagraphs as necessary to suit project.  Select english or metric dimensions.  Add paragraphs for other temporary equipment with specific requirements for the project.

B. Water Hoses:  inch (19 mm) heavy duty abrasive-resistant flexible rubber hoses, 100 feet (30 m) long with pressure rating greater than the maximum pressure of the water distribution system.  Provide adjustable shutoff nozzles at hose discharge.

C. Electric Outlets:  Properly configured NEMA-polarized outlets to prevent insertion of 110 to 120 Volt plugs into higher voltage outlets.  Provide receptacle outlets equipped with ground fault circuit interrupters, reset button and pilot light for connection of power tools and equipment.

D. Electric Power Cords:  Grounded extension cords.

1. Provide hard-service cords where exposed to abrasion or traffic.

2. Provide waterproof connectors to connect separate lengths of electric cords where single lengths will not reach areas of construction activity.

3. Do not exceed safe length-voltage ratio.

E. Lamps and Light Fixtures:  General service incandescent lamps of wattage required for adequate illumination.

1. Provide guard cages or tempered glass enclosures where exposed to breakage.

2. Provide exterior fixtures where exposed to moisture.

F. Heating Units:  Temporary heating units that have been tested and labeled by UL, FM or another recognized trade association related to the type of fuel consumed.

G. Temporary Offices:  Prefabricated or mobile units or similar job-built enclosures, inclusive of but not limited to lockable entrances, operable windows, serviceable finishes, heating and air conditioning, electric power and lighting, and foundations adequate for the loads.

Retain the following paragraph if construction personnel are not allowed to use existing toilet facilities or if toilet facilities are not available.

H. Self-Contained Toilet Units:  Temporary single-occupant toilet units of the chemical, aerated recirculation, or combustion type for use by all construction personnel.  Units shall be properly vented and fully enclosed with a glass-fiber-reinforced polyester shell or similar nonabsorbent material.

I. Fire Extinguishers:  Hand-carried portable UL-rated fire extinguishers.

1. Class A extinguishers for temporary offices and similar spaces.

2. Class ABC dry chemical extinguishers or a combination of extinguishers of NFPA recommended classes for the exposures in other locations.

3. Comply with NFPA 10 and NFPA 241 for classification, extinguishing agent and size required by location and class of fire exposure.

PART 3 -  EXECUTION

3.1 INSTALLATION

A. Use qualified personnel for installation of temporary facilities.

B. Locate facilities where they will serve the project adequately and result in minimum interference with performance of construction activities.  Maintain, relocate and modify facilities as required during the construction period.

3.2 TEMPORARY UTILITIES

A. Engage the appropriate local utility companies to install temporary services or connect to existing services.  Where a utility company provides only part of a service, provide the remainder with matching and compatible materials and equipment in compliance with utility company recommendations.

1. Arrange with the utility company and existing users for a time where service can be interrupted, if necessary, to make connections for temporary services.

2. Provide adequate capacity at each stage of construction.

3. Obtain easements to bring in temporary utilities to the site where Government easements are not available for that purpose.

Retain, delete, modify and add to the following paragraphs and subparagraphs of this article to suit project.

B. Temporary Water Service:  Install temporary water service and distribution piping of sizes and pressures adequate for construction needs.  Sterilize water piping prior to use.

C. Temporary Electric Power Service:  Provide weatherproof grounded electric power service and distribution system of sufficient size, capacity and power characteristics for construction needs.  Include meters, transformers, overload-protected disconnects, automatic ground-fault interrupters and main distribution switch gear.

D. Temporary Lighting:  Install and operate temporary lighting that will fulfill security and protection requirements without operating the entire system.  Provide lighting that provides adequate illumination for construction operations and traffic conditions.

E. Temporary Heat and Ventilation:  Provide temporary heat and ventilation required for the construction activities, including but not limited to curing or drying completed installations and protecting construction from adverse effects of low temperatures and high humidity.  Use safe equipment that will not have a harmful effect on elements being installed and on completed installations.  Coordinate ventilation requirements to produce the ambient condition required for the work and to minimize energy consumption, and to protect personnel from fumes and other harmful effects.

F. Heating Facilities:  Provide vented self-contained heaters with individual space thermostatic control.  Do not use gasoline-burning space heaters, open flame or salamander-type heating units.

G. Temporary Telephone Service:  Minimally provide a separate telephone line for each temporary office and first-aid station, and provide a dedicated telephone line for a fax machine in the Contractor's field office.

H. Temporary Sanitary Facilities:  Provide for toilets, wash facilities and drinking water fixtures in compliance with regulations and health codes for type, number, location, operation and maintenance of fixtures and facilities.  Provide toilet tissue, paper towels, paper cups and similar disposable materials as appropriate for each facility, and provide covered waste containers for used materials.

Retain one of the following two subparagraphs as suitable for the project.

1. Toilets:  Use of the Government's existing toilet facilities will be permitted.  Clean and maintain facilities in a condition acceptable to the Contracting Officer and, at completion of construction, restore facilities to condition prevalent at the time of initial use.

2. Toilets:  Install separate self-contained toilet units for male and female personnel.  Shield toilets to ensure privacy.

3. Wash Facilities:  Install wash facilities supplied with potable water at convenient locations for personnel involved in handling materials that require wash-up for a healthy and sanitary condition.

a. Dispose of drainage properly.

b. Supply cleaning compounds appropriate for each condition.

Retain the following sub-subparagraph where required by authorities for health and safety purposes.

c. Include safety showers, eyewash fountains and similar facilities for the convenience, safety and sanitation of personnel.

4. Drinking Water Facilities:  Provide drinking water fountains or containerized tap-dispenser bottled-drinking water units, complete with paper cup supplies.  Where power is accessible, provide electric water coolers to maintain dispensed water temperature at 45 to 55 degF (7 to 13 degC).

Retain one of the two following paragraphs.  Use first paragraph where project includes sufficient sitework that could accommodate drainage ditches or stabilization ponds.  Use second paragraph for other projects.  Edit if necessary.

I. Sewers and Drainage:  If sewers are available, provide temporary connections to remove effluent that can be discharged lawfully.  If sewers can not be used, provide drainage ditches, dry wells, stabilization ponds and similar facilities.  If neither sewers nor drainage facilities can be lawfully used for discharge of effluent, provide containers to remove and dispose of effluent off-site in a lawful manner.

J. Sewers and Drainage:  Where sewers are available, provide temporary connections to remove effluent that can be discharged lawfully.  If sewers are not available, provide containers to remove and dispose of effluent off-site in a lawful manner.

Retain the following subparagraphs with either paragraph above.  Edit to suit project.

1. Filter out excessive amounts of soil, construction debris, chemicals, oils and similar contaminants that might clog sewers or pollute waterways.

2. Connect temporary sewers as directed by sewer utility officials.

3. Maintain temporary sewers and facilities in a clean, sanitary condition.  Following heavy use, promptly restore sewers and facilities to normal conditions.

Retain the following paragraph for projects with sitework that includes excavation and subgrade construction.  Edit if required.

K. Storm Water Controls:  Provide earthen embankments and similar barriers in and around excavations and subgrade construction, sufficient to prevent flooding by runoff of storm water from heavy rains.

Add paragraphs if project includes requirements for other temporary utilities.

3.3 TEMPORARY SUPPORT FACILITIES

A. Locate field offices, storage sheds, sanitary facilities and other temporary construction and support facilities for easy access.

B. Provide incombustible construction for offices, shops and sheds located within the construction area or within 30 feet (9 m) of building lines.  Comply with NFPA 241.

Retain, delete, modify and add to the following paragraphs and subparagraphs of this article to suit project.

C. Field Offices:  Provide insulated, weathertight temporary offices of sufficient size to accommodate office personnel at the Project site.  Include space for small meetings.  Maintain offices clean and orderly.  Furnish and equip offices for use.

1. Furniture:  Minimally provide desks, chairs, file cabinets, plan table, plan rack and bookcase.

2. Sanitary Facilities:  Provide water cooler and a private toilet room with a water closet, lavatory and medicine cabinet.

3. Meeting Space:  Provide a room of not less than 240 sq. feet (22.5 sq. m) for project meetings.  Furnish meeting room with conference table, tack board and not less than 12 chairs.

D. Storage Sheds and Fabrication Shops:  Provide sheds and shops that are sized, furnished and equipped to accommodate materials and equipment involved.  Include complete temporary utility services for intended use.  Sheds and shops may be open shelters or fully enclosed spaces, as appropriate for use.

Retain the following paragraph if project involves temporary paving.  Edit to suit project.

E. Temporary Paving:  Construct and maintain temporary roads and paving to support indicated loadings, if any, and to withstand exposure to traffic during the construction period.

Retain the following subparagraphs that apply to project.  Edit to suit project.

1. Locate temporary paving for roads, storage areas and parking where same permanent facilities will be located, or obtain authorization from the Contracting Officer's Representative for proposed temporary paving not located at permanent paving locations.

2. Comply with Division 2 sections applicable to paving for construction and maintenance of temporary paving.

3. Coordinate temporary paving development with subgrade grading, compaction, installation and stabilization of subbase, and installation of permanent paving.

4. Install temporary paving to minimize the need to rework installations and result in permanent roads and paved areas without damage or deterioration when occupied by the Government.

5. Subject to Contraction Officer's Representative's approval, extend temporary paving in and around the construction area as necessary to accommodate delivery and storage of materials, equipment usage, administration and supervision.

6. Delay installation of permanent paving until acceptable to the Contracting Officer's Representative.  Coordinate with weather conditions to avoid unsatisfactory results.

Add subparagraphs for any additional temporary paving requirements.

Retain the following paragraph for projects with sitework that includes excavation and subgrade construction.

F. Dewatering Facilities and Drains:  Comply with dewatering requirements of applicable Division 2 sections for temporary drainage and dewatering facilities and operations not directly associated with construction activities.  Where feasible, use same facilities provided for the construction activities.  Maintain site, excavation and construction free of water.

G. Temporary Enclosures:  Provide temporary enclosures for protection of construction, in progress or completed, from exposure, inclimate weather, other construction operations and similar conditions.

1. Where heat is needed and the building enclosure is not complete, provide temporary enclosures where there is no other provision for containment of heat.  Coordinate enclosure with ventilating and material drying or curing requirements to avoid dangerous conditions or unacceptable effects.

2. Install tarpaulins securely with incombustible framing.  Close openings of 25 sq. ft. (2.3 sq. m.) or less with plywood or similar materials.

3. Close openings through floor or roof decks and other horizontal surfaces with load-bearing wood-framed construction.

4. Where enclosure exceeds 100 sq. ft. (9.2 sq. m) in area, use UL labeled fire-retardant-treated wood and plywood for framing and sheathing.

Retain the following paragraph for projects of 3 stories or more.

H. Temporary Lifts and Hoists:  Provide facilities for hoisting materials and personnel.

Add subparagraphs for temporary elevators and other special lift and hoist requirements, if any.

I. Project Identification and Other Temporary Signs:  Provide project identification and other signs of sizes, layout, content, graphics and colors indicated.  Locate signs where best to inform public and instruct persons seeking entrance to the project.  Support signs on posts or framing of steel or preservative-treated wood.

Provide sign layout drawing attached to this section.

1. Project Identification Sign:  Comply with sign design provided at the end of this section.  Engage an experienced sign painter to apply graphics.

Add requirements for any other required signs, such as security warnings, to the following subparagraph.

2. Other Temporary Signs:  Prepare signs to provide directional information to construction personnel and visitors.

3. Install exterior yard and sign lights so signs are visible at all times when work is being performed.

4. Do not permit installation of unauthorized signs.

J. Collection and Disposal of Waste:  Collect waste from construction areas and elsewhere daily.  Enforce requirements strictly and dispose of material lawfully.

1. Comply with NFPA 241 for removal of combustible waste material and debris.

2. Do not hold waste materials more than 7 days during periods when the ambient temperature remains continuously less than 80 degF (27 degC), or more than 3 days when the temperature exceeds or is expected to rise above 80 degF (27 degC).

3. Handle and properly containerize hazardous, dangerous or unsanitary waste materials separately from other waste.

Add subparagraphs for other disposal requirements of the project, if any.

K. Rodent and Pest Control:  Retain a local exterminator or pest control company to recommend practices to minimize attraction and harboring of rodents, roaches and other pests.  Employ this service to perform extermination and control procedures at regular intervals so the project will be free of pests and their residue at completion.  Perform control operations lawfully, using environmentally safe materials.

Retain the following paragraphs if applicable for project.

L. Temporary Stairs:  Until permanent stairs are available, provide temporary stairs where ladders are not adequate.  When permanent stairs are available, cover finishes with protective coverings to result in undamaged conditions at project completion.

Add additional paragraphs for special support facilities applicable to the project, if any.

3.4 TEMPORARY PROTECTION FACILITIES

A. Temporary Facility Changeover:  Except for using permanent fire protection facilities as soon as available, do not change over from temporary protection facilities until authorized by the Contracting Officer's Representative.

B. Temporary Fire Protection:  Until fire protection needs are supplied by permanent facilities, install and maintain temporary fire protection facilities of types needed to protect against reasonably predictable and controlled fire losses.

1. Comply with NFPA 10 "Standard for Portable Fire Extinguishers" and NFPA 241 "Standard for Safeguarding Construction, Alterations, and Demolition Operations".

Edit the following subparagraph by deleting ", including stairwells on each floor" if project building contains one story only.

2. Locate fire extinguishers where convenient and effective for their intended purpose, but not less than one extinguisher at or near each access route exit or entrance, including stairwells on each floor.

3. Store combustible materials in containers in fire-safe locations.

4. Maintain unobstructed access to fire extinguishers, fire hydrants, temporary fire protection facilities and access routes.  Prohibit smoking in hazardous fire-exposure areas.

5. Provide supervision of welding operations, combustion-type temporary heating units and other sources of fire ignition.

C. Permanent Fire Protection:  At the earliest feasible date in each area of the Project, complete installation of the permanent fire protection facilities, including connected services, and place into operation and use.  Instruct key personnel in the use of the facilities.

D. Barricades, Warning Signs, and Lights:  Comply with standards and code requirements for erecting structurally adequate barricades.  Paint with appropriate colors, graphics, and warning signs to inform personnel and the public of the hazard involved.  Where appropriate and needed, provide lighting, including flashing red or amber lights.

Retain the following paragraph where an enclosure fence is required for the project.

E. Enclosure Fence:  When excavation begins, install an enclosure fence with lockable entrance gates.  Install in a manner that will prevent people and animals from easily entering the site, except by the entrance gates.

1. Locate where indicated or, if not indicated, enclose the entire site or the portion determined sufficient to accommodate construction operations.

Retain one of the three following subparagraphs.  Edit as required for project.

2. Provide open-mesh chainlink fencing with posts set in a compacted mixture of gravel and earth.

3. Provide 8 feet (12.5 m) high plywood fence framed with four nominal 2-by-4-inch (50-by-100-mm) wood rails, and preservative-treated wood posts spaced not more than 8 feet (2.5 m) apart.

4. Provide either open-mesh chainlink fencing with posts set in a compacted mixture of gravel and earth, or provide 8 feet (2.5 m) high plywood fence framed with four nominal 2-by-4 inch (50-by-100 mm) wood rails and preservative-treated wood posts spaced not more than 8 feet (2.5 m) apart.

Retain the following paragraph and subparagraphs where covered walkway is required.  Edit for project.

F. Covered Walkway:  Erect a structurally adequate, protective covered walkway for passage of persons along the adjacent public street.  Coordinate with entrance gates, other facilities, and obstructions.  Comply with regulations of authorities having jurisdiction.

1. Construct covered walkways using scaffold or shoring framing.

2. Provide wood plank overhead decking, protective plywood enclosure walls, handrails, barricades, warning signs, lights, safe and well-drained walkways, and similar provisions for protection and safe passage.

3. Extend the back wall to complete the enclosure fence.

4. Paint and maintain in a manner acceptable to the Contracting Officer's Representative.

G. Security Enclosure and Lockup:  Install substantial temporary enclosure of partially completed areas of construction.  Provide locking entrances to prevent unauthorized entrance, vandalism, theft and similar violations of security.

1. Storage:  Provide a secure lockup for valuable stored materials and equipment.

2. Enforce discipline in connection with the installation and release of material to minimize the opportunity for theft and vandalism.

H. Environmental Protection:  Provide protection, operate temporary facilities, and conduct construction in ways and by methods that comply with environmental regulations, and minimize the possibility that air, waterways, and subsoil might be contaminated or polluted or that other undesirable effects might result.  Avoid using tools and equipment that produce harmful noise.  Restrict use of noise-making tools and equipment to hours that will minimize complaints from persons near the site.

Add additional paragraphs for any other special protection facilities.

3.5 OPERATION, TERMINATION, AND REMOVAL

A. Supervision:  Enforce strict discipline in use of temporary facilities.  Limit availability of temporary facilities to essential and intended uses to minimize waste and abuse.

B. Maintenance:  Maintain facilities in good operating condition until removal.  Protect from damage by freezing temperatures and similar elements.

1. Maintain operation of temporary enclosures, heating, cooling, humidity control, ventilation, and similar facilities on a 24-hour basis where required to achieve indicated results and to avoid possibility of damage.

2. Protection:  Prevent water-filled piping from freezing.  Maintain markers for underground lines.  Protect underground lines from damage during excavation operations.

C. Termination and Removal:  Unless the Contracting Officer's Representative requests that a temporary facility be maintained longer, each temporary facility shall be removed when the need for its service has ended and can be replaced by authorized use of a permanent facility.  Complete or, if necessary, restore permanent construction that may have been delayed because of interference with the temporary facility.  Repair damaged Work, clean exposed surfaces, and replace construction that cannot be satisfactorily repaired.

1. Materials and facilities that constitute temporary facilities are the property of the Contractor, except the Government reserves the right to take possession of project identification signs.

Retain the following subparagraph if temporary paving is required.  Edit for project.

2. Remove temporary paving not intended for or acceptable for integration into permanent paving.  Where the area is intended for landscape development, remove soil and aggregate fill that do not comply with requirements for subsoil or fill in the area.  Remove materials contaminated with road oil, asphalt and other petrochemical compounds, and other substances that might impair growth of plant materials or lawns.  Repair or replace street paving, curbs, and sidewalks at temporary entrances, in accordance with the requirements of the governing authority.

3. Prior to project completion, replace clean and restore permanent facilities used during the construction period including, but not limited to, the following:

a. Replace air filters and clean inside of ductwork and housings.

b. Replace significantly worn parts and parts subject to unusual operating conditions.

c. Replace lamps burned out or noticeably dimmed by hours of use.

END OF SECTION 01500

TEMPORARY FACILITIES AND CONTROLS
01500 - 3

