GSA SUPPLEMENT TO MASTERSPEC
GSA
9/99

Retain this section if the "Record Document Submittals" article is not included in Section 01770; use this section for large projects with requirements for detailed records.

SECTION 01781 - PROJECT RECORD DOCUMENTS
PART 1 -  GENERAL

1.1 SUMMARY

A. This section includes administrative and procedural requirements for Project Record Documents.  Required Project Record Documents include the following:

Edit, delete and add to the following subparagraphs to suit project.  Correspondingly edit articles that follow.

1. Marked-up copies of Contract Drawings.

2. Marked-up copies of Shop Drawings.

3. Newly prepared drawings.

4. Marked-up copies of Specifications, addenda, and Change Orders.

5. Marked-up Product Data submittals.

6. Record Samples.

7. Field records for variable and concealed conditions.

8. Record information on Work that is recorded only schematically.

B. Maintenance of Documents and Samples:  Store record documents and samples in the field office apart from the Contract Documents used for construction.  Do not use Project Record Documents for construction purposes.  Maintain record documents in good order and in a clean, dry, legible condition.  Make documents and samples available at all times for the Contracting Officer's Representative's inspections.

1.2 RECORD DRAWINGS

A. Markup Procedure:  During construction, maintain a set of blue- or black-line white prints of Contract Drawings and Shop Drawings for Project Record Document purposes.

1. Mark these Drawings to show the actual installation where the installation varies from the installation shown originally.  Give particular attention to information on concealed elements that would be difficult to identify or measure and record later.  Items required to be marked include, but are not limited to, the following:

Edit, delete and add to the following sub-subparagraphs to suit project.

a. Dimensional changes to the Drawings.

b. Revisions to details shown on the Drawings.

c. Depths of foundations below the first floor.

d. Locations and depths of underground utilities.

e. Revisions to routing of piping and conduits.

f. Revisions to electrical circuitry.

g. Actual equipment locations.

h. Duct size and routing.

i. Locations of concealed internal utilities.

j. Changes made by Change Order or Construction Change Directives.

k. Changes made following the Architect's written orders.

l. Details not on original Contract Drawings.

2. Mark record prints of Contract Drawings or Shop Drawings, whichever is most capable of showing actual physical conditions, completely and accurately.  Where Shop Drawings are marked, show cross-reference on Contract Drawings location.

3. Mark record sets with red erasable colored pencil.  Use other colors to distinguish between changes for different categories of the Work at the same location.

4. Mark important additional information that was either shown schematically or omitted from original Drawings.

5. Note alternate numbers, Change Order numbers, and similar identifications.

B. Responsibility for Markup:  The individual, installer, subcontractor or other entity who obtained the record data shall prepare the markup on record drawings.

1. Accurately record information in an understandable drawing technique.

2. Record data as soon as possible after obtaining it.  Record and check the markup prior to enclosing concealed installations.

Retain either the following subparagraph or the following two paragraphs with subparagraphs.

3. Prior to Final Acceptance, submit record drawings to the Contracting Officer for the Governments records.  Organize into sets, and bind and label.

Retain the following two paragraphs with subparagraphs if subparagraph above is deleted.  Retain the following if transparencies are required.

C. Preparation of Transparencies:  Prior to Final Acceptance, review completed marked-up record drawings with the Contracting Officer's Representative.  When authorized, prepare a full set of corrected transparencies of the Contract Drawings and Shop Drawings.

1. Incorporate changes and additional information previously marked on print sets.  Erase, redraw, and add details and notations where applicable.  Identify and date each drawing.  Include the printed designation "PROJECT RECORD DRAWINGS" in a prominent location on each drawing.

Retain or edit the two following sample subparagraphs to suit any specific requirements for locating and identifying changes.  Delete if recording final construction is important but tracking changes during the construction process is unimportant.

a. Encircle each area of change or additional information with a free-form cloud-shape drawn on the reverse side of the transparency.

b. Identify changes and additional information by printing the Change Order Number or other change reference designation, when applicable, within the cloud-shape encircled area.

2. Refer instances of uncertainty to the Contracting Officer's Representative for resolution.

Retain one of the two following paragraphs.

3. The Government will furnish the Contractor with one set of transparencies of original Contract Drawings for use in recording changes and additional information.  Other printing is the Contractor's responsibility.

4. The Contractor is responsible for printing original Contract Drawings and other drawings as required to produce transparencies.  The Contracting Officer's Representative will make original Contract Drawings available to the Contractor's print shop.

5. Review of Transparencies:  Before copying and distributing, submit corrected transparencies and the original marked-up prints to the Contracting Officer's Representative for review and acceptance of the general scope of changes, additional information recorded and quality of drafting.  If acceptable, the Contracting Officer's Representative will return transparencies and the original marked-up prints to the Contractor for organizing into sets, printing, binding, and final submittal.

Revise number of prints in the following paragraph if required for the project.

D. Copies and Distribution:  After completing the preparation of transparency record drawings, print 3 blue- or black-line prints of each drawing, whether or not changes and additional information were recorded.  Organize the copies into manageable sets.  Bind each set with durable-paper cover sheets.  Include appropriate identification, including titles, dates, and other information on the cover sheets.

1. Organize and bind original marked-up set of prints that were maintained during the construction period in the same manner.

2. Organize record transparencies into sets matching the print sets.  Place these sets in durable tube-type drawing containers with end caps.  Mark the end cap of each container with suitable identification.

3. Submit the marked-up record set, transparencies, and the copy sets to the Contracting Officer for the Government's records.

E. Newly Prepared Record Drawings:  Prepare new drawings instead of following procedures specified for preparing record drawings where new drawings are required when neither the original Contract Drawings nor Shop Drawings are suitable to show the actual installation.  New drawings may be required when a Change Order is issued as a result of accepting an alternate, substitution, or other modification.

1. Provide Drawings in a scale that allows for the scope of detailing and notations required to record the actual physical installation and its relationship to other construction.

2. When completed and accepted, integrate newly prepared Drawings with procedures specified for organizing, copying, binding and submitting record drawings.

1.3 RECORD SPECIFICATIONS

Revise the number of copies in the following paragraph if required for the project.

A. During the construction period, maintain 3 copies of the Project Specifications, including addenda and other modifications issued, for Project Record Document purposes.

1. Mark the Specifications to indicate the actual installation where the installation varies from that indicated in Specifications. Note related project record drawing information, where applicable.  Give particular attention to substitutions, selection of product options, and information on concealed installations that would be difficult to identify or measure and record later.

Retain the following sub-subparagraphs or edit or delete to suit project requirements.

a. In each Specification section where products, materials or units of equipment are specified or scheduled, mark the copy with the proprietary name and model number of the product furnished.

b. Record the name of the manufacturer, supplier, installer, and other information necessary to provide a record of selections made and to document coordination with record Product Data submittals and maintenance manuals.

2. Upon completion of markup, submit Record Specifications to the Contracting Officer for the Government's records.

1.4 RECORD PRODUCT DATA

A. During the construction period, maintain one copy of each Product Data submittal for Project Record Document purposes.

1. Mark Product Data to indicate the actual product installation where the installation varies substantially from that indicated in Product Data submitted.  Include significant changes in the product delivered to the site and changes in manufacturer's instructions and recommendations for installation.

2. Give particular attention to information about concealed products and installations that cannot be readily identified and recorded later.

3. Note related Change Orders and markup of record Drawings, where applicable.

4. Upon completion of markup, submit a complete set of record Product Data to the Contracting Officer for the Government's records.

5. Where record Product Data is required as part of maintenance manuals, submit marked-up Product Data as an insert in the manual instead of submittal as record Product Data.

Retain the following article if project includes samples that may be critical for future work.

1.5 RECORD SAMPLE SUBMITTAL

A. Immediately prior to the date of Substantial Completion, the Contractor shall meet with the Contracting Officer's Representative at the site to determine which of the Samples maintained during the construction period shall be transmitted to the Government for record purposes.

B. Comply with the Contracting Officer's Representative's instructions for packaging, identification marking and delivery to the Government's sample storage space.  Dispose of other samples in a manner specified for disposing of surplus and waste materials.

1.6 MISCELLANEOUS RECORD SUBMITTALS

A. Refer to other Specification sections for miscellaneous record-keeping requirements and submittals in connection with various construction activities.  Prior to Final Acceptance, complete miscellaneous records and place in good order, properly identified and bound or otherwise organized to allow for use and reference.

B. Submit to the Contracting Officer for the Government's records.

C. Miscellaneous records include, but are not limited to, the following:

Edit, delete and add to the following subparagraphs to suit the project.

1. Field records on excavations and foundations.

2. Field records on underground construction and similar work.

3. Survey showing locations and elevations of underground lines.

4. Invert elevations of drainage piping.

5. Surveys establishing building lines and levels.

6. Authorized measurements utilizing unit prices or allowances.

7. Records of plant treatment.

8. Ambient and substrate condition tests.

9. Certifications received in lieu of labels on bulk products.

10. Batch mixing and bulk delivery records.

11. Testing and qualification of tradespersons.

12. Documented qualification of installation firms.

13. Load and performance testing.

14. Inspections and certifications by governing authorities.

15. Leakage and water-penetration tests.

16. Fire-resistance and flame-spread test results.

17. Final inspection and correction procedures.

PART 2 -  PRODUCTS (Not Applicable)

PART 3 -  EXECUTION (Not Applicable)

END OF SECTION 01781

PROJECT RECORD DOCUMENTS
01781 - 1

