Copyright 1998 AIA
MASTERSPEC
9/98

NIH VERSION of MASTERSPEC based on GSA Masterspec dated 9/99

NIH Version dated January 2002

This section uses the term Contracting Officer, Contracting Officer's Technical Representative and/or Project Officer in specific locations within this section based on specific authorities granted each individual under the terms of the contract.  Any changes in these terms at specific locations within these specifications must be coordinated with the Contracting Officer.

<Text> requires insertion of data

[text] requires decision to delete, retain, or edit text in brackets

SECTION 01731 - CUTTING AND PATCHING
PART 1 -  GENERAL

1.1 RELATED DOCUMENTS

A. Drawings and general provisions of the Contract, including General and Supplementary Conditions apply to this Section.

1.2 SUMMARY

A. This section includes procedural requirements for cutting and patching.

B. Definition:  Cutting and patching includes cutting into existing construction to provide for the installation or performance of other work and subsequent fitting and repair required to restore surfaces to their original condition.

C. Refer to other sections for other requirements and limitations applicable to cutting and patching individual parts of the work.

Retain the following paragraph for projects with demolition work.

D. Coordinate cutting and patching with demolition requirements specified in Division 1 Section "Selective Demolition."

E. Related Sections:  This specification section is related to any and all specification sections with explicit or implicit reference to cutting and patching.  Specific submittal requirements of these related specification sections are not included in this section.  Related sections include but are not limited to the following specification sections:

Edit the following list to include pertinent Division 1 sections included in the project.

1. Division 1 Section "Summary"

2. Division 1 Section "Work Restrictions"

3. Division 1 Section "Project Management and Coordination"

4. Division 1 Section "Construction Progress Documentation"

5. Division 1 Section "Photographic Documentation"

6. Division 1 Section "Submittal Procedures"

7. Division 1 Section "Quality Requirements"

8. Division 1 Section "References"

9. Division 1 Section "Construction Quality Control"

10. Division 1 Section "Temporary Facilities and Controls"

11. Division 1 Section "Use, Handling, Storage, Transporting, Accumulation and Disposal of NIH Controlled Material"

12. Division 1 Section "Temporary Traffic Controls"

13. Division 1 Section "Safety and Health"

14. Division 1 Section "Product Requirements"

15. Division 1 Section "Execution Requirements"

16. Division 1 Section "Selective Demolition"

17. Division 1 Section "Fire Precautions for Hot Work"

18. Division 1 Section "Closeout Procedures"

19. Division 1 Section "Project Record Documents"

1.3 SUBMITTALS

Revise time in the following paragraph to suit project. NIH minimum time is indicted.  Coordinate with similar language contained in Division 1 Section "Project Management and Coordination."

A. Cutting and Patching Plan:  Submit a written plan to the Project Officer through the Quality Control manager describing procedures at least 21 calendar days in advance of the time cutting and patching will initially be performed.

1. Include the following information, as applicable:

a. Description of the extent of cutting and patching required.  Show how it will be performed and indicate why it cannot be avoided.

b. Description of the anticipated results in terms of changes to existing construction.  Include changes to structural elements and operating components as well as changes in appearance and other significant visual elements.

c. List of products to be used and entities that will perform work.

d. Dates and hours of operation when cutting and patching will be performed.

Retain the following sub-subparagraph if utilities are affected by cutting and patching.  Coordinate with Utility Interruption Plan requirements contained in Division 1 Section "Project Management and Coordination."

e. List of utilities that cutting and patching procedures will disturb or affect.  List utilities that will be relocated and those that will be temporarily out-of-service.  Indicate how long service will be disrupted.

f. Compatibility and cohesion characteristics of patching compounds with adjacent materials.

Retain the following sub-subparagraph if structure requires reinforcement as a result of cutting and patching.

g. Details and engineering calculations showing integration of reinforcement with the original structure, where cutting and patching involves adding reinforcement to structural elements.

2. Approval by the Project Officer to proceed with cutting and patching does not waive the right to later require complete removal and replacement of unsatisfactory work.

Retain the following paragraph if cutting and patching samples are required for historically significant or other special items.  Revise time to suit project.  NIH minimum required time is indicated.  If this paragraphs is used indicate special cutting and patching samples required in sub-subparagraphs.

3. Samples:  Provide cutting and patching samples for the following items within 14 calendar days after notice to proceed in order that special reviews and coordination can be arranged with approval authorities.

a. <List required samples.>

1.4 QUALITY REQUIREMENTS

A. Requirements for Structural Work:  Do not cut and patch structural elements in a manner that would change their load-carrying capacity or load-deflection ratio.

1. The cutting and patching plan shall include but not be necessarily limited to work required at the following structural elements:

Edit, delete or add to the following sub-subparagraphs to list the project's structural elements to be cut and patched.

a. Foundation construction.

b. Bearing and retaining walls.

c. Structural concrete.

d. Structural steel.

e. Lintels.

f. Timber and primary wood framing.

g. Structural decking.

h. Stair systems.

i. Miscellaneous structural metals.

j. Exterior curtain-wall construction.

k. Equipment supports.

l. Piping, ductwork, vessels, and equipment.

m. Structural systems of other construction.

Retain the following paragraph if modifications to operational elements and safety systems are extensive. Edit, delete or add to the sub-subparagraphs to list the project's operating elements and safety related systems to be cut and patched.

B. Operational Limitations:  Do not cut and patch operating elements, safety related systems, or related components in a manner that would result in reducing their capacity to perform as intended.  Do not cut and patch operating elements, safety related systems or related components in a manner that would result in increased maintenance or decreased operational life or safety. Operating elements or safety related systems include but are not limited to the following:

1. Primary operational systems and equipment.

2. Air or smoke barriers.

3. Water, moisture, or vapor barriers.

4. Membranes and flashings.

5. Fire protection systems.

6. Noise and vibration control elements and systems.

7. Control systems.

8. Communication systems.

9. Conveying systems.

10. Electrical wiring systems.

11. Operating systems of other construction.

C. Visual Requirements:  Do not cut and patch construction exposed on the exterior or in occupied spaces in a manner that would, in the Project Officer's opinion, reduce the building's aesthetic qualities.  Do not cut and patch construction in a manner that would result in visual evidence of cutting and patching.  Remove and replace construction that was previously cut and patched in a visually unsatisfactorily manner.  When requested by the Contracting Officer, engage a Specialist who is specifically experienced in the work.  Visual elements include but are not limited to the following:

1. Processed concrete finishes.

2. Stonework and stone masonry.

3. Ornamental metal.

4. Matched-veneer woodwork.

5. Preformed metal panels.

6. Firestopping.

7. Window wall system.

8. Stucco and ornamental plaster.

9. Acoustical ceilings.

10. Terrazzo.

11. Finished wood flooring.

12. Fluid-applied flooring.

13. Carpeting.

14. Aggregate wall coating.

15. Wall covering.

16. Mechanical system enclosures, cabinets, or covers.

1.5 EXISTING WARRANTIES

A. Remove, replace, patch, and repair material and surfaces cut or damaged during cutting and patching operations by methods and with materials in such a manner as not to void any existing warranties.

PART 2 -  PRODUCTS

2.1 MATERIALS

A. Comply with requirements specified in other Sections of these Specifications.

B. Use materials identical to existing materials.  For exposed surfaces, use materials that visually match existing adjacent surfaces to the fullest extent possible.

1. If identical materials are unavailable or cannot be used, use materials that, when installed, will match the visual and functional performance of existing materials.

Insert specific material requirements here if not specified elsewhere.  Material and installation requirements are better specified in individual Sections.

PART 3 -  EXECUTION

3.1 INSPECTION

A. Before cutting, examine surfaces to be cut and patched and conditions under which cutting and patching is to be performed.  If unsafe or unsatisfactory conditions are encountered, take corrective action before proceeding.

B. Before proceeding with cutting and patching involving two or more trades, meet at the project site with the Project Officer and the entities providing or affected by the cutting and patching.  Site meeting should be incorporated into the Three Phases of Quality Control process managed by the Contractor's Quality Control Manager.  Review areas of potential interference and conflict.  Coordinate procedures and resolve potential conflicts before proceeding.

C. Compatibility:  Before patching, verify compatibility with and suitability of substrates, including compatibility with existing finishes or primers.

3.2 PREPARATION

A. Provide temporary support of work to be cut.

B. Protect existing conditions during cutting and patching to prevent damage.  Provide protection from adverse weather conditions for portions of the Project that might be exposed during cutting and patching operations.

C. Avoid interference with use of adjoining areas or interruption of free passage to adjoining areas.

D. Where existing services are required to be removed, relocated, abandoned, bypass such services before cutting to avoid interruption of services to occupied areas.

3.3 PERFORMANCE

A. Employ skilled workmen to perform cutting and patching.  Proceed with cutting and patching at the earliest feasible time and complete without delay.

B. Cutting:  Cut existing construction using methods least likely to damage elements retained and adjoining construction.  Where possible, review proposed procedures with the original installer and comply with the original installer's recommendations.

Edit, delete and add to any of the following subparagraphs to suit the project.

1. In general, use hand or small power tools designed for sawing or grinding, not for hammering and chopping.

2. Cut holes and slots as small as possible, neatly to size required, and with minimum disturbance of adjacent surfaces.  Temporarily cover openings when not in use.

3. To avoid marring existing finished surfaces, cut or drill from the exposed or finished side into concealed surfaces.

4. Cut through concrete and masonry using a cutting machine, such as a Carborundum saw or a diamond-core drill.

5. Comply with requirements of applicable Division 2 sections where cutting and patching requires excavating and backfilling.

6. After utility services are bypassed, cut-off pipe or conduit in walls or partitions to be removed. Cap, valve or plug and seal the remaining portion of pipe or conduit to prevent entrance of moisture or other foreign matter after by-passing and cutting.

C. Patching:  Patch construction by filling, repairing, refinishing, closing up, and similar operations following performance of other work.  Patch with durable seams that are as invisible as possible.  Provide materials and comply with installation requirements specified in other Sections of these Specifications.

Edit, delete and add to the following subparagraphs to suit the project.

1. Where feasible, inspect and test patched areas to demonstrate integrity of the installation.

2. Restore exposed finishes of patched areas and extend finish restoration into adjoining construction in a manner that will eliminate evidence of patching and refinishing.

3. Where removed walls or partitions extends one finished area into another finished area, patch and repair floor and wall surfaces to provide an even surface of uniform color[, finish, texture,] and appearance.  Remove existing floor and wall coverings and replace with new materials, if necessary, to achieve uniform color and appearance.

4. Where patching occurs in a smooth painted surface, extend final paint coat over entire unbroken surface that contains the patch after the area has received primer and other undercoats.

5. Patch, repair or rehang existing ceilings as necessary to provide an even-plane surface of uniform appearance.

6. Maintain integrity of fire barriers, vapor barriers and insulation.

7. Exterior Building Enclosure:  Patch components in a manner that restores enclosure to a weathertight condition.

Retain the following paragraph when specific cutting and patching work is listed in Division 1 Section "Work Restrictions" to be performed during NIH unoccupied hours.  This may include work that generates noise or other disruptive activity.

D. Perform cutting and patching work listed in Division 1 Section "Work Restrictions" during NIH Unoccupied Hours.

3.4 CLEANING

A. Clean areas and spaces where cutting and patching are performed.  Completely remove paint, mortar, oils, putty, and similar items.

B. Thoroughly clean piping, conduit, and similar features before applying paint, restored pipe coverings, or other finishing materials.

END OF SECTION 01731

CUTTING AND PATCHING
01731 - 6

