Copyright 1998 AIA
MASTERSPEC
9/98

NIH Version based on AIA MASTERSPEC dated 9/98 and GSA MASTERSPEC dated 9/99

NIH Version dated January 2002

This section uses the term Contracting Officer, Contracting Officer's Technical Representative and/or Project Officer in specific locations within this section based on specific authorities granted each individual under the terms of the contract.  Any changes in these terms at specific locations within these specifications must be coordinated with the Contracting Officer.

<Text> requires insertion of data

[text] requires decision to delete, retain, or edit text in brackets

Retain this section if the "Record Document Submittals" article is not included in Division 1 Section "Closeout Procedures."  Use this section for large projects with requirements for detailed records.

SECTION 01781 - PROJECT RECORD DOCUMENTS
PART 1 -  GENERAL

1.1 RELATED DOCUMENTS

A. Drawings and general provisions of the Contract, including General and Supplementary Conditions apply to this Section.

1.2 SUMMARY

A. This section includes administrative and procedural requirements for Project Record Documents.  Required Project Record Documents include the following:

Edit, delete and add to the following subparagraphs to suit project.  Coordinate list with articles that follow in this section.

1. Marked-up copies of Contract Drawings.

2. Marked-up copies of Shop Drawings.

3. Newly prepared drawings.

4. Marked-up copies of Specifications, addenda, and Change Orders.

5. Marked-up Product Data submittals.

6. Construction Photographs

7. Record Samples.

8. Miscellaneous Record Submittals.

B. Related Sections:  This specification section is related to any and all specification sections with explicit or implicit reference to project record documents.  Specific submittal requirements of these related specification sections are not included in this section.  Related sections include but are not limited to the following specification sections:

Edit the following list to include pertinent Division 1 sections included in the project.

1. Division 1 Section "Summary"

2. Division 1 Section "Project Management and Coordination"

3. Division 1 Section "Construction Progress Documentation"

4. Division 1 Section "Photographic Documentation"

5. Division 1 Section "Submittal Procedures"

6. Division 1 Section "Quality Requirements"

7. Division 1 Section "Construction Quality Control"

8. Division 1 Section "Temporary Facilities and Controls"

9. Division 1 Section "Use, Handling, Storage, Transporting, Accumulation and Disposal of NIH Controlled Material"

10. Division 1 Section "Safety and Health"

11. Division 1 Section "Product Requirements"

12. Division 1 Section "Execution Requirements"

13. Division 1 Section "Closeout Procedures"

14. Division 1 Section "Demonstration and Training"

15. Divisions 2 through 16 sections for project record document requirements for products included in those sections.

C. Project Record Documents and samples are to be stored by the Contractor in the field office apart from the Contract Documents used for construction.  Project Record Documents are not to be used for construction purposes.  Record Documents shall be maintained in good order and in a clean, dry, legible condition and available at all times for the Contracting Officer's inspection.

1.3 SUBMITTALS

A. Record Drawings:  Comply with the following:

1. Number of Copies:  Submit [one] <Insert number> set[s] of marked-up Record Prints to the Contracting Officer for review and approval prior to final payment.

Retain subparagraph above for small projects with minimal Record Drawings.  Tailor following articles to eliminate descriptive sections for products not required.

Retain subparagraph and associated subparagraphs below for large projects with complex Record Drawings and or Specifications where the designer may have a significant role in their review and approval.  Below involves and initial submittal for verification of accuracy and quality before having the Contractor make any corrections and additional prints.  In the paragraph above, the ability to have corrections made is implied in the "for review and approval" context.

Coordinate the procedures and authority marking the initial submittal with the Project Officer.

2. Number of Copies:  Submit copies of Record Drawings as follows:

Tailor the following paragraph for the product and copies required.

a. Initial Submittal:  Prior to the Pre-final Acceptance Inspection, submit [one] <Insert number> set[s] of [corrected Record Transparencies] [plots from corrected Record CAD Drawings] and [one] <Insert number> set[s] of marked-up Record Prints to the Contracting Officer.  Contracting Officer will initial and date each [transparency] [plot] and mark whether general scope of changes, additional information recorded, and quality of drafting are acceptable.  The Contracting Officer will return [transparencies] [plots] and prints for correction and organizing into sets, printing, binding, and final submittal.

Revise number of sets in subparagraphs below to suit Project and eliminate products not required.  Coordinate with the Project Officer for number of prints required and type of products required.  Tailor paragraphs in "Products" article of this specification to reflect products specified, eliminating those not required.

Tailor the number of sets in the following subparagraph based on the following:  Four sets of drawings are required for work that includes NIH Building 10, three sets for all other NIH buildings.

b. Final Submittal:  Prior to final payment, submit [one] <Insert number> set[s] of marked-up Record Prints, <Insert number> set[s] of record transparencies, and [three] <Insert number> copies printed from Record Transparencies, [one] <Insert number> set[s] of Record CAD Drawing files, [one] <Insert number> set[s] of Record CAD Drawing plots, and [three] <Insert number> copies printed from record plots.  Incorporate all comments provided with return of the initial submission.  Plot and print each Drawing, whether or not changes and additional information were recorded.

Edit format in subparagraph below to match specific requirements for electronic media.  Coordinate with the Project Officer for electronic format requirements.

1) Electronic Media:  [3-1/2-inch diskettes, formatted for 1.44 MB] [CD-ROM].

Tailor the number of sets of Record Specifications and Record Product Data in the following paragraphs based on the following:  Two sets of documents are required for work that includes NIH Building 10, One set for all other NIH buildings.

B. Record Specifications:  Submit [one copy] <Insert number> [copies] of Record Specifications, including addenda and contract modifications.

C. Record Product Data:  Submit [one copy] <Insert number> [copies] of each Product Data submittal.

1. Where Record Product Data is required as part of operation and maintenance manuals, submit marked-up Product Data as an insert in the manual instead of submittal as Record Product Data.

PART 2 -  PRODUCTS

2.1 RECORD DRAWINGS

A. Record Prints:  During construction, maintain a set of blue or black-line white prints of Contract Drawings and Shop Drawings for Project Record Document purposes.

1. Mark Project Record Drawings to show the actual installation where the installation varies from the installation shown originally.  Give particular attention to information on concealed elements that would be difficult to identify or measure and record later.  Items required to be marked include, but are not limited to, the following:

Edit, delete and add to the following sub-subparagraphs to suit project.

a. Dimensional changes to the Drawings.

b. Revisions to details shown on the Drawings.

c. Depths of foundations below the first floor.

d. Locations and depths of underground utilities.

e. Revisions to routing of piping and conduits.

f. Revisions to electrical circuitry.

g. Actual equipment locations.

h. Duct size and routing.

i. Locations of concealed internal utilities.

j. Changes made by Change Order or Construction Change Directives.

k. Changes made following the Contracting Officer's written orders.

l. Details not on original Contract Drawings.

2. Mark record prints of Contract Drawings or Shop Drawings, whichever is most capable of showing actual physical conditions, completely and accurately.  Where Shop Drawings are marked, show cross-reference to and mark Contract Drawings location.

If the following paragraph is included, include the paragraphs regarding "Newly Prepared Record Drawings" in this section.

3. Where Contract Drawings or Shop Drawings cannot be modified to clearly indicate the actual conditions, prepare new drawings as specified in the section.

4. Mark record sets with red erasable colored pencil.  Use other colors to distinguish between changes for different categories of the work at the same location.

5. Mark important additional information that was either shown schematically or omitted from original Drawings.

6. Note alternate numbers, Change Order numbers, and similar identifications.

7. Responsibility for Markup:  The individual, installer, subcontractor or other entity who obtained the record data shall prepare the markup on record drawings.

a. Accurately record information in an understandable drawing technique.

b. Record data as soon as possible after obtaining it.  Record and check the markup prior to enclosing concealed installations.

Paragraph and subparagraphs below describe a procedure for providing reproducible record drawings.  Add requirements for special printing methods on specific material, such as moisture-sensitive prints on Mylar film.  Delete Record Transparency paragraph if Record Transparencies are not required.

B. Record Transparencies:  Prior to Final Acceptance, review completed marked-up record prints with the Project Officer.  When authorized, prepare a full set of corrected transparencies of the Contract Drawings and Shop Drawings.

1. Transparencies shall be based on approved record prints incorporating changes and additional information previously marked on the record prints.  Format shall be as specified for record prints.

Retain one of the two following paragraphs. Coordinate with the paragraph "Record CAD Drawings" in this section.

2. NIH will furnish the Contractor with one set of transparencies of original Contract Drawings for use in recording changes and additional information.  Other printing is the Contractor's responsibility.

3. The Contractor is responsible for printing original Contract Drawings and other drawings as required to produce transparencies.  The Project Officer will make original Contract Drawings available to the Contractor's print shop.

4. Review of transparencies:  Before copying and distributing copies of the corrected drawings, submit corrected transparencies with the original marked-up prints to the Contracting Officer for review and acceptance of the general scope of changes, additional information recorded and quality of drafting.  If acceptable, the Contracting Officer will return transparencies and the original marked-up prints to the Contractor for organizing into sets, printing, binding, and final submittal.

Revise number of prints in the following paragraph if required for the project.

5. Copies and Distribution:  After completing the preparation of transparency record drawings, print [three] blue- or black-line prints of each drawing, whether or not changes and additional information were recorded.  Organize the copies into manageable sets.  Bind each set with durable-paper cover sheets.  Include appropriate identification, including titles, dates, and other information on the cover sheets.

a. Organize and bind original marked-up set of prints that were maintained during the construction period in the same manner.

b. Organize record transparencies into sets matching the print sets.  Place these sets in durable tube-type drawing containers with end caps marked with suitable identification.

c. Submit the marked-up record set, transparencies, and the copy sets to the Contracting Officer for NIH records.

Include the following sub paragraph if Record CAD Drawings paragraph is used.

d. If project drawings were available in electronic media, then record drawings shall also be provided in electronic media as specified in this section.

The following paragraph should be used when CAD drawings are required in lieu of or in addition to print and/or transparencies.  Coordinate CAD information and formatting requirements with Submittals article and "Record Print" and "Record Transparency" paragraphs of this specification.  In paragraph and subparagraphs below, architects may want to delete title block information from CAD files provided to Contractor.  Consult with Project Officer for format required.  Note that DXF format may not provide a clean translation from one CAD program to another.

C. Record CAD Drawings:  Review marked-up Record Prints with the Contracting Officer.  When authorized, prepare a full set of corrected CAD Drawings of the Contract Drawings, as follows:

1. Format:  Use the same CAD program, version, and operating system as the original Contract Drawings.

Retain subparagraph above or below.  Edit paragraph below for correct electronic format and applicable references.

2. Format:  [DWG] [DXF], Version <Insert designation>, operating in [Windows 3.x] [Windows 95] [Windows 98] [Windows NT] [Macintosh OS7.x] [Macintosh OS8.x] operating system.

3. Incorporate changes and additional information previously marked on Record Prints.  Delete, redraw, and add details and notations where applicable.

4. Refer instances of uncertainty to the Contracting Officer for resolution.

5. NIH will furnish Contractor one set of CAD Drawings of the Contract Drawings for use in recording information.

a. NIH makes no representations as to the accuracy or completeness of CAD Drawings as they relate to the Contract Drawings.

b. CAD Software Program:  The Contract Drawings are available in <Insert name and version of CAD program and operating system>.

Paragraph below describes a logical requirement, but it cannot be effectively enforced unless specifically mentioned in Change Order and unless Contractor has included cost in the proposal.

D. Newly Prepared Record Drawings:  Prepare new drawings instead of following procedures specified for preparing record drawings where new drawings are required when neither the original Contract Drawings nor Shop Drawings are suitable to show the actual installation.  New drawings may be required when a Change Order is issued as a result of accepting an alternate, substitution, or other modification.

1. Provide Drawings in a scale that allows for the scope of detailing and notations required to record the actual physical installation and its relationship to other construction.

2. When completed and accepted, integrate newly prepared Drawings with the previous procedures specified for organizing, copying, binding and submitting record drawings.

Edit the following paragraph as necessary, eliminating the types of Record Documentation not required.  Coordinate with other paragraphs in this section.

E. Format:  Identify and date each Record Drawing; include the designation "PROJECT RECORD DRAWING" in a prominent location.

1. Record Prints:  Organize Record Prints and newly prepared Record Drawings into manageable sets.  Bind each set with durable paper cover sheets.  Include identification on cover sheets.

2. Record Transparencies:  Organize into unbound sets matching Record Prints.  Place transparencies in durable tube-type drawing containers with end caps.  Mark end cap of each container with identification.  If container does not include a complete set, identify Drawings included.

3. Record CAD Drawings:  Organize CAD information into separate electronic files that correspond to each sheet of the Contract Drawings.  Name each file with the sheet identification.  Include identification in each CAD file.

4. Identification:  As follows:

a. Project name.

b. Date.

c. Designation "PROJECT RECORD DRAWINGS."

d. Name of Architect.

e. Name of Contractor.

f. Name of Project Officer.

2.2 RECORD SPECIFICATIONS

Revise the number of copies in the following paragraph as required for the project.

A. During the construction period, maintain [three] copies of the Project Specifications, including addenda and other modifications issued, for Project Record Document purposes.

1. Mark the Specifications to indicate the actual installation where the installation varies from that indicated in Specifications. Note related project record drawing information, where applicable.  Give particular attention to substitutions, selection of product options, and information on concealed installations that would be difficult to identify or measure and record later.

Retain the following sub-subparagraphs or edit or delete to suit project requirements.

a. In each Specification section where products, materials or units of equipment are specified or scheduled, mark the copy with the proprietary name and model number of the product furnished.

b. Record the name of the manufacturer, supplier, installer, and other information necessary to provide a record of selections made and to document coordination with record Product Data submittals and maintenance manuals.

2. Upon completion of markup, submit Record Specifications to the Project Officer for NIH records.

2.3 RECORD PRODUCT DATA

A. During the construction period, maintain [one] copy of each Product Data submittal for Project Record Document purposes.

1. Mark Product Data to indicate the actual product installation where the installation varies substantially from that indicated in Product Data submitted.  Include significant changes in the product delivered to the site and changes in manufacturer's instructions and recommendations for installation.

2. Give particular attention to information about concealed products and installations that cannot be readily identified and recorded later.

3. Note related Change Orders and markup of Record Drawings, where applicable.

4. Upon completion of markup, submit a complete set of record Product Data to the Project Officer.

5. Where record Product Data is required as part of operation and maintenance manuals, submit marked-up Product Data as an insert in the manual instead of submittal as record Product Data.

Retain the following article if project includes samples that may be critical for future work.

2.4 RECORD SAMPLE SUBMITTAL

A. Immediately prior to the date of Substantial Completion, the Contractor shall meet with the Project Officer at the site to determine which of the Samples maintained during the construction period shall be transmitted to NIH for record purposes.

B. Comply with the Project Officer's instructions for packaging, identification marking and delivery to the NIH sample storage space.  Dispose of other samples in a manner specified for disposing of surplus and waste materials.

2.5 MISCELLANEOUS RECORD SUBMITTALS

A. Refer to other specification sections for miscellaneous record-keeping requirements and submittals in connection with various construction activities.  Prior to Final Acceptance, assemble completed miscellaneous records and place in good order, properly identified and bound or otherwise organized to allow for use and reference.

B. Submit to the Project Officer for NIH records.

C. Miscellaneous records include, but are not limited to, the following:

Edit, delete and add to the following subparagraphs to suit the project.

1. Field records on excavations and foundations.

2. Field records on underground construction and similar work.

3. Survey showing locations and elevations of underground lines.

4. Invert elevations of drainage piping.

5. Surveys establishing building lines and levels.

6. Authorized measurements utilizing unit prices or allowances.

7. Records of plant treatment.

8. Ambient and substrate condition tests.

9. Certifications received in lieu of labels on bulk products.

10. Batch mixing and bulk delivery records.

11. Testing and qualification of tradespersons.

12. Documented qualification of installation firms.

13. Load and performance testing.

14. Inspections and certifications by governing authorities.

15. Leakage and water-penetration tests.

16. Fire-resistance and flame-spread test results.

17. Construction photographs and videotapes.

18. Certifications of final disposition/treatment /disposal of wastes.

19. Final inspection and correction procedures.

PART 3 -  EXECUTION

3.1 RECORDING AND MAINTENANCE

A. Recording:  Maintain one copy of each submittal during the construction period for Project Record Document purposes.  Post changes and modifications to Project Record Documents as they occur; do not wait until the end of Project.

B. Maintenance of Record Documents and Samples:  Store Record Documents and Samples in the field office apart from the Contract Documents used for construction.  Do not use Project Record Documents for construction purposes.  Maintain Record Documents in good order and in a clean, dry, legible condition, protected from deterioration and loss.  Provide access to Project Record Documents for the Project Officer's reference during normal working hours.

END OF SECTION 01781

PROJECT RECORD DOCUMENTS
01781 - 9

