
SECTION 01500PRIVATE 




  CONSTRUCTION FACILITIES AND TEMPORARY CONTROLS

Provisions of the contract including General and Supplementary General Conditions and the General Requirements apply to the work of this section.
1 PART 1   GENERAL


.1

SECTION INCLUDES



A.
Temporary Utilities: Electricity, lighting, heat, ventilation, telephone service, water, and sanitary facilities.



B.
Temporary Controls: Barriers, enclosures and fencing, protection of the Work, and water control.



C.
Construction Facilities: progress cleaning, project signage, and temporary buildings.


.2

RELATED SECTIONS

A. Section 01700 ‑ Contract Closeout: Final cleaning.

B.
Reference to General Conditions.


.3

TEMPORARY ELECTRICITY



A.
Contractor may connect of Owner’s power system for normal amounts of construction power.  Temporary connections must be removed prior to substantial completion.



B.
Maintain power to other areas of building and vital equipment during construction.



C.
Provide power outlets for construction operations, with branch wiring and distribution boxes located as required to prevent need for joining extension cord.  Provide flexible power cords as required.



D.
Permanent convenience receptacles may not be utilized during construction.


.4

TEMPORARY LIGHTING



A.
Provide and maintain incandescent lighting for construction operations.



B.
Provide branch wiring from power source to distribution boxes with lighting conductors, pigtails, and lamps as required.



C.
Maintain lighting and provide routine repairs.



D.
Permanent building lighting may not be utilized during construction.


.5

TEMPORARY VENTILATION



A.
Ventilate enclosed areas to assist cure of materials, to dissipate humidity, and to prevent accumulation of dust, fumes, vapors, or gases.


.6

TELEPHONE SERVICE



A.
Provide, maintain and pay for telephone service to field office for use during construction. Provide means to contact engineer or Owner personnel who may be at site, for telephone calls.


.7

TEMPORARY WATER SERVICE



A.
Provide, maintain and pay for suitable quality water service required for construction operations.



B.
Extend branch piping with outlets located so water is available by hoses with threaded connections.  Provide temporary pipe insulation to prevent freezing.



C.
Maintain water supply to building areas not under renovation.


.8

TEMPORARY SANITARY FACILITIES



A.
Provide and maintain required facilities and enclosures. Existing facilities shall not be used.


.9

BARRIERS



A.
Provide barriers to prevent unauthorized entry to construction areas and to protect existing facilities and adjacent properties from damage from construction operations [and demolition.



B.
Provide protection for plant life designated to remain. Replace damaged plant life.



C.
Protect non‑owned vehicular traffic, stored materials, site and structures from damage.



.10
FENCING



A.
Construction: Commercial grade chain link fence.



B.
Provide 6 foot (1.8 m) high fence around construction site; equip with vehicular gates with locks.


.11
WATER CONTROL



A.
Grade site to drain.
Maintain excavations free of water. Provide, operate, and maintain pumping equipment.



B.
Protect site from puddling or running water. Provide water barriers as required to protect site from runoff from other areas.


.12
EXTERIOR ENCLOSURES



A.
Provide temporary  weather‑tight closure of exterior openings to accommodate acceptable working conditions and protection for Products, to allow for temporary heating and maintenance of required ambient temperatures identified in individual specification Sections, and to prevent entry of unauthorized persons. Provide access doors with self‑closing hardware and locks.


.13
INTERIOR ENCLOSURES



A.
Provide temporary partitions as required to separate work areas from Owner occupied areas, to prevent penetration of dust and moisture into Owner occupied areas, and to prevent damage to existing materials and equipment.



B.
Construction: Framing and plywood sheet materials with closed joints and sealed edges at intersections with existing surfaces.


.14
PROTECTION OF INSTALLED WORK



A.
Protect installed Work and provide special protection where specified in individual specification Sections.



B.
Provide temporary and removable protection for installed Products.  Control activity in immediate work area to minimize damage.



C.
Provide protective coverings at walls, projections, jambs, sills, and soffits of openings.



D.
Protect finished floors, stairs, and other surfaces from traffic, dirt, wear, damage, or movement of heavy objects, by protecting with durable sheet materials.



E.
Prohibit traffic or storage upon waterproofed or roofed surfaces.  If traffic or activity is necessary, obtain recommendations for protection from waterproofing or roofing material manufacturer.



F.
Prohibit traffic from landscaped areas.


.15
SECURITY



A.
Provide security and facilities to protect Work, and Owner's operations from unauthorized entry, vandalism, or theft.


.16
PARKING



A.
Provide temporary roadway and temporary contractor parking area on west side of facility. Remove and grade when complete.



.17
PROGRESS CLEANING



A.
Maintain areas free of waste materials, debris, and rubbish.  Maintain site in a clean and orderly condition.



B.
Remove debris and rubbish from pipe chases, plenums, attics, crawl spaces, and other closed or remote spaces, prior to enclosing the space.



C.
Broom and vacuum clean interior areas prior to start of surface finishing, and continue cleaning to eliminate dust.



D.
Remove waste materials, debris, and rubbish from site periodically and dispose off‑site.


.18
PROJECT IDENTIFICATION



A.
Provide 8 w x 6 h foot (2.4 w x 1.8 h m) project sign of exterior grade plywood and wood frame construction, painted, with exhibit lettering by professional sign painter, to Architect/Engineer's design and colors.

B. List title of project, names of Owner, and Police Jurors as provided by Architect, Engineer, and Contractor. 



C.
Erect on site at location established by Architect/Engineer.



D.
No other signs are allowed [without Owner permission] except those required by law.


.19
FIELD OFFICES AND SHEDS



A.
Office: Weather‑tight, with lighting, electrical outlets, heating, cooling and ventilating equipment, and equipped with sturdy furniture drawing rack and drawing display table.



B.
Locate offices and sheds a minimum distance of 30 feet (10  m) from existing and new structures.



.20
REMOVAL OF UTILITIES, FACILITIES, AND CONTROLS



A.
Remove temporary above grade or buried utilities, equipment, facilities, materials, prior to Substantial Completion inspection.



B.
Clean and repair damage caused by installation or use of temporary work.



C.
Restore existing facilities used during construction to original condition.  Restore permanent facilities used during construction to specified condition.


END OF SECTION
PAGE  
Section 01500 Page 1 of 1     

