
SECTION 05805PRIVATE 


EXPANSION JOINT ASSEMBLIES
Provisions of the contract including General and Supplementary General Conditions and the General Requirements apply to the work of this section.
1 PART 1   GENERAL


.1

SECTION INCLUDES



A.
Expansion joint assemblies for floor, wall and ceiling soft-fit surfaces.



B.
Refer to Schedule of items at end of Section.


.2
seq level2 \h \r0 
PRODUCTS FURNISHED BUT NOT INSTALLED UNDER THIS SECTION



A.
Section 03100 ‑ Concrete Formwork: Placement of joint assembly frames in formwork.



B.
Section 04300 ‑ Unit Masonry System: Placement of joint assembly frames in masonry.


.3

seq level2 \h \r0 

REFERENCES
seq level2 \h \r0 



A.
ANSI/ASTM B221 ‑ Aluminum‑Alloy, Extruded Bar, Rod, Wire, Shape, and Tube.



B.
ANSI/ASTM B308 ‑ Aluminum‑Alloy, Standard Structural Shapes, Rolled or Extruded.



C.
ANSI/ASTM B445 ‑ Copper‑Zinc‑Lead Alloy (Leaded‑Brass) Extruded Shapes.


D.
Reference to General Conditions.

.4
seq level2 \h \r0 


SUBMITTALS



A.
Submit under provisions of Section 01300.



B.
Product Data: Provide joint assembly profiles, dimensions, locations in the Work, affected adjacent construction, anchorage devices, [available colors and finish], and locations of splices.



C.
Manufacturer's Installation Instructions: Indicate rough‑in sizes.  Provide templates for cast‑in or placed frames or anchors, and indicate tolerances for item placement.



.5

FIELD MEASUREMENTS



D.
Verify that field measurements are as instructed by the manufacturer.


2

seq level1 \h \r0 

seq level2 \h \r0  PART 2   PRODUCTS


.1

MATERIALS



A.
Extruded Aluminum: ANSI/ASTM 6063-T5 alloy.



B.
Resilient Filler: Flexible PVC.



C.
Threaded Fasteners: Manufacturer’s standard.



D
.
Backing Paint:
Asphaltic type.

.2

seq level2 \h \r0 

FABRICATION



A.
Joint Covers:  Aluminum cover plate, aluminum frame construction, retainers with resilient vinyl filler strip, designed to permit plus or minus 50  percent joint movement with full recovery, flush and recess mounted; refer to Schedule.



B.
Back paint components in contact with cementitious materials.



C.
Galvanize embedded ferrous metal anchors and fastening devices.



D.
Shop assemble components and package with anchors and fittings.



E.
Provide joint components in single length wherever practical.  Minimize site splicing.


.3

seq level2 \h \r0 

FINISHES



A.
Floors:  Mill finish.



B.
Walls and Ceilings:  Clear anodized.



C.
Resilient Filler Exposed to View:  Gray.

3

seq level1 \h \r0 

seq level2 \h \r0  PART 3   EXECUTION


.1

EXAMINATION



A.
Verify existing conditions under provisions of Section 01039.



B.
Verify that joint preparation and affected dimensions are acceptable.


.2

seq level2 \h \r0 

PREPARATION



A.
Provide anchoring devices for installation and embedment.



B.
Provide templates and rough‑in measurements.


.3

seq level2 \h \r0 

INSTALLATION



A.
Install components and accessories in accordance with manufacturer's instructions.



B.
Align work plumb and level.



C.
Rigidly anchor components to substrate to prevent misalignment.


.4

seq level2 \h \r0 

PROTECTION OF FINISHED WORK



A.
Protect finished work under provisions of Section 01500. 



B.
Do not permit traffic over unprotected floor joint surfaces.



C.
Provide removable strippable coating reinforced cloth 




tape to protect finish surface.


END OF SECTION
PAGE  
Section 05805 Page 1 of 3

