
SECTION 06410PRIVATE 


CUSTOM CASEWORK

Provisions of the contract including General and Supplementary General Conditions and the General Requirements apply to the work of this section.
1 PART 1  GENERAL


.1

SECTION INCLUDES



A.
Special fabricated cabinet units.



B.
Cabinet hardware.



C.
Preparation for site finishing



D.
Preparation for installing utilities.


.2 
RELATED SECTIONS



A.
Section 06114 ‑ Rough Carpentry:
Grounds and support framing.



B.
Section 06200 ‑ Finish Carpentry: Related trim not specified in this section.



C.
Section 06200 – Finish Carpentry: Cast plastic countertops.



D.
Section 09900 ‑ Painting: Finishing cabinet exterior and interior.



E.
Section 1544 - Plumbing fixtures.



F.
Section 12301 ‑ Metal Casework.

G. Section 12390 ‑ Residential Casework.

H.
Reference to General Conditions.


.3

REFERENCES



A.
ANSI/BHMA A156.9 ‑ Cabinet Hardware.



B.
AWI ‑ Quality Standards.



C.
FS MM‑L‑736 ‑ Lumber, Hardwood.



D.
FS MMM‑A‑130 ‑ Adhesive, Contact.



E.
National Electric Manufacturers Association (NEMA) LD3 -




High Pressure Decorative Laminates.



F.
PS 1 ‑ Construction and Industrial Plywood.



G.
PS 20 ‑ American Softwood Lumber Standard.


.4

SUBMITTALS



A.
Submit under provisions of Section 01300.



B.
Shop Drawings: Indicate materials, component profiles and elevations, assembly methods, joint details, fastening methods, accessory listings, hardware location, and schedule of finishes.


.5

QUALITY ASSURANCE



A.
Perform work in accordance with AWI Custom quality.


.6

QUALIFICATIONS



A.
Manufacturer:  Company specializing in manufacturing the products specified in this section with minimum three years experience.


.7

MOCKUP



A.
Provide mockup of full size base cabinet and upper cabinet under provisions of Section 01400.



B.
Provide units with specified countertop; with hardware installed.



C.
Units will be examined to ascertain quality and conformity to AWI quality level standards and specification requirements.



D.
Mockup may not remain as part of the Work.


.8

DELIVERY, STORAGE, AND HANDLING



A.
Deliver, store and handle products to site under provisions of Section 01600.



B.
Protect units from moisture damage.


.9

FIELD MEASUREMENTS



A.
Verify that field measurements are as indicated on shop drawings.


.10
COORDINATION



A.
Coordinate work under provisions of Section 01039. 



B.
Coordinate the work with plumbing and electrical rough‑in.

2 PART 2 PRODUCTS 


.1

WOOD MATERIALS



A.
Softwood Lumber:
PS 20; graded in accordance with AWI Custom Economy; average moisture content of 6 percent



.2

SHEET MATERIALS



A.
Softwood Plywood:  PS 1; graded in accordance with AWI, core materials of lumber; species and cut as follows:




ITEM




   FACE SPECIES 
   CUT




Door and Drawer Fronts
Birch


Premium Rotary

a. LAMINATE MATERIALS

B. Plastic Laminate:  AWI, 0.050 inch General Purpose Commercial quality; color, pattern, and surface texture as selected.



C.
Laminate Backing Sheet:  0.020 inch (o.5 mm) backing sheet grade, undecorated plastic laminate.


 .5

ACCESSORIES



A.
Adhesive:  FS MMM‑A‑130 contact adhesive Type recommended by AWI to suit application.



B.
Fasteners:
Size and type to suit application.



C.
Bolts, Nuts, Washers, Lags, Pins, and Screws:  Of size and type to suit application; unfinished steel finish in concealed locations and unfinished steel finish in exposed locations.



E.
Concealed Joint Fasteners:
Threaded steel.


.6

HARDWARE

A. Shelf Standards and Rests: Heavy duty or type indicated, K-V, National or Grant.


B.
Drawer and Door Pulls: Engineered Products, MC-42, Stanley 4484, Baldwin 298 or 265.



C.
Cabinet Locks:
Pin Tumbler dead lock, Yale 5591 K-V 986/987, Russwin D696R, Sargent 1654.



D.
Catches:  K-V 916, Stanley 46 ALD, Hager 1437 or 1438, Engineered Products 590 and 591.



E.
Drawer Slides: K-V 1100 or Grant 328, Full extension, 50 lb. Capacity. 



F.
Hinges: Pivot type National Mfg. V465 and 466, Stanley 331 and 332, Hager 1043 and 1044.



G.
Coat Hooks – Raymond Engineer, Inc., No. 923 Alum, Clear stain anodized.



.7

FINISHING MATERIALS



A.
Stain, Varnish and Finishing Materials:  As specified in Section 09900.


.8

FABRICATION



A.
Shop assemble casework for delivery to site in units easily handled and to permit passage through building openings.



B.
Fit shelves, doors, and exposed edges with 3/8 inch (9.5 mm) matching hardwood edging.  Use one piece for full length only.



C.
Cap exposed plastic laminate finish edges with material of same finish and pattern.



D.
Door and Drawer Fronts: 3/4 inch (19 mm) thick; flush style.



E.
When necessary to cut and fit on site, provide materials with ample allowance for cutting.  Provide trim for scribing and site cutting.


.9

FINISHING



A.
Sand work smooth and set exposed nails and screws.



B.
Apply wood filler in exposed nail and screw indentations.



C.
On items to receive transparent finishes, use wood filler which matches surrounding surfaces and of types recommended for applied finishes.



D.
Seal, stain and varnish exposed to view surfaces.  Brush apply only.



E.
Seal stain and varnish internal exposed to view and semi‑concealed surfaces.  Brush apply only.



F.
Seal surfaces in contact with cementitious materials.

3 PART 3 EXECUTION


.1

EXAMINATION



A.
Verify adequacy of backing and support framing.


.2

INSTALLATION



A.
Set and secure casework in place; rigid, plumb, and level.



B.
Use fixture attachments in concealed locations for wall mounted components.



C.
Use concealed joint fasteners to align and secure adjoining cabinet units counter tops.



D.
Carefully scribe casework abutting other components, with maximum gaps of 1/32 inch (1 mm).  Do not use additional overlay trim for this purpose.



E.
Secure cabinet and counter bases to floor using appropriate angles and anchorages.



F.
Countersink anchorage devices at exposed locations.  Conceal with solid wood plugs of species to match surrounding wood; finish flush with surrounding surfaces.


.3

ADJUSTING



A.
Adjust work under provisions of Section 01700.



B.
Adjust moving or operating parts to function smoothly and correctly.


.4

CLEANING



A.
Clean work under provisions of 01700.



B.
Clean casework, counters, shelves, hardware, fittings and fixtures.


END OF SECTION
PAGE  
Section 06410 Page 5 of 5
Error! Main Document Only.

