
SECTION 07270PRIVATE 


FIRESTOPPING

1 PART 1   GENERAL


.1
SECTION INCLUDES



A.
Fireproof firestopping [and firesafing] materials[.] [and accessories.]


.2

seq level2 \h \r0 
RELATED SECTIONS



A.
Section [_____‑__________]:  Building substrate components.



B.
Section [______‑____________]:  Cutting and patching of [________________.]



C.
Section [07253 ‑ Cellulose Fireproofing] [07255 ‑ Cementitious Fireproofing]:   Spray Applied Fireproofing.



D.
Section [______‑____________]:
Plaster fireproofing.



E.
Section 09260 ‑ Gypsum Board Systems:  Gypsum wallboard fireproofing.



F.
Section [______‑____________]:  Mechanical work requiring firestopping.



G.
Section [______‑____________]:  Electrical work requiring firestopping.


.3

seq level2 \h \r0 
REFERENCES



A.
ASTM E84 ‑ Test Method for Surface Burning Characteristics of Building Materials.



B.
ASTM E119 ‑ Method for Fire Tests of Building Construction and Materials.



C.
ASTM E814 ‑ Test Method of Fire Tests of Through‑ Penetration Firestops.


.4

seq level2 \h \r0 
PERFORMANCE REQUIREMENTS



A.
Fireproofing Materials:  [ASTM E119] [ASTM E814] [________] to achieve a fire rating as noted on Drawings.


****** [OR] ******



A.
Fireproofing Materials:  [ASTM E119] [ASTM E814] [________] to achieve a fire rating [of [____] hour [floor edge] construction.] [noted in Schedule at end of this Section.] [in accordance with UL Design Number [_______].]



B.
Surface Burning:
[ASTM E84] [________] with a flame spread / fuel contributed / smoke developed rating of [___/___/___].


.5

seq level2 \h \r0 
SUBMITTALS



A.
Submit under provisions of Section [01300.] [________.]



B.
Product Data:  Provide data on product characteristics, performance and limitation criteria[,] [and][.] [________________.]



C.
Manufacturer's Installation Instructions:  Indicate preparation and installation instructions.



D.
Manufacturer's Certificate:  Certify that [products] [________] meet or exceed [specified requirements.] [___________________.]


.6

seq level2 \h \r0 
QUALIFICATIONS



A.
Manufacturer: Company specializing in manufacturing the products specified in this Section with minimum [three] [_______] years [documented] experience.



B.
Applicator: Company specializing in performing the work of this Section [with minimum [__________] years [documented] experience.] [approved by manufacturer.]


.7

seq level2 \h \r0 
REGULATORY REQUIREMENTS



A.
Conform to [applicable] [___________] code for fire resistance ratings and surface burning characteristics.



B.
Provide certificate of compliance from [authority having jurisdiction [________________] indicating approval of [combustibility.] [_______________.]


.8

seq level2 \h \r0 
MOCK‑UP



A.
Provide mock‑up of applied firestopping material under provisions of Section [01400.] [________.]



B.
Apply [1] [____] [sq] [lineal] ft ([1/3] [____] [sq] [lineal] m) to a representative substrate surface.



C.
If accepted, mock‑up will demonstrate minimum standard for the Work.



D.
Mock‑up may [not] remain as part of the Work.


.9

seq level2 \h \r0 
ENVIRONMENTAL REQUIREMENTS



A.
Do not apply materials when temperature of substrate material and ambient air is below [60] [____] degrees F ([15] [____] degrees C).



B.
Maintain this minimum temperature before, during, and for [3] [____] days after installation of materials.



C.
[Provide ventilation in areas to receive solvent cured materials.]


.10

seq level2 \h \r0 
SEQUENCING



A.
Sequence Work under the provisions of Section [01010.] [________.]



B.
Sequence Work to permit firestopping materials to be installed after adjacent and surrounding work is complete.

2

seq level1 \h \r0 

seq level2 \h \r0  PART 2   PRODUCTS


.1
MANUFACTURERS



A.
[__________________________________] Product [_________.]



B.
[__________________________________] Product [_________.]



C.
[__________________________________] Product [_________.]



D.
Substitutions: Under provisions of Section [01600.] [________.]


.2

seq level2 \h \r0 
MATERIALS



A.
Firestopping Material:  [[Single] [multiple] component [silicone elastomeric] [foam] compound] [Formulated compound mixed with incombustible non‑asbestos fibers] [Mineral fiber stuffing insulation] [Mechanical device with incombustible fillers]; conforming to the following:




1.
[Elongation] [Shrinkage]:  [____] percent.




2.
Adhesion and Bond To Substrate:  [____] psi ([____] kPa).




3.
Compressive Strength:  [____] psi ([____] kPa).




4.
Density:  [________] lb/cu ft ([________] kg/cu m).




5.
Surface Durability:  [________________.]




6.
Vapor and Air Permeability:  [____] perms ([____] ng/(Pa.s.sq m].




7.
Durability and Longevity:  [Permanent.] [________.]




8.
Side Effects During Installation:  [Non‑toxic.] [________.]




9.
Long Term Side Effects:  [None.] [________.]



B.seq level3 \h \r0 
Primer:  Type recommended by firestopping manufacturer for specific substrate surfaces.


.3

seq level2 \h \r0 
ACCESSORIES



A.
Dam Material:  [Mineral fiberboard] [Mineral fiber matting] [Sheet metal] [Plywood or particle board] [Alumina silicate fire board] [________________], [permanent.] [removable.]



B.
Retainers:  [________] clips to support [mineral fiber matting.] [________________.]


.4

seq level2 \h \r0 
FINISHES



A.
Color:
[Dark gray.] [Black.] [________.]

3

seq level1 \h \r0 

seq level2 \h \r0  PART 3   EXECUTION


.1
EXAMINATION



A.
Verify site conditions under provisions of Section [01039.] [________.]



B.
Verify that openings are ready to receive the Work of this Section.


.2

seq level2 \h \r0 
PREPARATION



A.
Clean substrate surfaces of dirt, dust, grease, oil, loose material, or other matter which may affect bond of firestopping material.



B.
Remove incompatible materials which affect bond.



C.
[Install backing materials to arrest liquid material leakage.]


.3

seq level2 \h \r0 
APPLICATION



A.
Apply [primer and] materials in accordance with manufacturer's instructions.



B.
Apply firestopping material [in sufficient thickness to achieve rating] [to a thickness of [____] inches ([____] mm)] to uniform density and texture.



C.
Install material at walls or partition openings which contain penetrating sleeves, piping, ductwork, conduit and other items requiring firestopping.



D.
[Remove dam material after firestopping material has cured.]  [Dam material to remain.]


.4

seq level2 \h \r0 
CLEANING



A.
Clean Work under provisions of Section [01700.] [_____.]



B.
Clean adjacent surfaces of firestopping materials.


.5

seq level2 \h \r0 
PROTECTION OF FINISHED WORK



A.
Protect finished Work under provisions of Section [01500.] [_____.]



B.
Protect adjacent surfaces from damage by material installation.


.6

seq level2 \h \r0 
SCHEDULES




LOCATION








FIRE RATING



A.
Main floor fire walls





 1 hour



B.
Stair walls 







 2 hours



C.
Room to room partitions 




 3/4 hour


END OF SECTION

PAGE  
Section 07270.doc Page 1 of 1

