
SECTION 07465PRIVATE 


PREFORMED METAL SIDING

1 PART 1 GENERAL


.1

SECTION INCLUDES



A.
Preformed metal siding system for walls, with related flashings and accessory components.


.2
seq level2 \h \r0 
RELATED SECTIONS



A.
Section 05120: Structural Steel: Structural steel building frame.



B.
Section 05400 ‑ Cold Formed Metal Framing: Stud wall framing system.



C.
Section 07213 ‑ Batt and Blanket Insulation.



D.
Section 07270 – Firestopping.



E.
Section 07421 ‑ Composite Metal Building Panels.



F.
Section 07620 ‑ Sheet Metal Flashing and Trim.



.3

seq level2 \h \r0 

REFERENCES



A.
ANSI/ASTM A153 ‑ Zinc Coating (Hot‑Dip) on Iron and Steel Hardware.



B.
ASTM A606 ‑ Steel Sheet and Strip, Hot‑Rolled and Cold‑Rolled, High‑Strength, Low‑Alloy, with Improved Corrosion Resistance.



C.
ASTM A792 ‑ Steel Sheet, Aluminum‑Zinc Alloy Coated by the Hot‑Dip Process, Structural (Physical) Quality.


.4

seq level2 \h \r0 

SYSTEM DESCRIPTION



A.
System:  Preformed and pre‑finished metal siding system of vertical profile; site assembled.


.5
seq level2 \h \r0 
DESIGN REQUIREMENTS



A.
Design and size components to withstand dead and live loads caused by positive and negative wind pressure acting normal to plane of wall as calculated in accordance with SBCC code.



B.
Maximum Allowable Deflection of Panel: 1/180.



C.
System to accommodate, without damage to components or deterioration of seals, movement within system; movement between system and perimeter components when subject to seasonal temperature cycling; dynamic loading and release of loads; deflection of structural support framing.



D.
Accommodate positive drainage for moisture entering or condensation occurring within panel system, to exterior.



E.
Products of this section shall provide continuity of thermal barrier at building enclosure elements in conjunction with thermal insulating materials in Section 07213.


.6

seq level2 \h \r0 

SUBMITTALS



A.
Submit under provisions of Section 01300.



B.
Shop Drawings: Indicate dimensions, layout, joints, construction details, methods of anchorage.



C.
Samples:
Submit two samples of siding 12 x 12 inch in size illustrating finish color, sheen and texture.


.7

seq level2 \h \r0 

DELIVERY, STORAGE, AND HANDLING seq level2 \h \r0 




A.
Deliver, store, protect and handle products to site under provisions of Section 01600. 



B.
Protect panels from accelerated weathering by removing or venting sheet plastic shipping wrap.



C.
Stack pre‑finished material to prevent twisting, bending, or abrasion, and to provide ventilation.  Slope metal sheets to ensure drainage.



D.
Prevent contact with materials which may cause discoloration or staining.


.8seq level2 \h \r0 

FIELD MEASUREMENTS



A.
Verify that field measurements are as indicated on shop 

seq level0 \h \r1 

seq level1 \h \r11 

seq level2 \h \r0 

seq level3 \h \r0 

seq level4 \h \r0 

seq level5 \h \r0 

seq level6 \h \r0 

seq level7 \h \r0 

.9seq level2 \h \r0 

COORDINATION



A.
Coordinate work under provisions of Section 01039.


.10seq level2 \h \r0 
WARRANTY



A.
Provide twenty-year warranty under provisions of Section 01700.



B.
Warranty:  Include coverage for degradation of panel finish including color fading caused by exposure to weather.

2

seq level1 \h \r0 

seq level2 \h \r0  PART 2 PRODUCTS

 .1

MANUFACTURERS




Not used


.2seq level2 \h \r0 

EXTERIOR SHEET MATERIALS



A.
Pre‑coated Galvanized Steel:
ANSI/ASTM A446, G90 zinc coating; shop pre‑coated with seq level0 \h \r2 

seq level1 \h \r2 

seq level2 \h \r0 

seq level3 \h \r0 

seq level4 \h \r0 

seq level5 \h \r0 

seq level6 \h \r0 

seq level7 \h \r0 

seq level0 \h \r2 

seq level1 \h \r2 

seq level2 \h \r0 

seq level3 \h \r0 

seq level4 \h \r0 

seq level5 \h \r0 

seq level6 \h \r0 

seq level7 \h \r0 Kynar coating.


.3
seq level2 \h \r0 
OTHER SHEET MATERIALS



A.
Galvanized Steel: ASTM A446, Grade G90 zinc coating.



seq level0 \h \r2 

seq level1 \h \r3 

seq level2 \h \r0 

seq level3 \h \r0 

seq level4 \h \r0 

seq level5 \h \r0 

seq level6 \h \r0 

seq level7 \h \r0 

.4

seq level2 \h \r0 

ACCESSORIES



A.
Insulation:  Glass fiber type specified in Section 07213; two inch thick, thermal resistance R of 6.



B.
Sealants:  Manufacturer's standard type suitable for use with installation of system; non‑staining.


C.
Fasteners:  Manufacturer's standard type to suit application; with soft neoprene washers, steel, hot dip galvanized in accordance with ANSI/ASTM A153; fastener cap same color as exterior panel.


D.
Field Touch‑up Paint: As recommended by panel manufacturer.

.5
seq level2 \h \r0 
COMPONENTS



A.
Exterior Sheet: Minimum 24 gage thick pre‑coated steel stock; profile as indicated.



B.
Internal and External Corners: Same material, thickness and finish as exterior sheets; profile to suit system; shop cut and factory-mitered brake formed to required angles.



C.
Trim, Closure Pieces, Caps, Flashings: Same material, thickness and finish as exterior sheets; brake formed to required profiles.



D.
Anchors:
Galvanized steel.


.6

seq level2 \h \r0 

FABRICATION



A.
Form sections true to shape, accurate in size, square, and free from distortion or defects.



B.
Form pieces in longest practical lengths.



C.
Form panels for standing batten seams.


.7

seq level2 \h \r0 

FINISH



A.
Exposed Exterior Surfaces: color as selected [from manufacturer's standard range.

3

seq level1 \h \r0 

seq level2 \h \r0  PART 3 EXECUTION


.1

EXAMINATION



A.
Verify substrate framing under provisions of Section 01039.



B.
Verify that building framing members are ready to receive panel system.


.2

seq level2 \h \r0 

INSTALLATION 



A.
Install metal siding system on walls and soffits in accordance with manufacturer's instructions.



B.
Protect surfaces in contact with cementitious materials and dissimilar metals with bituminous paint.  Allow to dry prior to installation.



C.
Fasten siding to structural supports; aligned, level, and plumb.



D.
Locate joints over supports.




E.
Seal and place gaskets to prevent weather penetration.  Maintain neat appearance.


.3seq level2 \h \r0 

TOLERANCES



A.
Maximum Offset from True Alignment Between Adjacent Members Butting or In Line: 1/16 inch (1.6 mm).



B.
Maximum Variation from Plane or Location Indicated on Drawings: 1/8 inch.


.3

seq level2 \h \r0 

CLEANING



A.
Clean work under provisions of 01700.



B.
Remove site cuttings from finish surfaces.



C.
Clean and wash pre‑finished surfaces with mild soap and water, rinse with clean water.


END OF SECTION
PAGE  
Section  07465 Page 4 of 4

