

SECTION 08112PRIVATE 


STANDARD STEEL FRAMES

Provisions of the contract including General and Supplementary General Conditions and the General Requirements apply to the work of this section.
1 PART 1 GENERAL


.1

SECTION INCLUDES



A.
Steel frames.



B.
Exterior glazed light frames.


.2

PRODUCTS FURNISHED BUT NOT INSTALLED UNDER THIS SECTION



A.
Section 04300 – Unit Masonry: Placement of anchors into wall construction.


.3

RELATED SECTIONS



A.
Section 04100 – Masonry Grout: Masonry mortar fill of metal frames.



B.
Section 08111 ‑ Standard Steel Doors.



C.
Section 08211 ‑ Wood Doors.



D.
Section 08712 ‑ Door Hardware.



E.
Section 08800 ‑ Glazing.



F.
Section 09900 ‑ Painting: Field painting of frames.


.4

REFERENCES



A.
ANSI A117.1 ‑ Specifications for Making Buildings and Facilities Accessible to and Usable by Physically Handicapped People.



B.
ANSI/SDI‑100 ‑ Standard Steel Doors and Frames.



C.
ASTM A525 ‑ Steel Sheet, Zinc‑Coated (Galvanized) by the Hot‑Dip Process.



D.
ASTM E152 ‑ Methods of Fire Tests of Door Assemblies.



E.
DHI ‑ Door Hardware Institute: The Installation of Commercial Steel Doors and Steel Frames, Insulated Steel Doors in Wood Frames and Builder's Hardware.



F.
NFPA 80 ‑ Fire Doors and Windows.



G.
NFPA 252 ‑ Fire Tests for Door Assemblies.

H. UL 10B ‑ Fire Tests of Door Assemblies.

I.
Reference to General Conditions.


.5

SUBMITTALS



A.
Submit under provisions of Section 01300.



B.
Shop Drawings: Indicate frame elevations, reinforcement, and finish.



C.
Product Data:  Indicate frame configuration, anchor types and spacing, location of cut‑outs for hardware, reinforcement.



D.
Manufacturer's Installation Instructions: Indicate special installation instructions.


.6

QUALITY ASSURANCE



A.
Conform to requirements of ANSI/SDI‑100 and ANSI A117.1.


.7

REGULATORY REQUIREMENTS



A.
Fire Rated Frame Construction: Conform to NFPA 252 UL 10B.



B.
Installed Frame Assembly: Conform to NFPA 80 for fire rated class same as fire door.


.8

DELIVERY, STORAGE, AND HANDLING

A. Deliver, store, protect, and handle products to site under provisions of Section 01600.



B.
Accept frames on site in manufacturer's packaging.  Inspect for damage.


.9

FIELD MEASUREMENTS



A.
Verify that field measurements are as indicated on shop drawings.


.10
COORDINATION



A.
Coordinate work under provisions of Section 01039.



B.
Coordinate the work with frame opening construction, door and hardware installation.

2 PART 2 PRODUCTS


.1

FRAME MANUFACTURERS



A.
Ceco.



B.
Republic.



C.
Substitutions: Under provisions of Section 01600. 


.2

FRAMES




A.
Exterior Frames: 16 gage thick material, base metal thickness.



B.
Interior Frames: 16 gage thick material, base metal thickness.


.3

ACCESSORIES



A.
Silencers:  Resilient rubber stick‑on type.



B.
Primer:  Zinc chromate type.


.4

FABRICATION



A.
Fabricate frames as welded unit.



B.
Transom Bars for Glazed Lights: Fixed type, of same profiles as jamb and head.



C.
Fabricate frames with hardware reinforcement plates welded in place.  Provide mortar guard boxes.



D.
Reinforce frames wider than 48 inches with roll formed steel channels fitted tightly into frame head, flush with top.



E.
Terminate door stops 6 inches (150 mm) above finished floor.  Cut stop at 45-degree angle and close.



F.
Provide three single silencers for single doors on strike side.  Provide two single silencers on frame head at double doors without mullions.



G.
Fabricate frames to suit masonry wall coursing with 4-inch head member.


.5

FINISH



A.
Steel Sheet: Galvanized to ASTM A525 G60.



B.
Primer:  Air-dried.



C.  Coat inside of frame profile with bituminous coating to a thickness of 1/16 inch (1.5 mm).

3 PART 3 EXECUTION


.1

EXAMINATION



A.
Verify substrate conditions under provisions of Section 01039.



B.
Verify that opening sizes and tolerances are acceptable.


.2

INSTALLATION



A.
Install frames in accordance with ANSI/SDI‑100 and DHI.



B.
Coordinate with masonry wall construction for anchor placement.



C.
Coordinate installation of glass and glazing.



D.
Coordinate installation of frames with installation of hardware specified in Section 08710 and doors in Section 08111.



E.
Install roll formed steel reinforcement channels between two abutting frames.  Anchor to structure and floor.


.3

ERECTION TOLERANCES



A.
Maximum Diagonal Distortion: 1/16 inch measured with straight edges, crossed corner to corner.


END OF SECTION
1
SError! Main Document Only.
Section 08112 Page 2 of 4

