

SECTION 09688PRIVATE 


CARPET ‑ GLUE DOWN

Provisions of the contract including General and Supplementary General Conditions and the General Requirements apply to the work of this section.

PART 1
GENERAL

.1

SECTION INCLUDES


A.
Carpet placed with glue down method.


B.
Accessories.


.2 
RELATED SECTIONS


A.
Section 03300 – Cast-in-Place concrete: floor substrate surface.

.3

REFERENCES


A. 
ASTM E648 ‑ Critical Radiant Flux of Floor Covering Systems using a Radiant Heat Energy Source.



B.
ASTM E64 – Surface Burning Characteristics of Building materials.

C. HUD Building Product certification program 24 CFR 200.935  (9/20/1979) except a modified by material bulletin (UM).


D.
Reference to General Conditions. 

.4

SUBMITTALS


  A.
Submit product date under provisions of Section 01300.


B.
Provided product data on specified products, describing physical and performance characteristics; sizes, patterns, colors available, and method of installation.


C.
Submit samples under provisions of Section 01300.

D. Submit two samples in size illustrating color and pattern for each carpet material specified.

E. Submit manufacturer’s installation instructions under provisions of section 01300.

F. Submit certification that carpet conforms to HUD-UM44D

.5

OPERATIONS AND MAINTENANCE DATA


A.
Submit operations and maintenance data under provisions of Section 01700.

B. Include maintenance procedures, recommended maintenance materials, and suggested schedule for cleaning and shampooing.

.6

REGULATORY REQUIREMENTS


A.
Conform to applicable State code for carpet flammability requirements of in accordance with ASTM E84.

.7

ENVIRONMENTAL REQUIREMENTS

A. Store materials for three days prior to installation in area of installation to achieve temperature stability


B.
Maintain minimum 70 degrees F (21 degrees C0 amibent temperature three days prior to, during, and 24 hours after installations of materials.

.8

EXTRA MATERIAL

A. Provide scraps of less than nine feet, larger than 3 S.F. 

    and wider than 8” of carpeting of each color specified,               under provisions of Section 01700.  Deliver to Owner’s storage        space as directed.

PART 2

PRODUCTS

.1

MANUFACTURERS


A.
Armstrong.


B.
Bigelow Sanford


C.
Keystone.


D.
Mohawk Bridgeport II.

.2

MATERIALS 


A.
Carpet:  Conforming to the following criteria:



Pile fiber:
100% solution dyed, continuos filament with build-in anti-microbial protection and static protection.




Static Control Fiber:  







Max. Electrostatic Charge: 3 Kv @ 20 % R.H.














Pile Weight: 26 oz/sq. yd.




 



Density Factor: 169,100 Kilotex






Stitch Count
: 10 stitches per inch






Primary Backing: Polypropylene





Secondary Backing:  Uniloc




Total Weight: 67.5 oz/sq. yd







Wear Warranty: 10 year

.3

ACCESSORIES


A.
Sub‑Floor Filler: White premix latex; type recommended by carpet manufacturer.

B. Primers and Adhesives: Waterproof; of types recommended by carpet manufacturer.


C.
Edge Strips: Non-metallic type, black color..

PART 3

EXECUTION

.1

EXAMINATION


A.
Verify that surfaces are smooth and flat with maximum variation of 1/8 inch in 10 ft (3] mm in 3 m), and are ready to receive work.


B.
Verify concrete floors are dry to maximum moisture content of 7 percent; and exhibit negative alkalinity, carbonization, or dusting.

.2

PREPARATION


A.
Remove sub‑floor ridges and bumps.  Fill minor or local low spots, cracks, joints, holes, and other defects with sub‑floor filler.


B.
Apply, trowel, and float filler to achieve smooth, flat, hard surface.



C.
Prohibit traffic until filler is cured.


D. Vacuum clean substrate.

.3

INSTALLATION


A.
Apply carpet and adhesive in accordance with manufacturers' instructions.


B.
Verify carpet match before cutting to ensure minimal variation between dye lots.


C.
Double cut carpet, to allow intended seam and pattern match. Make cuts straight, true, and unfrayed.  


D.
Locate seams in area of least traffic.


E.
Fit seams straight, not crowded or peaked, free of gaps.


F.
Lay carpet on floors with run pile in same direction as anticipated traffic.


G.
Do not change run of pile in any room where carpet is continuous through a wall opening into another room.  Locate change of color or pattern between rooms under door centerline.


H.
Cut and fit carpet around interruptions.


I.
Fit carpet tight to intersection with vertical surfaces without gaps.

.4
CLEANING


A.
Remove excess adhesive without damage, from floor, base, and wall surfaces.


B.
Clean and vacuum carpet surfaces.

.5

PROTECTION


A.
Prohibit traffic from carpet areas for 24 hours after        installation.


END OF SECTION
1
Error! Main Document Only.
Section 09688 Page 1 of 4      

