

SECTION 10160PRIVATE 


METAL TOILET COMPARTMENTS

PART 1
GENERAL

.1
SECTION INCLUDES


A.Metal toilet compartments, [floor mounted.] [head rail braced.] [ceiling hung.] [wall hung.]


B.[Urinal] [Vestibule] screens; wall mounted with floor to ceiling [pilaster] [tube] brace.


C.[Shower cubicles.]

.2

seq level2 \h \r0 
PRODUCTS INSTALLED BUT NOT FURNISHED UNDER THIS SECTION


A.Section [______‑____________]:  Placement of concealed supports.

.3

seq level2 \h \r0 
  RELATED SECTIONS


A.Section [______‑____________]:  [Framing and plates within walls] [Framing above ceiling] for partition panel support.


B.Section 10800 ‑ Toilet and Bath Accessories.


C.[Section [______‑____________]:  Shower cubicle construction.]

.4

seq level2 \h \r0 
REFERENCES


A.ANSI A117.1 ‑ Safety Standards for the Handicapped.


B.ASTM A167 ‑ Stainless and Heat Resisting Chromium‑Nickel Steel Plate, Sheet, and Strip.


C.ASTM A424 ‑ Steel Sheet for Porcelain Enameling.


D.ASTM A526 ‑ Steel Sheet, Zinc‑Coated (Galvanized) by the Hot‑Dip Process, Commercial Quality.


E.FS RR‑P‑1352 ‑ Partitions, Toilet, Complete.

.5

seq level2 \h \r0 
SUBMITTALS


A.Submit under provisions of Section [01300.] [________.]


B.Shop Drawings:  Indicate partition plan, elevation views, dimensions, details of [wall,] [floor,] [and] [ceiling] supports, door swings.


C.Product Data:  Provide data on panel construction, hardware, and accessories.


D.Samples:  Submit [two] [_______] samples of partition panels, [____x____] inch ([____x____] mm) in size illustrating panel finish, color, and sheen.


E.Manufacturer's Installation Instructions:  Indicate special procedures, perimeter conditions requiring special attention, and [________].

.6

seq level2 \h \r0 
REGULATORY REQUIREMENTS


A.Conform to [ANSI A117.1] [___________] code for access for the handicapped.

.7

seq level2 \h \r0 
FIELD MEASUREMENTS


A.Verify that field measurements are as [indicated [on shop drawings].] [instructed by the manufacturer.]

.8

seq level2 \h \r0 
COORDINATION


A.Coordinate work under provisions of Section [01039.] [________.]


B.
Coordinate the work with placement of support framing and anchors in [wall] [and] [ceiling].

PART 2

seq level1 \h \r0 

seq level2 \h \r0 
PRODUCTS

.1
ACCEPTABLE MANUFACTURERS


A.[__________________________________] Product [_________].


B.[__________________________________] Product [_________].


C.[__________________________________] Product [_________].


D.Substitutions: Under provisions of Section [01600.] [________.]

.2

seq level2 \h \r0 
MATERIALS


A.Steel Sheet:  [ASTM  A526, with [G90] [____] zinc coating.] [ASTM A424, Type I, commercial quality.] [ASTM A167, Type 304 stainless steel.]

.3

seq level2 \h \r0 
ACCESSORIES


A.Pilaster Shoe:  Formed [chromed steel with [polished] [satin] finish,] [ASTM A167 type 304 stainless steel with [No. 4] [____] finish,] [3] [____] inch ([175] [____] mm) high, with adjustable screw jack.


B.Head Rails:  Hollow [steel] [stainless steel] [aluminum] tube, [1 x 1‑5/8] [____] inch ([25 x 41] [____] mm) size, with [anti‑grip strips and] cast socket wall brackets.


C.Attachments, Screws, and Bolts:  Stainless steel; [tamper proof type,] heavy duty extruded aluminum brackets.


D.Hardware:  [Chrome plated non‑ferrous cast metal:] [Stainless steel:]


1.Pivot hinges, gravity type, adjustable for door close positioning.


2.Nylon bearings.


3.Thumb turn door latch [with exterior emergency access feature].


4.Door strike and keeper with rubber bumper.


5.Coat hook with rubber bumper.


6.[Door pull for outswinging doors.]

.4

seq level2 \h \r0 

seq level3 \h \r0 
FABRICATION


A.[Fabricate partitions in accordance with FS RR‑P‑1352.]


B.Fabricate components of steel sheet as follows:


1.Panel and Door Faces:  [20] [____] gage ([0.9] [____] mm).


2.Pilaster Faces:  [18] [____] gage ([1.2] [____] mm).


3.Reinforcement:  [12] [________] gage ([2.5] [____] mm).


C.seq level3 \h \r0 Doors and Panels:


1.Thickness:  [1] [____] inch ([25] [____] mm)


2.Door Width:  [24] [____] inch ([610] [____] mm)


3.Door Width for Handicapped Use:  [36] [____] inch ([915] [______] mm) [, out‑swinging].


4.Height:  [58] [____] inch (1 473] [____] mm)


D.seq level3 \h \r0 Pilasters:  1‑1/4 inch (32 mm) thick, of sizes required to suit cubicle width and spacing.


E.Door, Panel, and Pilaster Construction:  Sheet steel face, pressure bonded to sound deadening core, form and close edges, miter and weld corners, grind smooth.


F.Internal Reinforcement:  Provide in areas of attached hardware and fittings.  Mark locations of reinforcement for partition mounted washroom accessories.

.5

seq level2 \h \r0 
FINISHING


A.Clean, degrease, and neutralize panels.


B.Follow immediately with a phosphatizing treatment, prime coat and two finish coats baked enamel.


C.Single Color:  [________] color [as selected].


****** [OR] ******


C.Two Colors:  [________] color [as selected] for [doors] [________], [________] color [as selected] for [panels,] [pilasters,] [and] [________].


D.Stainless Steel Surfaces:  [No. 4] [____] finish.


E.Exposed Steel Surfaces:  [[Polished] [Satin] chrome plated] [________].


F.Aluminum:  Anodized to [________] color [as selected].


G.Non‑ferrous Surfaces:  [Polished] [Satin] chrome plated.

PART 3

seq level1 \h \r0 

seq level2 \h \r0 
EXECUTION

.1
EXAMINATION


A.Verify site conditions under provisions of Section [01039.] [________.]


B.Verify correct spacing of and between plumbing fixtures.


C.Verify correct location of built‑in framing, anchorage, and bracing.

.2

seq level2 \h \r0 
INSTALLATION


A.Install partitions secure, rigid, plumb, and level in accordance with manufacturer's instructions.


B.Maintain [3/8 to 1/2] [________] inch ([9 to 13] [________] mm) space between wall and panels and between wall and end pilasters.


C.Attached panel brackets securely to walls using anchor devices.


D.Attach panels and pilasters to brackets with [tamper proof] through bolts and nuts.  [Locate head rail joints at pilaster center lines.]


E.Install [30] [____] inch ([760] [____] mm) wide x [42] [____] inch ([1 066] mm) high  stainless steel protective splash panels on partitions adjacent to urinals.  Fasten with stainless steel screws spaced [8] [____] inches ([20] [____] mm) oc.


F.Anchor urinal screen panels to walls with [two] [____] panel brackets and vertical upright consisting of [pilaster] [tubular head rail stock and sockets] anchored to floor and ceiling.


G.Provide adjustment for floor variations with screw jack through steel saddles integral with pilaster.  Conceal floor fastenings with pilaster shoes.


H.Support pilasters from built‑in framing using [two] [____] adjustable hanging studs providing vertical leveling.  Conceal ceiling fastenings with pilaster shoe.


I.Equip each door with two hinges, one door latch, [one] [____] coat hook and bumper [; outswinging door with pull].


J.Install door strike and keeper with door bumper on each pilaster in alignment with door latch.


K.Field touch‑up of scratches or damaged enamel finish will not be permitted.


L.Replace damaged or scratched materials with new materials.

.3

seq level2 \h \r0 
ERECTION TOLERANCES


A.Maximum Variation From True Position:  [1/4] [____] inch ([6] [____] mm).


B.Maximum Variation From Plumb:  [1/8] [____] inch ([3] [____] mm).

.4

seq level2 \h \r0 
ADJUSTING


A.Adjust work under provisions of Section [01700.] [________.]


B.Adjust and align hardware to uniform clearance at vertical edge of  doors, not exceeding 3/16 inch (5 mm).


C.Adjust hinges to position doors in [partial opening] [full closed] position when unlatched.  [Return out swinging doors to closed position.]


D.Adjust adjacent components for consistency of line or plane.

.5

seq level2 \h \r0 
SCHEDULES


A.Toilet Room 204:  Three toilet stalls of floor mounted, head rail braced type; two urinal screens with pilaster bracing; two contrasting colors as selected.


B.Staff Washroom:  One toilet stall of ceiling mounted type; single color as selected.


END OF SECTION

Section 10160 Page 2 of 6

