
SECTION 14200

ELEVATOR MODIFICATIONS

PART 1

GENERAL

1.01 Modify existing elevators in buildings 3500 and 

Buildings 3510, in include the following.

A. Elevators presently have non-ADA telephones.  Install ADA approved emergency telephones in each Elevator, single push button activated dial to pre-selected station, with voice communication and automatic location announcement at monitoring station.

B.  Replace existing door edge re-opening system   

      with “Light screen door control”.

1. Control system shall be “Magic Edge” or equal.

                           Control System Specifications:

A. Light emitting diodes:  -G4 (two per detector 

Operating in a redundant mode.

B. Range:  Up to 8 ft, min.

C. Maximum misalignment:  12 degrees

D. Response Time:  50 Milliseconds 

E. Maximum Power Dissapation 12 VA

F. Built in Defogger

G. Power consumation 30 VA

Section 14200 Page 1 of 1

