ST. TAMMANY FIRE PROTECTION DISTRICT NO. 1

FIRE STATION #10

2745 LAKESHORE VISTA BLVD

SLIDELL, LA 70461


Copyright 2010 by The American Institute of Architects (AIA)

Exclusively published and distributed by Architectural Computer Services, Inc. (ARCOM) for the AIA
SECTION 101419
DIMENSIONAL LETTER SIGNAGE
Revise this Section by deleting and inserting text to meet Project-specific requirements.

This Section uses the term "Architect." Change this term to match that used to identify the design professional as defined in the General and Supplementary Conditions.

Verify that Section titles referenced in this Section are correct for this Project's Specifications; Section titles may have changed.
PART 1 -  GENERAL

1.1 SUMMARY

A. Section Includes:

1. Fabricated channel dimensional characters.

1.2 ACTION SUBMITTALS

A. Product Data: For each type of product.

B. Shop Drawings: For dimensional letter signs.

1. Include fabrication and installation details and attachments to other work.

2. Show sign mounting heights, locations of supplementary supports to be provided by others, and accessories.

3. Show message list, typestyles, graphic elements, and layout for each sign.
Retain two subparagraphs below if signs are electrically powered.
4. Show locations of electrical service connections.

5. Include diagrams for power, signal, and control wiring.

C. Samples: For each exposed product and for each color and texture specified.

D. INFORMATIONAL SUBMITTALS

E. Sample warranty.

1.3 CLOSEOUT SUBMITTALS

A. Maintenance data.

1.4 WARRANTY

A. Special Warranty: Manufacturer agrees to repair or replace components of signs that fail in materials or workmanship within specified warranty period.
Verify available warranties and warranty periods for units and components.
1. Warranty Period: Five years from date of Substantial Completion.

PART 2 -  PRODUCTS
See Editing Instruction No. 1 in the Evaluations for cautions about named manufacturers and products. For an explanation of options and Contractor's product selection procedures, see Section 016000 "Product Requirements."
2.1 PERFORMANCE REQUIREMENTS
Retain "Delegated Design" Paragraph below if Contractor is required to assume responsibility for design.
A. Delegated Design: Engage a qualified professional engineer, as defined in this Section, to design sign structure and anchorage of dimensional character to withstand design loads as indicated on Drawings.

B. Electrical Components, Devices, and Accessories: Listed and labeled as defined in NFPA 70, by a qualified testing agency, and marked for intended location and application.

2.2 DIMENSIONAL CHARACTERS
Retain this article for signs consisting of multiple, individually mounted characters.

Copy "Cast Characters," "Cutout Characters," "Fabricated Channel Characters," and "Molded-Plastic Characters" paragraphs below and re-edit for each product.

Insert number to complete drawing designation for each product required. Use these designations on Drawings to identify each product.
A. Cast Characters: Characters with uniform faces, sharp corners, and precisely formed lines and profiles, and as follows:
Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.
1. Character Material: Sheet or plate aluminum or stainless steel.

2. Character Height: As indicated on drawings.

3. Thickness: Manufacturer's standard for size of character.

4. Finishes:
Retain "Integral Metal Finish," "Integral Aluminum Finish," "Integral Stainless-Steel Finish, "Integral Acrylic Color," "Baked-Enamel or Powder-Coat Finish," and "Overcoat" subparagraphs below as required.

Retain "Integral Metal Finish" Subparagraph for integral finishes on metals other than aluminum and stainless steel.

Retain "Baked-Enamel or Powder-Coat Finish" Subparagraph below for applied finish on metals; revise to suit Project.
a. Baked-Enamel or Powder-Coat Finish: Manufacturer's standard, in color as selected by Architect from manufacturer's full range.
Retain "Overcoat" Subparagraph below if required.

Retain one option in "Mounting" Subparagraph below; revise to suit Project.
5. Mounting: Concealed, stainless-steel back bar or bracket assembly to allow minimum 1” stand-off from finish wall surface.
Retain "Illuminated Characters" Subparagraph below if required.
6. Illuminated Characters: Backlighted character construction with LED lighting including transformers, insulators, and other accessories for operability, with provision for servicing and concealing connections to building electrical system. Use tight or sealed joint construction to prevent unintentional light leakage. Space lamps apart from each other and away from character surfaces as needed to illuminate evenly.

a. Power: As indicated on electrical Drawings.

7. Character Material: Sheet or plate aluminum or stainless steel.
Retain "Translucent Face Sheet" Subparagraph below for translucent-face characters.
8. Character Height: As indicated.

9. Character Depth: 2”.

10. Finishes:
Retain "Integral Aluminum Finish" and "Baked-Enamel or Powder-Coat Finish" subparagraphs below as required.

Retain "Integral Aluminum Finish" Subparagraph for integral finishes on aluminum.

Retain "Baked-Enamel or Powder-Coat Finish" Subparagraph below for applied finish on metals; revise to suit Project.
a. Baked-Enamel or Powder-Coat Finish: Manufacturer's standard, in color as selected by Architect from manufacturer's full range.
Retain one option in "Mounting" Subparagraph below; revise to suit Project.
11. Mounting: Manufacturer's standard for size and design of character.
Retain subparagraph below for characters projecting from wall surface.
a. Hold characters at 2-inch (51-mm) distance from wall surface.

2.3 ACCESSORIES

A. Fasteners and Anchors: Manufacturer's standard as required for secure anchorage of signage, noncorrosive and compatible with each material joined, and complying with the following:

1. Use concealed fasteners and anchors unless indicated to be exposed.

2. For exterior exposure, furnish nonferrous-metal, stainless-steel or hot-dip galvanized  devices unless otherwise indicated.

3. Exposed Metal-Fastener Components, General:

a. Fabricated from same basic metal and finish of fastened metal unless otherwise indicated.

4. Sign Mounting Fasteners:

a. Concealed Studs: Concealed (blind), threaded studs welded or brazed to back of sign material, screwed into back of sign assembly, or screwed into tapped lugs cast integrally into back of cast sign material, unless otherwise indicated.

b. Projecting Studs: Threaded studs with sleeve spacer, welded or brazed to back of sign material, screwed into back of sign assembly, or screwed into tapped lugs cast integrally into back of cast sign material, unless otherwise indicated.

c. Through Fasteners: Exposed metal fasteners matching sign finish, with type of head indicated, installed in predrilled holes.
Retain first "Adhesives" Paragraph below if required for LEED-NC, LEED-CI, or LEED-CS Credit IEQ 4.1. VOC content limit is that for multipurpose construction adhesives. Revise VOC content limit based on application; see Section 018113.13 "Sustainable Design Requirements - LEED for New Construction and Major Renovations," Section 018113.16 "Sustainable Design Requirements - LEED for Commercial Interiors," or Section 018113.19 "Sustainable Design Requirements - LEED for Core and Shell Development" for a list of adhesive types and corresponding VOC content limits.
2.4 FABRICATION

A. General: Provide manufacturer's standard sign assemblies according to requirements indicated.

1. Mill joints to a tight, hairline fit. Form assemblies and joints exposed to weather to resist water penetration and retention.

2. Provide welds and brazes behind finished surfaces without distorting or discoloring exposed side. Clean exposed welded and brazed connections of flux, and dress exposed and contact surfaces.

3. Conceal connections if possible; otherwise, locate connections where they are inconspicuous.

4. Internally brace signs for stability and for securing fasteners.

5. Provide rebates, lugs, and brackets necessary to assemble components and to attach to existing work. Drill and tap for required fasteners. Use concealed fasteners where possible; use exposed fasteners that match sign finish.

6. Castings: Fabricate castings free of warp, cracks, blowholes, pits, scale, sand holes, and other defects that impair appearance or strength. Grind, wire brush, sandblast, and buff castings to remove seams, gate marks, casting flash, and other casting marks before finishing.

B. Brackets: Fabricate brackets, fittings, and hardware for bracket-mounted signs to suit sign construction and mounting conditions indicated. Modify manufacturer's standard brackets as required.

1. Aluminum Brackets: Factory finish brackets with baked-enamel or powder-coat finish to match sign-background color unless otherwise indicated.

2. Stainless-Steel Brackets: Factory finish brackets to match sign background finish unless otherwise indicated.

PART 3 -  EXECUTION

3.1 INSTALLATION
Retain mounting methods in this article that coordinate with mounting requirements in sign-description paragraphs retained in Part 2.
A. General: Install signs using mounting methods indicated and according to manufacturer's written instructions.

1. Install signs level, plumb, true to line, and at locations and heights indicated, with sign surfaces free of distortion and other defects in appearance.

2. Before installation, verify that sign surfaces are clean and free of materials or debris that would impair installation.

3. Corrosion Protection: Coat concealed surfaces of exterior aluminum in contact with grout, concrete, masonry, wood, or dissimilar metals, with a heavy coat of bituminous paint.

B. Mounting Methods:

1. Concealed Studs: Using a template, drill holes in substrate aligning with studs on back of sign. Remove loose debris from hole and substrate surface.

a. Masonry Substrates: Fill holes with adhesive. Leave recess space in hole for displaced adhesive. Place sign in position and push until flush to surface, embedding studs in holes. Temporarily support sign in position until adhesive fully sets.

b. Thin or Hollow Surfaces: Place sign in position and flush to surface, install washers and nuts on studs projecting through opposite side of surface, and tighten.

2. Projecting Studs: Using a template, drill holes in substrate aligning with studs on back of sign. Remove loose debris from hole and substrate surface.

a. Masonry Substrates: Fill holes with adhesive. Leave recess space in hole for displaced adhesive. Place spacers on studs, place sign in position, and push until spacers are pinched between sign and substrate, embedding the stud ends in holes. Temporarily support sign in position until adhesive fully sets.

b. Thin or Hollow Surfaces: Place spacers on studs, place sign in position with spacers pinched between sign and substrate, and install washers and nuts on stud ends projecting through opposite side of surface, and tighten.

3. Through Fasteners: Drill holes in substrate using predrilled holes in sign as template. Countersink holes in sign if required. Place sign in position and flush to surface. Install through fasteners and tighten.

4. Back Bar and Brackets: Remove loose debris from substrate surface and install backbar or bracket supports in position so that signage is correctly located and aligned.

5. Adhesive: Clean bond-breaking materials from substrate surface and remove loose debris. Apply linear beads or spots of adhesive symmetrically to back of sign and of suitable quantity to support weight of sign after cure without slippage. Keep adhesive away from edges to prevent adhesive extrusion as sign is applied and to prevent visibility of cured adhesive at sign edges. Place sign in position, and push to engage adhesive. Temporarily support sign in position until adhesive fully sets.

C. Remove temporary protective coverings and strippable films as signs are installed.

END OF SECTION 101419
	DIMENSIONAL LETTER SIGNAGE
	101419 - 5



