OPTIONAL
6 May 1999
08361opt
UPDATES  -

Overhead doors are used on some older design Storage Buildings.

If the high security version of an older Storage Building is not to be used, some or all of the following changes to the standard Storage Building plan may be necessary -

1.
Delete the inner door between the two rooms if the overhead door will not be locked by padlocks in the tracks

2.
Show a single lockset on the exterior man-door rather than two deadbolts

3.
Show the lift handle on the exterior of the overhead door rather than the inside

4.
Delete the bollards in front of the overhead door

5.
Delete the padlocks shown in the overhead door tracks

6.
Delete the tie-down anchor at the overhead door as well as the hole through the door to accommodate the tie-down

The high security version of the older Storage Building is preferred in virtually all circumstances.



SECTION  08361


SECTIONAL OVERHEAD DOORS

PART 1 GENERAL

 1.1
SUMMARY

A.
Products Supplied But Not Installed Under This Section

1.
Sectional overhead door for Storage Building.

Edit Paragraph ‘B’ to fit Project requirements.

B.
Related Sections

1.
Division 04  -  Preparation of masonry opening
04814 04815
1.
Division 06  -  Preparation of framed opening
06110 06221
2.
Division 06  -  Installation
06201
3.
Section 08712  -  Cylinder (Standard)
08712
3.
Padlocks by local Church PM Group (High security)
Owner
PART 2 PRODUCTS

 2.1
MANUFACTURED UNIT

A.
Overhead Door

1.
Panels  -  Galvanized steel, 20 gauge minimum.

2.
Counterbalance  -

a.
Oil-tempered torsion spring counterbalance with grooved cable drums mounted on steel shafts extending full width of opening.

b.
Attach to door with galvanized steel lift cables.

3.
Tracks  -  2 inch galvanized steel, inclined for weather-tight closing.  Ball-bearing rollers

4.
Finish  -  Primed.

5.
Operation  -

Include paragraphs 'a' and 'b' if the overhead door is to be locked with padlocks in the track as shown on the standard Storage Building drawings.

a.
Manual push-up from inside only.

b.
Do not use exterior operating or locking devices.  Security will be provided by padlocks in track inside building.

Include paragraph 'a' if the overhead door will be opened from the outside.

a.
Manual push-up.

 2.2
ACCEPTABLE MANUFACTURERS
Approved Products
A.
Atlas Door, Orlando, FL  (407) 857-0680  www.atlasdoor.com
B.
Clopay Corporation, Cincinnati, OH  (800) 225-6729  www.clopaydoor.com
C.
Overhead Door Corp, Dallas, TX  (800) 275-3290  www.overheaddoor.com
D.
Raynor Manufacturing Co, Dixon, IL  (800) 472-9667  www.raynor.com
E.
Wayne-Dalton Corp, Mt Hope, OH  (800) 827-3667  www.wayne-dalton.com
F.
Windsor Door, Little Rock, AR  (501) 570-9256  www.windsordoor.com
G.
Equal as approved by Architect before bidding.  See Section 01600.

PART 3 EXECUTION  -  Not Used
Installation

END OF SECTION

557-1804 (20020)
1
Section 08361

Welfare Services Center

Sectional Overhead Doors

