STANDARD
19 Aug 1999
09120

UPDATES  -

19 Aug 99
-
Corrected system designations



SECTION  09120


 CEILING SUSPENSION

PART 1 GENERAL

 1.1
SUMMARY

A.
Includes But Not Limited To

1.
Furnish and install ceiling suspension system as described in Contract Documents to receive gypsum board.

B.
Related Sections

1.
Section 09250  -  Gypsum Board
09250
PART 2 PRODUCTS

 2.1
ACCEPTABLE SYSTEMS AND MANUFACTURERS
Approved Products
A.
Drywall Grid by Armstrong World Industries, Lancaster, PA  (800) 448-1405 www.ceilings.com
B.
Drywall Furring Suspended Ceiling System by Chicago Metallic Corporation, Chicago, IL  (800) 323-7164  www.chicago-metallic.com
C.
Drywall Suspension System by USG Interiors Inc, Chicago, IL  (800) 950-3839 www.usg.com
D.
Equal meeting specified requirements as approved by Architect before bidding.  See Section 01600.

 2.2
MISCELLANEOUS MATERIALS

A.
Hanger Wire  -  12 ga minimum galvanized carbon steel, soft tempered, prestretched unless thicker required by ceiling loading in accordance with Suspension System Manufacturer's Literature.

B.
Joints  -  16 gauge galvanized for joints.

PART 3 EXECUTION

 3.1
INSTALLATION
Installation
A.
Follow Manufacturer's instructions with modifications listed below except where Manufacturer's instructions are more stringent.

1.
Space carrying channels 4 feet on center maximum.

2.
Space hanger wires 4 feet on center maximum.

3.
Maintain flatness of assembly to within 1/8 inch in 12 feet.

4.
Do not kink, twist, or bend hanger wires as a means of leveling assembly.

Delete paragraph ‘5’ if ceiling suspension system is not installed adjacent to adjustable headers for accordion folding partitions.

5.
Do not attach suspension system to adjustable folding partition headers.


END OF SECTION

557-1804 (20020)
1
Section 09120

Orlando Welfare Complex
October 11, 2000
Ceiling Suspension

