STANDARD
4 Feb 1999
10655

UPDATES  -

11 Feb 99
-
Added new color scheme information



SECTION  10655


ACCORDION FOLDING PARTITIONS

PART 1 GENERAL

 1.1
SUMMARY

A.
Includes But Not Limited To

1.
Furnish and install folding partitions as described in Contract Documents.

B.
Related Sections

1.
Division 06  -

The folding partition headers are to be the type detailed on approved standard plan drawings.  These headers are basically Won-Door headers.  The other folding partition manufacturers must adjust their systems to fit these details.  The submitted shop drawings should show the standard plan drawing details with the Manufacturer's suspension system shown attached.

a.
Headers and framing required to receive material furnished and installed by Folding Partition Manufacturer
06110

09936 09936ar.a 09936ar.b
 1.2
REFERENCES

A.
American Society For Testing And Materials
ASTM
1.
ASTM E 336-97, 'Standard Test Method for Measurement of Airborne Sound Insulation in Buildings'

2.
ASTM E 413-87 (1999), 'Classification for Rating Sound Insulation'

 1.3
SYSTEM DESCRIPTION

A.
Design Requirements  -  Total folding partition assembly shall be repairable at installation location without removal to repair shop or factory.

NIC requirements for Chapel and Cultural Center partitions installed in existing openings may need to be lowered depending on the height of the partitions and the surrounding structure.  This reduction will probably be to between 26 and 28 and should be determined by the Sound / Acoustic Consultant for the Project.

B.
Performance Requirements  -  Completed acoustic folding partition assembly shall have NIC rating of 39 in areas when tested in accordance with ASTM E 336 and calculated in accordance with ASTM E 413 and when installed on header configuration and surrounding construction shown on Contract Documents.

 1.4
SUBMITTALS

A.
Product Data
Product Data
1.
Manufacturer's literature or cut sheet

2.
Operating, repairing, and maintaining instructions

3.
Color and style selections

B.
Shop Drawings  -  Show attachment to framing and header configuration detailed on Drawings.
Shop Drawings
 1.5
QUALITY ASSURANCE

A.
Qualifications  -  Installation shall be performed by Manufacturer certified and approved personnel.

B.
Certification  -  Provide letter certifying that installation is complete and ready for acoustic testing.

1.7
WARRANTY
Warranty
A.
Manufacturer's 3 year minimum written warranty covering installation and complete folding partition assembly.

1.
Warranty becomes effective on date of substantial completion.  Date shall be entered by Manufacturer on warranty certificate.

2.
Warranty covers major maintenance during warranty period, defined as defects in folding partition or its operation that will not allow it to function for its intended use.  Provide requested maintenance service within 10 days of receiving request and at no additional cost to Owner.

 1.8
MAINTENANCE

A.
Provide maintenance and repair kit consisting of maintenance and repair manuals and video, repair parts, and tools.

1.
Parts and tools supplied will be those illustrated or demonstrated in the manuals and video.

2.
Amount of parts supplied shall be sufficient to perform maintenance and repair procedures for normal maintenance for five years.

3.
Deliver kit to Architect for Owner's Field Representative at Substantial Completion.

PART 2 PRODUCTS

 2.1
MANUFACTURED UNITS

A.
Acoustic Partitions
Approved Products
1.
Basis of Design - Modernfold  

a.
Rollers shall be one inch diameter minimum with double steel race open ball bearings and nylon tires.

b.
Grip type latch.  Provide pull-in latches where necessary.

c.
Lead posts and latching jambs shall be 16 ga minimum cold rolled vinyl clad steel.

d.
Track  -  14 gauge minimum cold rolled steel.

e. Panel Assembly  -  fabric, 9” profile pantograph connected by steel hinges.  

f. Acoustic Seals  -

1)
Continuous extruded vinyl sweep strips at top and bottom of partition.

g.
Weight  -  5 lbs per sq ft of partition surface area maximum.

h.
Height Adjustment  -  Suspension system shall be fully adjustable for one inch minimum up or down from original installed position.

i.
Stacking Depth  -  1-3/4 inch per lineal foot of full extended partition maximum, not including jamb and lead post.

j.
Approved Model  -  Soundmaster 8.  

Contact individual suppliers for memo supplies to determine color selections to be specified.  Select neutral colors without patterns from the Manufacturer’s standard offering.  Wood tones and patterned styles are not recommended.

B.
Color And Pattern Quality Standard  - to be selected by Architect from Manufacturer’s full color line.  

The flexible acoustic barrier is generally used only in conjunction with partitions in Chapels and Cultural Centers.

 2.2
MANUFACTURERS

A.
Hufcor, Janesville, WI (800) 542-2371 www.hufcor.com

B. Modernfold, Newcastle, IN  (800) 869-9685  www.modernfold.com
C. Won-Door, Salt Lake City, UT  (800) 453-8494  www.wondoor.com
 2.3
FABRICATION

A.
Fabricate partitions according to actual field measurements of fully prepared, finished openings.  Partition Manufacturer is responsible for field measurements and their accuracy.

PART 3 EXECUTION

 3.1
INSTALLATION
Installation
A.
Install so track is aligned, level, etc, to eliminate catching or binding of rollers.

See instruction for Article ‘2.2' above.


END OF SECTION

557-1804 (20020)
3
Section 10655

Orlando Welfare Complex
October 11, 2000
Accordion Folding Partitions

