STRUCTURAL INSPECTION

Date:
October 24, 2003

Type:
Bulk Head 

For:
Mary Najolia

             Century 21

Re: Lot # 87

      Lake View Drive

      Slidell, La. 70458

History:

Kay Willson wants to purchase this piece of property and asked us to inspect it. The property is 60 x 120 with a concrete bulkhead that is approximately 30 years old.

Findings:

This is a concrete bulkhead with another concrete bulkhead 15’ back from the water line. It is held together with steel rebar that eventually rusts away. Due to the age is has done so. Upon inspection, it was noted that the bulkhead has separated from each other. This is due to the tiebacks deteriorating and the force of the land pushing out against it. Please see the attached photos.

Analysis:

To correct the problem it will need to be repaired. The repair will include digging out the dirt around the bulkhead. And digging were the tiebacks are connected to both bulkheads. Once all dirt is removed the bulkhead will have to be pulled back into its place. It will then need new ¾” galvanized allthread rods reinstalled to both bulkheads and tie off, and the dirt filled back in.

Conclusion:

It is my professional opinion that when the bulkhead is repaired it will be structurally sound.

Sincerely,

Emmett G. (Pete) Dammon, P.E.

Louisiana License No. 8796
This inspection is limited to the apparent visual conditions of the structural components of this building.  It does not cover, nor attempts to cover, any components, items, and/or conditions which, by their nature or location, are concealed or are difficult or hazardous to inspect, or which require the moving of furniture, flooring materials, rugs, fixtures, appliances, or any component-part nailed, bolted, or screwed down or shut.  No opinions are expressed regarding conditions which could be discovered only by the disassembly of any component parts, special testing, or removal of any concealing objects.

Inspections are made under normal weather conditions, and are not opinions of the conditions of the property and/or structure which may exist under unusual weather conditions, such as, but not limited to floods, heavy rains or snows, high winds, temperature extremes, or any act of God.  Specific hazardous wastes, toxic substances, toxic mold, air and water quality, communicable diseases, asbestos, soil, environmental, radon, carbon monoxide, formaldehyde, building code and termite conditions are not included in this report unless otherwise stipulated.

This report is not a warranty or guarantee of the property inspected, but it is our opinion of its condition at the time inspected.  Our liability shall be limited to reimbursement of the total cost of inspection.


